# SECTION 27 41 16

INTEGRATED AUDIO-VIDEO SYSTEMS AND EQUIPMENT

Equipment Specified in this section:

DM-NVX-350C

DM-NVX-351C

DM-NVX-350

DM-NVX-351

DMF-CI-8

Table of Contents

PART 1	GENERAL	5

1.1	SUMMARY	5

A.	Section Includes	5

1.2	REFERENCES	5

A.	Abbreviations and Acronyms	5

PART 2	PRODUCTS	5

2.1	ENCODER/DECODER UNIT TYPE 1	5

A.	Real-Time 4K60 Video Distribution	5

B.	Single Component Design	5

C.	Auto-Switcher	5

D.	HDMI Output	6

E.	USB and KVM Integration	6

F.	7.1 Surround Sound Audio	6

G.	Analog Audio Embedding or De-embedding	6

H.	Breakaway Audio	7

I.	Text Overlay	7

J.	Video Wall Processing	7

K.	Copper or Fiber LAN Connectivity	7

L.	Enterprise-Grade Security	7

M.	CEC Device Control	8

N.	Web-Based Setup	8

O.	High-Density Card-Based Component	8

P.	Encoding/Decoding	8

Q.	Video	8

R.	Maximum Resolutions	9

S.	Audio	9

T.	Communications	9

U.	Connectors	10

V.	Controls & Indicators	10

W.	Construction	11

X.	Basis of design product: Crestron DM-NVX-350C	11

2.2	ENCODER/DECODER UNIT TYPE 2	11

A.	Real-Time 4K60 Video Distribution	11

B.	Single Component Design	11

C.	Auto-Switcher	11

D.	HDMI Output	11

E.	USB and KVM Integration	12

F.	7.1 Surround Sound Audio with Downmixing	12

G.	Analog Audio Embedding or De-embedding	12

H.	Breakaway Audio	13

I.	Text Overlay	13

J.	Video Wall Processing	13

K.	Copper or Fiber LAN Connectivity	13

L.	Enterprise-Grade Security	13

M.	CEC Device Control	13

N.	Web-Based Setup	14

O.	High-Density Card-Based Component	14

P.	Encoding/Decoding	14

Q.	Video	14

R.	Maximum Resolutions	14

S.	Audio	15

T.	Communications	15

U.	Connectors	15

V.	Controls & Indicators	16

W.	Construction	17

X.	Basis of design product: Crestron DM-NVX-351C	17

2.3	ENCODER/DECODER UNIT TYPE 3	17

A.	Real-Time 4K60 Video Distribution	17

B.	Single Component Design	17

C.	Auto-Switcher	17

D.	HDMI Output	17

E.	USB and KVM Integration	17

F.	7.1 Surround Sound Audio	18

G.	Analog Audio Embedding or De-embedding	18

H.	Breakaway Audio	18

I.	Text Overlay	18

J.	Video Wall Processing	19

K.	Copper or Fiber LAN Connectivity	19

L.	Enterprise-Grade Security	19

M.	CEC Device Control	19

N.	Device Control	19

O.	Web-Based Setup	20

P.	Low-Profile Installation	20

Q.	Encoding/Decoding	20

R.	Video	20

S.	Maximum Resolutions	20

T.	Audio	21

U.	Communications	21

V.	Connectors	22

W.	Controls & Indicators	23

X.	Power	23

Y.	Enclosure	24

Z.	Basis of design product: Crestron DM-NVX-350	24

2.4	ENCODER/DECODER UNIT TYPE 4	24

A.	Real-Time 4K60 Video Distribution	24

B.	Single Component Design	24

C.	Auto-Switcher	24

D.	HDMI Output	24

E.	USB and KVM Integration	25

F.	7.1 Surround Sound Audio with Downmixing	25

G.	Analog Audio Embedding or De-embedding	25

H.	Breakaway Audio	26

I.	Text Overlay	26

J.	Video Wall Processing	26

K.	Copper or Fiber LAN Connectivity	26

L.	Enterprise-Grade Security	26

M.	CEC Device Control	26

N.	Device Control	27

O.	Web-Based Setup	27

P.	Low-Profile Installation	27

Q.	Encoding/Decoding	27

R.	Video	27

S.	Maximum Resolutions	28

T.	Audio	28

U.	Communications	28

V.	Connectors	29

W.	Controls & Indicators	30

X.	Power	31

Y.	Enclosure	31

Z.	Mounting: Freestanding, surface mount, or attach to a single rack rail	31

AA.	Basis of design product: Crestron DM-NVX-351	31

2.5	CARD CHASSIS	31

A.	Card Chassis Design	31

B.	Communications	31

C.	Card Slots	31

D.	Connectors	31

E.	Controls & Indicators	32

F.	Power	32

G.	Environmental	32

H.	Enclosure	32

I.	Basis of design product: DMF-CI-8	32

PART 3	NOT USED	32

END OF SECTION 27 41 16	33

# SECTION 27 41 16

INTEGRATED AUDIO-VIDEO SYSTEMS AND EQUIPMENT

Specifier: The Specifier/Design Professional is responsible for the accuracy of all project specifications, including system application and coordination with related sections.  This guide specification is provided as a convenience and requires editing to match actual project requirements.  CRESTRON ELECTRONICS, INC. SHALL NOT BE LIABLE FOR ANY DAMAGES ARISING OUT OF THE USE OF ANY OF ITS GUIDE SPECIFICATIONS.  For Crestron design assistance and design review please contact Sales Support Services Department at   800.237.2041 or techsales@crestron.com.

GENERAL

SUMMARY

Section Includes

4K60 4:4:4 HDR Network AV Encoder/Decoder Unit

REFERENCES

Abbreviations and Acronyms

CEC:  Consumer Electronics Control

EDID:  Extended display identification data

HDCP:  High-bandwidth Digital Content Protection

HDMI:  High-Definition Multimedia Interface

PRODUCTS

ENCODER/DECODER UNIT TYPE 1

Specifier Note: DM-NVX-350C - DigitalMedia 4K60 4:4:4 HDR Network AV Encoder/Decoder Card

Real-Time 4K60 Video Distribution

The Encoder/Decoder end-to-end latency for 60 frame per second content shall be less than 1 frame.

Single Component Design

In a single card, the Encoder/Decoder shall be configurable to operate as:

Network AV encoder

Network AV decoder

The encoder/decoder mode shall be switchable while in use via a control system.

Standard 1Gb network operation

4K60 Video distribution

Web based control and management

Scalable network distribution system

System of encoder/decoder units shall be scalable via network switch.

Auto-Switcher

The Encoder/Decoder shall include two HDMI inputs. Switching between inputs shall be performed:

Automatically using an auto-switching mode

Programmatically via a control system

Through a web browser

HDMI Output

When configured as a decoder, the HDMI output is scaled up or down to match the native resolution of the display device.

When used as an encoder, the HDMI output shall function as a pass through output, with resolution matched to the encoded source.

USB and KVM Integration

The Encoder/Decoder shall support the extension of USB signals, which may be switched and routed alongside the AV signal or separately via a compatible control system.

USB 2.0 host and device ports are provided on each Encoder/Decoder, allowing a USB mouse, keyboard, or other device to be connected at one DM NVX endpoint and routed to a computer or other host at any other endpoint.

USB peripherals switching functionality shall support:

Whiteboards

Touch screens

Game controllers

Cameras

Mobile devices

Headsets

Flash drives

7.1 Surround Sound Audio

The Encoder/Decoder shall support lossless transport of 7.1 surround sound audio signals, including:

Dolby TrueHD

Dolby Atmos

DTS-HD

DTS:X

Uncompressed linear PCM.

In decoder mode, the Encoder/Decoder shall have the ability to receive both multichannel and 2-channel downmix signals from a DSP version of the Encoder/Decoder, allowing either signal to be selected at the HDMI output while the 2-channel signal is automatically routed to the analog output.

Analog Audio Embedding or De-embedding

A balanced stereo analog audio port shall be included, which may be configured as either an input or output.

As an input, it allows a stereo audio source to be connected and combined with the video signal from either HDMI input or the incoming network video stream.

As an output, it can provide a stereo line-level signal to feed a local sound system or analog audio switcher. The output volume is adjustable via a compatible control system or web browser.

Breakaway Audio

In decoder mode the Encoder/Decoder may select and combine separate video and audio signals from two different inputs, including two different encoders.

Text Overlay

The Encoder/Decoder shall be capable of displaying dynamic or fixed text on screen.

Video Wall Processing

The Encoder/Decoder shall support video wall functionality.

Video walls composed of up to 64 individual displays shall be supported with configurations using multiple Encoder/Decoder units.

Each Encoder/Decoder shall provide fully-adjustable zoom capability and bezel compensation.

One Encoder/Decoder is required per display, supporting configurations of up to eight wide by up to eight high.

Copper or Fiber LAN Connectivity

The Encoder/Decoder includes two RJ45 1000Base-T LAN ports. Either port may be used as the primary LAN connection, allowing the other to be used to provide a network connection for an additional device. These ports may also be used to daisy-chain multiple Encoder/Decoder units feeding a single-source video wall or individual displays all showing the same video image.

Encoder/Decoder unit shall support connection to a fiber optic network by inserting an appropriate SFP transceiver module into the SFP port on the Encoder/Decoder.

Encoder/Decoder manufacturer shall offer a selection of modules to accommodate various multimode and single-mode fiber types. An RJ45 module is also offered to provide a third RJ45 LAN port.

Enterprise-Grade Security

Encoder/Decoder shall employ advanced security features and protocols including:

802.1x authentication

Active Directory credential management

LDAP directory management

PKI certification

AES encryption

TLS, SSH

HTTPS

Encoder/Decoder shall run on a dedicated AV network, with fully-managed access to, or isolation from, the user’s LAN or the Internet.

CEC Device Control

Through a compatible control processor, the Encoder/Decoder unit shall include a gateway for controlling devices through their HDMI connections using the CEC signal embedded in HDMI.

Web-Based Setup

Setup of the Encoder/Decoder unit shall be accomplished using a computer web browser.

Full control and monitoring of the Encoder/Decoder unit is enabled through integration with a compatible control processor.

High-Density Card-Based Component

The Encoder/Decoder unit shall be a card type component utilizing a card chassis.

The compatible card chassis shall be a scalable unit capable of housing up to eight Encoder/Decoder units.

Encoding/Decoding

Video Codec: Pixel Perfect Processing

Video Resolutions: Up to 4096x2160@60Hz (DCI 4K60), 4:4:4 color sampling, HDR10 and Deep Color support

Audio Formats: Primary multichannel (up to 8-channel LPCM or encoded HBR 7.1 surround sound), secondary 2-channel LPCM

Bitrates: 100 to 990 Mbps

Streaming Protocols: RTP, RTSP, SDP

Container: MPEG-2 transport stream (.ts)

Session Initiation: Multicast via RTSP

Copy Protection: HDCP 2.2

Video

Input Signal Types: HDMI w/HDR10, Deep Color, and 4K60 4:4:4 support (Dual-Mode DisplayPort and DVI compatible)

Output Signal Types: HDMI w/HDR10, Deep Color, and 4K60 4:4:4 support (DVI compatible)

Switcher: 2x1 auto-switching

Scaler: 4K60 4:4:4 video scaler with motion-adaptive deinterlacing, intelligent frame rate conversion, Deep Color support, HDR10 support, widescreen format selection (zoom, stretch, maintain aspect-ratio, or 1:1), video wall processing up to 8 wide x up to 8 high, static or dynamic text overlay

Copy Protection: HDCP 2.2

Maximum Resolutions

Maximum Common Resolutions

| Scan Type | Resolution | Frame Rate | Color Sampling | Color Depth |
| --- | --- | --- | --- | --- |
| Progressive | 4096x2160 DCI 4K & 3840x2160 4K UHD | 24 Hz | 4:4:4 | 36 bit |
|  |  | 30 Hz | 4:4:4 | 36 bit |
|  |  | 60 Hz | 4:2:2 | 36 bit |
|  |  | 60 Hz | 4:4:4 | 24 bit |
|  | 2560x1600 WQXGA | 60 Hz | 4:4:4 | 36 bit |
|  | 1920x1080 HD 1080p | 60 Hz | 4:4:4 | 36 bit |
| Interlaced (Input only) | 1920x1080 HD 1080i | 30 Hz | 4:4:4 | 36 bit |

Encoder/Decoder unit shall support other custom resolutions at pixel clock rates up to 600 MHz

Audio

Input Signal Types: HDMI (Dual-Mode DisplayPort compatible [10]), analog stereo

Output Signal Types: HDMI, analog stereo

Digital Formats: Dolby Digital®, Dolby Digital EX, Dolby Digital Plus, Dolby TrueHD, Dolby Atmos, DTS®, DTS-ES, DTS 96/24, DTS-HD High Res, DTS-HD Master Audio, DTS:X, LPCM up to 8 channels

Analog Formats: Stereo 2-channel

Analog-To-Digital Conversion: 24-bit 48 kHz

Digital-To-Analog Conversion: 24-bit 48 kHz

Analog Performance:

Frequency Response: 20 Hz to 20 kHz ±0.5 dB

S/N Ratio: >95 dB 20 Hz to 20 kHz A-weighted

THD+N: <0.005% @ 1 kHz

Stereo Separation: >90 dB

Analog Output Volume Adjustment: -80 to +20 dB

Communications

Ethernet: 10/100/1000 Mbps, auto-switching, auto-negotiating, auto-discovery, full/half duplex, TCP/IP, UDP/IP, CIP, DHCP, SSL, TLS, SSH, SFTP (SSH File Transfer Protocol), IEEE 802.1x, IPv4, HTTPS web browser setup and control, Crestron control system integration

USB: USB 2.0 host or device signal extension

HDMI: HDCP 2.2, EDID, CEC

Integrated Control (via Ethernet): HDCP 2.2, AES, RTP, RTSP, SDP, ONVIF, IGMPv3, SMPTE 2022

Management of HDCP and EDID

Management of CEC between the connected HDMI devices and a control system

Connectors

USB DEVICE:

(1) USB Type B female;

USB 2.0 device port;

USB signal extender port for connection to a computer or any other USB 2.0 host

USB HOST:

(1) USB Type A female

USB 2.0 host port

USB signal extender port for connection of a mouse, keyboard, or any other USB 2.0 device

Available Power: 500 mA at 5 Volts DC

LAN 1 – 2:

(2) 8-pin RJ45 female

10Base-T/100Base-TX/1000Base-T Ethernet ports

LAN 3:

(1) SFP port

Accepts one SFP transceiver module

HDMI OUTPUT:

(1) 19-pin Type A HDMI female

HDMI digital video/audio output (DVI compatible)

HDMI INPUT 1 – 2:

(2) 19-pin Type A HDMI female

HDMI digital video/audio inputs

(DVI & Dual-Mode DisplayPort compatible)

AUDIO I/O:

(1) 5-pin 3.5 mm detachable terminal block

Balanced/unbalanced stereo line-level audio input or output

Input Impedance: 24k Ohms balanced/unbalanced

Maximum Input Level: 4 Vrms balanced, 2 Vrms unbalanced

Output Impedance: 200 Ohms balanced, 100 Ohms unbalanced

Maximum Output Level: 4 Vrms balanced, 2 Vrms unbalanced

Controls & Indicators

TX: (1) Green LED, indicates unit is in transmitter (encoder) mode

RX: (1) Green LED, indicates unit is in receiver (decoder) mode

OL: (1) Green LED, indicates an online connection to a control system via Ethernet

LAN 1 – 2: (4) LEDs, green indicates Ethernet link status, amber indicates Ethernet activity

LAN 3 LNK: (1) Green LED, indicates Ethernet link status

LAN 3 ACT: (1) Green LED, indicates Ethernet activity

HDMI OUTPUT: (1) Green LED, indicates video signal transmission at the HDMI output

HDMI INPUT 1 – 2: (2) Green LEDs, each indicates sync detection at the corresponding HDMI input

Construction

Plug-in card, occupies (1) card slot in a card chassis

Basis of design product: Crestron DM-NVX-350C

Product Description: DigitalMedia 4K60 4:4:4 HDR Network AV Encoder/Decoder Card

ENCODER/DECODER UNIT TYPE 2

Specifier Note: DM-NVX-351C - DigitalMedia™ 4K60 4:4:4 HDR Network AV Encoder/Decoder Card w/Downmixing

Real-Time 4K60 Video Distribution

The Encoder/Decoder end-to-end latency for 60 frame per second content shall be less than 1 frame.

Single Component Design

In a single card, the Encoder/Decoder shall be configurable to operate as:

Network AV encoder

Network AV decoder

The encoder/decoder mode shall be switchable while in use via a control system.

Standard 1Gb network operation

4K60 Video distribution

Web based control and management

Scalable network distribution system

System of encoder/decoder units shall be scalable via network switch.

Auto-Switcher

The Encoder/Decoder shall include two HDMI inputs. Switching between inputs shall be performed:

Automatically using an auto-switching mode

Programmatically via a control system

Through a web browser

HDMI Output

When configured as a decoder, the HDMI output is scaled up or down to match the native resolution of the display device.

When used as an encoder, the HDMI output shall function as a pass through output, with resolution matched to the encoded source.

USB and KVM Integration

The Encoder/Decoder shall support the extension of USB signals, which may be switched and routed alongside the AV signal or separately via a compatible control system.

USB 2.0 host and device ports are provided on each Encoder/Decoder, allowing a USB mouse, keyboard, or other device to be connected at one DM NVX endpoint and routed to a computer or other host at any other endpoint.

USB peripherals switching functionality shall support:

Whiteboards

Touch screens

Game controllers

Cameras

Mobile devices

Headsets

Flash drives

7.1 Surround Sound Audio with Downmixing

The Encoder/Decoder shall support lossless transport of 7.1 surround sound audio signals, including:

Dolby TrueHD

Dolby Atmos

DTS-HD

DTS:X

Uncompressed linear PCM.

The Encoder/Decoder shall include the ability to decode the incoming multichannel surround sound signal, from the network or an HDMI input, and downmix that signal to stereo. The stereo downmix signal is automatically routed to the onboard analog output, while the HDMI output can be configured to output either stereo or multichannel.

As an encoder, the Encoder/Decoder unit distributes both stereo and multichannel signals simultaneously over the network, allowing either signal to be selected at any decoder on the network.

Analog Audio Embedding or De-embedding

A balanced stereo analog audio port shall be included, which may be configured as either an input or output.

As an input, it allows a stereo audio source to be connected and combined with the video signal from either HDMI input or the incoming network video stream.

As an output, it can provide a stereo line-level signal to feed a local sound system or analog audio switcher. The output volume is adjustable via a compatible control system or web browser.

Breakaway Audio

In decoder mode the Encoder/Decoder may select and combine separate video and audio signals from two different inputs, including two different encoders.

Text Overlay

The Encoder/Decoder shall be capable of displaying dynamic or fixed text on screen.

Video Wall Processing

The Encoder/Decoder shall support video wall functionality.

Video walls composed of up to 64 individual displays shall be supported with configurations using multiple Encoder/Decoder units.

Each Encoder/Decoder shall provide fully-adjustable zoom capability and bezel compensation.

One Encoder/Decoder is required per display, supporting configurations of up to eight wide by up to eight high.

Copper or Fiber LAN Connectivity

The Encoder/Decoder includes two RJ45 1000Base-T LAN ports. Either port may be used as the primary LAN connection, allowing the other to be used to provide a network connection for an additional device. These ports may also be used to daisy-chain multiple Encoder/Decoder units feeding a single-source video wall or individual displays all showing the same video image.

Encoder/Decoder unit shall support connection to a fiber optic network by inserting an appropriate SFP transceiver module into the SFP port on the Encoder/Decoder.

Encoder/Decoder manufacturer shall offer a selection of modules to accommodate various multimode and single-mode fiber types. An RJ45 module is also offered to provide a third RJ45 LAN port.

Enterprise-Grade Security

Encoder/Decoder shall employ advanced security features and protocols including:

802.1x authentication

Active Directory credential management

LDAP directory management

PKI certification

AES encryption

TLS, SSH

HTTPS

Encoder/Decoder shall run on a dedicated AV network, with fully-managed access to, or isolation from, the user’s LAN or the Internet.

CEC Device Control

Through a compatible control processor, the Encoder/Decoder unit shall include a gateway for controlling devices through their HDMI connections using the CEC signal embedded in HDMI.

Web-Based Setup

Setup of the Encoder/Decoder unit shall be accomplished using a computer web browser.

Full control and monitoring of the Encoder/Decoder unit is enabled through integration with a compatible control processor.

High-Density Card-Based Component

The Encoder/Decoder unit shall be a card type component utilizing a card chassis.

The compatible card chassis shall be a scalable unit capable of housing up to eight Encoder/Decoder units.

Encoding/Decoding

Video Codec: Pixel Perfect Processing

Video Resolutions: Up to 4096x2160@60Hz (DCI 4K60), 4:4:4 color sampling, HDR10 and Deep Color support

Audio Formats: Primary multichannel (up to 8-channel LPCM or encoded HBR 7.1 surround sound), secondary 2-channel LPCM

Bitrates: 100 to 990 Mbps

Streaming Protocols: RTP, RTSP, SDP

Container: MPEG-2 transport stream (.ts)

Session Initiation: Multicast via RTSP

Copy Protection: HDCP 2.2

Video

Input Signal Types: HDMI w/HDR10, Deep Color, and 4K60 4:4:4 support (Dual-Mode DisplayPort and DVI compatible)

Output Signal Types: HDMI w/HDR10, Deep Color, and 4K60 4:4:4 support (DVI compatible)

Switcher: 2x1 auto-switching

Scaler: 4K60 4:4:4 video scaler with motion-adaptive deinterlacing, intelligent frame rate conversion, Deep Color support, HDR10 support, widescreen format selection (zoom, stretch, maintain aspect-ratio, or 1:1), video wall processing up to 8 wide x up to 8 high, static or dynamic text overlay

Copy Protection: HDCP 2.2

Maximum Resolutions

Maximum Common Resolutions

| Scan Type | Resolution | Frame Rate | Color Sampling | Color Depth |
| --- | --- | --- | --- | --- |
| Progressive | 4096x2160 DCI 4K & 3840x2160 4K UHD | 24 Hz | 4:4:4 | 36 bit |
|  |  | 30 Hz | 4:4:4 | 36 bit |
|  |  | 60 Hz | 4:2:2 | 36 bit |
|  |  | 60 Hz | 4:4:4 | 24 bit |
|  | 2560x1600 WQXGA | 60 Hz | 4:4:4 | 36 bit |
|  | 1920x1080 HD 1080p | 60 Hz | 4:4:4 | 36 bit |
| Interlaced (Input only) | 1920x1080 HD 1080i | 30 Hz | 4:4:4 | 36 bit |

Encoder/Decoder unit shall support other custom resolutions at pixel clock rates up to 600 MHz

Audio

Input Signal Types: HDMI (Dual-Mode DisplayPort compatible [10]), analog stereo

Output Signal Types: HDMI, analog stereo

Digital Formats: Dolby Digital®, Dolby Digital EX, Dolby Digital Plus, Dolby TrueHD, Dolby Atmos, DTS®, DTS-ES, DTS 96/24, DTS-HD High Res, DTS-HD Master Audio, DTS:X, LPCM up to 8 channels

Analog Formats: Stereo 2-channel

Analog-To-Digital Conversion: 24-bit 48 kHz

Digital-To-Analog Conversion: 24-bit 48 kHz

Analog Performance:

Frequency Response: 20 Hz to 20 kHz ±0.5 dB

S/N Ratio: >95 dB 20 Hz to 20 kHz A-weighted

THD+N: <0.005% @ 1 kHz

Stereo Separation: >90 dB

Analog Output Volume Adjustment: -80 to +20 dB

Communications

Ethernet: 10/100/1000 Mbps, auto-switching, auto-negotiating, auto-discovery, full/half duplex, TCP/IP, UDP/IP, CIP, DHCP, SSL, TLS, SSH, SFTP (SSH File Transfer Protocol), IEEE 802.1x, IPv4, HTTPS web browser setup and control, Crestron control system integration

USB: USB 2.0 host or device signal extension

HDMI: HDCP 2.2, EDID, CEC

Integrated Control (via Ethernet): HDCP 2.2, AES, RTP, RTSP, SDP, ONVIF, IGMPv3, SMPTE 2022

Management of HDCP and EDID

Management of CEC between the connected HDMI devices and a control system

Connectors

USB DEVICE:

(1) USB Type B female;

USB 2.0 device port;

USB signal extender port for connection to a computer or any other USB 2.0 host

USB HOST:

(1) USB Type A female

USB 2.0 host port

USB signal extender port for connection of a mouse, keyboard, or any other USB 2.0 device

Available Power: 500 mA at 5 Volts DC

LAN 1 – 2:

(2) 8-pin RJ45 female

10Base-T/100Base-TX/1000Base-T Ethernet ports

LAN 3:

(1) SFP port

Accepts one SFP transceiver module

HDMI OUTPUT:

(1) 19-pin Type A HDMI female

HDMI digital video/audio output (DVI compatible)

HDMI INPUT 1 – 2:

(2) 19-pin Type A HDMI female

HDMI digital video/audio inputs

(DVI & Dual-Mode DisplayPort compatible)

AUDIO I/O:

(1) 5-pin 3.5 mm detachable terminal block

Balanced/unbalanced stereo line-level audio input or output

Input Impedance: 24k Ohms balanced/unbalanced

Maximum Input Level: 4 Vrms balanced, 2 Vrms unbalanced

Output Impedance: 200 Ohms balanced, 100 Ohms unbalanced

Maximum Output Level: 4 Vrms balanced, 2 Vrms unbalanced

Controls & Indicators

TX: (1) Green LED, indicates unit is in transmitter (encoder) mode

RX: (1) Green LED, indicates unit is in receiver (decoder) mode

OL: (1) Green LED, indicates an online connection to a control system via Ethernet

LAN 1 – 2: (4) LEDs, green indicates Ethernet link status, amber indicates Ethernet activity

LAN 3 LNK: (1) Green LED, indicates Ethernet link status

LAN 3 ACT: (1) Green LED, indicates Ethernet activity

HDMI OUTPUT: (1) Green LED, indicates video signal transmission at the HDMI output

HDMI INPUT 1 – 2: (2) Green LEDs, each indicates sync detection at the corresponding HDMI input

Construction

Plug-in card, occupies (1) card slot in a card chassis

Basis of design product: Crestron DM-NVX-351C

Product Description: DigitalMedia 4K60 4:4:4 HDR Network AV Encoder/Decoder Card w/Downmixing

ENCODER/DECODER UNIT TYPE 3

Specifier Note: DM-NVX-350 - DigitalMedia 4K60 4:4:4 HDR Network AV Encoder/Decoder

Real-Time 4K60 Video Distribution

The Encoder/Decoder end-to-end latency for 60 frame per second content shall be less than 1 frame.

Single Component Design

In a single card, the Encoder/Decoder shall be configurable to operate as:

Network AV encoder

Network AV decoder

The encoder/decoder mode shall be switchable while in use via a control system.

Standard 1Gb network operation

4K60 Video distribution

Web based control and management

Scalable network distribution system

System of encoder/decoder units shall be scalable via network switch.

Auto-Switcher

The Encoder/Decoder shall include two HDMI inputs. Switching between inputs shall be performed:

Automatically using an auto-switching mode

Manually using input select button

Programmatically via a control system

Through a web browser

HDMI Output

When configured as a decoder, the HDMI output is scaled up or down to match the native resolution of the display device.

When used as an encoder, the HDMI output shall function as a pass through output, with resolution matched to the encoded source.

USB and KVM Integration

The Encoder/Decoder shall support the extension of USB signals, which may be switched and routed alongside the AV signal or separately via a compatible control system.

USB 2.0 host and device ports are provided on each Encoder/Decoder, allowing a USB mouse, keyboard, or other device to be connected at one DM NVX endpoint and routed to a computer or other host at any other endpoint.

USB peripherals switching functionality shall support:

Whiteboards

Touch screens

Game controllers

Cameras

Mobile devices

Headsets

Flash drives

7.1 Surround Sound Audio

The Encoder/Decoder shall support lossless transport of 7.1 surround sound audio signals, including:

Dolby TrueHD

Dolby Atmos

DTS-HD

DTS:X

Uncompressed linear PCM.

In decoder mode, the Encoder/Decoder shall have the ability to receive both multichannel and 2-channel downmix signals from a DSP version of the Encoder/Decoder, allowing either signal to be selected at the HDMI output while the 2-channel signal is automatically routed to the analog output.

Analog Audio Embedding or De-embedding

A balanced stereo analog audio port shall be included, which may be configured as either an input or output.

As an input, it allows a stereo audio source to be connected and combined with the video signal from either HDMI input or the incoming network video stream.

As an output, it can provide a stereo line-level signal to feed a local sound system or analog audio switcher. The output volume is adjustable via a compatible control system or web browser.

Breakaway Audio

In decoder mode the Encoder/Decoder may select and combine separate video and audio signals from two different inputs, including two different encoders.

Text Overlay

The Encoder/Decoder shall be capable of displaying dynamic or fixed text on screen.

Video Wall Processing

The Encoder/Decoder shall support video wall functionality.

Video walls composed of up to 64 individual displays shall be supported with configurations using multiple Encoder/Decoder units.

Each Encoder/Decoder shall provide fully-adjustable zoom capability and bezel compensation.

One Encoder/Decoder is required per display, supporting configurations of up to eight wide by up to eight high.

Copper or Fiber LAN Connectivity

The Encoder/Decoder includes two RJ45 1000Base-T LAN ports. Either port may be used as the primary LAN connection, allowing the other to be used to provide a network connection for an additional device. These ports may also be used to daisy-chain multiple Encoder/Decoder units feeding a single-source video wall or individual displays all showing the same video image.

Encoder/Decoder unit shall support connection to a fiber optic network by inserting an appropriate SFP transceiver module into the SFP port on the Encoder/Decoder.

Encoder/Decoder manufacturer shall offer a selection of modules to accommodate various multimode and single-mode fiber types. An RJ45 module is also offered to provide a third RJ45 LAN port.

Enterprise-Grade Security

Encoder/Decoder shall employ advanced security features and protocols including:

802.1x authentication

Active Directory credential management

LDAP directory management

PKI certification

AES encryption

TLS, SSH

HTTPS

Encoder/Decoder shall run on a dedicated AV network, with fully-managed access to, or isolation from, the user’s LAN or the Internet.

CEC Device Control

Through a compatible control processor, the Encoder/Decoder unit shall include a gateway for controlling devices through their HDMI connections using the CEC signal embedded in HDMI.

Device Control

The Encoder/Decoder Unit shall include built-in RS-232 and IR control ports for control of the connected display or device.

Web-Based Setup

Setup of the Encoder/Decoder unit shall be accomplished using a computer web browser.

Full control and monitoring of the Encoder/Decoder unit is enabled through integration with a compatible control processor.

Low-Profile Installation

The Encoder/Decoder Unit mounts conveniently to a flat surface or rack rail. All connections and LED indicators are positioned on the top and bottom.

Power is provided using the included 100-240V universal power pack or an optional power injector.

Encoding/Decoding

Video Codec: Pixel Perfect Processing

Video Resolutions: Up to 4096x2160@60Hz (DCI 4K60), 4:4:4 color sampling, HDR10 and Deep Color support

Audio Formats: Primary multichannel (up to 8-channel LPCM or encoded HBR 7.1 surround sound), secondary 2-channel LPCM

Bitrates: 100 to 990 Mbps

Streaming Protocols: RTP, RTSP, SDP

Container: MPEG-2 transport stream (.ts)

Session Initiation: Multicast via RTSP

Copy Protection: HDCP 2.2

Video

Input Signal Types: HDMI w/HDR10, Deep Color, and 4K60 4:4:4 support (Dual-Mode DisplayPort and DVI compatible)

Output Signal Types: HDMI w/HDR10, Deep Color, and 4K60 4:4:4 support (DVI compatible)

Switcher: 2x1 auto-switching

Scaler: 4K60 4:4:4 video scaler with motion-adaptive deinterlacing, intelligent frame rate conversion, Deep Color support, HDR10 support, widescreen format selection (zoom, stretch, maintain aspect-ratio, or 1:1), video wall processing up to 8 wide x up to 8 high, static or dynamic text overlay

Copy Protection: HDCP 2.2

Maximum Resolutions

Maximum Common Resolutions

| Scan Type | Resolution | Frame Rate | Color Sampling | Color Depth |
| --- | --- | --- | --- | --- |
| Progressive | 4096x2160 DCI 4K & 3840x2160 4K UHD | 24 Hz | 4:4:4 | 36 bit |
|  |  | 30 Hz | 4:4:4 | 36 bit |
|  |  | 60 Hz | 4:2:2 | 36 bit |
|  |  | 60 Hz | 4:4:4 | 24 bit |
|  | 2560x1600 WQXGA | 60 Hz | 4:4:4 | 36 bit |
|  | 1920x1080 HD 1080p | 60 Hz | 4:4:4 | 36 bit |
| Interlaced (Input only) | 1920x1080 HD 1080i | 30 Hz | 4:4:4 | 36 bit |

Encoder/Decoder unit shall support other custom resolutions at pixel clock rates up to 600 MHz

Audio

Input Signal Types: HDMI (Dual-Mode DisplayPort compatible), analog stereo

Output Signal Types: HDMI, analog stereo

Digital Formats: Dolby Digital®, Dolby Digital EX, Dolby Digital Plus, Dolby TrueHD, Dolby Atmos, DTS®, DTS-ES, DTS 96/24, DTS-HD High Res, DTS-HD Master Audio, DTS:X, LPCM up to 8 channels

Analog Formats: Stereo 2-channel

Analog-To-Digital Conversion: 24-bit 48 kHz

Digital-To-Analog Conversion: 24-bit 48 kHz

Analog Performance:

Frequency Response: 20 Hz to 20 kHz ±0.5 dB

S/N Ratio: >95 dB 20 Hz to 20 kHz A-weighted

THD+N: <0.005% @ 1 kHz

Stereo Separation: >90 dB

Analog Output Volume Adjustment: -80 to +20 dB

Communications

Ethernet: 10/100/1000 Mbps, auto-switching, auto-negotiating, auto-discovery, full/half duplex, TCP/IP, UDP/IP, CIP, DHCP, SSL, TLS, SSH, SFTP (SSH File Transfer Protocol), IEEE 802.1x, IPv4, HTTPS web browser setup and control, Crestron control system integration

USB: USB 2.0 host or device signal extension

HDMI: HDCP 2.2, EDID, CEC

Integrated Control (via Ethernet): HDCP 2.2, AES, RTP, RTSP, SDP, ONVIF, IGMPv3, SMPTE 2022

Management of HDCP and EDID

Management of CEC between the connected HDMI devices and a control system

RS-232: 2-way device control and monitoring up to 115.2k baud with hardware and software handshaking (via control system); computer console (for setup)

IR/Serial: 1-way device control via infrared up to 1.1 MHz or serial TTL/RS-232 (0-5 Volts) up to 19.2k baud (via compatible control system)

Connectors

USB DEVICE:

(1) USB Type B female;

USB 2.0 device port;

USB signal extender port for connection to a computer or any other USB 2.0 host

USB HOST:

(1) USB Type A female

USB 2.0 host port

USB signal extender port for connection of a mouse, keyboard, or any other USB 2.0 device

Available Power: 500 mA at 5 Volts DC

LAN 1 – 2:

(2) 8-pin RJ45 female

10Base-T/100Base-TX/1000Base-T Ethernet ports

LAN 3:

(1) SFP port

Accepts one SFP transceiver module

HDMI OUTPUT:

(1) 19-pin Type A HDMI female

HDMI digital video/audio output (DVI compatible)

HDMI INPUT 1 – 2:

(2) 19-pin Type A HDMI female

HDMI digital video/audio inputs

(DVI & Dual-Mode DisplayPort compatible)

AUDIO I/O:

(1) 5-pin 3.5 mm detachable terminal block

Balanced/unbalanced stereo line-level audio input or output

Input Impedance: 24k Ohms balanced/unbalanced

Maximum Input Level: 4 Vrms balanced, 2 Vrms unbalanced

Output Impedance: 200 Ohms balanced, 100 Ohms unbalanced

Maximum Output Level: 4 Vrms balanced, 2 Vrms unbalanced

CONSOLE, SERIAL:

(1) 8-pin RJ45 female

RS-232 computer console port (for setup)

CONSOLE, USB:

(1) USB Type B female

USB 2.0 computer console port (for setup)

IR 1 – 2: (1) 4-pin 3.5 mm detachable terminal block

Comprises (2) IR/Serial ports

IR output up to 1.1 MHz

1-way serial TTL/RS-232 (0-5 Volts) up to 19200 baud

COM: (1) 5-pin 3.5 mm detachable terminal block

Bidirectional RS-232 port

Up to 115.2k baud, hardware and software handshaking support

24VDC 2.0A: (1) 2.1 x 5.5 mm DC power connector

24 Volt DC power input

PW-2420RU power pack included

Chassis ground lug

Controls & Indicators

TX: (1) Green LED, indicates unit is in transmitter (encoder) mode

RX: (1) Green LED, indicates unit is in receiver (decoder) mode

OL: (1) Green LED, indicates an online connection to a control system via Ethernet

LAN 1 – 2: (4) LEDs, green indicates Ethernet link status, amber indicates Ethernet activity

LAN 3 LNK: (1) Green LED, indicates Ethernet link status

LAN 3 ACT: (1) Green LED, indicates Ethernet activity

HDMI OUTPUT: (1) Green LED, indicates video signal transmission at the HDMI output

HDMI INPUT 1 – 2: (2) Green LEDs, each indicates sync detection at the corresponding HDMI input

PWR: (1) Bi-color green/amber LED, indicates operating power supplied via the power pack or injector, illuminates amber while booting and green when operating

SETUP: (1) Red LED and (1) recessed pushbutton for Ethernet setup

RESET: (1) Recessed pushbutton for hardware reset

INPUT SEL: (1) Pushbutton for manual input selection and (2) bi-color green/amber LEDs to indicate the current active input and signal presence at each corresponding input

Power

Power Pack (included):

Input: 1.5 Amps maximum @ 100-240 Volts AC, 50/60 Hz

Output: 2 Amps @ 24 Volts DC

Power over Ethernet:

Power Consumption: 35 Watts typical

Environmental

Temperature: 32° to 104° F (0° to 40° C)

Humidity: 10% to 90% RH (non-condensing)

Heat Dissipation: 85 BTU/hr

Enclosure

Chassis: Metal, black finish, integral mounting flanges, fan cooled; vented top, front, bottom, and sides

Mounting: Freestanding, surface mount, or attach to a single rack rail

Basis of design product: Crestron DM-NVX-350

Product Description: DigitalMedia 4K60 4:4:4 HDR Network AV Encoder/Decoder

ENCODER/DECODER UNIT TYPE 4

Specifier Note: DM-NVX-351 - DigitalMedia™ 4K60 4:4:4 HDR Network AV Encoder/Decoder w/Downmixing

Real-Time 4K60 Video Distribution

The Encoder/Decoder end-to-end latency for 60 frame per second content shall be less than 1 frame.

Single Component Design

In a single card, the Encoder/Decoder shall be configurable to operate as:

Network AV encoder

Network AV decoder

The encoder/decoder mode shall be switchable while in use via a control system.

Standard 1Gb network operation

4K60 Video distribution

Web based control and management

Scalable network distribution system

System of encoder/decoder units shall be scalable via network switch.

Auto-Switcher

The Encoder/Decoder shall include two HDMI inputs. Switching between inputs shall be performed:

Automatically using an auto-switching mode

Programmatically via a control system

Through a web browser

HDMI Output

When configured as a decoder, the HDMI output is scaled up or down to match the native resolution of the display device.

When used as an encoder, the HDMI output shall function as a pass through output, with resolution matched to the encoded source.

USB and KVM Integration

The Encoder/Decoder shall support the extension of USB signals, which may be switched and routed alongside the AV signal or separately via a compatible control system.

USB 2.0 host and device ports are provided on each Encoder/Decoder, allowing a USB mouse, keyboard, or other device to be connected at one DM NVX endpoint and routed to a computer or other host at any other endpoint.

USB peripherals switching functionality shall support:

Whiteboards

Touch screens

Game controllers

Cameras

Mobile devices

Headsets

Flash drives

7.1 Surround Sound Audio with Downmixing

The Encoder/Decoder shall support lossless transport of 7.1 surround sound audio signals, including:

Dolby TrueHD

Dolby Atmos

DTS-HD

DTS:X

Uncompressed linear PCM.

The Encoder/Decoder shall include the ability to decode the incoming multichannel surround sound signal, from the network or an HDMI input, and downmix that signal to stereo. The stereo downmix signal is automatically routed to the onboard analog output, while the HDMI output can be configured to output either stereo or multichannel.

As an encoder, the Encoder/Decoder unit distributes both stereo and multichannel signals simultaneously over the network, allowing either signal to be selected at any decoder on the network.

Analog Audio Embedding or De-embedding

A balanced stereo analog audio port shall be included, which may be configured as either an input or output.

As an input, it allows a stereo audio source to be connected and combined with the video signal from either HDMI input or the incoming network video stream.

As an output, it can provide a stereo line-level signal to feed a local sound system or analog audio switcher. The output volume is adjustable via a compatible control system or web browser.

Breakaway Audio

In decoder mode the Encoder/Decoder may select and combine separate video and audio signals from two different inputs, including two different encoders.

Text Overlay

The Encoder/Decoder shall be capable of displaying dynamic or fixed text on screen.

Video Wall Processing

The Encoder/Decoder shall support video wall functionality.

Video walls composed of up to 64 individual displays shall be supported with configurations using multiple Encoder/Decoder units.

Each Encoder/Decoder shall provide fully-adjustable zoom capability and bezel compensation.

One Encoder/Decoder is required per display, supporting configurations of up to eight wide by up to eight high.

Copper or Fiber LAN Connectivity

The Encoder/Decoder includes two RJ45 1000Base-T LAN ports. Either port may be used as the primary LAN connection, allowing the other to be used to provide a network connection for an additional device. These ports may also be used to daisy-chain multiple Encoder/Decoder units feeding a single-source video wall or individual displays all showing the same video image.

Encoder/Decoder unit shall support connection to a fiber optic network by inserting an appropriate SFP transceiver module into the SFP port on the Encoder/Decoder.

Encoder/Decoder manufacturer shall offer a selection of modules to accommodate various multimode and single-mode fiber types. An RJ45 module is also offered to provide a third RJ45 LAN port.

Enterprise-Grade Security

Encoder/Decoder shall employ advanced security features and protocols including:

802.1x authentication

Active Directory credential management

LDAP directory management

PKI certification

AES encryption

TLS, SSH

HTTPS

Encoder/Decoder shall run on a dedicated AV network, with fully-managed access to, or isolation from, the user’s LAN or the Internet.

CEC Device Control

Through a compatible control processor, the Encoder/Decoder unit shall include a gateway for controlling devices through their HDMI connections using the CEC signal embedded in HDMI.

Device Control

The Encoder/Decoder Unit shall include built-in RS-232 and IR control ports for control of the connected display or device.

Web-Based Setup

Setup of the Encoder/Decoder unit shall be accomplished using a computer web browser.

Full control and monitoring of the Encoder/Decoder unit is enabled through integration with a compatible control processor.

Low-Profile Installation

The Encoder/Decoder Unit mounts conveniently to a flat surface or rack rail. All connections and LED indicators are positioned on the top and bottom.

Power is provided using the included 100-240V universal power pack or an optional power injector.

Encoding/Decoding

Video Codec: Pixel Perfect Processing

Video Resolutions: Up to 4096x2160@60Hz (DCI 4K60), 4:4:4 color sampling, HDR10 and Deep Color support

Audio Formats: Primary multichannel (up to 8-channel LPCM or encoded HBR 7.1 surround sound), secondary 2-channel LPCM

Bitrates: 100 to 990 Mbps

Streaming Protocols: RTP, RTSP, SDP

Container: MPEG-2 transport stream (.ts)

Session Initiation: Multicast via RTSP

Copy Protection: HDCP 2.2

Video

Input Signal Types: HDMI w/HDR10, Deep Color, and 4K60 4:4:4 support (Dual-Mode DisplayPort and DVI compatible)

Output Signal Types: HDMI w/HDR10, Deep Color, and 4K60 4:4:4 support (DVI compatible)

Switcher: 2x1 auto-switching

Scaler: 4K60 4:4:4 video scaler with motion-adaptive deinterlacing, intelligent frame rate conversion, Deep Color support, HDR10 support, widescreen format selection (zoom, stretch, maintain aspect-ratio, or 1:1), video wall processing up to 8 wide x up to 8 high, static or dynamic text overlay

Copy Protection: HDCP 2.2

Maximum Resolutions

Maximum Common Resolutions

| Scan Type | Resolution | Frame Rate | Color Sampling | Color Depth |
| --- | --- | --- | --- | --- |
| Progressive | 4096x2160 DCI 4K & 3840x2160 4K UHD | 24 Hz | 4:4:4 | 36 bit |
|  |  | 30 Hz | 4:4:4 | 36 bit |
|  |  | 60 Hz | 4:2:2 | 36 bit |
|  |  | 60 Hz | 4:4:4 | 24 bit |
|  | 2560x1600 WQXGA | 60 Hz | 4:4:4 | 36 bit |
|  | 1920x1080 HD 1080p | 60 Hz | 4:4:4 | 36 bit |
| Interlaced (Input only) | 1920x1080 HD 1080i | 30 Hz | 4:4:4 | 36 bit |

Encoder/Decoder unit shall support other custom resolutions at pixel clock rates up to 600 MHz

Audio

Input Signal Types: HDMI (Dual-Mode DisplayPort compatible [10]), analog stereo

Output Signal Types: HDMI, analog stereo

Digital Formats: Dolby Digital®, Dolby Digital EX, Dolby Digital Plus, Dolby TrueHD, Dolby Atmos, DTS®, DTS-ES, DTS 96/24, DTS-HD High Res, DTS-HD Master Audio, DTS:X, LPCM up to 8 channels

Analog Formats: Stereo 2-channel

Analog-To-Digital Conversion: 24-bit 48 kHz

Digital-To-Analog Conversion: 24-bit 48 kHz

Analog Performance:

Frequency Response: 20 Hz to 20 kHz ±0.5 dB

S/N Ratio: >95 dB 20 Hz to 20 kHz A-weighted

THD+N: <0.005% @ 1 kHz

Stereo Separation: >90 dB

Analog Output Volume Adjustment: -80 to +20 dB

Communications

Ethernet: 10/100/1000 Mbps, auto-switching, auto-negotiating, auto-discovery, full/half duplex, TCP/IP, UDP/IP, CIP, DHCP, SSL, TLS, SSH, SFTP (SSH File Transfer Protocol), IEEE 802.1x, IPv4, HTTPS web browser setup and control, Crestron control system integration

USB: USB 2.0 host or device signal extension

HDMI: HDCP 2.2, EDID, CEC

Integrated Control (via Ethernet): HDCP 2.2, AES, RTP, RTSP, SDP, ONVIF, IGMPv3, SMPTE 2022

Management of HDCP and EDID

Management of CEC between the connected HDMI devices and a control system

RS-232: 2-way device control and monitoring up to 115.2k baud with hardware and software handshaking (via control system); computer console (for setup)

IR/Serial: 1-way device control via infrared up to 1.1 MHz or serial TTL/RS-232 (0-5 Volts) up to 19.2k baud (via compatible control system)

Connectors

USB DEVICE:

(1) USB Type B female;

USB 2.0 device port;

USB signal extender port for connection to a computer or any other USB 2.0 host

USB HOST:

(1) USB Type A female

USB 2.0 host port

USB signal extender port for connection of a mouse, keyboard, or any other USB 2.0 device

Available Power: 500 mA at 5 Volts DC

LAN 1 – 2:

(2) 8-pin RJ45 female

10Base-T/100Base-TX/1000Base-T Ethernet ports

LAN 3:

(1) SFP port

Accepts one SFP transceiver module

HDMI OUTPUT:

(1) 19-pin Type A HDMI female

HDMI digital video/audio output (DVI compatible)

HDMI INPUT 1 – 2:

(2) 19-pin Type A HDMI female

HDMI digital video/audio inputs

(DVI & Dual-Mode DisplayPort compatible)

AUDIO I/O:

(1) 5-pin 3.5 mm detachable terminal block

Balanced/unbalanced stereo line-level audio input or output

Input Impedance: 24k Ohms balanced/unbalanced

Maximum Input Level: 4 Vrms balanced, 2 Vrms unbalanced

Output Impedance: 200 Ohms balanced, 100 Ohms unbalanced

Maximum Output Level: 4 Vrms balanced, 2 Vrms unbalanced

CONSOLE, SERIAL:

(1) 8-pin RJ45 female

RS-232 computer console port (for setup)

CONSOLE, USB:

(1) USB Type B female

USB 2.0 computer console port (for setup)

IR 1 – 2: (1) 4-pin 3.5 mm detachable terminal block

Comprises (2) IR/Serial ports

IR output up to 1.1 MHz

1-way serial TTL/RS-232 (0-5 Volts) up to 19200 baud

COM: (1) 5-pin 3.5 mm detachable terminal block

Bidirectional RS-232 port

Up to 115.2k baud, hardware and software handshaking support

24VDC 2.0A: (1) 2.1 x 5.5 mm DC power connector

24 Volt DC power input

PW-2420RU power pack included

Chassis ground lug

Controls & Indicators

TX: (1) Green LED, indicates unit is in transmitter (encoder) mode

RX: (1) Green LED, indicates unit is in receiver (decoder) mode

OL: (1) Green LED, indicates an online connection to a control system via Ethernet

LAN 1 – 2: (4) LEDs, green indicates Ethernet link status, amber indicates Ethernet activity

LAN 3 LNK: (1) Green LED, indicates Ethernet link status

LAN 3 ACT: (1) Green LED, indicates Ethernet activity

HDMI OUTPUT: (1) Green LED, indicates video signal transmission at the HDMI output

HDMI INPUT 1 – 2: (2) Green LEDs, each indicates sync detection at the corresponding HDMI input

PWR: (1) Bi-color green/amber LED, indicates operating power supplied via the power pack or injector, illuminates amber while booting and green when operating

SETUP: (1) Red LED and (1) recessed pushbutton for Ethernet setup

RESET: (1) Recessed pushbutton for hardware reset

INPUT SEL: (1) Pushbutton for manual input selection and (2) bi-color green/amber LEDs to indicate the current active input and signal presence at each corresponding input

Power

Power Pack (included):

Input: 1.5 Amps maximum @ 100-240 Volts AC, 50/60 Hz

Output: 2 Amps @ 24 Volts DC

Power over Ethernet:

Power Consumption: 35 Watts typical

Environmental

Temperature: 32° to 104° F (0° to 40° C)

Humidity: 10% to 90% RH (non-condensing)

Heat Dissipation: 85 BTU/hr

Enclosure

Chassis: Metal, black finish, integral mounting flanges, fan cooled; vented top, front, bottom, and sides

Mounting: Freestanding, surface mount, or attach to a single rack rail

Basis of design product: Crestron DM-NVX-351

Product Description: DigitalMedia 4K60 4:4:4 HDR Network AV Encoder/Decoder w/Downmixing

CARD CHASSIS

Specifier Note: DMF-CI-8 DigitalMedia™ Card Chassis for DM-NVX-C & DMCF, 8 Slots

Card Chassis Design

The Card Chassis shall be a 2-space rack-mountable chassis designed to house up to eight Encoder/Decoder cards. Any of these cards may be installed in any slot.

Communications

USB Host: For loading of firmware via a USB mass storage device

USB Device: Computer console for setup

RS-232: Computer console for setup

Card Slots

Card Slots: eight Card slots per chassis

Each slot accepts one encoder/decoder card

Connectors

Chassis ground lug

100-240V~4.3-1.8A 50/60Hz: (1) IEC 60320 C14 main power inlet mates with removable power cord, included

CONSOLE, SERIAL (front): (1) 8-pin RJ45 female

RS-232 computer console port

CONSOLE, USB (front): (1) USB Type-B female

USB computer console port

USB (front): (1) USB Type A female

Supports USB mass storage devices for firmware update

Controls & Indicators

PWR: (1) Bi-color green/amber LED, indicates operating power supplied from AC line, turns amber while booting and green when operating

STATUS: (1) Bi-color green/red LED, indicates a normal (green) or critical error (red) condition

Display: (1) 2 inch (52 mm) diagonal, 220 x 176 pixels, 16-bit TFT active matrix color LCD, displays IP address settings for each installed card

Navigation Pad: (1) 5-way navigation pad for menu navigation and adjustment of settings

HOME: (1) Pushbutton, enters or returns to the home menu

BACK: (1) Pushbutton, steps menu back one level

CARD SLOT STATUS, TX 1 – 8: (8) Green LEDs, each indicates the corresponding card is in transmitter mode

CARD SLOT STATUS, RX 1 – 8: (8) Green LEDs, each indicates the corresponding card is in receiver mode

CARD SLOT STATUS, OL 1 – 8: (8) Green LEDs, each indicates the corresponding card is online

Power

Main Power: 4.3-1.8 Amps @ 100-240 Volts AC, 50/60 Hz

Power Consumption: 350 Watts fully loaded

Environmental

Temperature: 32° to 104° F (0° to 40° C)

Humidity: 10% to 90% RH (non-condensing)

Heat Dissipation: 1195 BTU/hr fully loaded

Enclosure

Chassis: Metal with black finish, vented sides, fan-cooled

Front Panel: Metal, black finish with polycarbonate label overlay

Mounting: Freestanding or 2 RU 19-inch rack-mountable (rack ears included)

Basis of design product: DMF-CI-8

DigitalMedia Card Chassis for DM-NVX-C & DMCF, 8 Slots

NOT USED

END OF SECTION 27 41 16