SECTION 27 41 16

INTEGRATED AUDIO-VIDEO SYSTEMS AND EQUIPMENT

GUIDE SPECIFICATION

Specifier: The Specifier/Design Professional is responsible for the accuracy of all project specifications, including system application and coordination with related sections.  This guide specification is provided as a convenience and requires editing to match actual project requirements.  CRESTRON ELECTRONICS, INC. SHALL NOT BE LIABLE FOR ANY DAMAGES ARISING OUT OF THE USE OF ANY OF ITS GUIDE SPECIFICATIONS.  For Crestron design assistance and design review please contact Sales Support Services Department at   800.237.2041 or techsales@crestron.com.

Specifier:  Please see PART 4 for a listing of products specified in this Guide Specification.

Table of Contents

PART 1	GENERAL	4

PART 2	PRODUCTS	4

2.1	CEILING SPEAKER TYPE 1:	4

A.	Basis of Design:	4

B.	Device Architecture	4

C.	Functions	5

D.	Compliance	6

2.2	CEILING SPEAKER TYPE 2:	6

A.	Basis of Design:	6

B.	Device Architecture	6

C.	Functions	7

D.	Compliance	8

2.3	CEILING SPEAKER TYPE 3:	9

A.	Basis of Design:	9

B.	Device Architecture	9

C.	Functions	10

D.	Compliance	10

2.4	CEILING SPEAKER TYPE 4:	11

A.	Basis of Design:	11

B.	Device Architecture	11

C.	Functions	12

D.	Compliance	13

2.5	CEILING SPEAKER TYPE 5:	13

A.	Basis of Design:	13

B.	Device Architecture	13

C.	Functions	14

D.	Compliance	15

2.6	CEILING SPEAKER TYPE 6:	15

A.	Basis of Design Product:	15

B.	Device Architecture	15

C.	Functions	16

D.	Compliance	17

2.7	CEILING SPEAKER TYPE 7:	17

A.	Basis of Design Product:	17

B.	Device Architecture	17

C.	Functions	18

D.	Compliance	18

2.8	CEILING SPEAKER TYPE 8:	19

A.	Basis of Design Product:	19

B.	Device Architecture	19

C.	Functions	19

D.	Compliance	20

PART 3	EXECUTION	20

PART 4	APPENDICES	20

4.1	SPECIFIED PRODUCTS	20

A.	Crestron SAROS IC4T	20

B.	Crestron SAROS IC6T	20

C.	Crestron SAROS IC6LPT	20

D.	Crestron SAROS IC8T	20

E.	Crestron SAROS ICS8T	20

F.	Crestron SAROS IC8LPT	20

G.	Crestron SAROS ICT5T-12	20

H.	Crestron SAROS ICT5T-22	21

GENERAL

NOT USED in this Guide Specification.  Specifier shall Specify PART 1 administrative and procedural requirements as needed.

PRODUCTS

CEILING SPEAKER TYPE 1:

Specifier Note:

The Saros IC4T model is a 2-way in-ceiling speaker featuring a 4 inch woofer and wide-dispersion, horn-loaded 3/4 inch dome tweeter. Saros® speakers by Crestron® deliver professional grade performance and flexible installation in a range of popular sizes for demanding commercial applications.

Basis of Design:

Crestron SAROS IC4T

Device Architecture

Enclosure material: Aluminum, plenum rated.

Baffle: ABS UL94V-0 plastic, low frequency ported.

Grille:

Steel

Frameless magnetically affixed with safety tether.

Finish: Re-paintable, powder coated.

Mounting:

Flush ceiling mount.

Four integral two-step toggle mounting clamps.

Maximum ceiling thickness: 2.4 inches (61 mm).

Cutout diameter: 7.75 inches (197 mm).

Minimum ceiling cavity depth: 6.9 inches (176 mm).

Environmental Operating Conditions:

Temperature: -2° to 120°F (-19° to 49°C)

Humidity: 5% to 95% RH (non-condensing)

Included Accessories:

Support bridge for tile ceiling installation.

Adjustable for off center installation.

Collapsible design allowing bridge support to be inserted from below ceiling.

Optional Accessories:

Safety Tether

Have a minimum of two (2) safety wire installation points.

Black Grille

Termination:

4-pin 5 mm screw down conductor detachable terminal block with speaker input and parallel pass-through connections.

Maximum wire size: 12 AWG.

Conduit knockouts:

One 0.5-inch knockout.

One 0.75-inch knockout.

Dimensions:

Diameter: 8.53 inches (217 mm), not including toggles.

Depth: 7.13 inches (181 mm).

Weight: 7.02 lb. (3.2 kg)

Drivers:

4-inch (102 mm) polypropylene woofer with decoupled cloth surround and steel basket

.79-inch (20 mm) titanium dome horn loaded tweeter

Functions

Speaker unit shall be equipped for Low Impedance 8-Ohm or High Impedance 70V or 100V operation.

Accessible front mounted low impedance, high impedance selector switch.

High Impedance Line Transformer:

Five level taps at 70V:

1.88 Watts

3.75 Watts

7.5 Watts

15 Watts

30 Watts

Four level taps at 100V:

3.75 Watts

7.5 Watts

15 Watts

30 Watts

Low Impedance transformer by-passed:

8 Ω

Sensitivity: 85 dB at one meter, with 1-W input.

Frequency Response: Within plus or minus 3 dB from 60 to 20,000 Hz.

Frequency Range: Within minus 10 dB from 45 to 20,000 Hz.

Crossover Frequency: 2 kHz

Nominal Coverage: 110 degrees conical.

Power Handling: 100 Watts Program at 8-Ohms.

Specifier: Crestron Saros speakers utilize a tweeter protection method that does not require replacement of internal components.  In the event of the tweeter being “over-powered” the protection device will limit power to the tweeter as long as the condition exists.  Many competitor speakers utilize a fuse or fuse-bulb type component, which is capable of absorbing some over-limit power, but will eventually rupture, requiring the entire unit to be removed and disassemble for replacement of the fuse/fuse-bulb.

Driver Protection:

Integral tweeter protection circuit.

Self-resetting protection device.

Speakers utilizing driver protection methods requiring physical replacement of ruptured protection components including but not limited to, fuses or fuse bulbs shall not be accepted.

Compliance

UL Listed for US & Canada

UL 1480

UL 2043

CE

FCC Part 15 Class B

CEILING SPEAKER TYPE 2:

Specifier Note: The Saros IC6T model is a 2-way in-ceiling speaker featuring a 6.50 in. (165.1 mm) woofer and wide-dispersion, horn-loaded 1 in. (25.4 mm) dome tweeter. Saros® speakers by Crestron® deliver professional grade performance and flexible installation in a range of popular sizes for demanding commercial applications.

Basis of Design:

Crestron SAROS IC6T

Device Architecture

Enclosure material: Aluminum, plenum rated.

Baffle: ABS UL94V-0 plastic, low frequency ported.

Grille:

Steel

Frameless magnetically affixed with safety tether.

Finish: Re-paintable, powder coated.

Mounting:

Flush ceiling mount.

Four integral two-step toggle mounting clamps.

Maximum ceiling thickness: 2.4 inches (61 mm).

Cutout diameter: 9.68 inches (246 mm).

Minimum ceiling cavity depth: 8.1 inches (206 mm).

Environmental Operating Conditions:

Temperature: -2° to 120°F (-19° to 49°C)

Humidity: 5% to 95% RH (non-condensing)

Included Accessories:

Support bridge for tile ceiling installation.

Adjustable for off center installation.

Collapsible design allowing bridge support to be inserted from below ceiling.

Optional Accessories:

Safety Tether

Have a minimum of two (2) safety wire installation points.

Black Grille

Termination:

4-pin 5 mm screw down conductor detachable terminal block with speaker input and parallel pass-through connections.

Maximum wire size: 12 AWG.

Conduit knockouts:

One 0.5-inch knockout.

One 0.75-inch knockout.

Dimensions:

Diameter: 10.57 inches (269 mm), not including toggles.

Depth: 8.31 inches (211 mm).

Weight: 10.38 lb. (4.7 kg)

Drivers:

6.5-inch (165 mm) polypropylene woofer with decoupled cloth surround and steel basket

.98-inch (25 mm) titanium dome horn loaded tweeter

Functions

Speaker unit shall be equipped for Low Impedance 8-Ohm or High Impedance 70V or 100V operation.

Accessible front mounted low impedance, high impedance selector switch.

High Impedance Line Transformer:

Five level taps at 70V:

3.75 Watts

7.5 Watts

15 Watts

30 Watts

60 Watts

Four level taps at 100V:

7.5 Watts

15 Watts

30 Watts

60 Watts

Low Impedance transformer by-passed:

8 Ω

Sensitivity: 88.5 dB at one meter, with 1-W input.

Frequency Response: Within plus or minus 3 dB from 50 to 20,000 Hz.

Frequency Range: Within minus 10 dB from 40 to 20,000 Hz.

Crossover Frequency: 2.5 kHz

Nominal Coverage: 100 degrees conical.

Power Handling: 125 Watts Program at 8-Ohms.

Specifier: Crestron Saros speakers utilize a tweeter protection method that does not require replacement of internal components.  In the event of the tweeter being “over-powered” the protection device will limit power to the tweeter as long as the condition exists.  Many competitor speakers utilize a fuse or fuse-bulb type component, which is capable of absorbing some over-limit power, but will eventually rupture, requiring the entire unit to be removed and disassemble for replacement of the fuse/fuse-bulb.

Driver Protection:

Integral tweeter protection circuit.

Self-resetting protection device.

Speakers utilizing driver protection methods requiring physical replacement of ruptured protection components including but not limited to, fuses or fuse bulbs shall not be accepted.

Compliance

UL 1480

UL 2043

CEILING SPEAKER TYPE 3:

Specifier Note: The Saros IC6LPT model is a 2-way in-ceiling speaker featuring a 6-1/2 inch woofer and wide-dispersion, horn-loaded 1 inch dome tweeter. Its low-profile enclosure allows for installation in constricted ceiling spaces as shallow as 5-1/8 inches (130 mm). Saros® speakers by Crestron® deliver professional grade performance and flexible installation in a range of popular sizes for demanding commercial applications.

Basis of Design:

Crestron SAROS IC6LPT

Device Architecture

Enclosure material: Zinc-plated steel, plenum rated with side entry cable clamp

Baffle: ABS UL94V-0 plastic.

Grille:

Steel

Magnetically held frameless design

White textured finish, paintable

Mounting:

Flush ceiling mount.

Four integral two-step swiveling dogs

Maximum ceiling thickness: 2.4 inches (61 mm).

Recommended cutout diameter: 9.875 inches (250 mm).

Minimum ceiling cavity depth: 5.2 inches (130 mm).

Environmental Operating Conditions:

Temperature: -2° to 120°F (-19° to 49°C)

Humidity: 5% to 95% RH (non-condensing)

Included Accessories:

Support bridge for tile ceiling installation.

Adjustable for off center installation.

Optional Accessories:

Safety Tether

Have a minimum of two (2) safety wire installation points.

Termination:

4-pin 5 mm screw down conductor detachable terminal block with speaker input and parallel pass-through connections.

Maximum wire size: 12 AWG.

Dimensions:

Diameter: 10.64 inches (270 mm), not including mounting dogs.

Depth: 5.30 inches (135 mm).

Weight: 8.2 lb. (3.7 kg)

Drivers:

6.5-inch (165 mm) polypropylene woofer with ring mode decoupled cloth surround and steel basket

.98-inch (25 mm) titanium dome horn loaded tweeter

Functions

Speaker unit shall be equipped for Low Impedance 8 Ω or High Impedance 70 V or 100 V operation.

Accessible front mounted low impedance, high impedance selector switch.

High Impedance Line Transformer:

Five level taps at 70 V:

3.75 Watts

7.5 Watts

15 Watts

30 Watts

60 Watts

Four level taps at 100 V:

7.5 Watts

15 Watts

30 Watts

60 Watts

Low Impedance transformer:

8 Ω

Sensitivity: 88 dB at 1 W, 1 m away.

Frequency Response: Within plus or minus 3 dB from 65 to 20,000 Hz.

Frequency Range: Within minus 10 dB from 55 to 20,000 Hz.

Crossover frequency: 2.5 kHz.

Nominal Coverage: 95 degrees conical.

Power Handling: 50 W program at 8 Ω

Compliance

UL Listed for US & Canada

UL 1480

UL 2043

CE

FCC Part 15 Class B

CEILING SPEAKER TYPE 4:

Specifier Note: The Saros IC8T model is a 2-way in-ceiling speaker featuring an 8 inch woofer and wide-dispersion, horn-loaded 1 inch dome tweeter. Saros® speakers by Crestron® deliver professional grade performance and flexible installation in a range of popular sizes for demanding commercial applications.

Basis of Design:

Crestron SAROS IC8T

Device Architecture

Enclosure material: Aluminum, plenum rated.

Baffle: ABS UL94V-0 plastic, low frequency ported.

Grille:

Steel

Frameless magnetically affixed with safety tether.

Finish: Re-paintable, powder coated.

Mounting:

Flush ceiling mount.

Four integral two-step toggle mounting clamps.

Maximum ceiling thickness: 2.4 inches (61 mm).

Cutout diameter: 11.3125 inches (287 mm).

Minimum ceiling cavity depth: 8.9 inches (226 mm).

Environmental Operating Conditions:

Temperature: -2° to 120°F (-19° to 49°C)

Humidity: 5% to 95% RH (non-condensing)

Included Accessories:

Support bridge for tile ceiling installation.

Adjustable for off center installation.

Collapsible design allowing bridge support to be inserted from below ceiling.

Optional Accessories:

Safety Tether

Have a minimum of two (2) safety wire installation points.

Black Grille

Termination:

4-pin 5 mm screw down conductor detachable terminal block with speaker input and parallel pass-through connections.

Maximum wire size: 12 AWG.

Conduit knockouts:

One 0.5-inch knockout.

One 0.75-inch knockout.

Dimensions:

Diameter: 12.07 inches (307 mm), not including toggles.

Depth: 9.10 inches (231 mm).

Weight: 12.5 lb. (5.7 kg)

Drivers:

8-inch (203 mm) polypropylene woofer with ring mode decoupled cloth surround and steel basket

.98-inch (25 mm) titanium dome horn loaded tweeter

Functions

Speaker unit shall be equipped for Low Impedance 8-Ohm or High Impedance 70V or 100V operation.

Accessible front mounted low impedance, high impedance selector switch.

High Impedance Line Transformer:

Five level taps at 70V:

3.75 Watts

7.5 Watts

15 Watts

30 Watts

60 Watts

Four level taps at 100V:

7.5 Watts

15 Watts

30 Watts

60 Watts

Low Impedance transformer by-passed:

8-Ohms.

Sensitivity: 90.5 dB at one meter, with 1-W input.

Frequency Response: Within plus or minus 3 dB from 50 to 20,000 Hz.

Frequency Range: Within minus 10 dB from 40 to 20,000 Hz.

Crossover Frequency: 2.5 kHz

Nominal Dispersion Angle: 100 degrees conical.

Power Handling: 150 Watts Program at 8-Ohms.

Specifier: Crestron Saros speakers utilize a tweeter protection method that does not require replacement of internal components.  In the event of the tweeter being “over-powered” the protection device will limit power to the tweeter as long as the condition exists.  Many competitor speakers utilize a fuse or fuse-bulb type component, which is capable of absorbing some over-limit power, but will eventually rupture, requiring the entire unit to be removed and disassemble for replacement of the fuse/fuse-bulb.

Driver Protection:

Integral tweeter protection circuit.

Self-resetting protection device.

Speakers utilizing driver protection methods requiring physical replacement of ruptured protection components including but not limited to, fuses or fuse bulbs shall not be accepted.

Compliance

UL Listed for US & Canada

UL 1480

UL 2043

CE

FCC Part 15 Class B

CEILING SPEAKER TYPE 5:

Specifier Note: The Saros ICS8T model is an 8 inch in-ceiling subwoofer designed to provide extended low-frequency response for music and sound reinforcement applications. Saros® speakers by Crestron® deliver professional grade performance and flexible installation in a range of popular sizes for demanding commercial applications.

Basis of Design:

Crestron SAROS ICS8T

Device Architecture

Enclosure material: Zinc-plated steel, plenum rated.

Baffle: ABS UL94V-0 plastic.

Grille:

Steel

Magnetically held frameless design

White textured finish, paintable

Mounting:

Flush ceiling mount.

Four integral two-step toggle mounting clamps.

Maximum ceiling thickness: 2.4 inches (61 mm).

Cutout diameter: 11.3125 inches (287 mm).

Minimum ceiling cavity depth: 8.9 inches (226 mm).

Environmental Operating Conditions:

Temperature: -2° to 120°F (-19° to 49°C)

Humidity: 5% to 95% RH (non-condensing)

Included Accessories:

Support bridge for tile ceiling installation.

Adjustable for off center installation.

Optional Accessories:

Safety Tether

Have a minimum of two (2) safety wire installation points.

Black Grille

Termination:

4-pin 5 mm screw down conductor detachable terminal block with speaker input and parallel pass-through connections.

Maximum wire size: 12 AWG.

Conduit knockouts:

One 0.5-inch knockout.

One 0.75-inch knockout.

Dimensions:

Diameter: 12.07 inches (307 mm), not including toggles.

Depth: 9.10 inches (231 mm).

Weight: 14.0 lb. (6.4 kg)

Drivers:

8-inch (203 mm) polypropylene woofer with rubber surround, steel basket and phase plug

Functions

Speaker unit shall be equipped for Low Impedance 8 Ω or High Impedance 70 V or 100 V operation.

Accessible front mounted low impedance, high impedance selector switch.

High Impedance Line Transformer:

Two level taps at 70 V:

60 Watts

120 Watts

One level tap at 100 V:

120 Watts

Low Impedance transformer by-passed:

8 Ω

Sensitivity: 90 dB at 1 W, 1 m away.

Frequency Response: Within plus or minus 3 dB from 45 to 125 Hz.

Frequency Range: Within minus 10 dB from 35 to 125 Hz.

125 Hz 2nd order Low-Pass Filter

Power Handling: 100 Watts Program at 8 Ω.

Compliance

UL Listed for US & Canada

UL 1480

UL 1480A

UL 2043

CE

FCC Part 15 Class B

CEILING SPEAKER TYPE 6:

Specifier Note: The Saros IC8LPT model is a 2-way in-ceiling speaker featuring an 8 inch woofer and wide-dispersion, horn-loaded 1 inch dome tweeter. Its low-profile enclosure allows for installation in constricted ceiling spaces as shallow as 5-1/8 inches (130 mm). Saros® speakers by Crestron® deliver professional grade performance and flexible installation in a range of popular sizes for demanding commercial applications.

Basis of Design Product:

Crestron SAROS IC8LPT

Device Architecture

Enclosure material: Zinc-plated steel, plenum rated with side entry cable clamp

Baffle: ABS UL94V-0 plastic.

Grille:

Steel

Magnetically held frameless design

White textured finish, paintable

Mounting:

Flush ceiling mount.

Four integral two-step swiveling dogs

Maximum ceiling thickness: 2.4 inches (61 mm).

Recommended cutout diameter: 11.3125 inches (287 mm).

Minimum ceiling cavity depth: 5.2 inches (130 mm).

Environmental Operating Conditions:

Temperature: -2° to 120°F (-19° to 49°C)

Humidity: 5% to 95% RH (non-condensing)

Included Accessories:

Support bridge for tile ceiling installation.

Adjustable for off center installation.

Optional Accessories:

Safety Tether

Have a minimum of two (2) safety wire installation points.

Black Grille

Termination:

4-pin 5 mm screw down conductor detachable terminal block with speaker input and parallel pass-through connections.

Maximum wire size: 12 AWG.

Dimensions:

Diameter: 12.13 inches (308 mm), not including mounting dogs

Depth: 5.30 inches (135 mm).

Weight: 11.2 lb. (5.0 kg)

Drivers:

8-inch (203 mm) polypropylene woofer with ring mode decoupled cloth surround and steel basket

.98-inch (25 mm) titanium dome horn loaded tweeter

Functions

Speaker unit shall be equipped for Low Impedance 8 Ω or High Impedance 70 V or 100 V operation.

Accessible front mounted low impedance, high impedance selector switch.

High Impedance Line Transformer:

Five level taps at 70 V:

3.75 Watts

7.5 Watts

15 Watts

30 Watts

60 Watts

Four level taps at 100 V:

7.5 Watts

15 Watts

30 Watts

60 Watts

Low Impedance transformer:

8 Ω

Sensitivity: 90 dB at 1 W, 1 m away.

Frequency Response: Within plus or minus 3 dB from 65 to 20,000 Hz.

Frequency Range: Within minus 10 dB from 55 to 20,000 Hz.

Crossover frequency: 2.5 kHz.

Nominal Coverage: 95 degrees conical.

Power Handling: 50 W program at 8 Ω

Compliance

UL Listed for US & Canada

UL 1480

UL 1480A

UL 2043

CE

FCC Part 15 Class B

CEILING SPEAKER TYPE 7:

Specifier Note: The Saros ICT5T-12 is a 1 ft x 2 ft ceiling mount loudspeaker designed for multimedia presentation and speech reinforcement in classrooms, meeting spaces, and most indoor distributed sound system applications. It features a coaxial 5 in. two-way driver in a low profile enclosure that’s ideal for use in plenum spaces that are packed with preexisting systems and utilities, leaving limited space for ceiling loudspeakers.

Basis of Design Product:

Crestron SAROS ICT5T-12

Device Architecture

Enclosure material: Steel, painted black.

Baffle: Steel, painted black.

Grille: Steel, painted white.

Mounting:

Lays onto suspended ceiling grid

Environmental Operating Conditions:

For indoor use only

Included Accessories:

One (1) cross tee accessory per loudspeaker.

24-inch length, 15/16-inch face profile

Staked-on end clips

Optional Accessories:

Safety Tether

Have a minimum of two (5) safety wire installation points.

Termination:

4-pin 5 mm screw down conductor detachable terminal block with speaker input and parallel pass-through connections.

Maximum wire size: 12 AWG.

Dimensions:

Height: 4.00 inches

Width: 11.88 inches

Depth: 23.75 inches

Weight: 8.4 lb. (3.8 kg)

Transducer: 5-inch woofer with dome tweeter

Functions

Speaker unit shall be equipped for Low Impedance 8 Ω or High Impedance 70 V or 100 V operation.

Rotary knob on plenum side of loudspeaker enclosure for low impedance, high impedance mode selection.

High Impedance Line Transformer:

Five level taps at 70 V:

1 Watt

2 Watts

4 Watts

8 Watts

16 Watts

Four level taps at 100 V:

2 Watts

4 Watts

8 Watts

16 Watts

Low Impedance transformer:

8 Ω

Sensitivity: 90 dB at 1 W, 1 m away.

Frequency Range: Within plus 3 to minus 10 dB from 70 to 18,000 Hz.

Beamwidth: 120 degrees

Power Handling: 30 W program at 8 Ω

Compliance

UL 1480A

UL 2043

CEILING SPEAKER TYPE 8:

Specifier Note: The Saros ICT5T-22 is a 2 ft x 2 ft ceiling mount loudspeaker designed for multimedia presentation and speech reinforcement in classrooms, meeting spaces, and most indoor distributed sound system applications. It features a coaxial 5 in. two-way driver in a low profile enclosure that’s ideal for use in plenum spaces that are packed with preexisting systems and utilities, leaving limited space for ceiling loudspeakers.

Basis of Design Product:

Crestron SAROS ICT5T-22

Device Architecture

Enclosure material: Steel, painted black.

Baffle: Steel, painted black.

Grille: Steel, painted white.

Mounting:

Lays onto suspended ceiling grid

Environmental Operating Conditions:

For indoor use only

Included Accessories:

One (1) cross tee accessory per loudspeaker.

24-inch length, 15/16-inch face profile

Staked-on end clips

Optional Accessories:

Safety Tether

Have a minimum of two (5) safety wire installation points.

Termination:

4-pin 5 mm screw down conductor detachable terminal block with speaker input and parallel pass-through connections.

Maximum wire size: 12 AWG.

Dimensions:

Height: 4.00 inches

Width: 23.75 inches

Depth: 23.75 inches

Weight: 12.45 lb. (5.6 kg)

Transducer: 5-inch woofer with dome tweeter

Functions

Speaker unit shall be equipped for Low Impedance 8 Ω or High Impedance 70 V or 100 V operation.

Rotary knob on plenum side of loudspeaker enclosure for low impedance, high impedance mode selection.

High Impedance Line Transformer:

Five level taps at 70 V:

1 Watt

2 Watts

4 Watts

8 Watts

16 Watts

Four level taps at 100 V:

2 Watts

4 Watts

8 Watts

16 Watts

Low Impedance transformer:

8 Ω

Sensitivity: 90 dB at 1 W, 1 m away.

Frequency Range: Within plus 3 to minus 10 dB from 70 to 18,000 Hz.

Beamwidth: 120 degrees

Power Handling: 30 W program at 8 Ω

Compliance

UL 1480A

UL 2043

EXECUTION

NOT USED in this Guide Specification.  Specifier shall Specify PART 3 On-Site work as needed.

APPENDICES

SPECIFIED PRODUCTS

Crestron SAROS IC4T

Crestron SAROS IC6T

Crestron SAROS IC6LPT

Crestron SAROS IC8T

Crestron SAROS ICS8T

Crestron SAROS IC8LPT

Crestron SAROS ICT5T-12

Crestron SAROS ICT5T-22