SECTION 27 41 16

INTEGRATED AUDIO-VIDEO SYSTEMS AND EQUIPMENT

GUIDE SPECIFICATION

Specifier: The Specifier/Design Professional is responsible for the accuracy of all project specifications, including system application and coordination with related sections.  This guide specification is provided as a convenience and requires editing to match actual project requirements.  CRESTRON ELECTRONICS, INC. SHALL NOT BE LIABLE FOR ANY DAMAGES ARISING OUT OF THE USE OF ANY OF ITS GUIDE SPECIFICATIONS.  For Crestron design assistance and design review please contact Sales Support Services Department at   800.237.2041 or techsales@crestron.com.

Specifier: Please see PART 4 for a listing of products specified in this Guide Specification.

Table of Contents

1	GENERAL	4

2	PRODUCTS	4

2.1	USB 3.2 Extender over CAT6a/7 Cable, Local	4

2.1.1	Basis of Design	4

2.1.2	Device Definition	4

2.1.3	Device Architecture	5

2.1.4	Functions	5

2.1.5	Compliance	7

2.2	USB 3.2 Extender over CAT 6a/7 Cable, Remote	7

2.2.1	Basis of Design	8

2.2.2	Device Definition	8

2.2.3	Device Architecture	8

2.2.4	Functions	9

2.2.5	Compliance	12

2.3	USB 3.2 Extender over CAT 6a/7 Cable, Local and Remote	12

2.3.1	Basis of Design	12

2.3.2	Device Definition	12

2.3.3	Device Architecture	13

2.3.4	Functions	13

2.3.5	Compliance	17

2.4	USB 3.2 Extender over CAT6A/7 Cable, Local, 1-Gang	17

2.4.1	Basis of Design	17

2.4.2	Device Definition	17

2.4.3	Device Architecture	18

2.4.4	Functions	19

2.4.5	Compliance	21

2.5	USB 3.2 Extender over CAT6A/7 Cable, Remote, 1-Gang	21

2.5.1	Basis of Design	21

2.5.2	Device Definition	21

2.5.3	Device Architecture	22

2.5.4	Functions	23

2.5.5	Compliance	25

3	EXECUTION	25

4	APPENDICES	26

4.1	SPECIFIED PRODUCTS	26

4.1.1	Crestron USB-EXT-3S-LOCAL	26

4.1.2	Crestron USB-EXT-3S-REMOTE	26

4.1.3	Crestron USB-EXT-3S SYSTEM	26

4.1.4	Crestron USB-EXT-3S-LOCAL-1G	26

4.1.5	Crestron USB-EXT-3S-REMOTE-1G	26

# GENERAL

NOT USED in this Guide Specification.  Specifier shall Specify PART 1 administrative and procedural requirements as needed.

# PRODUCTS

## USB 3.2 Extender over CAT6a/7 Cable, Local

Specifier Note:

The USB-EXT-3S-LOCAL is a USB 3.2 Gen 1 extender with connectivity for a single USB host such as a computer. The extender supports plug-and-play functionality with hosts running Windows®, macOS®, Linux®, or Chrome OS™ operating systems. No additional driver installation is required.

### Basis of Design

#### Crestron USB-EXT-3S-LOCAL

Specifier Note:

USB-EXT-3S-LOCAL
https://www.crestron.com/Products/Catalog/Audio-Video-and-USB/USB/Extender/USB-EXT-3S-LOCAL

### Device Definition

#### USB local endpoint with the following capabilities and features:

##### USB signal extension over CAT 6A/7 cable up to 330 ft (100 m)

##### USB 3.2 Gen 1 data rate up to 5 Gbps

##### USB Type-A and USB Type-C connectivity

##### Simultaneous USB 3.2, 2.0, and 1.1 signal support

##### Support for USB transfer types: control, interrupt, isochronous (ISO), and bulk

##### RS-232 serial pass-through

##### Plug-and-play functionality without installation of additional drivers

##### Compatibility with Windows®, macOS®, Linux®, and Chrome OS™ operating systems

##### TAA compliance

##### Freestanding or surface mountable form factor

##### 20 VDC power pack included

### Device Architecture

#### Environmental

##### Operating Temperature: 32° to 122°F (0° to 50° C)

##### Storage Temperature: -4° to 158°F (-20° to 70° C)

##### Humidity: 10% to 90% RH (non-condensing)

#### Construction

##### Chassis: Metal, black finish, vented sides

##### Mounting: Freestanding or surface mountable

#### Dimensions

##### Height: 0.89 in. (23 mm)

##### Width: 5.12 in. (130 mm)

##### Depth: 4.90 in. (125 mm)

#### Weight: 12.32 oz (350 g)

### Functions

#### Communications

##### LINK: Proprietary link for point-to-point connection between endpoints

##### USB Speeds

###### USB 3.2 Gen 1 up to 5 Gbps

###### USB 2.0 up to 480 Mbps

###### USB 1.1 up to 12 Mbps

##### RS-232: Bidirectional serial pass-through

#### Connectors

##### 20 V 3.0 A: (1) Locking DC power connector; 20 VDC power input

Specifier Note:

Only one endpoint should be connected to a local power supply. Either the local or remote endpoint can be powered locally. The other endpoint will receive power over the LINK connection. When the remote endpoint receives power over the LINK connection from the local endpoint, the total USB current available to connected peripherals will be less than 3.5 A.

##### TO HOST USB 3

###### (1) USB Type-C® connector, female

###### USB 3.2 Gen 1 host port for connection to computer, media server, game console, or other USB host

Specifier Note:

For the LINK connection between the local and remote extenders, use CAT6a/7 UTP (unshielded twisted pair), FTP (foiled twisted pair), or STP (shielded twisted pair) as determined by the specific installation requirements. The maximum cable length is 330 ft (100 m). If using on-premise cabling, up to 33 ft (10 m) of CAT6a/7 patch cord can be used for each extender, ensuring that the total cable length does not exceed 330 ft (100 m). The remaining 264 ft (80 m) distance must consist of solid-core premise cabling.

###### Backward compatibility with USB 2.0 and USB 1.1

##### LINK

###### (1) 8-pin RJ45 connector, female

###### Connects directly to LINK port on Remote endpoint by same manufacturer

Specifier Note:

The LINK ports of the local and the remote endpoints must be connected to each other. Do not connect the LINK ports to any other device or to a network.

###### Up to 330 ft (100 m) over CAT6a/7 cable

###### If powered locally, supplies power to connected REMOTE endpoint

###### If not powered locally, receives power from connected REMOTE endpoint

##### COM

###### (1) 4-pin 3.5 mm detachable terminal block

###### Bidirectional RS-232 port up to 115.2k baud

###### Fourth pin (labeled NC) not used

##### SERVICE: For factory use only

#### Indicators

##### PWR: (1) Green LED, indicates that power is being supplied from the included power pack or via the LINK connection

##### LINK: (1) Green LED, indicates that a link is established between the local and remote endpoints

##### USB

###### (1) Green LED

###### Solid indicates that only USB 3 (or greater) devices are connected and the link is active

###### Flashing green indicates that one or more USB 2 (or lower) devices are connected and the link is active

###### Off indicates that no USB devices are connected

#### Power

##### Power Pack

###### Input: 100-240 VAC, 50/60 Hz

###### Output: 3.0 A @ 20 VDC

###### Power Consumption

31.28 W total between LOCAL and REMOTE

Specifier Note:

When a remote endpoint receives power over the LINK connection from a local endpoint, the total USB power available to connected peripherals will be less than 17.5 W (3.5 A).

### Compliance

#### Regulatory Model: M202444001

#### FCC Part 15 Class B, IC Class B, CE, Intertek® Listed for US and Canada

## USB 3.2 Extender over CAT 6a/7 Cable, Remote

Specifier Note:

The USB-EXT-3S-REMOTE is a USB 3.2 Gen 1 extender with connectivity for up to four USB devices such as cameras or video capture devices at one location. The extender is expandable to support additional USB devices with the connection of USB hubs (not included).

### Basis of Design

#### Crestron USB-EXT-3S-REMOTE

Specifier Note:

USB-EXT-3S-REMOTE 
https://www.crestron.com/Products/Catalog/Audio-Video-and-USB/USB/Extender/USB-EXT-3S-REMOTE

### Device Definition

#### USB point-to-point extension solution with the following capabilities and features:

##### USB signal extension over CAT 6A/7 cable up to 330 ft (100 m)

##### USB 3.2 Gen 1 data rate up to 5 Gbps

##### USB Type-A and USB Type-C connectivity

##### Simultaneous USB 3.2, 2.0, and 1.1 signal support

##### Support for USB transfer types: control, interrupt, isochronous (ISO), and bulk

##### RS-232 serial pass-through

##### Plug-and-play functionality without installation of additional drivers

##### Compatibility with Windows®, macOS®, Linux®, and Chrome OS™ operating systems

##### TAA compliance

##### Freestanding or surface mountable form factor

##### 20 VDC power pack included

### Device Architecture

#### Environmental

##### Operating Temperature: 32° to 122°F (0° to 50° C)

##### Storage Temperature: -4° to 158°F (-20° to 70° C)

##### Humidity: 10% to 90% RH (non-condensing)

#### Construction

##### Chassis: Metal, black finish, vented sides

##### Mounting: Freestanding or surface mountable

#### Dimensions

##### Height: 0.89 in. (23 mm)

##### Width: 5.12 in. (130 mm)

##### Depth: 4.90 in. (125 mm)

#### Weight: 12 oz (340 g)

### Functions

#### Communications

##### LINK: Proprietary link for point-to-point connection between endpoints

##### USB Speeds

###### USB 3.2 Gen 1 up to 5 Gbps

###### USB 2.0 up to 480 Mbps

###### USB 1.1 up to 12 Mbps

##### RS-232: Bidirectional serial pass-through

#### Connectors – Remote Endpoint

##### 20 V 3.0 A: (1) locking DC power connector; 20 VDC power input

Specifier Note:

Only one endpoint should be connected to a local power supply. Either the local or remote endpoint can be powered locally. The other endpoint will receive power over the LINK connection. When the remote endpoint receives power over the LINK connection from the local endpoint, the total USB current available to connected peripherals will be less than 3.5 A.

##### TO DEVICE USB3 1-2

###### (2) USB Type-A connectors, female

###### USB 3.2 Gen 1 device ports for connector USB cameras, video capture devices, flash drives, keyboards, mice, hubs, or other USB devices

Specifier Note:

The USB extenders are engineered to deliver maximum compatibility with the widest possible range of USB hosts and devices. Crestron does not guarantee that all USB hosts or devices are compatible with the USB extenders. Certain USB hosts and devices may require additional driver installation if not directly supported by the operating system of the computer.

###### Backward compatibility with USB 2.0 and USB 1.1

###### Available power: 1 A per USB port

##### TO DEVICE USB 3 3

###### (1) USB Type-A connector, female

###### USB 3.2 Gen 1 device port for connection to USB cameras, video capture devices, flash drives, keyboards, mice hubs, or other USB devices

###### Backward compatibility with USB 2.0 and USB 1.1

###### Available Power: 1.5 A

##### TO DEVICE USB3 4

###### (1) USB Type-C connector, female

###### USB 3.2 Gen 1 device port for connection to USB cameras, video capture devices, flash drives, keyboards, mice, hubs, or other USB devices

###### Backward compatibility with USB 2.0 and USB 1.1

###### Available Power: 1.5 A

##### LINK

###### (1) 8-pin RJ45 connector, female

###### Connects directly to LINK port on Remote endpoint by same manufacturer

Specifier Note:

The LINK ports of the local and the remote endpoints must be connected to each other. Do not connect the LINK ports to any other device or to a network.

###### Up to 330 ft (100 m) over CAT6a/7 cable

Specifier Note:

For the LINK connection between the local and remote extenders, use CAT6a/7 UTP (unshielded twisted pair), FTP (foiled twisted pair), or STP (shielded twisted pair) as determined by the specific installation requirements. The maximum cable length is 330 ft (100 m). If using on-premise cabling, up to 33 ft (10 m) of CAT6a/7 patch cord can be used for each extender, ensuring that the total cable length does not exceed 330 ft (100 m). The remaining 264 ft (80 m) distance must consist of solid-core premise cabling.

###### If powered locally, supplies power to connected LOCAL endpoint

###### If not powered locally, receives power from connected LOCAL endpoint

##### COM

###### (1) 4-pin 3.5 mm detachable terminal block

###### Bidirectional RS-232 port

###### Up to 115.2k baud

###### Fourth pin (labeled NC) not used

##### SERVICE: For factory use only

#### Indicators

##### PWR: (1) Green LED, indicates that power is being supplied from the included power pack or via the LINK connection

##### LINK: (1) Green LED, indicates that a link is established between the local and remote endpoints

##### USB

###### (1) Green LED

###### Solid indicates that only USB 3 (or greater) devices are connected and the link is active

###### Flashing green indicates that one or more USB 2 (or lower) devices are connected and the link is active

###### Off indicates that no USB devices are connected

#### Power

##### Power Pack

###### Input: 100-240 VAC, 50/60 Hz

###### Output: 3.0 A @ 20 VDC

###### Power Consumption

31.28 W total between LOCAL and REMOTE

Specifier Note:

When a remote endpoint receives power over the LINK connection from a local endpoint, the total USB power available to connected peripherals will be less than 17.5 W (3.5 A).

### Compliance

#### Regulatory Model: M202444001

#### FCC Part 15 Class B, IC Class B, CE, Intertek® Listed for US and Canada

## USB 3.2 Extender over CAT 6a/7 Cable, Local and Remote

Specifier Note:

The Crestron USB-EXT-3S SYSTEM offers a versatile USB 3.2 extender to support USB peripherals such as UC soundbars and cameras. A standard USB 3.2 cable has a maximum length of 6 ft (1.8 m), which is not adequate for most integrated solutions used in conference rooms and other meeting spaces. The USB-EXT-3S SYSTEM provides a solution for the simultaneous extension of USB signals up to 330 ft (100 m) over CAT 6a/7 cable at speeds of up to 5 Gbps.

### Basis of Design

#### Crestron USB-EXT-3S SYSTEM

Specifier Note:

USB-EXT-3S SYSTEM
https://www.crestron.com/Products/Catalog/Audio-Video-and-USB/USB/Extender-Kit/USB-EXT-3S-SYSTEM

### Device Definition

#### USB point-to-point extension solution with the following capabilities and features:

##### USB signal extension over CAT 6A/7 cable up to 330 ft (100 m)

##### USB 3.2 Gen 1 data rate up to 5 Gbps

##### USB Type-A and USB Type-C connectivity

##### Simultaneous USB 3.2, 2.0, and 1.1 signal support

##### Support for USB transfer types: control, interrupt, isochronous (ISO), and bulk

##### RS-232 serial pass-through

##### Plug-and-play functionality without installation of additional drivers

##### Compatibility with Windows®, macOS®, Linux®, and Chrome OS™ operating systems

##### TAA compliance

##### Freestanding or surface mountable form factor

##### 20 VDC power pack included

### Device Architecture

#### Environmental

##### Operating Temperature: 32° to 122°F (0° to 50° C)

##### Storage Temperature: -4° to 158°F (-20° to 70° C)

##### Humidity: 10% to 90% RH (non-condensing)

#### Construction

##### Chassis: Metal, black finish, vented sides

##### Mounting: Freestanding or surface mountable

#### Dimensions

##### Height: 0.89 in. (23 mm), local and remote endpoints

##### Width: 5.12 in. (130 mm), local and remote endpoints

##### Depth: 4.90 in. (125 mm), local and remote endpoints

#### Weight: 12.32 oz (350 g) local, 12 oz (340 g) remote

### Functions

#### Communications

##### LINK: Proprietary link for point-to-point connection between endpoints

##### USB Speeds

###### USB 3.2 Gen 1 up to 5 Gbps

###### USB 2.0 up to 480 Mbps

###### USB 1.1 up to 12 Mbps

##### RS-232: Bidirectional serial pass-through

#### Connectors – Local Endpoint

##### 20 V 3.0 A: (1) Locking DC power connector; 20 VDC power input

Specifier Note:

Only one endpoint should be connected to a local power supply. Either the local or remote endpoint can be powered locally. The other endpoint will receive power over the LINK connection. When the remote endpoint receives power over the LINK connection from the local endpoint, the total USB current available to connected peripherals will be less than 3.5 A.

##### TO HOST USB 3

###### (1) USB Type-C® connector, female

###### USB 3.2 Gen 1 host port for connection to computer, media server, game console, or other USB host

Specifier Note:

For the LINK connection between the local and remote extenders, use CAT6a/7 UTP (unshielded twisted pair), FTP (foiled twisted pair), or STP (shielded twisted pair) as determined by the specific installation requirements. The maximum cable length is 330 ft (100 m). If using on-premise cabling, up to 33 ft (10 m) of CAT6a/7 patch cord can be used for each extender, ensuring that the total cable length does not exceed 330 ft (100 m). The remaining 264 ft (80 m) distance must consist of solid-core premise cabling.

###### Backward compatibility with USB 2.0 and USB 1.1

##### LINK

###### (1) 8-pin RJ45 connector, female

###### Connects directly to LINK port on Remote endpoint by same manufacturer

Specifier Note:

The LINK ports of the local and the remote endpoints must be connected to each other. Do not connect the LINK ports to any other device or to a network.

###### Up to 330 ft (100 m) over CAT6a/7 cable

###### If powered locally, supplies power to connected REMOTE endpoint

###### If not powered locally, receives power from connected REMOTE endpoint

##### COM

###### (1) 4-pin 3.5 mm detachable terminal block

###### Bidirectional RS-232 port up to 115.2k baud

###### Fourth pin (labeled NC) not used

##### SERVICE: For factory use only

#### Connectors – Remote Endpoint

##### 20 V 3.0 A: (1) locking DC power connector; 20 VDC power input

##### TO DEVICE USB3 1-2

###### (2) USB Type-A connectors, female

###### USB 3.2 Gen 1 device ports for connector USB cameras, video capture devices, flash drives, keyboards, mice, hubs, or other USB devices

Specifier Note:

The USB extenders are engineered to deliver maximum compatibility with the widest possible range of USB hosts and devices. Crestron does not guarantee that all USB hosts or devices are compatible with the USB extenders. Certain USB hosts and devices may require additional driver installation if not directly supported by the operating system of the computer.

###### Backward compatibility with USB 2.0 and USB 1.1

###### Available power: 1 A per USB port

##### TO DEVICE USB 3 3

###### (1) USB Type-A connector, female

###### USB 3.2 Gen 1 device port for connection to USB cameras, video capture devices, flash drives, keyboards, mice hubs, or other USB devices

###### Backward compatibility with USB 2.0 and USB 1.1

###### Available Power: 1.5 A

##### TO DEVICE USB3 4

###### (1) USB Type-C connector, female

###### USB 3.2 Gen 1 device port for connection to USB cameras, video capture devices, flash drives, keyboards, mice, hubs, or other USB devices

###### Backward compatibility with USB 2.0 and USB 1.1

###### Available Power: 1.5 A

##### LINK

###### (1) 8-pin RJ45 connector, female

###### Connects directly to LINK port on Remote endpoint by same manufacturer

###### Up to 330 ft (100 m) over CAT6a/7 cable

###### If powered locally, supplies power to connected LOCAL endpoint

###### If not powered locally, receives power from connected LOCAL endpoint

##### COM

###### (1) 4-pin 3.5 mm detachable terminal block

###### Bidirectional RS-232 port

###### Up to 115.2k baud

###### Fourth pin (labeled NC) not used

##### SERVICE: For factory use only

#### Indicators

##### PWR: (1) Green LED, indicates that power is being supplied from the included power pack or via the LINK connection

##### LINK: (1) Green LED, indicates that a link is established between the local and remote endpoints

##### USB

###### (1) Green LED

###### Solid indicates that only USB 3 (or greater) devices are connected and the link is active

###### Flashing green indicates that one or more USB 2 (or lower) devices are connected and the link is active

###### Off indicates that no USB devices are connected

#### Power

##### Power Pack

###### Input: 100-240 VAC, 50/60 Hz

###### Output: 3.0 A @ 20 VDC

###### Power Consumption

31.28 W total between LOCAL and REMOTE

25 W allocated to 4 USB devices

Specifier Note:

When a remote endpoint receives power over the LINK connection from a local endpoint, the total USB power available to connected peripherals will be less than 17.5 W (3.5 A).

### Compliance

#### Regulatory Model: M202444001

#### FCC Part 15 Class B, IC Class B, CE, Intertek® Listed for US and Canada

## USB 3.2 Extender over CAT6A/7 Cable, Local, 1-Gang

Specifier Note:

The USB-EXT-3S-LOCAL-1G is a USB 3.2 Gen 1 extender wall plate with connectivity for a single USB host such as a computer. The extender supports plug-and-play functionality with hosts running Windows®, macOS®, Linux®, or Chrome OS™ operating systems. No additional driver installation is required.

### Basis of Design

#### Crestron USB-EXT-3S-LOCAL-1G

Specifier Note:

USB-EXT-3S-LOCAL-1G
https://www.crestron.com/Products/Catalog/Audio-Video-and-USB/USB/Extender/USB-EXT-3S-LOCAL-1G

### Device Definition

#### USB extension local endpoint with the following capabilities and features:

##### USB signal extension over CAT 6A/7 cable up to 330 ft (100 m)

##### USB 3.2 Gen 1 data rate up to 5 Gbps

##### USB Type-A and USB Type-C connectivity

##### Simultaneous USB 3.2, 2.0, and 1.1 signal support

##### Support for USB transfer types: control, interrupt, isochronous (ISO), and bulk

##### RS-232 serial pass-through

##### Plug-and-play functionality without installation of additional drivers

##### Compatibility with Windows®, macOS®, Linux®, and Chrome OS™ operating systems

##### TAA compliance

##### Mountable in a standard US electrical box

##### Swappable front panels

##### Compatibility with decorator style faceplates by same manufacturer

##### 20 VDC power pack compatibility

### Device Architecture

#### Environmental

##### Operating Temperature: 32° to 122°F (0° to 50° C)

##### Storage Temperature: -4° to 158°F (-20° to 70° C)

##### Humidity: 10% to 90% RH (non-condensing)

#### Construction

##### Chassis: Metal housing with detachable white and black plastic face plates included

##### Mounting

###### Mounts in an unobstructed North American 1-gang electrical box

###### Minimum mounting depth of 3.5 in. (89 mm) required

###### Front faceplate by same manufacturer required

#### Dimensions

##### Height: 4.16 in. (106 mm)

##### Width: 1.77 in. (45 mm)

##### Depth: 2.36 in. (60 mm)

#### Weight: 6.61 oz (187 g)

### Functions

#### Communications

##### LINK: Proprietary link for point-to-point connection between USB extension endpoints by same manufacturer

##### USB Speeds

###### USB 3.2 Gen 1 up to 5 Gbps

###### USB 2.0 up to 480 Mbps

###### USB 1.1 up to 12 Mbps

##### RS-232: Bidirectional serial pass-through

#### Connectors

###### 20 V 3.0 A

(1) 2-pin DC power connector

20 VDC power input

Specifier Note:

If a USB-EXT-3S-LOCAL-1G is connected to a USB-EXT-3S-REMOTE-1G, both devices must be connected to a local power supply. If a wall plate endpoint is connected to a room box endpoint (USB-EXT-3S-LOCAL or USB-EXT-3S-REMOTE), it can receive power over the LINK connection. When a remote endpoint receives power over the LINK connection from a local endpoint, the total USB current available to connected peripherals will be less than 3.5 A.

###### TO HOST USB3

(1) USB Type-C® connector, female

USB 3.2 Gen 1 host port for connection to computer, media server, game console, or other USB host

Specifier Note:

For the LINK connection between the local and remote extenders, use CAT6A/7 UTP (unshielded twisted pair), FTP (foiled twisted pair), or STP (shielded twisted pair) as determined by the specific installation requirements. The maximum cable length is 330 ft (100 m).

Backward compatibility with USB 2.0 and USB 1.1

###### LINK

(1) 8-pin RJ45 connector, female

Connects directly to LINK port on remote endpoint by same manufacturer

Specifier Note:

The LINK ports of the local and the remote endpoints must be connected to each other. Do not connect the LINK ports to any other device or to a network.

Up to 330 ft (100 m) over CAT6a/7 cable

If not powered locally, receives power from connected REMOTE endpoint

###### COM

(1) 4-pin 3.5 mm detachable terminal block

Bidirectional RS-232 port

Up to 115.2k baud

Fourth pin (labeled NC) not used

###### SERVICE: For factory use only

#### Indicators

##### PWR: (1) Green LED, indicates that power is being supplied from the included power pack or via the LINK connection

##### LINK: (1) Green LED, indicates that a link is established between the local and remote endpoints

##### USB

###### (1) Green LED

###### Solid indicates that only USB 3 (or greater) devices are connected and the link is active

###### Flashing green indicates that one or more USB 2 (or lower) devices are connected and the link is active

###### Off indicates that no USB devices are connected

#### Power

##### Power Pack

###### Input: 100-240 VAC, 50/60 Hz

###### Output: 3.0 A @ 20 VDC

##### Power Consumption: 31.28 W total between local and remote endpoints

Specifier Note:

When a remote endpoint receives power over the LINK connection from a local endpoint, the total USB power available to connected peripherals will be less than 17.5 W (3.5 A).

### Compliance

#### Regulatory Model: M202444002

#### FCC Part 15 Class B, IC Class B, CE, Intertek® Listed for US and Canada

## USB 3.2 Extender over CAT6A/7 Cable, Remote, 1-Gang

Specifier Note:

The USB-EXT-3S-REMOTE-1G is a USB 3.2 Gen 1 extender wall plate with connectivity for up to four USB devices such as cameras or video capture devices at one location. The extender is expandable to support additional USB devices with the connection of USB hubs (not included).

### Basis of Design

#### Crestron USB-EXT-3S-REMOTE-1G

Specifier Note:

USB-EXT-3S-REMOTE-1G
https://www.crestron.com/Products/Catalog/Audio-Video-and-USB/USB/Extender/USB-EXT-3S-REMOTE-1G

### Device Definition

#### USB extension local endpoint with the following capabilities and features:

##### USB signal extension over CAT 6A/7 cable up to 330 ft (100 m)

##### USB 3.2 Gen 1 data rate up to 5 Gbps

##### USB Type-A and USB Type-C connectivity

##### Simultaneous USB 3.2, 2.0, and 1.1 signal support

##### Support for USB transfer types: control, interrupt, isochronous (ISO), and bulk

##### RS-232 serial pass-through

##### Plug-and-play functionality without installation of additional drivers

##### Compatibility with Windows®, macOS®, Linux®, and Chrome OS™ operating systems

##### TAA compliance

##### Mountable in a standard US electrical box

##### Swappable front panels

##### Compatibility with decorator style faceplates by same manufacturer

##### 20 VDC power pack compatibility

### Device Architecture

#### Environmental

##### Operating Temperature: 32° to 122°F (0° to 50° C)

##### Storage Temperature: -4° to 158°F (-20° to 70° C)

##### Humidity: 10% to 90% RH (non-condensing)

#### Construction

##### Chassis: Metal housing with detachable white and black plastic face plates included

##### Mounting

###### Mounts in an unobstructed North American 1-gang electrical box

###### Minimum mounting depth of 3.5 in. (89 mm) required

###### Front faceplate by same manufacturer required

#### Dimensions

##### Height: 4.16 in. (106 mm)

##### Width: 1.77 in. (45 mm)

##### Depth: 2.36 in. (60 mm)

#### Weight: 8.29 oz (235 g)

### Functions

#### Communications

##### LINK: Proprietary link for point-to-point connection between USB extension endpoints by same manufacturer

##### USB Speeds

###### USB 3.2 Gen 1 up to 5 Gbps

###### USB 2.0 up to 480 Mbps

###### USB 1.1 up to 12 Mbps

##### RS-232: Bidirectional serial pass-through

#### Connectors

###### 20 V 3.0 A

(1) 2-pin DC power connector

20 VDC power input

Specifier Note:

If a USB-EXT-3S-LOCAL-1G is connected to a USB-EXT-3S-REMOTE-1G, both devices must be connected to a local power supply. If a wall plate endpoint is connected to a room box endpoint (USB-EXT-3S-LOCAL or USB-EXT-3S-REMOTE), it can receive power over the LINK connection. When a remote endpoint receives power over the LINK connection from a local endpoint, the total USB current available to connected peripherals will be less than 3.5 A.

###### TO DEVICE USB3 1-3

(3) USB Type-A connectors, female

USB 3.2 Gen 1 device ports for connection to USB cameras, video capture devices, flash drives, keyboards, mice, hubs, or other USB devices

Specifier Note:

The USB extenders are engineered to deliver maximum compatibility with the widest possible range of USB hosts and devices. Crestron does not guarantee that all USB hosts or devices are compatible with the USB extenders. Certain USB hosts and devices may require additional driver installation if not directly supported by the operating system of the computer.

Backward compatibility with USB 2.0 and USB 1.1

Available Power: 1 A per USB port

###### TO DEVICE USB3 4

(1) USB Type-C connector, female

USB 3.2 Gen 1 device port for connection to USB cameras, video capture devices, flash drives, keyboards, mice, hubs, or other USB devices

Backward compatibility with USB 2.0 and USB 1.1

Available Power: 1.5 A

###### LINK

(1) 8-pin RJ45 connector, female

Connects directly to LINK port on remote endpoint by same manufacturer

Specifier Note:

The LINK ports of the local and the remote endpoints must be connected to each other. Do not connect the LINK ports to any other device or to a network.

Up to 330 ft (100 m) over CAT6a/7 cable

Specifier Note:

For the LINK connection between the local and remote extenders, use CAT6a/7 UTP (unshielded twisted pair), FTP (foiled twisted pair), or STP (shielded twisted pair) as determined by the specific installation requirements. The maximum cable length is 330 ft (100 m).

If not powered locally, receives power from connected LOCAL endpoint

###### COM

(1) 4-pin 3.5 mm detachable terminal block

Bidirectional RS-232 port

Up to 115.2k baud

Fourth pin (labeled NC) not used

###### SERVICE: For factory use only

#### Indicators

##### PWR: (1) Green LED, indicates that power is being supplied from the included power pack or via the LINK connection

##### LINK: (1) Green LED, indicates that a link is established between the local and remote endpoints

##### USB

###### (1) Green LED

###### Solid indicates that only USB 3 (or greater) devices are connected and the link is active

###### Flashing green indicates that one or more USB 2 (or lower) devices are connected and the link is active

###### Off indicates that no USB devices are connected

#### Power

##### Power Pack

###### Input: 100-240 VAC, 50/60 Hz

###### Output: 3.0 A @ 20 VDC

##### Power Consumption: 31.28 W total between local and remote endpoints

Specifier Note:

When a remote endpoint receives power over the LINK connection from a local endpoint, the total USB power available to connected peripherals will be less than 17.5 W (3.5 A).

### Compliance

#### Regulatory Model: M202444002

#### FCC Part 15 Class B, IC Class B, CE, Intertek® Listed for US and Canada

# EXECUTION

NOT USED in this Guide Specification.  Specifier shall Specify PART 3 On-Site work as needed.

# APPENDICES

## SPECIFIED PRODUCTS

Specifier Note: This Article includes Crestron products specified in this Guide Specification document.  This Article is for reference only and should not be required in actual project manual unless included in an overall system equipment list.

### Crestron USB-EXT-3S-LOCAL

### Crestron USB-EXT-3S-REMOTE

### Crestron USB-EXT-3S SYSTEM

### Crestron USB-EXT-3S-LOCAL-1G

### Crestron USB-EXT-3S-REMOTE-1G