SECTION 27 41 16

INTEGRATED AUDIO-VIDEO SYSTEMS AND EQUIPMENT

GUIDE SPECIFICATION

Specifier: The Specifier/Design Professional is responsible for the accuracy of all project specifications, including system application and coordination with related sections.  This guide specification is provided as a convenience and requires editing to match actual project requirements.  CRESTRON ELECTRONICS, INC. SHALL NOT BE LIABLE FOR ANY DAMAGES ARISING OUT OF THE USE OF ANY OF ITS GUIDE SPECIFICATIONS.  For Crestron design assistance and design review please contact Sales Support Services Department at   800.237.2041 or techsales@crestron.com.

Specifier:  Please see PART 4 for a listing of products specified in this Guide Specification.

Table of Contents

1	GENERAL	5

1.1	Description: Single cable 4K HDMI transmitter	5

2	PRODUCTS	5

2.1	Transmitter Type 1	5

2.1.1	Basis of Design	5

2.1.2	Device Architecture	5

2.1.3	Functions	5

2.1.4	Connectors	6

2.1.5	Power	7

2.1.6	Compliance	7

2.2	Transmitter Type 2	7

2.2.1	Basis of Design	7

2.2.2	Device Architecture	8

2.2.3	Functions	8

2.2.4	Connectors	8

2.2.5	Power	10

2.2.6	Compliance	10

2.3	Transmitter Type 3	10

2.3.1	Basis of Design	10

2.3.2	Device Architecture	10

2.3.3	Functions	11

2.3.4	Connectors	11

2.3.5	Power	12

2.3.6	Compliance	12

2.4	Transmitter Type 4	13

2.4.1	Basis of Design	13

2.4.2	Device Architecture	13

2.4.3	Functions	13

2.4.4	Connectors	13

2.4.5	Power	14

2.4.6	Compliance	15

2.5	Transmitter Type 5	15

2.5.1	Basis of Design	15

2.5.2	Device Architecture	15

2.5.3	Functions	15

2.5.4	Connectors	15

2.5.5	Power	17

2.5.6	Compliance	17

2.6	Transmitter Type 6	17

2.6.1	Basis of Design	17

2.6.2	Device Architecture	17

2.6.3	Functions	17

2.6.4	Connectors	18

2.6.5	Power	19

2.6.6	Compliance	19

2.7	Transmitter Type 7	19

2.7.1	Basis of Design	19

2.7.2	Device Architecture	19

2.7.3	Functions	20

2.7.4	Connectors	20

2.7.5	Power	21

2.7.6	Compliance	21

2.8	Transmitter Type 8	21

2.8.1	Basis of Design	21

2.8.2	Device Architecture	22

2.8.3	Functions	22

2.8.4	Connectors	22

2.8.5	Power	23

2.8.6	Compliance	23

2.9	Transmitter Type 9	23

2.9.1	Basis of Design	23

2.9.2	Device Architecture	23

2.9.3	Functions	24

2.9.4	Connectors	24

2.9.5	Power	25

2.9.6	Compliance	25

2.10	Transmitter Type 10	25

2.10.1	Basis of Design	25

2.10.2	Device Architecture	25

2.10.3	Functions	26

2.10.4	Connectors	26

2.10.5	Power	27

2.10.6	Compliance	28

2.11	Transmitter Type 11	28

2.11.1	Basis of Design	28

2.11.2	Device Architecture	28

2.11.3	Functions	28

2.11.4	Connectors	28

2.11.5	Power	30

2.11.6	Compliance	30

2.12	Transmitter Type 12	30

2.12.1	Basis of Design	30

2.12.2	Device Architecture	30

2.12.3	Functions	30

2.12.4	Connectors	31

2.12.5	Power	33

2.12.6	Compliance	33

2.13	Transmitter Type 13	33

2.13.1	Basis of Design	33

2.13.2	Device Architecture	33

2.13.3	Functions	33

2.13.4	Connectors	34

2.13.5	Power	35

2.13.6	Compliance	36

2.14	Transmitter Type 14	36

2.14.1	Basis of Design	36

2.14.2	Device Architecture	36

2.14.3	Functions	36

2.14.4	Connectors	37

2.14.5	Power	38

2.14.6	Compliance	38

3	EXECUTION	38

4	APPENDICES	38

4.1	SPECIFIED PRODUCTS	38

4.1.1	Crestron HD-TX-4KZ-401	38

4.1.2	Crestron HD-TX-4KZ-211-2G	38

4.1.3	Crestron HD-TX-4KZ-201	38

4.1.4	Crestron HD-TX-4KZ-101	38

4.1.5	Crestron HD-TXC-4KZ-101	38

4.1.6	Crestron HD-TXC-4KZ-101-1G	39

4.1.7	Crestron HD-TX-4KZ-111	39

4.1.8	Crestron HD-TX-4KZ-111-1G	39

4.1.9	Crestron HD-TX-4KZ-101-1G	39

4.1.10	Crestron HD-TXCA-4KZ-101	39

4.1.11	Crestron HD-TXA-4KZ-101	39

4.1.12	Crestron HD-TX-4KZ-211-CHGR	39

4.1.13	Crestron HD-TX-4KZ-421-CHGR	39

4.1.14	Crestron HD-TXA-4KZ-101-1G	39

# GENERAL

NOT USED in this Guide Specification.  Specifier shall Specify PART 1 administrative and procedural requirements as needed.

## Description: Single cable 4K HDMI transmitter

# PRODUCTS

## Transmitter Type 1

Specifier Note:

The HD‑TX‑4KZ‑401 is a DM Lite® 4K60 4:4:4 transmitter designed to interoperate with a DM Lite® receiver or DMPS Lite™ switcher for the transmission of an HDMI® signal.

### Basis of Design

#### Crestron HD-TX-4KZ-401

Specifier Note:

HD-TX-4KZ-401
https://www.crestron.com/Products/Video/HDMI-Solutions/HDMI-Extenders/HD-TX-4KZ-401

### Device Architecture

#### Physical Form factor

##### Device enclosure to be made of black finished metal with vented sides

##### Device weight: 1.54 lb. (0.70 kg)

#### Mounting Options

##### Freestanding

##### Surface mount via attachable mounting bracket

##### Attachment to single rack rail

#### Environmental Operating Conditions

##### 32° to 104° F (0° to 40° C)

##### 20% to 90% RH (non-condensing)

### Functions

#### The transmitter shall support input switching with the following options:

##### Automatic switching to active input

##### Manual switching via integrated selection buttons

#### Remote switching via control processor by same manufacturer when connected to receiver switching device by same manufacturer

This option requires a LAN connection at the compatible receiver device connected to a control processor by same manufacturer.

#### Single UTP/STP cable transmission

##### Signal transmission up to 230 feet.

##### Minimum cable type supported: UTP/STP CAT5e

### Controls and Indicators

#### The device shall include the following indicators:

##### One (1) Green LED indicating a valid link to device by same manufacturer

##### One (1) LED indicating that power is provided to device with the following modes:

###### Green - device is operational

###### Amber - device is booting

##### One (1) Green LED indicating reception of HDMI signal

##### Two (2) LEDs on RJ-45 connector with the following modes:

###### Green - Connection established to device by same manufacturer

###### Flashing Amber - non-HDCP video

###### Amber - HDCP video

### Connectors

#### The device shall include the following connectors:

##### Four (4) HDMI inputs with support for the following:

###### CEC Pass-through

###### HDMI with HDR10, HDR10+ Dolby Vision, Deep Color, and 4K60 4:4:4

###### HDCP 2.3 compliant

###### DisplayPort Dual-Mode

###### Dolby Digital, Dolby Digital EX, Dolby Digital Plus, Dolby TrueHD, Dolby Atmos DTS, DTS-ES, DTS 96/24, DTS HD High Res, DTS-HD Master Audio, DTS:X, LPCM up to 8 channels

###### DVI-D with adaptor

###### EDID

###### CEC Pass-through

###### Common resolutions: 1920x1080 30Hz to 4096x2160 DCI 4K 4:4:4 60Hz

###### Custom resolutions: pixel clock rates up to 600 MHz

##### One (1) RJ45 shielded female connector with support for the following:

###### Link port for connection to compatible transmission device by same manufacturer

###### Power sharing option for connected remote transmission device

##### One (1) 2.1 x 5.5 mm DC power connector supporting the following characteristic:

###### 24 VDC 2.5 A power input

##### One (1) USB Type A connector with support for:

###### Firmware loading and configuration management

###### USB host port for compatible control connection interface by same manufacturer

###### 5 VDC, 500 mA power to USB powered device

##### One (1) Chassis grounding lug

### Power

#### Power supply

##### Power supply shall support device and connected equipment

#### Power supply modes

##### Local or remote DC power source

###### Remote power supplied by opposite end of TX/RX pair.

### Compliance

#### UL Listed for US & Canada, CE, IC, FCC Part 15 Class B digital device

#### Regulatory Model M202047002

## Transmitter Type 2

Specifier Note:

The HD-TX-4KZ-211-2G is a DM Lite® 4K60 4:4:4 wall plate transmitter designed to interoperate with a DM Lite® receiver or DMPS Lite™ switcher. The HDMI® input of the HD-TX-4KZ-211-2G enables the transmission of an HDMI signal. The USB-C® input with DisplayPort™ Alt Mode capability enables the transmission of a DisplayPort AV signal.

### Basis of Design

#### Crestron HD-TX-4KZ-211-2G

Specifier Note:

HD-TX-4KZ-211-2G
https://www.crestron.com/Products/Video/HDMI-Solutions/HDMI-Extenders/HD-TX-4KZ-211-2G-B

### Device Architecture

#### Physical Form factor

##### Device enclosure and bracket to be composed of metal with black polycarbonate front label overlay

##### Device weight: 7.33 oz (208 g)

#### Mounting Option

##### Wall mount device, mounts in a 2-gang (or larger) 2 inch (51 mm) deep U.S. electrical box or plaster ring (not included)

#### Environmental Operating Conditions

##### 32° to 104° F (0° to 40° C)

##### 20% to 90% RH (non-condensing)

### Functions

#### The transmitter shall support input switching with the following options:

##### Automatic switching to active input

##### Manual switching via integrated selection button

##### Remote switching via control processor by same manufacturer when connected to receiver switching device by same manufacturer

This option requires a LAN connection at the compatible receiver device connected to a control processor by same manufacturer.

#### Single UTP/STP cable transmission

##### Signal transmission up to 230 feet.

##### Minimum cable type supported: UTP/STP CAT5e

### Controls and Indicators

#### The device shall include the following indicators:

##### One (1) LED indicating that power is provided to device with the following modes:

###### Green - device is operational

###### Amber - device is booting

##### One (1) Pushbutton for enabling or disabling automatic switching of inputs

##### One (1) Green LED indicating automatic switching of inputs is enabled

##### One (1) Pushbutton for Ethernet setup (applicable only when device is used in conjunction with switcher from same manufacturer)

##### One (1) Red LED indicating that control described in 2.2.4.1.4 has been pressed

##### One (1) Green LED indicating a valid link to device by same manufacturer

##### One (1) Pushbutton for manual input selection

##### One (1) LED indicating HDMI status with the following modes:

###### Green - HDMI input is routed to an output

###### Amber - Source is detected but not routed to an output

##### One (1) LED indicating USB-C DisplayPort status with the following modes:

###### Green - USB-C DisplayPort input is routed to an output

###### Amber - Source is detected but not routed to an output

##### One (1) Pushbutton for restoring the device to factory default settings

##### Two (2) LEDs on RJ-45 connector with the following modes:

###### Green - Connection established to device by same manufacturer

###### Flashing Amber - non-HDCP video

###### Amber - HDCP video

### Connectors

#### The device shall include the following connectors:

##### One (1) HDMI input with support for the following:

###### CEC Pass-through

###### HDMI with HDR10, HDR10+ Dolby Vision, Deep Color, and 4K60 4:4:4

###### HDCP 2.3 compliant

###### DisplayPort Dual-Mode

###### Dolby Digital, Dolby Digital EX, Dolby Digital Plus, Dolby TrueHD, Dolby Atmos DTS, DTS-ES, DTS 96/24, DTS HD High Res, DTS-HD Master Audio, DTS:X, LPCM up to 8 channels

###### DVI-D with adaptor

###### EDID

###### Common resolutions: 1920x1080 30Hz to 4096x2160 DCI 4K 4:4:4 60Hz

###### Custom resolutions: pixel clock rates up to 600 MHz

##### One (1) USB-C connector for Digital video / audio input with support for the following:

###### DisplayPort Alt Mode with HDR10, HDR10+, Dolby Vision, Deep Color

###### HDCP 2.3 compliant

###### CEC Pass-through

###### EDID

###### Dolby Digital, Dolby Digital EX, Dolby Digital Plus, Dolby True HD, Dolby Atmos DTS, DTS-ES, DTS 96/24, DTS HD High Res, DTS HD Master Audio, DTS:X, LPCM up to 8 channels

###### Common resolutions: 1920x1080 30Hz to 4096x2160 DCI 4K 4:4:4 60Hz

###### Custom resolutions: pixel clock rates up to 600 MHz

##### One (1) RJ45 shielded female connector with support for the following:

###### Link port for connection to compatible transmission device by same manufacturer

###### Power sharing option for connected remote transmission device

##### One (1) 2-pin 3.5 mm detachable terminal block supporting the following characteristic:

###### 24 VDC 0.7 A power input

##### One (1) Chassis grounding lug

### Power

#### Power supply

##### Power supply shall support device and connected equipment

#### Power supply modes

##### Local or remote DC power source

###### Remote power supplied by opposite end of TX/RX pair.

### Compliance

#### CE, IC, FCC Part 15 Class B digital device

#### Regulatory Model M202046004

## Transmitter Type 3

Specifier Note:

The HD‑TX‑4KZ‑201 is a DM Lite® 4K60 4:4:4 transmitter and 2x1 HDMI Auto-Switcher designed to interoperate with a DM Lite® receiver or DMPS Lite™ switcher for the transmission of an HDMI® signal.

### Basis of Design

#### Crestron HD-TX-4KZ-201

Specifier Note:

HD-TX-4KZ-201
https://www.crestron.com/Products/Video/HDMI-Solutions/HDMI-Extenders/HD-TX-4KZ-201

### Device Architecture

#### Physical Form factor

##### Device enclosure to be composed of black finish metal with vented top, front, rear, and sides

##### Device weight: 1.54 lb. (0.70 kg)

#### Mounting Options

##### Freestanding

##### Surface mount via attachable mounting bracket

##### Attachment to single rack rail

#### Environmental Operating Conditions

##### 32° to 104° F (0° to 40° C)

##### 20% to 90% RH (non-condensing)

### Functions

#### The transmitter shall support input switching with the following options:

##### Automatic switching to active input

##### Manual switching via integrated selection buttons

##### Remote switching via control processor by same manufacturer when connected to receiver switching device by same manufacturer

This option requires a LAN connection at the compatible receiver device connected to a control processor by same manufacturer.

#### Single UTP/STP cable transmission

##### Signal transmission up to 230 feet.

##### Minimum cable type supported: UTP/STP CAT5e

### Controls and Indicators

#### The device shall include the following indicators:

##### One (1) LED indicating that power is provided to device with the following modes:

###### Green - device is operational

###### Amber - device is booting

##### One (1) Pushbutton for enabling or disabling automatic switching of inputs

##### One (1) Green LED indicating automatic switching of inputs is enabled

##### One (1) Pushbutton for Ethernet setup (applicable only when device is used in conjunction with switcher from same manufacturer)

##### One (1) Red LED indicating that control described in 2.3.4.1.4 has been pressed

##### One (1) Green LED indicating a valid link to device by same manufacturer

##### Two (2) Pushbuttons for manual input selection

##### Two (2) LEDs indicating HDMI status with the following modes:

###### Green - HDMI input is routed to an output

###### Amber - Source is detected but not routed to an output

##### Two (2) LEDs on RJ-45 connector with the following modes:

###### Green - Connection established to device by same manufacturer

###### Flashing Amber - non-HDCP video

###### Amber - HDCP video

### Connectors

#### The device shall include the following connectors:

##### Two (2) HDMI inputs with support for the following:

###### CEC Pass-through

###### HDMI with HDR10, HDR10+ Dolby Vision, Deep Color, and 4K60 4:4:4

###### HDCP 2.3 compliant

###### DisplayPort Dual-Mode

###### Dolby Digital, Dolby Digital EX, Dolby Digital Plus, Dolby TrueHD, Dolby Atmos DTS, DTS-ES, DTS 96/24, DTS HD High Res, DTS-HD Master Audio, DTS:X, LPCM up to 8 channels

###### DVI-D with adaptor

###### EDID

###### Common resolutions: 1920x1080 30Hz to 4096x2160 DCI 4K 4:4:4 60Hz

###### Custom resolutions: pixel clock rates up to 600 MHz

##### One (1) RJ45 shielded female connector with support for the following:

###### Link port for connection to compatible transmission device by same manufacturer

###### Power sharing option for connected remote transmission device

##### One (1) 2.1 x 5.5 mm DC power connector supporting the following characteristic:

###### 24 VDC 2.5 A power input

##### One (1) Chassis grounding lug

### Power

#### Power supply

##### Power supply shall support device and connected equipment

#### Power supply modes

##### Local or remote DC power source

###### Remote power supplied by opposite end of TX/RX pair.

### Compliance

#### UL Listed for US & Canada, CE, IC, FCC Part 15 Class B digital device

#### Regulatory Model M202046002

## Transmitter Type 4

Specifier Note:

The HD‑TX‑4KZ‑101 is a DM Lite® 4K60 4:4:4 transmitter designed to interoperate with a DM Lite® receiver or DMPS Lite™ switcher for the transmission of an HDMI® signal.

### Basis of Design

#### Crestron HD-TX-4KZ-101

Specifier Note:

HD-TX-4KZ-101
https://www.crestron.com/Products/Video/HDMI-Solutions/HDMI-Extenders/HD-TX-4KZ-101

### Device Architecture

#### Physical Form factor

##### Device enclosure to be composed of black finish metal with vented sides

##### Device weight: 11.92 oz (338 g)

#### Mounting Options

##### Freestanding

##### Surface mount via attachable mounting bracket

##### Attachment to single rack rail

#### Environmental Operating Conditions

##### 32° to 104° F (0° to 40° C)

##### 20% to 90% RH (non-condensing)

### Functions

#### Single UTP/STP cable transmission

##### Signal transmission up to 230 feet.

##### Minimum cable type supported: UTP/STP CAT5e

### Controls and Indicators

#### The device shall include the following indicators:

##### One (1) Green LED indicating a valid link to device by same manufacturer

##### One (1) LED indicating that power is provided to device with the following modes:

###### Green - device is operational

###### Amber - device is booting

##### One (1) Green LED indicating device is receiving HDMI signal

##### Two (2) LEDs on RJ-45 connector with the following modes:

###### Green - Connection established to device by same manufacturer

###### Flashing Amber - non-HDCP video

###### Amber - HDCP video

### Connectors

#### The device shall include the following connectors:

##### One (1) HDMI input with support for the following:

###### CEC Pass-through

###### HDMI with Deep Color and 4K60 4:4:4

###### HDCP 2.3 compliant

###### DisplayPort Dual-Mode

###### Dolby Digital, Dolby Digital EX, Dolby Digital Plus, Dolby True HD, Dolby Atmos DTS, DTS-ES, DTS 96/24, DTS HD High Res, DTS HD Master Audio, DTS:X, LPCM up to 8 channels

###### DVI-D with adaptor

###### EDID

###### Common resolutions: 1920x1080 30Hz to 4096x2160 DCI 4K 4:4:4 60Hz

###### Custom resolutions: pixel clock rates up to 600 MHz

##### One (1) RJ45 shielded female connector with support for the following:

###### Link port for connection to compatible transmission device by same manufacturer

###### Power sharing option for connected remote transmission device

##### One (1) 2.1 x 5.5 mm DC power connector supporting the following characteristic:

###### 24 VDC 1.25 A power input

### Power

#### Power supply

##### Power supply shall support device and connected equipment

#### Power supply modes

##### Local or remote DC power source

###### Remote power supplied by opposite end of TX/RX pair.

### Compliance

#### UL Listed for US & Canada, CE, IC, FCC Part 15 Class B digital device

#### Regulatory Model M202047002

## Transmitter Type 5

Specifier Note:

The HD‑TXC‑4KZ‑101 is a DM Lite® 4K60 4:4:4 transmitter designed to interoperate with a DM Lite® receiver or DMPS Lite™ switcher for the transmission of HDMI®, RS-232, and IR signals.

### Basis of Design

#### Crestron HD-TXC-4KZ-101

Specifier Note:

HD-TXC-4KZ-101 
https://www.crestron.com/Products/Video/HDMI-Solutions/HDMI-Extenders/HD-TXC-4KZ-101

### Device Architecture

#### Physical Form factor

##### Device enclosure to be composed of black finish metal with vented sides

##### Device weight: 11.92 oz (338 g)

#### Mounting Options

##### Freestanding

##### Surface mount via attachable mounting bracket

##### Attachment to single rack rail

#### Environmental Operating Conditions

##### 32° to 104° F (0° to 40° C)

##### 20% to 90% RH (non-condensing)

### Functions

#### Single UTP/STP cable transmission

##### Signal transmission up to 230 feet.

##### Minimum cable type supported: UTP/STP CAT5e

### Controls and Indicators

#### The device shall include the following indicators:

##### One (1) Green LED indicating a valid link to device by same manufacturer

##### One (1) LED indicating that power is provided to device with the following modes:

###### Green - device is operational

###### Amber - device is booting

##### One (1) Green LED indicating device is receiving HDMI signal

##### Two (2) LEDs on RJ-45 connector with the following modes:

###### Green - Connection established to device by same manufacturer

###### Flashing Amber - non-HDCP video

###### Amber - HDCP video

### Connectors

#### The device shall include the following connectors:

##### One (1) HDMI input with support for the following:

###### CEC Pass-through

###### HDMI with HDR10, HDR10+, Dolby Vision, Deep Color and 4K60 4:4:4

###### HDCP 2.3 compliant

###### DisplayPort Dual-Mode

###### Dolby Digital, Dolby Digital EX, Dolby Digital Plus, Dolby True HD, Dolby Atmos DTS, DTS-ES, DTS 96/24, DTS HD High Res, DTS HD Master Audio, DTS:X, LPCM up to 8 channels

###### DVI-D with adaptor

###### EDID

###### Common resolutions: 1920x1080 30Hz to 4096x2160 DCI 4K60 4:4:4

###### Custom resolutions: pixel clock rates up to 600 MHz

##### One (1) RJ45 shielded female connector with support for the following:

###### Link port for connection to compatible transmission device by same manufacturer

###### Power sharing option for connected remote transmission device

##### One (1) 2.1 x 5.5 mm DC power connector supporting the following characteristic:

###### 24 VDC 1.25 A power input

##### One (1) 4-pin 3.5 mm detachable terminal block IR input/output control port supporting the following characteristics:

###### IR input connects to IR signal source and transmits signal to IR out port on receiver device from same manufacturer

###### IR output connects to IR emitter and transmits IR signal from source connected to receiver device from same manufacturer

###### IR up to 60 kHz

##### One (1) 3-pin 3.5 mm detachable terminal block Serial control port supporting the following characteristics:

###### Bidirectional RS-232 up to 115.2k baud

###### Passes RS-232 TD/RD data to/from RS-232 port on receiver device from same manufacturer

### Power

#### Power supply

##### Power supply shall support device and connected equipment

#### Power supply modes

##### Local or remote DC power source

###### Remote power supplied by opposite end of TX/RX pair.

### Compliance

#### UL Listed for US & Canada, CE, IC, FCC Part 15 Class B digital device

#### Regulatory Model M202047002

## Transmitter Type 6

Specifier Note:

The HD‑TXC‑4KZ‑101‑1G is a DM Lite® 4K60 4:4:4 wall plate transmitter designed to interoperate with a DM Lite® receiver or DMPS Lite™ switcher for the transmission of HDMI®, RS-232, and IR signals.

### Basis of Design

#### Crestron HD-TXC-4KZ-101-1G

Specifier Note:

HD-TXC-4KZ-101-1G 
https://www.crestron.com/Products/Video/HDMI-Solutions/HDMI-Extenders/HD-TXC-4KZ-101-1G-B

### Device Architecture

#### Physical Form factor

##### Device enclosure and bracket to be composed of metal with black polycarbonate front label overlay

##### Device enclosure to be composed of black finish metal with vented sides

##### Device weight: 5.54 oz (157 g)

#### Mounting Option

##### Wall mount device, mounts in a 1-gang (or larger) 2-1/4 inch (57 mm) deep U.S. electrical box or plaster ring (not included)

#### Environmental Operating Conditions

##### 32° to 104° F (0° to 40° C)

##### 20% to 90% RH (non-condensing)

### Functions

#### Single UTP/STP cable transmission

##### Signal transmission up to 230 feet.

##### Minimum cable type supported: UTP/STP CAT5e

### Controls and Indicators

#### The device shall include the following indicators:

##### One (1) Green LED indicating a valid link to device by same manufacturer

##### One (1) LED indicating that power is provided to device with the following modes:

###### Green - device is operational

###### Amber - device is booting

##### One (1) Green LED indicating device is receiving HDMI signal

##### Two (2) LEDs on RJ-45 connector with the following modes:

###### Green - Connection established to device by same manufacturer

###### Flashing Amber - non-HDCP video

###### Amber - HDCP video

### Connectors

#### The device shall include the following connectors:

##### One (1) HDMI input with support for the following:

###### CEC Pass-through

###### HDMI with HDR10, HDR10+, Dolby Vision, Deep Color, and 4K60 4:4:4

###### HDCP 2.3 compliant

###### DisplayPort Dual-Mode

###### Dolby Digital, Dolby Digital EX, Dolby Digital Plus, Dolby True HD, Dolby Atmos DTS, DTS-ES, DTS 96/24, DTS HD High Res, DTS HD Master Audio, DTS:X, LPCM up to 8 channels

###### DVI-D with adaptor

###### EDID

###### Common resolutions: 1920x1080 30Hz to 4096x2160 DCI 4K 4:4:4 60Hz

###### Custom resolutions: pixel clock rates up to 600 MHz

##### One (1) RJ45 shielded female connector with support for the following:

###### Link port for connection to compatible transmission device by same manufacturer

###### Power sharing option for connected remote transmission device

##### One (1) 2-pin 3.5 mm detachable terminal block supporting the following characteristic:

###### 24 VDC 0.75 A power input

##### One (1) 4-pin 3.5 mm detachable terminal block IR input/output control port supporting the following characteristics:

###### IR input connects to IR signal source and transmits signal to IR out port on receiver device from same manufacturer

###### IR output connects to IR emitter and transmits IR signal from source connected to receiver device from same manufacturer

###### IR up to 60 kHz

##### One (1) 3-pin 3.5 mm detachable terminal block Serial control port supporting the following characteristics:

###### Bidirectional RS-232 up to 115.2k baud

###### Passes RS-232 TD/RD data to/from RS-232 port on receiver device from same manufacturer

##### One (1) Chassis grounding lug

### Power

#### Power supply

##### Power supply shall support device and connected equipment

#### Power supply modes

##### Local or remote DC power source

###### Remote power supplied by opposite end of TX/RX pair.

### Compliance

#### UL Listed for US & Canada, CE, IC, FCC Part 15 Class B digital device

#### Regulatory Model M202045001

## Transmitter Type 7

Specifier Note:

The HD‑TX‑4KZ‑111 is a DM Lite® 4K60 4:4:4 transmitter designed to interoperate with a DM Lite® receiver or DMPS Lite™ switcher. The HD‑TX‑4KZ‑111 includes a USB-C® input with DisplayPort™ Alt Mode capability, enabling the transmission of a DisplayPort AV signal.

### Basis of Design

#### Crestron HD-TX-4KZ-111

Specifier Note:

HD-TX-4KZ-111 
https://www.crestron.com/Products/Video/HDMI-Solutions/HDMI-Extenders/HD-TX-4KZ-111

### Device Architecture

#### Physical Form factor

##### Device enclosure to be composed of black finish metal with vented sides

##### Device weight: 11.96 oz (339 g)

#### Mounting Options

##### Freestanding

##### Surface mount via attachable mounting bracket

##### Attachment to single rack rail

#### Environmental Operating Conditions

##### 32° to 104° F (0° to 40° C)

##### 20% to 90% RH (non-condensing)

### Functions

#### Single UTP/STP cable transmission

##### Signal transmission up to 230 feet.

##### Minimum cable type supported: UTP/STP CAT5e

### Controls and Indicators

#### The device shall include the following indicators:

##### One (1) Green LED indicating a valid link to device by same manufacturer

##### One (1) LED indicating that power is provided to device with the following modes:

###### Green - device is operational

###### Amber - device is booting

##### One (1) Green LED indicating device is receiving a DisplayPort over USB-C input signal

##### Two (2) LEDs on RJ-45 connector with the following modes:

###### Green - Connection established to device by same manufacturer

###### Flashing Amber - non-HDCP video

###### Amber - HDCP video

### Connectors

#### The device shall include the following connectors:

##### One (1) USB-C connector for Digital video / audio input with support for the following:

###### DisplayPort Alt Mode with HDR10, HDR10+, Dolby Vision, Deep Color

###### HDCP 2.3 compliant

###### CEC Pass-through

###### EDID

###### Dolby Digital, Dolby Digital EX, Dolby Digital Plus, Dolby True HD, Dolby Atmos DTS, DTS-ES, DTS 96/24, DTS HD High Res, DTS HD Master Audio, DTS:X, LPCM up to 8 channels

###### Common resolutions: 1920x1080 30Hz to 4096x2160 DCI 4K 4:4:4 60Hz

###### Custom resolutions: pixel clock rates up to 600 MHz

##### One (1) RJ45 shielded female connector with support for the following:

###### Link port for connection to compatible transmission device by same manufacturer

###### Power sharing option for connected remote transmission device

##### One (1) 2.1 x 5.5 mm DC power connector supporting the following characteristic:

###### 24 VDC 1.25 A power input

### Power

#### Power supply

##### Power supply shall support device and connected equipment

#### Power supply modes

##### Local or remote DC power source

###### Remote power supplied by opposite end of TX/RX pair.

### Compliance

#### UL Listed for US & Canada, CE, IC, FCC Part 15 Class B digital device

#### Regulatory Model M202047002

## Transmitter Type 8

Specifier Note:

The HD-TX-4KZ-111-1G is a DM Lite® 4K60 4:4:4 wall plate transmitter designed to interoperate with a DM Lite® receiver or DMPS Lite™ switcher. The HD‑TX‑4KZ‑111-1G includes a USB-C® input with DisplayPort™ Alt Mode capability, enabling the transmission of a DisplayPort AV signal.

### Basis of Design

#### Crestron HD-TX-4KZ-111-1G

Specifier Note:

HD-TX-4KZ-111-1G 
https://www.crestron.com/Products/Video/HDMI-Solutions/HDMI-Extenders/HD-TX-4KZ-111-1G-B

### Device Architecture

#### Physical Form factor

##### Device enclosure and bracket to be composed of metal with black polycarbonate front label overlay

##### Device weight: 5.33 oz (152 g)

#### Mounting Option

##### Wall mount device, mounts in a 1-gang (or larger) 2-1/4 inch (57 mm) deep U.S. electrical box or plaster ring (not included)

#### Environmental Operating Conditions

##### 32° to 104° F (0° to 40° C)

##### 20% to 90% RH (non-condensing)

### Functions

#### Single UTP/STP cable transmission

##### Signal transmission up to 230 feet.

##### Minimum cable type supported: UTP/STP CAT5e

### Controls and Indicators

#### The device shall include the following indicators:

##### One (1) Green LED indicating a valid link to device by same manufacturer

##### One (1) LED indicating that power is provided to device with the following modes:

###### Green - device is operational

###### Amber - device is booting

##### One (1) Green LED indicating device is receiving a DisplayPort over USB-C input signal

##### Two (2) LEDs on RJ-45 connector with the following modes:

###### Green - Connection established to device by same manufacturer

###### Flashing Amber - non-HDCP video

###### Amber - HDCP video

### Connectors

#### The device shall include the following connectors:

##### One (1) USB-C connector for Digital video / audio input with support for the following:

###### DisplayPort Alt Mode with HDR10, HDR10+, Dolby Vision, Deep Color

###### HDCP 2.3 compliant

###### CEC Pass-through

###### EDID

###### Dolby Digital, Dolby Digital EX, Dolby Digital Plus, Dolby True HD, Dolby Atmos DTS, DTS-ES, DTS 96/24, DTS HD High Res, DTS HD Master Audio, DTS:X, LPCM up to 8 channels

###### Common resolutions: 1920x1080 30Hz to 4096x2160 DCI 4K 4:4:4 60Hz

###### Custom resolutions: pixel clock rates up to 600 MHz

##### One (1) RJ45 shielded female connector with support for the following:

###### Link port for connection to compatible transmission device by same manufacturer

###### Power sharing option for connected remote transmission device

##### One (1) 2-pin 3.5 mm detachable terminal block supporting the following characteristic:

###### 24 VDC 0.75 A power input

##### One (1) Chassis grounding lug

### Power

#### Power supply

##### Power supply shall support device and connected equipment

#### Power supply modes

##### Local or remote DC power source

###### Remote power supplied by opposite end of TX/RX pair.

### Compliance

#### UL Listed for US & Canada, CE, IC, FCC Part 15 Class B digital device

#### Regulatory Model M202045001

## Transmitter Type 9

Specifier Note:

The HD-TX-4KZ-101-1G is a DM Lite® 4K60 4:4:4 wall plate transmitter designed to interoperate with a DM Lite® receiver or DMPS Lite™ switcher for the transmission of an HDMI® signal.

### Basis of Design

#### Crestron HD-TX-4KZ-101-1G

Specifier Note:

HD-TX-4KZ-101-1G 
https://www.crestron.com/Products/Video/HDMI-Solutions/HDMI-Extenders/HD-TX-4KZ-101-1G-B

### Device Architecture

#### Physical Form factor

##### Device enclosure and bracket to be composed of metal with black polycarbonate front label overlay

##### Device weight: 5.33 oz (152 g)

#### Mounting Option

##### Wall mount device, mounts in a 1-gang (or larger) 2-1/4 inch (57 mm) deep U.S. electrical box or plaster ring (not included)

#### Environmental Operating Conditions

##### 32° to 104° F (0° to 40° C)

##### 20% to 90% RH (non-condensing)

### Functions

#### Single UTP/STP cable transmission

##### Signal transmission up to 230 feet.

##### Minimum cable type supported: UTP/STP CAT5e

### Controls and Indicators

#### The device shall include the following indicators:

##### One (1) Green LED indicating a valid link to device by same manufacturer

##### One (1) LED indicating that power is provided to device with the following modes:

###### Green - device is operational

###### Amber - device is booting

##### One (1) Green LED indicating device is receiving HDMI signal

##### Two (2) LEDs on RJ-45 connector with the following modes:

###### Green - Connection established to device by same manufacturer

###### Flashing Amber - non-HDCP video

###### Amber - HDCP video

### Connectors

#### The device shall include the following connectors:

##### One (1) HDMI input with support for the following:

###### CEC Pass-through

###### HDMI with HDR10, HDR10+, Dolby Vision, Deep Color and 4K60 4:4:4

###### HDCP 2.3 compliant

###### DisplayPort Dual-Mode

###### Dolby Digital, Dolby Digital EX, Dolby Digital Plus, Dolby True HD, Dolby Atmos DTS, DTS-ES, DTS 96/24, DTS HD High Res, DTS HD Master Audio, DTS:X, LPCM up to 8 channels

###### DVI-D with adaptor

###### EDID

###### Common resolutions: 1920x1080 30Hz to 4096x2160 DCI 4K 4:4:4 60Hz

###### Custom resolutions: pixel clock rates up to 600 MHz

##### One (1) RJ45 shielded female connector with support for the following:

###### Link port for connection to compatible transmission device by same manufacturer

###### Power sharing option for connected remote transmission device

##### One (1) 2-pin 3.5 mm detachable terminal block supporting the following characteristic:

###### 24 VDC 0.75 A power input

##### One (1) Chassis grounding lug

### Power

#### Power supply

##### Power supply shall support device and connected equipment

#### Power supply modes

##### Local or remote DC power source

###### Remote power supplied by opposite end of TX/RX pair.

### Compliance

#### UL Listed for US & Canada, CE, IC, FCC Part 15 Class B digital device

#### Regulatory Model M202045001

## Transmitter Type 10

Specifier Note:

The HD‑TXCA‑4KZ‑101 is a DM Lite® 4K60 4:4:4 transmitter designed to interoperate with a DM Lite® receiver or DMPS Lite™ switcher for the transmission of HDMI®, RS-232, IR, and analog audio signals.

### Basis of Design

#### Crestron HD-TXCA-4KZ-101

Specifier Note:

HD-TXCA-4KZ-101 
https://www.crestron.com/Products/Video/HDMI-Solutions/HDMI-Extenders/HD-TXCA-4KZ-101

### Device Architecture

#### Physical Form factor

##### Device enclosure to be composed of black finish metal with vented sides

##### Device weight: 11.92 oz (338 g)

#### Mounting Options

##### Freestanding

##### Surface mount via attachable mounting bracket

##### Attachment to single rack rail

#### Environmental Operating Conditions

##### 32° to 104° F (0° to 40° C)

##### 20% to 90% RH (non-condensing)

### Functions

#### Single UTP/STP cable transmission

##### Signal transmission up to 230 feet.

##### Minimum cable type supported: UTP/STP CAT5e

### Controls and Indicators

#### The device shall include the following indicators:

##### One (1) Green LED indicating a valid link to device by same manufacturer

##### One (1) LED indicating that power is provided to device with the following modes:

###### Green - device is operational

###### Amber - device is booting

##### One (1) Pushbutton for enabling selection of HDMI or analog audio

##### One (1) Green LED indicating selection of HDMI audio

##### One (1) Green LED indicating selection of analog audio

##### One (1) Green LED indicating device is receiving HDMI signal

##### Two (2) LEDs on RJ-45 connector with the following modes:

###### Green - Connection established to device by same manufacturer

###### Flashing Amber - non-HDCP video

###### Amber - HDCP video

### Connectors

#### The device shall include the following connectors:

##### One (1) HDMI input with support for the following:

###### CEC Pass-through

###### HDMI with HDR10, HDR10+, Dolby Vision, Deep Color, and 4K60 4:4:4

###### HDCP 2.3 compliant

###### DisplayPort Dual-Mode

###### Dolby Digital, Dolby Digital EX, Dolby Digital Plus, Dolby True HD, Dolby Atmos DTS, DTS-ES, DTS 96/24, DTS HD High Res, DTS HD Master Audio, DTS:X, LPCM up to 8 channels

###### DVI-D with adaptor

###### EDID

###### Common resolutions: 1920x1080 30Hz to 4096x2160 DCI 4K 4:4:4 60Hz

###### Custom resolutions: pixel clock rates up to 600 MHz

##### One (1) RJ45 shielded female connector with support for the following:

###### Link port for connection to compatible transmission device by same manufacturer

###### Power sharing option for connected remote transmission device

##### One (1) 2.1 x 5.5 mm DC power connector supporting the following characteristic:

###### 24 VDC 1.25 A power input

##### One (1) 4-pin 3.5 mm detachable terminal block IR input/output control port supporting the following characteristics:

###### IR input connects to IR signal source and transmits signal to IR out port on receiver device from same manufacturer

###### IR output connects to IR emitter and transmits IR signal from source connected to receiver device from same manufacturer

###### IR up to 60 kHz

##### One (1) 3-pin 3.5 mm detachable terminal block Serial control port supporting the following characteristics:

###### Bidirectional RS-232 up to 115.2k baud

###### Passes RS-232 TD/RD data to/from RS-232 port on receiver device from same manufacturer

##### One (1) 5-pin 3.5 mm detachable terminal block balanced/unbalanced Stereo line-level audio input supporting the following characteristics:

###### Maximum Input Level: 4 Vrms balanced, 2 Vrms unbalanced

###### Input Impedance: 24 kΩ balanced/unbalanced

##### S/PDIF Optical Digital Audio Output

###### Supports Dolby Digital, Dolby Digital EX, DTS, DTS-ES, LPCM up to 6 channels

### Power

#### Power supply

##### Power supply shall support device and connected equipment

#### Power supply modes

##### Local or remote DC power source

###### Remote power supplied by opposite end of TX/RX pair.

### Compliance

#### UL Listed for US & Canada, CE, IC, FCC Part 15 Class B digital device

#### Regulatory Model M202047002

## Transmitter Type 11

Specifier Note:

The HD‑TXA‑4KZ‑101 is a DM Lite® 4K60 4:4:4 transmitter designed to interoperate with a DM Lite® receiver or DMPS Lite™ switcher for the transmission of HDMI® and analog audio signals.

### Basis of Design

#### Crestron HD-TXA-4KZ-101

Specifier Note:

HD-TXA-4KZ-101 
https://www.crestron.com/Products/Video/HDMI-Solutions/HDMI-Extenders/HD-TXA-4KZ-101

### Device Architecture

#### Physical Form factor

##### Device enclosure to be composed of black finish metal with vented sides

##### Device weight: 11.96 oz (339 g)

#### Mounting Options

##### Freestanding

##### Surface mount via attachable mounting bracket

##### Attachment to single rack rail

#### Environmental Operating Conditions

##### 32° to 104° F (0° to 40° C)

##### 20% to 90% RH (non-condensing)

### Functions

#### Single UTP/STP cable transmission

##### Signal transmission up to 230 feet.

##### Minimum cable type supported: UTP/STP CAT5e

### Controls and Indicators

#### The device shall include the following indicators:

##### One (1) Green LED indicating a valid link to device by same manufacturer

##### One (1) LED indicating that power is provided to device with the following modes:

###### Green - device is operational

###### Amber - device is booting

##### One (1) Pushbutton for enabling selection of HDMI or analog audio

##### One (1) Green LED indicating selection of HDMI audio

##### One (1) Green LED indicating selection of analog audio

##### One (1) Green LED indicating device is receiving HDMI signal

##### Two (2) LEDs on RJ-45 connector with the following modes:

###### Green - Connection established to device by same manufacturer

###### Flashing Amber - non-HDCP video

###### Amber - HDCP video

### Connectors

#### The device shall include the following connectors:

##### One (1) HDMI input with support for the following:

###### CEC Pass-through

###### HDMI with HDR10, HDR10+, Dolby Vision, Deep Color, and 4K60 4:4:4

###### HDCP 2.3 compliant

###### DisplayPort Dual-Mode

###### Dolby Digital, Dolby Digital EX, Dolby Digital Plus, Dolby True HD, Dolby Atmos DTS, DTS-ES, DTS 96/24, DTS HD High Res, DTS HD Master Audio, DTS:X, LPCM up to 8 channels

###### DVI-D with adaptor

###### EDID

###### Common resolutions: 1920x1080 30Hz to 4096x2160 DCI 4K 4:4:4 60Hz

###### Custom resolutions: pixel clock rates up to 600 MHz

##### One (1) RJ45 shielded female connector with support for the following:

###### Link port for connection to compatible transmission device by same manufacturer

###### Power sharing option for connected remote transmission device

##### One (1) 2.1 x 5.5 mm DC power connector supporting the following characteristic:

###### 24 VDC 1.25 A power input

##### One (1) 5-pin 3.5 mm detachable terminal block balanced/unbalanced Stereo line-level audio input supporting the following characteristics:

###### Maximum Input Level: 4 Vrms balanced, 2 Vrms unbalanced

###### Input Impedance: 24 kΩ balanced/unbalanced

### Power

#### Power supply

##### Power supply shall support device and connected equipment

#### Power supply modes

##### Local or remote DC power source

###### Remote power supplied by opposite end of TX/RX pair.

### Compliance

#### UL Listed for US & Canada, CE, IC, FCC Part 15 Class B digital device

#### Regulatory Model M202047002

## Transmitter Type 12

Specifier Note:

The HD-TX-4KZ-211-CHGR is a DM Lite® 4K60 4:4:4 transmitter designed to interoperate with a DM Lite® receiver or DMPS Lite™ switcher. The HDMI® input of the HD-TX-4KZ-211-CHGR enables the transmission of an HDMI signal. The USB-C® input with DisplayPort™ Alt Mode capability enables the transmission of a DisplayPort AV signal.

### Basis of Design

#### Crestron HD-TX-4KZ-211-CHGR

Specifier Note:

HD-TX-4KZ-211-CHGR
https://www.crestron.com/Products/Video/HDMI-Solutions/HDMI-Extenders/HD-TX-4KZ-211-CHGR

### Device Architecture

#### Physical Form factor

##### Device enclosure to be composed of black finish metal with vented top, front, rear, and sides

##### Device weight: 1.67 lb. (0.76 kg)

#### Mounting Options

##### Freestanding

##### Surface mount via attachable mounting bracket

##### Attachment to single rack rail

#### Environmental Operating Conditions

##### 32° to 104° F (0° to 40° C)

##### 20% to 90% RH (non-condensing)

### Functions

#### The transmitter shall support input switching with the following options:

##### Automatic switching to active input

##### Manual switching via integrated selection button

##### Remote switching via control processor by same manufacturer when connected to receiver switching device by same manufacturer

This option requires a LAN connection at the compatible receiver device connected to a control processor by same manufacturer.

#### Single UTP/STP cable transmission

##### Signal transmission up to 230 feet.

##### Minimum cable type supported: UTP/STP CAT5e

### Controls and Indicators

#### The device shall include the following indicators:

##### One (1) LED indicating that power is provided to device with the following modes:

###### Green - device is operational

###### Amber - device is booting

##### One (1) Pushbutton for enabling or disabling automatic switching of inputs

##### One (1) Green LED indicating automatic switching of inputs is enabled

##### One (1) Pushbutton for Ethernet setup (applicable only when device is used in conjunction with switcher from same manufacturer)

##### One (1) Red LED indicating that control described in 2.12.4.1.4 has been pressed

##### Two (2) Pushbuttons for manual input selection

##### Two (2) LEDs indicating input status corresponding to Pushbuttons in 2.12.4.1.6 with the following modes:

###### Green - Input is routed to an output

###### Amber - Source is detected but not routed to an output

##### Two (2) LEDs on RJ-45 connector with the following modes:

###### Green - Connection established to device by same manufacturer

###### Flashing Amber - non-HDCP video

###### Amber - HDCP video

### Connectors

#### The device shall include the following connectors:

##### One (1) HDMI input with support for the following:

###### CEC Pass-through

###### HDMI with HDR10, HDR10+ Dolby Vision, Deep Color, and 4K60 4:4:4

###### HDCP 2.3 compliant

###### DisplayPort Dual-Mode

###### Dolby Digital, Dolby Digital EX, Dolby Digital Plus, Dolby TrueHD, Dolby Atmos DTS, DTS-ES, DTS 96/24, DTS HD High Res, DTS-HD Master Audio, DTS:X, LPCM up to 8 channels

###### DVI-D with adaptor

###### EDID

###### Common resolutions: 1920x1080 30Hz to 4096x2160 DCI 4K 4:4:4 60Hz

###### Custom resolutions: pixel clock rates up to 600 MHz

##### One (1) USB-C connector for Digital video / audio input with support for the following:

###### DisplayPort Alt Mode with HDR10, HDR10+, Dolby Vision, Deep Color

###### 60 W Power Delivery

###### HDCP 2.3 compliant

###### CEC Pass-through

###### EDID

###### Dolby Digital, Dolby Digital EX, Dolby Digital Plus, Dolby True HD, Dolby Atmos DTS, DTS-ES, DTS 96/24, DTS HD High Res, DTS HD Master Audio, DTS:X, LPCM up to 8 channels

###### Common resolutions: 1920x1080 30Hz to 4096x2160 DCI 4K 4:4:4 60Hz

###### Custom resolutions: pixel clock rates up to 600 MHz

##### One (1) RJ45 shielded female connector with support for the following:

###### Link port for connection to compatible transmission device by same manufacturer

###### Power sharing option for connected remote transmission device

##### One (1) 2.5 x 5.5 mm DC power connector supporting the following characteristic:

###### 24 VDC 7.5 A power input

##### One (1) USB Type A connector with support for:

###### Firmware loading and configuration management

###### USB host port for compatible control connection interface by same manufacturer

###### 5 VDC, 500 mA power to USB powered device

##### One (1) Chassis grounding lug

### Power

#### Power supply

##### Power supply shall support device and connected equipment

#### Power supply modes

##### Local or remote DC power source

###### Remote power supplied by opposite end of TX/RX pair.

### Compliance

#### CE, IC, FCC Part 15 Class B digital device

#### Regulatory Model M202046003

## Transmitter Type 13

Specifier Note:

The HD-TX-4KZ-421-CHGR is a DM Lite® 4K60 4:4:4 transmitter designed to interoperate with a DM Lite® receiver or DMPS Lite™ switcher. The HDMI® inputs of the HD-TX-4KZ-421-CHGR enable the transmission of an HDMI signal. The USB-C® inputs with DisplayPort™ Alt Mode capability enable the transmission of a DisplayPort AV signal.

### Basis of Design

#### Crestron HD-TX-4KZ-421-CHGR

Specifier Note:

HD-TX-4KZ-211-CHGR
https://www.crestron.com/Products/Video/HDMI-Solutions/HDMI-Extenders/HD-TX-4KZ-421-CHGR

### Device Architecture

#### Physical Form factor

##### Device enclosure to be composed of black finish metal with vented top, front, rear, and sides

##### Device weight: 1.69 lb. (0.77 kg)

#### Mounting Options

##### Freestanding

##### Surface mount via attachable mounting bracket

##### Attachment to single rack rail

#### Environmental Operating Conditions

##### 32° to 104° F (0° to 40° C)

##### 20% to 90% RH (non-condensing)

### Functions

#### The transmitter shall support input switching with the following options:

##### Automatic switching to active input

##### Manual switching via integrated selection button

##### Remote switching via control processor by same manufacturer when connected to receiver switching device by same manufacturer

This option requires a LAN connection at the compatible receiver device connected to a control processor by same manufacturer.

#### Single UTP/STP cable transmission

##### Signal transmission up to 230 feet.

##### Minimum cable type supported: UTP/STP CAT5e

### Controls and Indicators

#### The device shall include the following indicators:

##### One (1) LED indicating that power is provided to device with the following modes:

###### Green - device is operational

###### Amber - device is booting

##### One (1) Pushbutton for enabling or disabling automatic switching of inputs

##### One (1) Green LED indicating automatic switching of inputs is enabled

##### One (1) Pushbutton for Ethernet setup (applicable only when device is used in conjunction with switcher from same manufacturer)

##### One (1) Red LED indicating that control described in 2.12.4.1.4 has been pressed

##### Four (4) Pushbuttons for manual input selection

##### Four (4) LEDs indicating input status corresponding to Pushbuttons in 2.12.4.1.6 with the following modes:

###### Green - Input is routed to an output

###### Amber - Source is detected but not routed to an output

##### Two (2) LEDs on RJ-45 connector with the following modes:

###### Green - Connection established to device by same manufacturer

###### Flashing Amber - non-HDCP video

###### Amber - HDCP video

### Connectors

#### The device shall include the following connectors:

##### Two (2) HDMI inputs with support for the following:

###### CEC Pass-through

###### HDMI with HDR10, HDR10+ Dolby Vision, Deep Color, and 4K60 4:4:4

###### HDCP 2.3 compliant

###### DisplayPort Dual-Mode

###### Dolby Digital, Dolby Digital EX, Dolby Digital Plus, Dolby TrueHD, Dolby Atmos DTS, DTS-ES, DTS 96/24, DTS HD High Res, DTS-HD Master Audio, DTS:X, LPCM up to 8 channels

###### DVI-D with adaptor

###### EDID

###### Common resolutions: 1920x1080 30Hz to 4096x2160 DCI 4K 4:4:4 60Hz

###### Custom resolutions: pixel clock rates up to 600 MHz

##### Two (2) USB-C connectors for Digital video / audio input with support for the following:

###### DisplayPort Alt Mode with HDR10, HDR10+, Dolby Vision, Deep Color

###### 60 W Power Delivery

###### HDCP 2.3 compliant

###### CEC Pass-through

###### EDID

###### Dolby Digital, Dolby Digital EX, Dolby Digital Plus, Dolby True HD, Dolby Atmos DTS, DTS-ES, DTS 96/24, DTS HD High Res, DTS HD Master Audio, DTS:X, LPCM up to 8 channels

###### Common resolutions: 1920x1080 30Hz to 4096x2160 DCI 4K 4:4:4 60Hz

###### Custom resolutions: pixel clock rates up to 600 MHz

##### One (1) RJ45 shielded female connector with support for the following:

###### Link port for connection to compatible transmission device by same manufacturer

###### Power sharing option for connected remote transmission device

##### One (1) 2.5 x 5.5 mm DC power connector supporting the following characteristic:

###### 24 VDC 7.5 A power input

##### One (1) USB Type A connector with support for:

###### Firmware loading and configuration management

###### USB host port for compatible control connection interface by same manufacturer

###### 5 VDC, 500 mA power to USB powered device

##### One (1) Chassis grounding lug

### Power

#### Power supply

##### Power supply shall support device and connected equipment

#### Power supply modes

##### Local or remote DC power source

###### Remote power supplied by opposite end of TX/RX pair.

### Compliance

#### CE, IC, FCC Part 15 Class B digital device

#### Regulatory Model M202046003

## Transmitter Type 14

Specifier Note:

The HD‑TXA‑4KZ‑101-1G is a DM Lite® 4K60 4:4:4 wall plate transmitter designed to interoperate with a DM Lite® receiver or DMPS Lite™ switcher for the transmission of HDMI® and analog audio signals.

### Basis of Design

#### Crestron HD-TXA-4KZ-101-1G

Specifier Note:

HD-TXA-4KZ-101-1G
https://www.crestron.com/Products/Video/HDMI-Solutions/HDMI-Extenders/HD-TXA-4KZ-101-1G-B

### Device Architecture

#### Physical Form factor

##### Device enclosure and bracket to be composed of metal with black polycarbonate front label overlay

##### Wall mount device, mounts in a 1-gang (or larger) 2-1/4 inch (57 mm) deep U.S. electrical box

##### Device weight: 5.39 oz (153 g)

#### Mounting Option

##### Wall mount device, mounts in a 1-gang (or larger) 2-1/4 inch (57 mm) deep U.S. electrical box or plaster ring (not included)

#### Environmental Operating Conditions

##### 32° to 104° F (0° to 40° C)

##### 20% to 90% RH (non-condensing)

### Functions

#### Single UTP/STP cable transmission

##### Signal transmission up to 230 feet.

##### Minimum cable type supported: UTP/STP CAT5e

### Controls and Indicators

#### The device shall include the following indicators:

##### One (1) Green LED indicating a valid link to device by same manufacturer

##### One (1) LED indicating that power is provided to device with the following modes:

###### Green - device is operational

###### Amber - device is booting

##### One (1) Pushbutton for enabling selection of HDMI or analog audio

##### One (1) Green LED indicating selection of HDMI audio

##### One (1) Green LED indicating selection of analog audio

##### One (1) Green LED indicating device is receiving HDMI signal

##### Two (2) LEDs on RJ-45 connector with the following modes:

###### Green - Connection established to device by same manufacturer

###### Flashing Amber - non-HDCP video

###### Amber - HDCP video

### Connectors

#### The device shall include the following connectors:

##### One (1) HDMI input with support for the following:

###### CEC Pass-through

###### HDMI with HDR10, HDR10+, Dolby Vision, Deep Color, and 4K60 4:4:4

###### HDCP 2.3 compliant

###### DisplayPort Dual-Mode

###### Dolby Digital, Dolby Digital EX, Dolby Digital Plus, Dolby True HD, Dolby Atmos DTS, DTS-ES, DTS 96/24, DTS HD High Res, DTS HD Master Audio, DTS:X, LPCM up to 8 channels

###### DVI-D with adaptor

###### EDID

###### Common resolutions: 1920x1080 30Hz to 4096x2160 DCI 4K 4:4:4 60Hz

###### Custom resolutions: pixel clock rates up to 600 MHz

##### One (1) RJ45 shielded female connector with support for the following:

###### Link port for connection to compatible transmission device by same manufacturer

###### Power sharing option for connected remote transmission device

#### One (1) 2-pin 3.5 mm detachable terminal block supporting the following characteristic:

###### 24 VDC 0.75 A power input

##### One (1) 5-pin 3.5 mm detachable terminal block balanced/unbalanced Stereo line-level audio input supporting the following characteristics:

###### Maximum Input Level: 4 Vrms balanced, 2 Vrms unbalanced

###### Input Impedance: 24 kΩ balanced/unbalanced

##### One (1) Chassis grounding lug

### Power

#### Power supply

##### Power supply shall support device and connected equipment

#### Power supply modes

##### Local or remote DC power source

###### Remote power supplied by opposite end of TX/RX pair.

### Compliance

#### UL Listed for US & Canada, CE, IC, FCC Part 15 Class B digital device

#### Regulatory Model M202047003

# EXECUTION

NOT USED in this Guide Specification.  Specifier shall Specify PART 3 On-Site work as needed.

# APPENDICES

## SPECIFIED PRODUCTS

### Crestron HD-TX-4KZ-401

### Crestron HD-TX-4KZ-211-2G

### Crestron HD-TX-4KZ-201

### Crestron HD-TX-4KZ-101

### Crestron HD-TXC-4KZ-101

### Crestron HD-TXC-4KZ-101-1G

### Crestron HD-TX-4KZ-111

### Crestron HD-TX-4KZ-111-1G

### Crestron HD-TX-4KZ-101-1G

### Crestron HD-TXCA-4KZ-101

### Crestron HD-TXA-4KZ-101

### Crestron HD-TX-4KZ-211-CHGR

### Crestron HD-TX-4KZ-421-CHGR

### Crestron HD-TXA-4KZ-101-1G