# SECTION 27 41 16

# INTEGRATED AUDIO-VIDEO SYSTEMS AND EQUIPMENT

Equipment Specified in this section:

High Definition Digital Video Processor:

DVPHD-8

DVPHD-4

DVPHD-DUAL-R

DVPHD-QUAD-R

DVPHD-PRO-R

DVPHD-CUSTOM

# Table of Contents

PART 1	GENERAL	3

1.1	SECTION INCLUDES	3

A.	Multi-function video processor	3

1.2	RELATED REQUIREMENTS	3

A.	Sections:	3

1.3	REFERENCES	3

A.	Abbreviations and Acronyms	3

PART 2	PRODUCTS	4

2.1	MANUFACTURERS	4

A.	Basis-of-Design Manufacturer:	4

2.2	HIGH-DEFINITION DIGITAL VIDEO PROCESSOR (HDDVP)	4

A.	Basis of Design Product:	4

B.	Functional Requirements:	4

C.	Architecture	6

D.	Video Connections	6

E.	HDCP management	10

F.	User set-up and configuration	10

G.	Remote control system integration.	10

PART 3	EXECUTION	11

END OF SECTION 27 41 16	11

# SECTION 27 41 16

INTEGRATED AUDIO-VIDEO SYSTEMS AND EQUIPMENT

Specifier: The Specifier/Design Professional is responsible for the accuracy of all project specifications, including system application and coordination with related sections.  This guide specification is provided as a convenience and requires editing to match actual project requirements.  CRESTRON ELECTRONICS, INC. SHALL NOT BE LIABLE FOR ANY DAMAGES ARISING OUT OF THE USE OF ANY OF ITS GUIDE SPECIFICATIONS.  For Crestron design assistance and design review please contact Sales Support Services Department at   800.237.2041 or techsales@crestron.com.

GENERAL

SECTION INCLUDES

Multi-function video processor

Multi-function video processor with video processing and touch screen GUI processing functions.

Support for up to eight HD with HDCP sources

Configurable inputs, allowing analog and HD/HDCP source connection.

RELATED REQUIREMENTS

Sections:

25 08 00 — Commissioning of Integrated Automation

25 10 00 — Integrated Automation Network Equipment

25 11 13 — Integrated Automation Network Servers

25 13 13 — Integrated Automation Control and Monitoring Network Supervisory Control

25 13 16 — Integrated Automation Control and Monitoring Network Integration Panels

25 13 19 — Integrated Automation Control and Monitoring Network Interoperability

25 15 16 — Integrated Automation Software for Control and Monitoring Networks

27 15 00 — Communications Horizontal Cabling

27 41 00 — Audio-Video Systems

REFERENCES

Abbreviations and Acronyms

EDID:  Extended display identification data

EDTV: Enhanced-Definition Television

GUI: Graphic User Interface

HD: High Definition.

HDCP:  High-bandwidth Digital Content Protection

HDCP: High-Bandwidth Digital Content Protection

HDDVP: High Definition Digital Video Processor

HDMI:  High-Definition Multimedia Interface

HDTV: High-Definition Television

KSV: Key Selection Vector

UI objects: User interface objects, i.e. on screen buttons, bar graphs, invisible buttons, etc.

PRODUCTS

MANUFACTURERS

Basis-of-Design Manufacturer:

Subject to compliance with requirements, provide products of Crestron Electronics, Inc., Rockleigh, NJ 07647, Phone 800-237-2041, Fax: 201-767-1903, www.crestron.com [or comparable products from a single manufacturer approved by A/E prior to bidding], with the following components and characteristics.

HIGH-DEFINITION DIGITAL VIDEO PROCESSOR (HDDVP)

Basis of Design Product:

Crestron, DVPHD- [8, 4, DUAL, QUAD, QUAD-BNC, PRO, PRO-BNC, PRO-QM, PRO-SDI, CUSTOM-PRO, CUSTOM, CUSTOM-GB, and custom configurations] Digital Media Presentation System.

Functional Requirements:

Multi-window display mode:

HDDVP output shall simultaneously display multiple input sources windowed in configurable frames.

Any number of active and connected input sources shall be available for multi-window output.

Custom configuration of eight Inputs composed of:

HDMI

DVI

DisplayPort Multimode

SDI

RGB

Component (YPbPr)

S-Video (Y/C)

Composite

Balanced Analog video

SPECIFIER: Based on the configuration of inputs the DVPHD is capable of 0-8 windows.

HDDVP Output shall support:

Full motion video

Custom background graphics

Dynamic text labeling

Each window allows independent control of:

Position

Size

Aspect ratio

Source selection

Seamless Switching – facilitated by maintained HDCP connection with HDCP compliant sources.

Transition effects shall be available for creating transitional switching between video input sources.

Source Processing:

Scaling: Gennum Visual Excellence Processing with Reality Expansion 30-bit image processing.

De-interlacing: Gennum TrueMotionHD.

Custom Graphics Processing – The following custom graphic elements shall be supported:

Object transparency.

Background textures

Full color images

Custom Logos

Dynamic graphics

Dynamic text

Pop-up windows

Animations

Crawling text

Touch screen graphics engine – HDDVP shall provide the following functions when integrated with a compatible touch enabled display device:

Simultaneous real-time display of UI objects and source video windows.

Real-time bidirectional communication between control processor and touch enabled display.  HDDVP shall:

Communicate control object button presses to control processor.

Receive data from control processor and display real-time feedback via UI object, custom graphic or initiate HDDVP action.

User annotation: real-time HD annotation over the active display output via touch enabled display.

Remote user annotation: real-time HD annotation over the active display output via remote compatible touch enabled displays.

HDDVP shall support up to 90 remote annotation users.

Built-in video test patterns shall be internally generated and available for testing and calibration of video windows and main output.

Architecture

The HDDVP unit shall be composed of the following fundamental elements:

Front panel user interface.

Built-in power supply.

Built-in communications ports.

SPECIFIER: All input and output cards are factory installed, configured and tested.  Actual configuration must be selected prior to order.  The DVPHD input and output cards are NOT user swappable.  Inputs may be configured in several configurations, see http://www.crestron.com/resources/product_and_programming_resources/catalogs_and_brochures/online_catalog/default.asp?jump=1&model=DVPHD  for additional configuration information.

Configurable inputs – configurable, 1 to 8 inputs using:

Single analog input card module.

Dual analog input cards module.

Dual digital input card module.

Single analog input and single digital input card module.

Configurable outputs – configurable, 1 output or 2 outputs using:

Single output configuration.

Dual output configuration.

Video Connections

Input Type 1: Analog Input

Minimum requirements:

Three female BNC connectors configurable to one of the following:

Component (YPbPr)

S-Video (Y/C)

Composite Video

Formats: 480i (NTSC), 576i (PAL), 576p, 720p, 1080i

Input Impedance: 75 ohms

Input Type 2: Analog/Digital Input

Minimum requirements:

One female DVI connector configurable to one of the following:

DVI

HDMI

DisplayPort Multimode

RGBHV (VGA)

Component (YPbPr)

Digital formats:

DVI 1.0

HDMI v.1.2 with Deep Color

HDCP v.1.2

Analog formats:

RGBHV

RGBS

RGsB

Component (YPbPr)

EDTV 480p

EDTV 576p

HDTV 720p

HDTV 1080i

HDTV1080p

Input resolution: Progressive: 640x400 to 1920x1200

Input Type 3: SDI/HD-SDI Input

Minimum requirements:

One female BNC connector configurable to one of the following:

SDI

SMPTE 125M

HD-SDI

SMPTE 274M, 295M, 296M

Formats:

SDI and HD-SDI interlaced:

487i

576i

1080i25

1080i30

HD-SDI Progressive:

720p24

720p25

720p30

720p50

720p60

1080p24

1080p25

1080p30

Input Type 4: Balanced Analog Input

Minimum requirements:

One female RJ-45 [QM (QuickMedia)] connector configurable to one of the following:

RGBHV (VGA)

RGBS

RGsB

Component (YPbPr)

S-Video (Y/C)

Composite Video

RGB Input resolution: Progressive: 640x400 to 1920x1200

Input Type 5: HDMI Input

Minimum requirements:

One female HDMI input connector configurable to one of the following:

HDMI

DVI

DisplayPort Multimode

Digital formats:

DVI

HDMI with Deep Color

HDCP

Input resolution: Progressive:

640x400 to 1920x1200

480P

576P

720P

1080P

Input resolution: Interlaced:

480i

576i

1080i

Specifier:  HDMI audio is routed to HDMI pass-thru output on same card only, the DVPHD does not pass audio to any switched/processed outputs.  The Crestron HD-DA-2 may be utilized for HDMI to analog audio conversion.

HDMI Digital Audio:

Support for pass-through from HDMI input to HDMI pass-through output.

Specifier:  video (and audio if included) on the DHDC-HDRGBVI HDMI input are passed through to the “HDMI OUT” connector.

One female HDMI pass-through output connector with support for sources connected to HDMI input.

HDMI output signal types:

HDMI

DVI

Output

Minimum requirements:

One female DVI connector configurable to one of the following:

DVI

HDMI

RGBHV (VGA)

Digital formats:

DVI 1.0

HDMI v.1.2 with Deep Color

HDCP v.1.2

EDID v.1.3

Digital Output resolution:

Interlaced: 1080i

Progressive: 640x400 to 1920x1200

Analog formats:

RGBHV

EDTV 480p

EDTV 576p

HDTV 720p

HDTV 1080p

Analog Output resolution:

Progressive: 640x400 to 1920x1200

Control and Communication Ports

One RS-232, bidirectional communications port for touchscreen communication.

Two USB, Type A USB 1.1 communications ports for touchscreen communication.

One LAN, 10/100BaseT port for control communication.

One Net, [Cresnet control network] port for control communication.

HDCP management

Minimum requirements:

Up to 8 HDCP compliant video inputs.

Combine HDCP sources with non-HDCP sources in multi-window layouts.

User set-up and configuration

Control and operating modes shall be configurable and controllable through the following user interfaces:

Front panel.

Compatible touch screen.

Web browser.

Remote control system integration.

The HDDVP shall be controllable by one or multiple external control processors.

All control communication shall be through the HDDVP unit’s native control protocol.

EXECUTION

Not Used

END OF SECTION 27 41 16