SECTION 27 41 16

INTEGRATED AUDIO-VIDEO SYSTEMS AND EQUIPMENT

GUIDE SPECIFICATION

Specifier: The Specifier/Design Professional is responsible for the accuracy of all project specifications, including system application and coordination with related sections.  This guide specification is provided as a convenience and requires editing to match actual project requirements.  CRESTRON ELECTRONICS, INC. SHALL NOT BE LIABLE FOR ANY DAMAGES ARISING OUT OF THE USE OF ANY OF ITS GUIDE SPECIFICATIONS.  For Crestron design assistance and design review please contact Sales Support Services Department at   800.237.2041 or techsales@crestron.com.

Specifier: Please see PART 4 for a listing of products specified in this Guide Specification.

Table of Contents

1	GENERAL	3

2	PRODUCTS	3

2.1	PoE Speaker and Microphone Pod with support for proprietary audio transmission standard by same manufacturer	3

2.1.1	Basis of Design	3

2.1.2	Device Definition	3

2.1.3	Device Architecture	3

2.1.4	Functions	4

2.1.5	Compliance	6

2.2	PoE Microphone Pod with support for proprietary audio transmission standard by same manufacturer	6

2.2.1	Basis of Design	6

2.2.2	Device Definition	6

2.2.3	Device Architecture	7

2.2.4	Functions	7

2.2.5	Compliance	8

3	EXECUTION	9

4	APPENDICES	9

4.1	SPECIFIED PRODUCTS	9

4.1.1	Crestron DM-NAX-POD-MIC-SPK	9

4.1.2	Crestron DM-NAX-POD-MIC	9

# GENERAL

NOT USED in this Guide Specification.  Specifier shall Specify PART 1 administrative and procedural requirements as needed.

# PRODUCTS

## PoE Speaker and Microphone Pod with support for proprietary audio transmission standard by same manufacturer

Specifier Note:

The DM-NAX-POD-MIC-SPK is a tabletop microphone and speaker pod for unified communications and commercial audio applications. The DM-NAX-POD-MIC-SPK connects to a dedicated audio-over-IP network to pair with a DM-NAX-AP-100 Intelligent Audio Processor (sold separately) and other microphone or speaker pods.

### Basis of Design

#### Crestron DM-NAX-POD-MIC-SPK

Specifier Note:

DM-NAX-POD-MIC-SPK
https://www.crestron.com/Products/Catalog/AV-Over-IP/DM-NVX-AV-Over-IP/DM-NAX-Intelligent-Audio/DM-NAX-POD-MIC-SPK

### Device Definition

#### PoE speaker and microphone pod with the following capabilities and features:

##### 12-element array microphone and built-in speaker

##### In-call volume and mic mute controls

##### Speaker volume indicator and LED indicator ring for display of operational status

##### AEC and noise reduction

##### Automatic gain control and mixing

##### Connects via network to audio processor device by same manufacturer, which sends and receives audio to a BYOM computer via USB-C® connection

##### Power over Ethernet (PoE+) network powered

### Device Architecture

#### Environmental

##### Temperature: 41° to 104°F (5° to 40° C)

##### Humidity: 10% to 95% RH (noncondensing)

#### Construction

##### Enclosure: Lightly textured black finish, plastic housing, perforated stainless steel grilles, and die-cast aluminum bottom cover

##### Mounting: Tabletop mounting only

#### Dimensions

##### Height: 2.50 in. (64 mm)

##### Width: 5.52 in. (140 mm)

##### Depth: 5.52 in. (140 mm)

### Functions

#### Communications

##### Ethernet: For control and PoE, AoIP, and/or console, 100/1000 Mbps, auto-switching, auto-negotiating, auto-discovery, full/half duplex, DHCP

#### Microphones

##### Microphones: 360° 12-element array

##### Pickup Range: 7.5 ft (2.3 m)

#### Speaker

##### Driver: One full range 2 in. speaker

##### Frequency Response: 100 Hz to 7 kHz

##### Output Level: 74 dB SPL @ 1 m

#### Audio

##### DSP

###### Acoustic echo cancellation, automatic gain control, dynamic noise suppression, and speaker volume control

###### Full-duplex IEEE 1329 Type 1 compliant

#### Connectors

##### LAN 1 (PoE+)

###### (1) 8-pin RJ45 connector, female

###### 100BASE-TX/1000BASE-T Ethernet port

###### IEEE 802.3at Type 2 PoE Class 4 (25.5 W) compliant

###### Compatible with PoE+ compliant Ethernet switch or third-party PoE+ PSE

###### Dedicated for Audio-over-IP traffic only

#### Controls and Indicators

##### Mic Mute

###### (1) Button, capacitive, backlit, mutes or unmutes internal microphone

###### (1) Bicolor red/green LED, indicates whether microphone is muted or unmuted

##### Volume Level

###### (2) Buttons, capacitive, backlit, increase (+) or decrease (-) speaker volume level

###### (5) White LEDs, indicate current speaker volume level

##### Status Ring

###### (1) Multicolor LED ring on top panel, indicates status conditions such as pairing, updating, or errors

##### RESET

###### (1) Push button: Pressing and holding the RESET button for 15 seconds with power supplied clears network settings and restores the default DHCP mode

Specifier Note:

To perform a factory restore, press and hold the RESET button with power disconnected, then connect the power supply and continue to hold the RESET button for 30 seconds

#### Power

##### PoE+

###### IEEE 802.3at Class 4 (25.5 W) compliant

###### Compatible with IEEE 802.3at compliant Ethernet switch or third-party PoE+ compliant PSE

### Compliance

#### Regulatory Model: M202439002

## PoE Microphone Pod with support for proprietary audio transmission standard by same manufacturer

Specifier Note:

The DM-NAX-POD-MIC is a tabletop microphone pod for unified communications and commercial audio applications. The DM-NAX-POD-MIC connects to a dedicated audio-over-IP network to pair with a DM-NAX-AP-100 Intelligent Audio Processor (sold separately) and other microphone or speaker pods.

### Basis of Design

#### Crestron DM-NAX-POD-MIC

Specifier Note:

DM-NAX-POD-MIC
https://www.crestron.com/Products/Catalog/AV-Over-IP/DM-NVX-AV-Over-IP/DM-NAX-Intelligent-Audio/DM-NAX-POD-MIC

### Device Definition

#### PoE microphone pod with the following capabilities and features:

##### 12-element array microphone

##### In-call volume and mic mute controls

##### LED indicator ring for display of operational status

##### AEC and noise reduction

##### Automatic gain control and mixing

##### Connects via network to audio processor device by same manufacturer, which sends and receives audio to a BYOM computer via USB-C® connection

##### Power over Ethernet (PoE+) network powered

### Device Architecture

#### Environmental

##### Temperature: 41° to 104°F (5° to 40° C)

##### Humidity: 10% to 95% RH (noncondensing)

#### Construction

##### Enclosure: Lightly textured black finish, plastic housing, perforated stainless steel grilles, and die-cast aluminum bottom cover

##### Mounting: Tabletop mounting only

#### Dimensions

##### Height: 1.3 in. (33 mm)

##### Width: 4.70 in. (120 mm)

##### Depth: 4.70 in. (120 mm)

### Functions

#### Communications

##### Ethernet: For control and PoE, AoIP, and/or console, 100/1000 Mbps, auto-switching, auto-negotiating, auto-discovery, full/half duplex, DHCP

#### Microphones

##### Microphones: 360° 12-element array

##### Pickup Range: 7.5 ft (2.3 m)

#### Audio

##### DSP

###### Automatic gain control and dynamic noise suppression

###### Full-duplex IEEE 1329 Type 1 compliant

#### Connectors

##### LAN 1 (PoE+)

###### (1) 8-pin RJ45 connector, female

###### 100BASE-TX/1000BASE-T Ethernet port

###### IEEE 802.3af Type 1 PoE Class 0 (12.95 W) compliant

###### Compatible with PoE compliant Ethernet switch or third-party PoE PSE

###### Dedicated for Audio-over-IP traffic only

#### Controls and Indicators

##### Mic Mute

###### (1) Button, capacitive, backlit, mutes or unmutes internal microphone

###### (1) Bicolor red/green LED, indicates whether microphone is muted or unmuted

##### Status Ring

###### (1) Multicolor LED ring on top panel, indicates status conditions such as pairing, updating, or errors

##### RESET

###### (1) Push button: Pressing and holding the RESET button for 15 seconds with power supplied clears network settings and restores the default DHCP mode

Specifier Note:

To perform a factory restore, press and hold the RESET button with power disconnected, then connect the power supply and continue to hold the RESET button for 30 seconds

#### Power

##### PoE

###### IEEE 802.3af Class 0 (12.95 W) compliant

###### Compatible with IEEE 802.3af compliant Ethernet switch or third-party PoE compliant PSE

### Compliance

#### Regulatory Model: M202439001

# EXECUTION

NOT USED in this Guide Specification.  Specifier shall Specify PART 3 On-Site work as needed.

# APPENDICES

## SPECIFIED PRODUCTS

Specifier Note: This Article includes Crestron products specified in this Guide Specification document.  This Article is for reference only and should not be required in actual project manual unless included in an overall system equipment list.

### Crestron DM-NAX-POD-MIC-SPK

### Crestron DM-NAX-POD-MIC