- SECTION 27 41 16

- INTEGRATED AUDIO-VIDEO SYSTEMS AND EQUIPMENT

- GUIDE SPECIFICATION

- Specifier: The Specifier/Design Professional is responsible for the accuracy of all project specifications, including system application and coordination with related sections.  This guide specification is provided as a convenience and requires editing to match actual project requirements.  CRESTRON ELECTRONICS, INC. SHALL NOT BE LIABLE FOR ANY DAMAGES ARISING OUT OF THE USE OF ANY OF ITS GUIDE SPECIFICATIONS.  For Crestron design assistance and design review please contact Sales Support Services Department at   800.237.2041 or techsales@crestron.com.

- Specifier:  Please see PART 4 for a listing of products specified in this Guide Specification.

- Table of Contents

- 1    GENERAL	4

- 1.1    Description: Single cable 4K HDMI transmitter	4

- 2    PRODUCTS	4

- 2.1    Transmitter Type 1	4

- 2.1.1    Basis of Design	4

- 2.1.2    Device Architecture	4

- 2.1.3    Functions	4

- 2.1.4    Connectors	5

- 2.1.5    Power	7

- 2.1.6    Compliance	7

- 2.2    Transmitter Type 2	7

- 2.2.1    Basis of Design	7

- 2.2.2    Device Architecture	7

- 2.2.3    Functions	8

- 2.2.4    Connectors	8

- 2.2.5    Power	10

- 2.2.6    Compliance	10

- 2.3    Transmitter Type 3	11

- 2.3.1    Basis of Design	11

- 2.3.2    Device Architecture	11

- 2.3.3    Functions	11

- 2.3.4    Connectors	11

- 2.3.5    Power	14

- 2.3.6    Compliance	14

- 2.4    Transmitter Type 4	14

- 2.4.1    Basis of Design	14

- 2.4.2    Device Architecture	14

- 2.4.3    Functions	14

- 2.4.4    Connectors	15

- 2.4.5    Power	16

- 2.4.6    Compliance	16

- 2.5    Transmitter Type 5	16

- 2.5.1    Basis of Design	16

- 2.5.2    Device Architecture	16

- 2.5.3    Functions	17

- 2.5.4    Connectors	17

- 2.5.5    Power	18

- 2.5.6    Compliance	18

- 2.6    Transmitter Type 6	18

- 2.6.1    Basis of Design	18

- 2.6.2    Device Architecture	18

- 2.6.3    Functions	19

- 2.6.4    Connectors	19

- 2.6.5    Power	20

- 2.6.6    Compliance	21

- 2.7    Transmitter Type 7	21

- 2.7.1    Basis of Design	21

- 2.7.2    Device Architecture	21

- 2.7.3    Functions	21

- 2.7.4    Connectors	21

- 2.7.5    Power	23

- 2.7.6    Compliance	23

- 3    EXECUTION	23

- 4    APPENDICES	23

- 4.1    SPECIFIED PRODUCTS	23

- 4.1.1    Crestron HD-TX-101-C-1G-E	23

- 4.1.2    Crestron HD-TX-101-C-E	23

- 4.1.3    Crestron HD-TX-201-C-2G-E	23

- 4.1.4    Crestron HD-TX-301-C-E	23

- 4.1.5    Crestron HD-TXC-101-C-1G-E	23

- 4.1.6    Crestron HD-TXC-101-C-E	24

- 4.1.7    Crestron HDI-TX-301-C-2G-E	24

# GENERAL

- NOT USED in this Guide Specification.  Specifier shall Specify PART 1 administrative and procedural requirements as needed.

## Description: Single cable 4K HDMI transmitter

# PRODUCTS

## Transmitter Type 1

### Basis of Design

#### Crestron HD-TX-301-C-E

### Device Architecture

#### Physical Form factor

##### Freestanding low-profile enclosure

##### Surface mount or single rack rail mount via integrated flange

### Functions

#### The transmitter shall support input switching with the following options:

##### Automatic switching to active input

##### Manual switching via integrated selection buttons

##### Remote switching via control processor by same manufacturer

- This option requires a LAN connection at the compatible receiver device connected to a Crestron control processor.

#### Single UTP/STP cable transmission

##### Signal transmission up to 230 feet.

##### Minimum cable type supported: UTP/STP CAT5e

### Connectors

#### The device shall include the following connectors:

##### Two (2) HDMI inputs with support for the following:

###### CEC Pass-through and CEC management

###### HDMI with Deep Color, 3D and 4K

###### HDCP 1.4 compliant

###### DisplayPort Dual-Mode

###### Dolby Digital, Dolby Digital EX, DTS, DTS-ES, DTS 96/24, LPCM up to 8 channels

###### DVI-D with adaptor

###### EDID

###### Common resolutions: 1920x1080 30Hz to 4096x2160 DCI 4K 4:4:4 30Hz

###### Custom resolutions: pixel clock rates up to 300 MHz

##### One (1) DB15 input with support for the following:

###### VGA video

###### Component video via breakout adaptor

###### Formats: RGBHV, RGBS, RGsB, YPbPr

###### Common resolutions: up to progressive 1920x1200 WUXGA 60Hz

###### Custom resolutions: pixel clock rates up to 165 MHz

##### One (1) 3.5 mm TRS mini phone jack input:

###### Signals supported:

Unbalanced stereo line-level analog audio

###### Audio requirements:

Input Impedance: 44k Ohms

Analog-To-Digital Conversion:	24-bit 48 kHz

Maximum Input Level: 2 Vrms

###### Minimum Audio Performance:

Frequency Response: 20 Hz to 20 kHz ±0.5 dB

S/N Ratio: >93 dB, 20 Hz to 20 kHz A-weighted

THD+N: <0.005% @ 1 kHz

Stereo Separation: >80 dB

##### One (1) RJ45 shielded female connector with support for the following:

###### Link port for connection to compatible transmission device by same manufacturer

###### Power sharing option for connected remote transmission device

##### One (1) DC power input jack with support for:

###### Power connection shall support power sharing with connected transmission device

##### One (1) USB Type A connector with support for:

###### USB host port for compatible control connection interface by same manufacturer

###### Service and firmware update connection

### Power

#### Power supply modes:

##### Local or remote DC power source

##### Remote power supplied by opposite end of TX/RX pair.

### Compliance

#### UL Listed for US & Canada, CE, IC, FCC Part 15 Class B digital device

## Transmitter Type 2

### Basis of Design

#### Crestron HDI-TX-301-C-2G-E

### Device Architecture

#### Physical Form factor

##### European 2-gang device,  mounts in a 45 mm (1-25/32 inch) deep UK or European electrical box.

### Functions

#### The transmitter shall support input switching with the following options:

##### Automatic switching to active input

##### Manual switching via integrated selection buttons

##### Remote switching via control processor by same manufacturer

- This option requires a LAN connection at the compatible receiver device connected to a Crestron control processor.

#### Single UTP/STP cable transmission

##### Signal transmission up to 230 feet.

##### Minimum cable type supported: UTP/STP CAT5e

### Connectors

#### The device shall include the following connectors:

##### Two (2) HDMI inputs with support for the following:

###### CEC Pass-through and CEC management

###### HDMI with Deep Color, 3D and 4K

###### HDCP 1.4 compliant

###### DisplayPort Dual-Mode

###### Dolby Digital, Dolby Digital EX, DTS, DTS-ES, DTS 96/24, LPCM up to 8 channels

###### DVI-D with adaptor

###### EDID

###### Common resolutions: 1920x1080 30Hz to 4096x2160 DCI 4K 4:4:4 30Hz

###### Custom resolutions: pixel clock rates up to 300 MHz

##### One (1) DB15 input with support for the following:

###### VGA video

###### Component video via breakout adaptor

###### Formats: RGBHV, RGBS, RGsB, YPbPr

###### Common resolutions: up to progressive 1920x1200 WUXGA 60Hz

###### Custom resolutions: pixel clock rates up to 165 MHz

##### One (1) 3.5 mm TRS mini phone jack input:

###### Signals supported:

Unbalanced stereo line-level analog audio

###### Audio requirements:

Input Impedance: 44k Ohms

Analog-To-Digital Conversion:	24-bit 48 kHz

Maximum Input Level: 2 Vrms

###### Minimum Audio Performance:

Frequency Response: 20 Hz to 20 kHz ±0.5 dB

S/N Ratio: >93 dB, 20 Hz to 20 kHz A-weighted

THD+N: <0.005% @ 1 kHz

Stereo Separation: >80 dB

##### One (1) RJ45 shielded female connector with support for the following:

###### Link port for connection to compatible transmission device by same manufacturer

###### Power sharing option for connected remote transmission device

##### One (1) RJ45 pass-through LAN connection

##### One (1) DC power input jack with support for:

###### Power connection shall support power sharing with connected transmission device

### Power

#### Power supply modes:

##### Local or remote DC power source

##### Remote power supplied by opposite end of TX/RX pair.

### Compliance

#### CE, IC, FCC Part 15 Class B digital device

## Transmitter Type 3

### Basis of Design

#### Crestron HD-TX-201-C-2G-E

### Device Architecture

#### Physical Form factor

##### Wall mount device,  mounts in a 2-gang, 2-1/4 inch (57 mm) deep U.S. electrical box

### Functions

#### The transmitter shall support input switching with the following options:

##### Automatic switching to active input

##### Manual switching via integrated selection buttons

##### Remote switching via control processor by same manufacturer

- This option requires a LAN connection at the compatible receiver device connected to a Crestron control processor.

#### Single UTP/STP cable transmission

##### Signal transmission up to 230 feet.

##### Minimum cable type supported: UTP/STP CAT5e

### Connectors

#### The device shall include the following connectors:

##### One (1) HDMI input with support for the following:

###### CEC Pass-through and CEC management

###### HDMI with Deep Color, 3D and 4K

###### HDCP 1.4 compliant

###### DisplayPort Dual-Mode

###### Dolby Digital, Dolby Digital EX, DTS, DTS-ES, DTS 96/24, LPCM up to 8 channels

###### DVI-D with adaptor

###### EDID

###### Common resolutions: 1920x1080 30Hz to 4096x2160 DCI 4K 4:4:4 30Hz

###### Custom resolutions: pixel clock rates up to 300 MHz

##### One (1) DB15 input with support for the following:

###### VGA video

###### Component video via breakout adaptor

###### Formats: RGBHV, RGBS, RGsB, YPbPr

###### Common resolutions: up to progressive 1920x1200 WUXGA 60Hz

###### Custom resolutions: pixel clock rates up to 165 MHz

##### One (1) 3.5 mm TRS mini phone jack input:

###### Signals supported:

Unbalanced stereo line-level analog audio

###### Audio requirements:

Input Impedance: 44k Ohms

Analog-To-Digital Conversion:	24-bit 48 kHz

Maximum Input Level: 2 Vrms

###### Minimum Audio Performance:

Frequency Response: 20 Hz to 20 kHz ±0.5 dB

S/N Ratio: >93 dB, 20 Hz to 20 kHz A-weighted

THD+N: <0.005% @ 1 kHz

Stereo Separation: >80 dB

##### One (1) RJ45 shielded female connector with support for the following:

###### Link port for connection to compatible transmission device by same manufacturer

###### Power sharing option for connected remote transmission device

##### One (1) RJ45 pass-through LAN connection

##### One (1) DC power input jack with support for:

###### Power connection shall support power sharing with connected transmission device

### Power

#### Power supply modes:

##### Local or remote DC power source

##### Remote power supplied by opposite end of TX/RX pair.

### Compliance

#### UL Listed for US & Canada, CE, IC, FCC Part 15 Class B digital device

## Transmitter Type 4

### Basis of Design

#### Crestron HD-TX-101-C-E

### Device Architecture

#### Physical Form factor

##### Freestanding low-profile enclosure

##### Surface mount or single rack rail mount via integrated flange

### Functions

#### Single UTP/STP cable transmission

##### Signal transmission up to 230 feet.

##### Minimum cable type supported: UTP/STP CAT5e

### Connectors

#### The device shall include the following connectors:

##### One (1) HDMI input with support for the following:

###### CEC Pass-through and CEC management

###### HDMI with Deep Color, 3D and 4K

###### HDCP 1.4 compliant

###### DisplayPort Dual-Mode

###### Dolby Digital, Dolby Digital EX, DTS, DTS-ES, DTS 96/24, LPCM up to 8 channels

###### DVI-D with adaptor

###### EDID

###### Common resolutions: 1920x1080 30Hz to 4096x2160 DCI 4K 4:4:4 30Hz

###### Custom resolutions: pixel clock rates up to 300 MHz

##### One (1) RJ45 shielded female connector with support for the following:

###### Link port for connection to compatible transmission device by same manufacturer

###### Power sharing option for connected remote transmission device

##### One (1) DC power input jack with support for:

###### Power connection shall support power sharing with connected transmission device

##### One (1) USB Type A connector with support for:

###### USB power port (500 mA maximum output)

###### Supplies power to a USB powered device

### Power

#### Power supply modes:

##### Local or remote DC power source

##### Remote power supplied by opposite end of TX/RX pair.

### Compliance

#### UL Listed for US & Canada, CE, IC, FCC Part 15 Class B digital device

## Transmitter Type 5

### Basis of Design

#### Crestron HD-TX-101-C-1G-E

### Device Architecture

#### Physical Form factor

##### Wall mount device, mounts in a 1-gang (or larger) 2-1/4 inch (57 mm) deep U.S. electrical box

### Functions

#### Single UTP/STP cable transmission

##### Signal transmission up to 230 feet.

##### Minimum cable type supported: UTP/STP CAT5e

### Connectors

#### The device shall include the following connectors:

##### One (1) HDMI input with support for the following:

###### CEC Pass-through and CEC management

###### HDMI with Deep Color, 3D and 4K

###### HDCP 1.4 compliant

###### DisplayPort Dual-Mode

###### Dolby Digital, Dolby Digital EX, DTS, DTS-ES, DTS 96/24, LPCM up to 8 channels

###### DVI-D with adaptor

###### EDID

###### Common resolutions: 1920x1080 30Hz to 4096x2160 DCI 4K 4:4:4 30Hz

###### Custom resolutions: pixel clock rates up to 300 MHz

##### One (1) RJ45 shielded female connector with support for the following:

###### Link port for connection to compatible transmission device by same manufacturer

###### Power sharing option for connected remote transmission device

##### One (1) DC power input jack with support for:

###### Power connection shall support power sharing with connected transmission device

##### One (1) USB Type A connector with support for:

###### USB power port (500 mA maximum output)

###### Supplies power to a USB powered device

### Power

#### Power supply modes:

##### Local or remote DC power source

##### Remote power supplied by opposite end of TX/RX pair.

### Compliance

#### UL Listed for US & Canada, CE, IC, FCC Part 15 Class B digital device

## Transmitter Type 6

### Basis of Design

#### Crestron HD-TXC-101-C-E

### Device Architecture

#### Physical Form factor

##### Freestanding low-profile enclosure

##### Surface mount or single rack rail mount via integrated flange

### Functions

#### Single UTP/STP cable transmission

##### Signal transmission up to 230 feet.

##### Minimum cable type supported: UTP/STP CAT5e

### Connectors

#### The device shall include the following connectors:

##### One (1) HDMI input with support for the following:

###### CEC Pass-through and CEC management

###### HDMI with Deep Color, 3D and 4K

###### HDCP 1.4 compliant

###### DisplayPort Dual-Mode

###### Dolby Digital, Dolby Digital EX, DTS, DTS-ES, DTS 96/24, LPCM up to 8 channels

###### DVI-D with adaptor

###### EDID

###### Common resolutions: 1920x1080 30Hz to 4096x2160 DCI 4K 4:4:4 30Hz

###### Custom resolutions: pixel clock rates up to 300 MHz

##### One (1) RJ45 shielded female connector with support for the following:

###### Link port for connection to compatible transmission device by same manufacturer

###### Power sharing option for connected remote transmission device

##### One (1) DC power input jack with support for:

###### Power connection shall support power sharing with connected transmission device

##### One (1) USB Type A connector with support for:

###### USB power port (500 mA maximum output)

###### Supplies power to a USB powered device

##### One (1) Control IR output port

##### One (1) Control IR input port

##### One (1) Control Serial port

### Power

#### Power supply modes:

##### Local or remote DC power source

##### Remote power supplied by opposite end of TX/RX pair.

### Compliance

#### UL Listed for US & Canada, CE, IC, FCC Part 15 Class B digital device

## Transmitter Type 7

### Basis of Design

#### Crestron HD-TXC-101-C-1G-E

### Device Architecture

#### Physical Form factor

##### Wall mount device, mounts in a 1-gang (or larger) 2-1/4 inch (57 mm) deep U.S. electrical box

### Functions

#### Single UTP/STP cable transmission

##### Signal transmission up to 230 feet.

##### Minimum cable type supported: UTP/STP CAT5e

### Connectors

#### The device shall include the following connectors:

##### One (1) HDMI input with support for the following:

###### CEC Pass-through and CEC management

###### HDMI with Deep Color, 3D and 4K

###### HDCP 1.4 compliant

###### DisplayPort Dual-Mode

###### Dolby Digital, Dolby Digital EX, DTS, DTS-ES, DTS 96/24, LPCM up to 8 channels

###### DVI-D with adaptor

###### EDID

###### Common resolutions: 1920x1080 30Hz to 4096x2160 DCI 4K 4:4:4 30Hz

###### Custom resolutions: pixel clock rates up to 300 MHz

##### One (1) RJ45 shielded female connector with support for the following:

###### Link port for connection to compatible transmission device by same manufacturer

###### Power sharing option for connected remote transmission device

##### One (1) DC power input jack with support for:

###### Power connection shall support power sharing with connected transmission device

##### One (1) USB Type A connector with support for:

###### USB power port (500 mA maximum output)

###### Supplies power to a USB powered device

##### One (1) Control IR output port

##### One (1) Control IR input port

##### One (1) Control Serial port

### Power

#### Power supply modes:

##### Local or remote DC power source

##### Remote power supplied by opposite end of TX/RX pair.

### Compliance

#### UL Listed for US & Canada, CE, IC, FCC Part 15 Class B digital device

# EXECUTION

- NOT USED in this Guide Specification.  Specifier shall Specify PART 3 On-Site work as needed.

# APPENDICES

## SPECIFIED PRODUCTS

### Crestron HD-TX-101-C-1G-E

### Crestron HD-TX-101-C-E

### Crestron HD-TX-201-C-2G-E

### Crestron HD-TX-301-C-E

### Crestron HD-TXC-101-C-1G-E

### Crestron HD-TXC-101-C-E

### Crestron HDI-TX-301-C-2G-E