SECTION 27 41 16

INTEGRATED AUDIO-VIDEO SYSTEMS AND EQUIPMENT

GUIDE SPECIFICATION

Specifier: The Specifier/Design Professional is responsible for the accuracy of all project specifications, including system application and coordination with related sections.  This guide specification is provided as a convenience and requires editing to match actual project requirements.  CRESTRON ELECTRONICS, INC. SHALL NOT BE LIABLE FOR ANY DAMAGES ARISING OUT OF THE USE OF ANY OF ITS GUIDE SPECIFICATIONS.  For Crestron design assistance and design review please contact Sales Support Services Department at   800.237.2041 or techsales@crestron.com.

Specifier:  Please see PART 4 for a listing of products specified in this Guide Specification.

Table of Contents

1	GENERAL	3

2	PRODUCTS	3

2.1	Wireless Presentation Adaptor	3

2.1.1	Basis of Design	3

2.1.2	Device Definition	3

2.1.3	Device Architecture	4

2.1.4	Functions	5

2.1.5	Connectors	5

2.1.6	Controls and Indicators	6

2.1.7	Power	7

2.1.8	Compliance	7

3	EXECUTION	7

4	APPENDICES	7

4.1	SPECIFIED PRODUCTS	7

4.1.1	Crestron AM-TX3-200	7

# GENERAL

NOT USED in this Guide Specification.  Specifier shall Specify PART 1 administrative and procedural requirements as needed.

# PRODUCTS

## Wireless Presentation Adaptor

Specifier Note:

The AirMedia® Series 3 Connect Endpoint (AM-TX3-200) enables wireless presentation and conferencing when paired with a Wi-Fi® network capable AirMedia Series 3 Receiver. Deploy the AM-TX3-200 alongside a compatible AirMedia receiver in conference rooms, huddle spaces, classrooms, or almost any space to establish a productive meeting or learning environment.

### Basis of Design

#### Crestron AM-TX3-200

Specifier Note:

AM-TX3-200
https://www.crestron.com/Products/Workspace-Solutions/Wireless-Presentation-Solutions/AirMedia-Presentation-System-Accessories/AM-TX3-200

### Device Definition

#### Presentation Connection Endpoint with the following capabilities:

##### Interface with source device via USB or HDMI and connection to Wi‑Fi network capable wireless receiver from same manufacturer to provide access to conferencing peripherals and display device

Specifier Note:

Wireless Conferencing functionality through the AirMedia Connect Endpoint requires a Wi-Fi network and Wireless Conferencing compatible AirMedia Series 3 receiver.

##### Support for content up to 4K30 resolution

##### Flexible installation and deployment options

##### Versatile power options, including PoE, DC power, or USB-C via connected user device

Specifier Note:

When using USB-C to power the device, only the USB-C input is active. The AM-TX3-200 wired network interfaces will not function in this mode.

##### PoE+ passthrough under DC power to provide power to touch screen by same manufacturer

##### Communication to Wi-Fi network capable wireless receiver by same manufacturer via Wi-Fi Direct, infrastructure Wi-Fi, or wired network

##### No requirement for software or driver installation

##### Provisioning and management via software suite by same manufacturer

### Device Architecture

#### Environmental

##### Temperature: 32° to 104°F (0° to 40° C)

##### Humidity: 10% to 90% RH (non-condensing)

##### Heat Dissipation: 47.5 BTU/hr

#### Construction

##### Chassis: Plastic, black finish, with (4) integrated mounting holes, vented sides

##### Mounting: Freestanding, surface mount

##### Mounting Bracket: Metal

#### Dimensions (Device)

##### Height: 1.18 in. (30 mm)

##### Width: 7.87 in. (200 mm)

##### Depth: 7.08 in. (180 mm)

#### Dimensions (Device with Attached Mounting Bracket)

##### Height: 1.26 in. (32 mm)

##### Width: 8.85 in. (225 mm)

##### Depth: 7.08 in. (180 mm)

#### Weight

##### Device: 21.06 oz (597 g)

##### Mounting Bracket: 11.18 oz (317 g)

### Functions

#### Communications

##### Ethernet: 100/1000 Mbps, autoswitching, autonegotiating, full/half duplex, TCP/IP, UDP/IP, DHCP, SSL, TLS, 802.3af compliant

##### Wi-Fi

###### Dual-band Wi-Fi 6 802.11a/b/g/n/ac/ax (2.4 GHz & 5 GHz)

###### Up to 70 ft (23 m) range at 80 Mbps, subject to site-specific conditions

##### HDMI Input: HDCP 1.4, EDID

##### USB: USB 2.0

##### OS Support: Windows 10, Window 11, macOS® 11 or newer

#### Video

##### Input Signal Types: DisplayPort™ Alt Mode over USB-C, HDMI

##### Output Signal Types: Network stream, received by paired Wi-Fi network capable wireless receiver by same manufacturer

##### Maximum Resolution: 3840x2160@30Hz (2160p30); output resolution is determined by the HDMI® output on Wi-Fi network capable wireless receiver by same manufacturer

Specifier Note:

All video inputs will be scaled to the selected HDMI output resolution.

#### Audio

##### Input Signal Types: DisplayPort Alt Mode over USB-C, HDMI

##### Output Signal Type: Network stream, received by paired Wi-Fi network capable wireless receiver by same manufacturer

##### Input/Output Format: 2 channel LPCM

### Connectors

#### HDMI INPUT

##### (1) HDMI Type A connector

##### HDMI digital video/audio input

##### Connect personal device for content sharing

#### USB2 TO HOST

##### (1) USB-C connector, provides access to peripherals

##### Connect a USB-C to USB-A cable for use with laptops without USB-C

#### USB2 / DP-S TO HOST: (1) USB-C connector, supports presentation using DisplayPort Alt-Mode and access to peripherals

#### USB2 TO DEVICE: Reserved for future use

#### MAIN PoE PD: (1) 8-pin RJ45 connector; 100BASE-TX/1000BASE-T Ethernet port and PoE Class 0

#### AUX PoE PSE

##### (1) 8-pin RJ45 connector

##### 100BASE-TX/1000BASE-T Ethernet port and PoE+ Class 4

##### If enabled, connect a device such as a touch panel to provide passthrough network access. Provides PoE+ when the device is powered via DC power supply.

#### 24V 2.5A (1) 2.1 x 5.5 mm DC power connector; 24VDC power input

### Controls and Indicators

#### Power: (1) White LED, power supplied via the local power pack, PoE, or connected USB-C device

#### MAIN LAN: (1) White LED, indicates a network connection on the Main network interface

#### AUX LAN: (1) White LED, indicates a network connection on the Auxiliary network interface

#### VIDEO IN: (1) White LED indicates the presence of a USB-C or HDMI video signal

#### PERIPHERAL: (1) White LED indicates that conferencing peripherals are in use by the device

#### SETUP: (1) White LED and (1) push button for pairing to a Wi-Fi network capable wireless receiver by same manufacturer

#### RESET: (1) Recessed push button for restoring device to factory defaults

### Power

#### Power Supply

##### USB: Powered via USB-C connection to personal device

##### PoE: IEEE 802.3af Class 0 Powered Device

##### Power Pack

###### Input: 100-240 VAC, 50/60 Hz

###### Output: 2.5 A @ 24 VDC

#### Power Consumption (Idle): 4 W

#### Power Consumption (In use): 12.5 W

#### Power Consumption (In use and AUX PoE+ PSE passthrough): 45 W

### Compliance

#### Regulatory Model: M202106002

#### Intertek® Listed for US & Canada, CE, IC, FCC Part 15 Class B digital device

# EXECUTION

NOT USED in this Guide Specification.  Specifier shall Specify PART 3 On-Site work as needed.

# APPENDICES

## SPECIFIED PRODUCTS

Specifier Note: This Article includes Crestron products specified in this Guide Specification document.  This Article is for reference only and should not be required in actual project manual unless included in an overall system equipment list.

### Crestron AM-TX3-200