SECTION 27 41 16

INTEGRATED AUDIO-VIDEO SYSTEMS AND EQUIPMENT

GUIDE SPECIFICATION

Specifier: The Specifier/Design Professional is responsible for the accuracy of all project specifications, including system application and coordination with related sections.  This guide specification is provided as a convenience and requires editing to match actual project requirements.  CRESTRON ELECTRONICS, INC. SHALL NOT BE LIABLE FOR ANY DAMAGES ARISING OUT OF THE USE OF ANY OF ITS GUIDE SPECIFICATIONS.  For Crestron design assistance and design review please contact Sales Support Services Department at   800.237.2041 or techsales@crestron.com.

Specifer:  Please see PART 4 for a listing of products specified in this Guide Specification.

Table of Contents

1	GENERAL	4

1.1	SYSTEM DESCRIPTION	4

1.1.1	Skype Room	4

1.2	RELATED REQUIREMENTS	4

1.2.1	Section 25 08 00 - Commissioning of Integrated Automation	4

1.2.2	Section 25 10 00 - Integrated Automation Network Equipment	4

1.2.3	Section 25 11 13 - Integrated Automation Network Servers	4

1.2.4	Section 25 13 13 - Integrated Automation Control and Monitoring Network Supervisory Control	4

1.2.5	Section 25 13 16 - Integrated Automation Control and Monitoring Network Integration Panels	5

1.2.6	Section 25 13 19 - Integrated Automation Control and Monitoring Network Interoperability	5

1.2.7	Section 25 15 16 - Integrated Automation Software for Control and Monitoring Networks	5

1.2.8	Section 26 09 43.13 - Digital-Network Lighting Controls	5

1.2.9	Section 27 15 00 - Communications Horizontal Cabling	5

1.2.10	Section 27 41 00 - Audio-Video Systems	5

1.3	REFERENCES	5

1.3.1	Abbreviations and Acronyms	5

2	PRODUCTS	5

2.1	Amplifier	5

2.1.1	Basis of Design Amplifier:	5

2.1.2	Amplifier Input/Output Requirements	5

2.1.3	Audio Input and Output	6

2.2	User Interfaces	6

2.2.1	Touch Screen	6

2.3	Wall mounted Hallway Room Availability Sign	6

2.3.1	Basis of Design Room Availability SIgn:	6

2.3.2	Functional Requirements	6

2.3.3	Physical	7

2.4	Network Occupancy Sensor:	7

2.4.1	Basis of Design Occupancy Sensor:	7

2.4.2	Functional	7

2.4.3	Physical	7

2.5	UC Codec System	8

2.5.1	Basis of Design UC Products:	8

2.6	Speakers	8

2.6.1	Basis of Design Speaker:	8

2.6.2	Ceiling mounted 2-way speaker with the following specifications:	8

2.7	Display Device	9

2.7.1	Display device shall be controllable via a built-in RS-232 connection port.	9

2.7.2	Display device shall support 4K video through built-in HDMI input connector(s).	9

2.8	Computer	9

2.8.1	Computer or Laptop shall include an HDMI video output connection port.	9

2.9	HD Camera	9

2.9.1	Basis of Design USB Camera:	9

2.9.2	System shall include one High Definition video camera.	9

3	EXECUTION	9

4	APPENDICES	10

4.1	SPECIFIED PRODUCTS	10

4.1.1	Crestron TSW-760	10

4.1.2	Crestron SSW	10

4.1.3	Crestron CCS-UC-CODEC-300	10

4.1.4	Crestron CCS-CAM-USB-F-100	10

4.1.5	Crestron AMP-150-70	10

4.1.6	Crestron SAROS IC4T	10

4.1.7	Crestron GLS-ODT-C-CN	10

# GENERAL

## SYSTEM DESCRIPTION

### Skype Room

#### See AV System Drawings for device quantities and connection details.

Specifier Note: See example AV System Drawing for Skype Room

#### Display System

##### Video Display Device - Room shall include One video display.  Display shall have the capability of viewing any in room video source via the switching system.

#### AV Sources

##### Computer (OFE)

##### Laptop Computers

#### Audio System

##### Audio - Room shall include control of, audio sourcing, and amplification.

#### AV Systems Control

##### Control User Interface - Room shall include a table top touch screen user interface.

##### Device Controller - Device and equipment controller for scheduling and management servers.

SPECIFIER: Retain this article if the AV controller specified in this section is connected to or integrated with a system scheduling and management server such as Crestron FUSION, where the controller in this section provides interfacing and control of equipment being scheduled and managed.

##### Room System Control - Room system and scheduling through touch screen user interface(s) with on-screen information display viewable on room display(s).

Specifier: Retain this article if room system includes Fusion integration.

##### Wall Mounted Control User Interface - Room shall include a wall mounted scheduling and control user interface with an integrated room occupancy indicator sign.

## RELATED REQUIREMENTS

### Section 25 08 00 - Commissioning of Integrated Automation

### Section 25 10 00 - Integrated Automation Network Equipment

### Section 25 11 13 - Integrated Automation Network Servers

### Section 25 13 13 - Integrated Automation Control and Monitoring Network Supervisory Control

### Section 25 13 16 - Integrated Automation Control and Monitoring Network Integration Panels

### Section 25 13 19 - Integrated Automation Control and Monitoring Network Interoperability

### Section 25 15 16 - Integrated Automation Software for Control and Monitoring Networks

### Section 26 09 43.13 - Digital-Network Lighting Controls

### Section 27 15 00 - Communications Horizontal Cabling

### Section 27 41 00 - Audio-Video Systems

## REFERENCES

### Abbreviations and Acronyms

#### CEC:  Consumer Electronics Control

#### EDID:  Extended display identification data

#### HDCP:  High-bandwidth Digital Content Protection

#### HDMI:  High-Definition Multimedia Interface

#### IDF: Intermediate Distribution Frame

#### KSV: Key Selection Vector

#### MDF: Main Distribution Frame

#### NVP: Nominal Velocity of Propagation

# PRODUCTS

## Amplifier

### Basis of Design Amplifier:

#### Crestron AMP-150-70

### Amplifier Input/Output Requirements

#### Amplifier shall deliver a maximum 50 watts, high impedance.

#### Amplifier shall have an input sensitivity of 1.23V, +4dBu balanced for the rated power.

#### Output Impedance:  high impedance for 70V speaker configurations

#### Amplifier shall be a 1/4 width single rack space unit.

### Audio Input and Output

#### Amplifier shall provide two balanced line level input connections.

#### and one speaker level output connections

#### The amplifier shall provide 70V high impedance output for distributed speaker systems.

##### High impedance outputs shall be direct coupled.

## User Interfaces

### Touch Screen

#### Basis of Design Touch Screen User Interface:

##### Crestron TSW-760

#### Functions

##### Provide custom user interface with controls for AV systems, integrated hardware, and integrated scheduling and management system.

#### Communication:

##### Bidirectional 10/100 Mbps Ethernet communication.

#### Physical

##### 7 inch TFT active-matrix color LCD touch screen 1280 by 800 WXGA resolution display.

##### Power:  IEEE 802.3af Class 3 PoE Powered Device

## Wall mounted Hallway Room Availability Sign

### Basis of Design Room Availability SIgn:

#### Crestron SSW

### Functional Requirements

#### The sign shall illuminate green when the room is available and red when the room is occupied.

#### Custom RGB Color Mixing - The sign shall support custom colors through control system programming.

#### The sign shall support custom color alternating functions, color fade in and out functions, or blink on and off functions through custom system programming.

#### For room scheduling applications, the sign will connect to a compatible system touch screen via USB. No programming or configuration is required. The sign color shall be controlled by the Room Scheduling system by the same manufacturer.

### Physical

#### Sign shall mount to the wall outside a meeting room to indicate room status.

#### The sign shall include tri-color RGB LEDs to allow for a full spectrum of colors by blending the red, green, and blue elements in 1% increments per color.

#### Custom Engraving Option

##### The sign shall be available with or without custom engraving. The engraving shall appear on both sides of the acrylic sign face to identify the room by name or number. When illuminated, the engraving shall be visible at a distance of 50 feet or 15 meters.

## Network Occupancy Sensor:

### Basis of Design Occupancy Sensor:

#### Crestron GLS-ODT-C-CN

### Functional

#### Combination of ultrasonic motion detection and passive infrared detection with internal microprocessor.

#### Coverage:  360 deg., 2000 sq. ft.

#### Delayed time off adjustment.

#### Detects movement within space while reducing false triggering or shutoffs while space is occupied.

#### Sensor shall be independently adjustable for installed conditions.

#### Walk-through mode.

### Physical

#### Additional Interfacing: includes connection port for additional photocell.

#### Equipped with 4-wire interface for direct connection to control bus

#### Mounting:  3” octagon box

#### Photocell: Built-in ambient light photocell

#### Set-up and commissioning: parameters shall be configurable via a handheld wireless remote.

## UC Codec System

### Basis of Design UC Products:

#### Crestron CCS-UC-CODEC-300

#### Microsoft SRS

## Speakers

### Basis of Design Speaker:

#### Crestron SAROS IC4T

### Ceiling mounted 2-way speaker with the following specifications:

#### Performance:

##### Sensitivity: 85 dB at one meter, with 1-W input

##### Frequency Response: Within plus or minus 3 dB from 60 to 20,000 Hz

##### Frequency Range: Within minus 3 dB from 40 to 20,000Hz

##### Nominal Dispersion Angle: 120 degrees conical

##### Power Handling: 100 Watts Program at 8-Ohms.

##### Speaker unit shall be equipped for Low Impedance 8-Ohm or High Impedance 70V or 100V operation.

###### Accessible front mounted low impedance, high impedance selector switch.

##### High Impedance Line Transformer level taps at 70V and 100V:

###### 1.875 Watts; 70 Watt only

###### 3.75 Watts

###### 7.5 Watts

###### 15 Watts

###### 30 Watts

#### Enclosures: Aluminum, plenum rated.

#### Termination:

##### 4-pin screw down conductor terminal removable terminal block with speaker input and parallel pass-through connections.

##### Conduit knockouts:

###### Two 0.5-inch or 0.75-inch knockout

#### Grille:

##### Steel

##### Frameless magnetically affixed with safety tether.

##### Finish: Re-paintable, powder coated.

#### Drivers:

##### 4-inch polypropylene woofer with steel basket

##### .79-inch titanium dome tweeter

Specifier: Crestron Saros speakers utilize a tweeter protection method that does not re-quire replacement of internal components. In the event of the tweeter being “over-powered” the protection device will limit power to the tweeter as long as the condition exists. Many competitor speakers utilize a fuse or fuse-bulb type component, which is capable of absorbing some over-limit power, but will eventually rupture, requiring the entire unit to be removed and disassemble for replacement of the fuse/fuse-bulb.

##### Crossover Frequency: 2000 Hz.

## Display Device

### Display device shall be controllable via a built-in RS-232 connection port.

### Display device shall support 4K video through built-in HDMI input connector(s).

## Computer

### Computer or Laptop shall include an HDMI video output connection port.

## HD Camera

### Basis of Design USB Camera:

#### Crestron CCS-CAM-USB-F-100

### System shall include one High Definition video camera.

# EXECUTION

NOT USED in this Guide Specification. Specifier shall Specify PART 3 On-Site work as needed.

# APPENDICES

## SPECIFIED PRODUCTS

Specifier Note: This Article includes Crestron products specified in this Guide Specification document. This Article is for reference only and should not be required in actual project manual unless included in an overall system equipment list.

### Crestron TSW-760

### Crestron SSW

### Crestron CCS-UC-CODEC-300

### Crestron CCS-CAM-USB-F-100

### Crestron AMP-150-70

### Crestron SAROS IC4T

### Crestron GLS-ODT-C-CN