SECTION 27 41 16

INTEGRATED AUDIO-VIDEO SYSTEMS AND EQUIPMENT

GUIDE SPECIFICATION

Specifier: The Specifier/Design Professional is responsible for the accuracy of all project specifications, including system application and coordination with related sections.  This guide specification is provided as a convenience and requires editing to match actual project requirements.  CRESTRON ELECTRONICS, INC. SHALL NOT BE LIABLE FOR ANY DAMAGES ARISING OUT OF THE USE OF ANY OF ITS GUIDE SPECIFICATIONS.  For Crestron design assistance and design review please contact Sales Support Services Department at   800.237.2041 or techsales@crestron.com.

Specifier: Please see PART 4 for a listing of products specified in this Guide Specification.

Table of Contents

1	GENERAL	3

2	PRODUCTS	3

2.1	Wall Mounted Media Presentation Controller	3

2.1.1	Basis of Design	3

2.1.2	Device Definition	3

2.1.3	Device Architecture	4

2.1.4	Functions	4

2.1.5	Compliance	6

3	EXECUTION	7

4	APPENDICES	7

4.1	SPECIFIED PRODUCTS	7

4.1.1	Crestron MPC4-301	7

# GENERAL

NOT USED in this Guide Specification.  Specifier shall Specify PART 1 administrative and procedural requirements as needed.

# PRODUCTS

## Wall Mounted Media Presentation Controller

Specifier Note:

The MPC4-301 is a wall-mounted, PoE-powered 4-Series® Control System automated processor and control panel in one. Its customizable backlit capacitive buttons provide an ideal user interface for controlling AV and other functions. It offers fully-programmable functionality, supports web and cloud-based configuration and management, and integrates with XiO Cloud® software as part of a complete managed enterprise solution.

### Basis of Design

#### Crestron MPC4-301

Specifier Note:

MPC4-301
https://www.crestron.com/Products/Catalog/Control-and-Management/Control-System/Wall-Mount-Keypad/MPC4-301-B

### Device Definition

#### Wall Mount Media Presentation Controller with the following capabilities and features:

##### Control processor and control panel functionality

##### Fits into 2-gang U.S. electrical box; ability to be flush mounted

##### Customizable buttons, rotary volume controls, power/mute buttons

##### Fully programmable functionality

##### Web-based configuration software by same manufacturer

##### High-speed Ethernet LAN port

##### iPhone®, iPad®, and Android™ device control app support

##### Computer and web-based control

##### Support for cloud-based device provisioning and management software by same manufacturer

##### SNMP remote IT management support

##### Native BACnet™ network/IP support

##### Installer setup via software, web browser, or cloud

##### Drag-and-drop, C#, and symbol-based programming environments with full Unicode (multilanguage) support

##### Enterprise-grade security and authentication

##### IPv6 ready

##### PoE+ network powered

### Device Architecture

#### Environmental

##### Operating Temperature: 41° to 95°F (5° to 35° C)

##### Humidity: 10% to 90% RH (non-condensing)

##### Heat Dissipation: 23 BTU/hr

#### Construction

##### Chassis: Plastic with metal mounting plate, removable front panel with optional security screw

##### Flush Wall Mount: Mounts in a 2-gang U.S. electrical box

#### Dimensions

##### Height: 4.50 in. (114 mm)

##### Width: 5.00 in. (126 mm)

##### Depth: 2.40 in. (61 mm)

#### Weight: 12.8 oz (363 g)

### Functions

#### Control Engine

##### Control processor by same manufacturer

##### Real-time, preemptive multithreaded/multitasking kernel

##### Transaction-safe Extended FAT file system

##### Battery-backed nonvolatile real-time clock

##### Support for up to 10 simultaneously running programs

##### Native support for web-based configuration software by same manufacturer

#### Communications

##### Ethernet: 1000 Mbps, autoswitching, autonegotiating, autodiscovery, full/half duplex, TCP/IP, UDP/IP, CIP, DHCP, SSL, TLS, SSH, SFTP (SSH File Transfer Protocol), FIPS 140-2 compliant encryption, IEEE 802.1X SNMP, BACnet/IP2, IPv4 or IPv6, Active Directory authentication, SMTP email client, HTTPS web server, HTTPS web server, HTTPS web browser setup and device provisioning and monitoring program (by same manufacturer) client, IEEE802.3at Type 2 PoE+ compliant

#### Connectors

##### LAN PoE

###### (1) 8-pin RJ45 connector, female

###### 1000BASE-TX Ethernet port

###### PoE+ (Power over Ethernet Plus) PD (Powered Device) port

##### Ground

###### (1) 6-32 screw

###### Chassis ground lug (flying lead with ring terminal included)

#### Controls and Indicators

##### Assignable Buttons

###### (5) Push buttons, customizable via labels (25 predesignated labels and five blank included)

###### Custom laser-etched labels available in black or white

##### Mute: (1) Projected capacitive push button with mute icon

##### Volume: (1) Knob (continuous turn rotary encoder) for volume control

##### Volume Gauge: (1) Blue LED multisegment circular graph surrounding the volume knob for volume level indication

##### Power: (1) Projected capacitive push button labeled with power icon

##### HW-R: (1) Push button (behind front panel) for hardware reset (reboots the processor)

##### SW-R: (1) Push button (behind front panel) for software reset (restarts the software program)

##### LAN PoE: (2) LEDs (on rear panel LAN port), green LED indicates Ethernet link status, amber LED indicates Ethernet activity

##### Illumination

###### Each front panel button is individually backlit – white when inactive and blue when active

###### Autobrightness control adjusts all backlighting in three levels according to the ambient light level

###### Backlighting may be dimmed or extinguished when unit enters idle state

Specifier Note:

All front panel buttons and the volume knob are custom programmable.

#### Memory

##### SDRAM: 2 GB

#### Light Sensor: Photosensor detects the ambient light level to enable autobrightness control

#### Power

##### PoE (Power over Ethernet)

###### IEEE 802.3at Type 2 compliant PoE+ PD (Powered Device)

###### Requests 15 W from an 802.3at Type 2 PSE with LLDP advanced power management

###### Requests 30 W (PoE+ Class 4) from an 802.3at Type 2 PSE without LLDP

##### Available power via proprietary cabling from same manufacturer: 2.5 W (104 mA @ 24 VDC)

##### Power Consumption: 8.5 W typical

### Compliance

#### Regulatory Model: M201902003

#### UL® Standards Listed, CE, IC, FCC Part 15 Class B digital device; Class 1M laser product, complies with safety regulations of IEC-60825-1, FDA 21 CFR 1040.11, and FDA 21 CRF 1040.10

# EXECUTION

NOT USED in this Guide Specification.  Specifier shall Specify PART 3 On-Site work as needed.

# APPENDICES

## SPECIFIED PRODUCTS

Specifier Note: This Article includes Crestron products specified in this Guide Specification document.  This Article is for reference only and should not be required in actual project manual unless included in an overall system equipment list.

### Crestron MPC4-301