SECTION 27 41 16

INTEGRATED AUDIO-VIDEO SYSTEMS AND EQUIPMENT

GUIDE SPECIFICATION

Specifier: The Specifier/Design Professional is responsible for the accuracy of all project specifications, including system application and coordination with related sections.  This guide specification is provided as a convenience and requires editing to match actual project requirements.  CRESTRON ELECTRONICS, INC. SHALL NOT BE LIABLE FOR ANY DAMAGES ARISING OUT OF THE USE OF ANY OF ITS GUIDE SPECIFICATIONS.  For Crestron design assistance and design review please contact Sales Support Services Department at   800.237.2041 or techsales@crestron.com.

Specifier: Please see PART 4 for a listing of products specified in this Guide Specification.

Table of Contents

1	GENERAL	3

2	PRODUCTS	3

2.1	Video Conferencing Integration Solution	3

2.1.1	Basis of Design	3

2.1.2	Device Definition	3

2.1.3	Device Architecture	4

2.1.4	Functions	4

2.1.5	Compliance	7

2.2	Video Conferencing Integration Solution	7

2.2.1	Basis of Design	7

2.2.2	Device Definition	7

2.2.3	Device Architecture	8

2.2.4	Functions	8

2.2.5	Compliance	11

2.3	Video Conferencing Integration Solution	11

2.3.1	Basis of Design	11

2.3.2	Device Definition	11

2.3.3	Device Architecture	12

2.3.4	Functions	12

2.3.5	Compliance	14

2.4	Video Conferencing Integration Solution	15

2.4.1	Basis of Design	15

2.4.2	Device Definition	15

2.4.3	Device Architecture	16

2.4.4	Functions	16

2.4.5	Compliance	18

3	EXECUTION	19

4	APPENDICES	19

4.1	SPECIFIED PRODUCTS	19

4.1.1	Crestron CCC-110-T	19

4.1.2	Crestron CCC-110-Z	19

4.1.3	Crestron CCC-120-T	19

4.1.4	Crestron CCC-120-Z	19

# GENERAL

NOT USED in this Guide Specification.  Specifier shall Specify PART 1 administrative and procedural requirements as needed.

# PRODUCTS

## Video Conferencing Integration Solution

Specifier Note:

The CCC-110-T provides a customizable video conference solution for use with Microsoft Teams® Rooms software. Featuring the Intel® Core™ Ultra 5 processor, the CCC-110-T is an AI-ready solution for intelligent meeting spaces. The CCC-110-T supports up to two displays, four USB video and audio devices, and three TS(W)-1080-UC series touch screens (sold separately).

### Basis of Design

#### Crestron CCC-110-T

Specifier Note:

CCC-110-T
https://www.crestron.com/Products/Catalog/Unified-Communications/Foundation/Compute/CCC-110-T

### Device Definition

#### Video Conferencing Integration Solution with the following capabilities and features:

##### AI computing via processor

##### Single or dual video display support

##### Native support for Microsoft Teams® Rooms software

##### Support for up to four USB audio and video devices

###### Support for Intelligent Audio and Video devices by same manufacturer

##### PoE+ port to support connectivity with touch panel by same manufacturer

##### Bring Your Own Device (BYOD)

###### Presentation from user device via direct HDMI connection, or from user device via HDMI or USB-C extension solutions from same manufacturer

###### Platform agnostic video conferencing via USB-C connection to touch panel (by same manufacturer) associated with this device

##### Optional toolless mounting bracket accessory

##### Support for cloud-based provisioning service by same manufacturer

##### Enterprise grade security

### Device Architecture

#### Environmental

##### Temperature: 32° to 104°F (0° to 40° C)

##### Humidity: 10% to 95% RH (non-condensing)

##### Heat Dissipation: 360 BTU/hr idle, 430 BTU/hr typical; 553 BTU/hr maximum

#### Construction

##### Chassis: Metal, black finish, fan cooled; Vented front and rear

##### Mounting: Freestanding or wall/tabletop with mounting bracket by same manufacturer

#### Dimensions

##### Height: 1.72 in. (44 mm)

##### Width: 8.67 in. (220 mm)

##### Depth: 15.32 in. (389 mm)

#### Weight: 8.06 lb. (3.66 kg)

### Functions

#### Computer

##### Processor: Intel® Core™ Ultra 5 125H

##### RAM: 32 GB Dual Channel DDR5 SODIMM

##### Storage: 256 GB

##### Graphics: Intel® ARC

##### Network

###### 1x Intel I219-LM 100/1000 Mbps Ethernet

###### 2x Intel I210 100/1000 Mbps Ethernet

##### Operating System: Windows® 11 Enterprise IoT Edition OS

#### Communications

##### Ethernet: 100/1000 Mbps

##### USB Type-C® (DisplayPort Alt Mode) (Rear): HDCP 2.3, EDID, CEC, USB3

##### USB Type-A (Rear): USB 3 10 Gbps

##### USB Type-A (Front): USB 2 480 Mbps

##### Wi-Fi: Intel BE200 (Wi-Fi 7 2x2 MIMO)

##### Bluetooth® Communications: Bluetooth version 5.4

##### Proprietary HDBaseT-based communications format by same manufacturer: HDCP 2.3, EDID pass-through

##### HDMI: HDCP 2.3, EDID, CEC; supports management of HDCP and EDID

#### Buttons and Indicators

##### Power: (1) Push button with green LED backlight; for power on and off

##### Content In: (1) Green LED, indicates that a HDMI® source is detected

##### Output: (4) Green LEDs, indicates active connection(s) to displays

##### 1G (Rear)

###### (2) Bicolor green/amber and (2) Amber LEDs

###### Green/amber LED indicates Ethernet link status and connection speed

###### Amber LED indicates Ethernet activity

##### PoE+

###### (1) Bicolor green/amber and (1) Amber LED

###### Green/amber LED indicates Ethernet link status and connection speed

###### Amber LED indicates Ethernet activity

#### Connectors

##### USB Type-C (Rear)

###### (1) USB Type-C® 3.2 (Gen 2, 5V/3A) connector, female

###### USB 3 signal for connections to a 5K 21:9 display or USB 3 devices

###### DisplayPort™ single stream video inputs

##### HDMI (HDMI Out): (3) HDMI 2.0b connectors, female; HDMI digital video/audio output

##### HDMI (Content In): (1) HDMI 2.0b connector, female; HDMI digital video/audio input

##### USB Type-A (Rear): (5) USB 3 Type-A 10 Gbps connector, female; connects to cameras, microphones, DSPs, and accessories

##### USB Type-A (Front): (2) USB 2 Type-A connectors, female; connects to a keyboard, mouse for system configuration

##### Proprietary HDBaseT-based communications format by same manufacturer: (1) 8-pin RJ45 yellow connector, female, shielded; input port for connection to a transmitter using same communications format

##### LAN (1G): (2) RJ45 connectors, female; 100/1000 Mbps 100BASE-TX/1000BASE-T Ethernet ports

##### LAN (PoE+)

###### (1) RJ45 connector, female

###### PoE+ PSE (power sourcing equipment) port, IEEE 802.3at Type 2 (30 W) compliant

##### ANT-A & ANT-B

###### (2) SMA connectors for external Wi-Fi® antennas

###### Wi-Fi antennas included with 6.56 ft (2 m) cables

#### Power

##### 105 W idle, 126 W typical, 162 W maximum, 100-240 VAC

### Compliance

#### Regulatory Model: M202504001

#### FCC Part 15 Class B, IC Class B, CE

## Video Conferencing Integration Solution

Specifier Note:

The CCC-110-Z provides a customizable video conference solution for use with Zoom Rooms® software. Featuring the Intel® Core™ Ultra 5 processor, the CCC-110-Z is an AI-ready solution for intelligent meeting spaces. The CCC-110-Z supports up to three displays, four USB video and audio devices, and three TS(W)-1080-UC series touch screens (sold separately).

### Basis of Design

#### Crestron CCC-110-Z

Specifier Note:

CCC-110-Z
https://www.crestron.com/Products/Catalog/Unified-Communications/Foundation/Compute/CCC-110-Z

### Device Definition

#### Video Conferencing Integration Solution with the following capabilities and features:

##### AI computing via processor

##### Support for up to three video displays

##### Native support for Zoom Rooms® software

##### Support for up to four USB audio and video devices

###### Support for Intelligent Audio and Video devices by same manufacturer

##### PoE+ port to support connectivity with touch panel by same manufacturer

##### Bring Your Own Device (BYOD)

###### Presentation from user device via direct HDMI connection, or from user device via HDMI or USB-C extension solutions from same manufacturer

###### Platform agnostic video conferencing via USB-C connection to touch panel (by same manufacturer) associated with this device

##### Optional toolless mounting bracket accessory

##### Support for cloud-based provisioning service by same manufacturer

##### Enterprise grade security

### Device Architecture

#### Environmental

##### Temperature: 32° to 104°F (0° to 40° C)

##### Humidity: 10% to 95% RH (non-condensing)

##### Heat Dissipation: 360 BTU/hr idle, 430 BTU/hr typical; 553 BTU/hr maximum

#### Construction

##### Chassis: Metal, black finish, fan cooled; Vented front and rear

##### Mounting: Freestanding or wall/tabletop with mounting bracket by same manufacturer

#### Dimensions

##### Height: 1.72 in. (44 mm)

##### Width: 8.67 in. (220 mm)

##### Depth: 15.32 in. (389 mm)

#### Weight: 8.06 lb. (3.66 kg)

### Functions

#### Computer

##### Processor: Intel® Core™ Ultra 5 125H

##### RAM: 32 GB Dual Channel DDR5 SODIMM

##### Storage: 256 GB

##### Graphics: Intel® ARC

##### Network

###### 1x Intel I219-LM 100/1000 Mbps Ethernet

###### 2x Intel I210 100/1000 Mbps Ethernet

##### Operating System: Windows® 11 Enterprise IoT Edition OS

#### Communications

##### Ethernet: 100/1000 Mbps

##### USB Type-C® (DisplayPort Alt Mode) (Rear): HDCP 2.3, EDID, CEC, USB3

##### USB Type-A (Rear): USB 3 10 Gbps

##### USB Type-A (Front): USB 2 480 Mbps

##### Wi-Fi: Intel BE200 (Wi-Fi 7 2x2 MIMO)

##### Bluetooth® Communications: Bluetooth version 5.4

##### Proprietary HDBaseT-based communications format by same manufacturer: HDCP 2.3, EDID pass-through

##### HDMI: HDCP 2.3, EDID, CEC; supports management of HDCP and EDID

#### Buttons and Indicators

##### Power: (1) Push button with green LED backlight; for power on and off

##### Content In: (1) Green LED, indicates that a HDMI® source is detected

##### Output: (4) Green LEDs, indicates active connection(s) to displays

##### 1G (Rear)

###### (2) Bicolor green/amber and (2) Amber LEDs

###### Green/amber LED indicates Ethernet link status and connection speed

###### Amber LED indicates Ethernet activity

##### PoE+

###### (1) Bicolor green/amber and (1) Amber LED

###### Green/amber LED indicates Ethernet link status and connection speed

###### Amber LED indicates Ethernet activity

#### Connectors

##### USB Type-C (Rear)

###### (1) USB Type-C® 3.2 (Gen 2, 5V/3A) connector, female

###### USB 3 signal for connections to a 5K 21:9 display or USB 3 devices

###### DisplayPort™ single stream video inputs

##### HDMI (HDMI Out): (3) HDMI 2.0b connectors, female; HDMI digital video/audio output

##### HDMI (Content In): (1) HDMI 2.0b connector, female; HDMI digital video/audio input

##### USB Type-A (Rear): (5) USB 3 Type-A 10 Gbps connector, female; connects to cameras, microphones, DSPs, and accessories

##### USB Type-A (Front): (2) USB 2 Type-A connectors, female; connects to a keyboard, mouse for system configuration

##### Proprietary HDBaseT-based communications format by same manufacturer: (1) 8-pin RJ45 yellow connector, female, shielded; input port for connection to a transmitter using same communications format

##### LAN (1G): (2) RJ45 connectors, female; 100/1000 Mbps 100BASE-TX/1000BASE-T Ethernet ports

##### LAN (PoE+)

###### (1) RJ45 connector, female

###### PoE+ PSE (power sourcing equipment) port, IEEE 802.3at Type 2 (30 W) compliant

##### ANT-A & ANT-B

###### (2) SMA connectors for external Wi-Fi® antennas

###### Wi-Fi antennas included with 6.56 ft (2 m) cables

#### Power

##### 105 W idle, 126 W typical, 162 W maximum, 100-240 VAC

### Compliance

#### Regulatory Model: M202504001

#### FCC Part 15 Class B, IC Class B, CE

## Video Conferencing Integration Solution

Specifier Note:

The CCC-120-T provides a customizable video conference solution for use with Microsoft Teams® Rooms software. Featuring the Intel® Core™ Ultra 7 processor, the CCC-120-T is an AI-ready and scalable solution for intelligent meeting spaces. The CCC-120-T supports up to two displays, four USB video and audio devices, and three TS(W)-1080-UC series touch screens (sold separately).

### Basis of Design

#### Crestron CCC-120-T

Specifier Note:

CCC-120-T
https://www.crestron.com/Products/Catalog/Unified-Communications/Foundation/Compute/CCC-120-T

### Device Definition

#### Video Conferencing Integration Solution with the following capabilities and features:

##### AI computing via processor

##### Single or dual video display support

##### Native support for Microsoft Teams® Rooms software

##### Support for up to four USB audio and video devices

###### Support for Intelligent Audio and Video devices by same manufacturer

##### PoE+ port to support connectivity with touch panel by same manufacturer

##### Bring Your Own Device (BYOD)

###### Presentation from user device via direct HDMI connection, or from user device via HDMI or USB-C extension solutions from same manufacturer

###### Platform agnostic video conferencing via USB-C connection to touch panel (by same manufacturer) associated with this device

##### Optional toolless mounting bracket accessory

##### Support for cloud-based provisioning service by same manufacturer

##### Enterprise grade security

### Device Architecture

#### Environmental

##### Temperature: 32° to 104°F (0° to 40° C)

##### Humidity: 10% to 95% RH (non-condensing)

##### Heat Dissipation: 360 BTU/hr idle, 430 BTU/hr typical; 553 BTU/hr maximum

#### Construction

##### Chassis: Metal, black finish, fan cooled; Vented front and rear

##### Mounting: Freestanding or wall/tabletop with mounting bracket by same manufacturer

#### Dimensions

##### Height: 1.72 in. (44 mm)

##### Width: 8.67 in. (220 mm)

##### Depth: 15.32 in. (389 mm)

#### Weight: 8.06 lb. (3.66 kg)

### Functions

#### Computer

##### Processor: Intel® Core™ Ultra 7 155H

##### RAM: 32 GB Dual Channel DDR5 SODIMM

##### Storage: 256 GB

##### Graphics: Intel® ARC

##### Network

###### 1x Intel I219-LM 100/1000 Mbps Ethernet

###### 2x Intel I210 100/1000 Mbps Ethernet

##### Operating System: Windows® 11 Enterprise IoT Edition OS

#### Communications

##### Ethernet: 100/1000 Mbps

##### USB Type-C® (DisplayPort Alt Mode) (Rear): HDCP 2.3, EDID, CEC, USB3

##### USB Type-A (Rear): USB 3 10 Gbps

##### USB Type-A (Front): USB 2 480 Mbps

##### Wi-Fi: Intel BE200 (Wi-Fi 7 2x2 MIMO)

##### Bluetooth® Communications: Bluetooth version 5.4

##### Proprietary HDBaseT-based communications format by same manufacturer: HDCP 2.3, EDID pass-through

##### HDMI: HDCP 2.3, EDID, CEC; supports management of HDCP and EDID

#### Buttons and Indicators

##### Power: (1) Push button with green LED backlight; for power on and off

##### Content In: (1) Green LED, indicates that a HDMI® source is detected

##### Output: (4) Green LEDs, indicates active connection(s) to displays

##### 1G (Rear)

###### (2) Bicolor green/amber and (2) Amber LEDs

###### Green/amber LED indicates Ethernet link status and connection speed

###### Amber LED indicates Ethernet activity

##### PoE+

###### (1) Bicolor green/amber and (1) Amber LED

###### Green/amber LED indicates Ethernet link status and connection speed

###### Amber LED indicates Ethernet activity

#### Connectors

##### USB Type-C (Rear)

###### (1) USB Type-C® 3.2 (Gen 2, 5V/3A) connector, female

###### USB 3 signal for connections to a 5K 21:9 display or USB 3 devices

###### DisplayPort™ single stream video inputs

##### HDMI (HDMI Out): (3) HDMI 2.0b connectors, female; HDMI digital video/audio output

##### HDMI (Content In): (1) HDMI 2.0b connector, female; HDMI digital video/audio input

##### USB Type-A (Rear): (5) USB 3 Type-A 10 Gbps connector, female; connects to cameras, microphones, DSPs, and accessories

##### USB Type-A (Front): (2) USB 2 Type-A connectors, female; connects to a keyboard, mouse for system configuration

##### Proprietary HDBaseT-based communications format by same manufacturer: (1) 8-pin RJ45 yellow connector, female, shielded; input port for connection to a transmitter using same communications format

##### LAN (1G): (2) RJ45 connectors, female; 100/1000 Mbps 100BASE-TX/1000BASE-T Ethernet ports

##### LAN (PoE+)

###### (1) RJ45 connector, female

###### PoE+ PSE (power sourcing equipment) port, IEEE 802.3at Type 2 (30 W) compliant

##### ANT-A & ANT-B

###### (2) SMA connectors for external Wi-Fi® antennas

###### Wi-Fi antennas included with 6.56 ft (2 m) cables

#### Power

##### 105 W idle, 126 W typical, 162 W maximum, 100-240 VAC

### Compliance

#### Regulatory Model: M202504001

#### FCC Part 15 Class B, IC Class B, CE

## Video Conferencing Integration Solution

Specifier Note:

The CCC-120-Z provides a customizable video conference solution for use with Zoom Rooms® software. Featuring the Intel® Core™ Ultra 7 processor, the CCC-120-Z is an AI-ready and scalable solution for intelligent meeting spaces. The CCC-120-Z supports up to three displays, four USB video and audio devices, and three TS(W)-1080-UC series touch screens (sold separately).

### Basis of Design

#### Crestron CCC-120-Z

Specifier Note:

CCC-120-Z
https://www.crestron.com/Products/Catalog/Unified-Communications/Foundation/Compute/CCC-120-Z

### Device Definition

#### Video Conferencing Integration Solution with the following capabilities and features:

##### AI computing via processor

##### Support for up to three video displays

##### Native support for Zoom Rooms® software

##### Support for up to four USB audio and video devices

###### Support for Intelligent Audio and Video devices by same manufacturer

##### PoE+ port to support connectivity with touch panel by same manufacturer

##### Bring Your Own Device (BYOD)

###### Presentation from user device via direct HDMI connection, or from user device via HDMI or USB-C extension solutions from same manufacturer

###### Platform agnostic video conferencing via USB-C connection to touch panel (by same manufacturer) associated with this device

##### Optional toolless mounting bracket accessory

##### Support for cloud-based provisioning service by same manufacturer

##### Enterprise grade security

### Device Architecture

#### Environmental

##### Temperature: 32° to 104°F (0° to 40° C)

##### Humidity: 10% to 95% RH (non-condensing)

##### Heat Dissipation: 360 BTU/hr idle, 430 BTU/hr typical; 553 BTU/hr maximum

#### Construction

##### Chassis: Metal, black finish, fan cooled; Vented front and rear

##### Mounting: Freestanding or wall/tabletop with mounting bracket by same manufacturer

#### Dimensions

##### Height: 1.72 in. (44 mm)

##### Width: 8.67 in. (220 mm)

##### Depth: 15.32 in. (389 mm)

#### Weight: 8.06 lb. (3.66 kg)

### Functions

#### Computer

##### Processor: Intel® Core™ Ultra 7 155H

##### RAM: 32 GB Dual Channel DDR5 SODIMM

##### Storage: 256 GB

##### Graphics: Intel® ARC

##### Network

###### 1x Intel I219-LM 100/1000 Mbps Ethernet

###### 2x Intel I210 100/1000 Mbps Ethernet

##### Operating System: Windows® 11 Enterprise IoT Edition OS

#### Communications

##### Ethernet: 100/1000 Mbps

##### USB Type-C® (DisplayPort Alt Mode) (Rear): HDCP 2.3, EDID, CEC, USB3

##### USB Type-A (Rear): USB 3 10 Gbps

##### USB Type-A (Front): USB 2 480 Mbps

##### Wi-Fi: Intel BE200 (Wi-Fi 7 2x2 MIMO)

##### Bluetooth® Communications: Bluetooth version 5.4

##### Proprietary HDBaseT-based communications format by same manufacturer: HDCP 2.3, EDID pass-through

##### HDMI: HDCP 2.3, EDID, CEC; supports management of HDCP and EDID

#### Buttons and Indicators

##### Power: (1) Push button with green LED backlight; for power on and off

##### Content In: (1) Green LED, indicates that a HDMI® source is detected

##### Output: (4) Green LEDs, indicates active connection(s) to displays

##### 1G (Rear)

###### (2) Bicolor green/amber and (2) Amber LEDs

###### Green/amber LED indicates Ethernet link status and connection speed

###### Amber LED indicates Ethernet activity

##### PoE+

###### (1) Bicolor green/amber and (1) Amber LED

###### Green/amber LED indicates Ethernet link status and connection speed

###### Amber LED indicates Ethernet activity

#### Connectors

##### USB Type-C (Rear)

###### (1) USB Type-C® 3.2 (Gen 2, 5V/3A) connector, female

###### USB 3 signal for connections to a 5K 21:9 display or USB 3 devices

###### DisplayPort™ single stream video inputs

##### HDMI (HDMI Out): (3) HDMI 2.0b connectors, female; HDMI digital video/audio output

##### HDMI (Content In): (1) HDMI 2.0b connector, female; HDMI digital video/audio input

##### USB Type-A (Rear): (5) USB 3 Type-A 10 Gbps connector, female; connects to cameras, microphones, DSPs, and accessories

##### USB Type-A (Front): (2) USB 2 Type-A connectors, female; connects to a keyboard, mouse for system configuration

##### Proprietary HDBaseT-based communications format by same manufacturer: (1) 8-pin RJ45 yellow connector, female, shielded; input port for connection to a transmitter using same communications format

##### LAN (1G): (2) RJ45 connectors, female; 100/1000 Mbps 100BASE-TX/1000BASE-T Ethernet ports

##### LAN (PoE+)

###### (1) RJ45 connector, female

###### PoE+ PSE (power sourcing equipment) port, IEEE 802.3at Type 2 (30 W) compliant

##### ANT-A & ANT-B

###### (2) SMA connectors for external Wi-Fi® antennas

###### Wi-Fi antennas included with 6.56 ft (2 m) cables

#### Power

##### 105 W idle, 126 W typical, 162 W maximum, 100-240 VAC

### Compliance

#### Regulatory Model: M202504001

#### FCC Part 15 Class B, IC Class B, CE

# EXECUTION

NOT USED in this Guide Specification.  Specifier shall Specify PART 3 On-Site work as needed.

# APPENDICES

## SPECIFIED PRODUCTS

Specifier Note: This Article includes Crestron products specified in this Guide Specification document.  This Article is for reference only and should not be required in actual project manual unless included in an overall system equipment list.

### Crestron CCC-110-T

### Crestron CCC-110-Z

### Crestron CCC-120-T

### Crestron CCC-120-Z