# SECTION 27 41 16

INTEGRATED AUDIO-VIDEO SYSTEMS AND EQUIPMENT

Equipment Specified in this section:

DSP-860

DSP-1280

DSP-1281

DSP-1282

DSP-1283

Table of Contents

PART 1	GENERAL	3

1.1	SUMMARY	3

A.	Section Includes	3

PART 2	PRODUCTS	3

2.1	AUDIO DSP	3

A.	Audio Digital Signal Processor	3

B.	Physical	3

C.	Communications	3

D.	Audio I/O	4

E.	Audio Configuration	4

F.	Filters for all models shall include	4

G.	Auxiliary channel strips	5

H.	DSP Type 4 and DSP Type 5 shall include:	5

I.	DSP Type 3 and DSP Type 5 shall include:	5

J.	User Interface Export	5

K.	Compliance	6

L.	Warranty	6

PART 3	NOT USED	6

END OF SECTION 27 41 16	6

# SECTION 27 41 16

INTEGRATED AUDIO-VIDEO SYSTEMS AND EQUIPMENT

Specifier: The Specifier/Design Professional is responsible for the accuracy of all project specifications, including system application and coordination with related sections.  This guide specification is provided as a convenience and requires editing to match actual project requirements.  CRESTRON ELECTRONICS, INC. SHALL NOT BE LIABLE FOR ANY DAMAGES ARISING OUT OF THE USE OF ANY OF ITS GUIDE SPECIFICATIONS.  For Crestron design assistance and design review please contact Sales Support Services Department at   800.237.2041 or techsales@crestron.com.

GENERAL

SUMMARY

Section Includes

Audio Digital Signal Processor (DSP)

PRODUCTS

AUDIO DSP

Audio Digital Signal Processor

The Audio Digital Signal Processor (DSP) will be purpose built for professional use. Each DSP shall include an internal processor that will provide advance audio controls and shall operate with or without a control system.

Physical

The DSP’s shall be in a 1RU EIA standard enclosure for rack mounting.

Unit depth shall be 14.35 inches, 365mm

Internal fan cooling shall provide side-to-side airflow.

The DSP shall have a front panel bi-color LED to indicate unit status

Connection to AC mains

Connection to a LAN.

The front panel Reset shall return the unit to the last saved configuration

The rear panel LAN connector shall provide network communications

The DSP shall include a Universal Power Supply

Operational 50-60Hz, 100-240VAC.

Communications

The DSP’s will be configured, monitored and controlled by the DSP software tool.

The DSP software tool shall allow multiple instances to be active on a common network.

The DSP software tool channel strip architecture will not require compiling in order to pass audio.

Remote communications with the DSP software tool and Touch Panels by same manufacturer will be via standard Ethernet.

shall support: 10/100/1000 Mbps, auto-switching, auto-negotiating, auto-discovery, full/half duplex, TCP/IP, UDP/IP, CIP, DHCP, SSL, SSH, SFTP (SSH File Transfer Protocol)

The DSP shall provide a local Universal Serial Bus (USB) console port connection on a standard USB-B type connector.

Audio I/O

DSP 12X8 Type

Shall provide 12 balanced input connections with selectable phantom power for condenser microphones.

Shall provide 8 balanced output connections.

DSP 8x6 Type

Shall provide 8 balanced input connections with selectable phantom power for condenser microphones

Shall provide 6 balanced output connections.

All Types

Input connections shall be selectable for balanced line level analog audio signals on Phoenix style removable connectors.

Each individual channel shall have its own dedicated connection.

Analog to Digital and Digital to Analog Conversion

24 bit, 48kHz

Audio Configuration

The channel strip software shall be configurable for instant audio delivery without requiring configuration code compile.

Presets shall recall any system configuration

Views shall recall any system control screen configuration

Channel Strips shall allow for the creation of unique configurations that are repeatable and portable.

The Digital Signal Processor shall provide:

Spectrum analyzer

Gain controls (Input/Output)

Any-to-Any Matrix Routing

Matrix In/Out level trim

Matrix crosspoint level

Filters for all models shall include

Equalization

Limiters

Gates

Compressors

Crossovers

Delays

Hi Pass filters

Low Pass filters

Lo and Hi Shelf

Gating style Auto Mixer

Auxiliary channel strips

An additional 8 Auxiliary channel strips with comprehensive signal processing shall be provided.

All Auxiliary Strips shall be routable to any Matrix crosspoint or output Bus.

Specifier Note:

DSP TYPES

DSP TYPE 1 – Crestron DSP-860

DSP TYPE 2 – Crestron DSP-1280

DSP TYPE 3 – Crestron DSP-1281

DSP TYPE 4 – Crestron DSP-1282

DSP TYPE 5 – Crestron DSP-1283

DSP Type 4 and DSP Type 5 shall include:

Acoustic Echo Cancellation on all 12 input channels

Separate POTS connector RJ11

Separate VoIP connector RJ45

USB 2.0 audio 8x8 with type B connector

DSP Type 3 and DSP Type 5 shall include:

Dante audio connectivity

Separate Primary and Secondary Dante network ports, RJ45

Front panel 5 segment LED indicators referencing:

Microphone and line in

Line Out

User Interface Export

Touch Panels by same manufacturer shall integrate natively with the DSP for a rich graphic user interface.

Graphic control elements will be selectable and exportable directly from the DSP software tool.

The User Interface Export file shall enable the building of touch panels with a drag and drop process.

Compliance

The DSP shall be UL listed, Commercial Audio Equipment

Compliant with the RoHS directive.

Warranty

The product warranty shall be 3 years.

NOT USED

END OF SECTION 27 41 16