# SECTION 27 41 16

# INTEGRATED AUDIO-VIDEO SYSTEMS AND EQUIPMENT

Specifier: The Specifier/Design Professional is responsible for the accuracy of all project specifications, including system application and coordination with related sections.  This guide specification is provided as a convenience and requires editing to match actual project requirements.  CRESTRON ELECTRONICS, INC. SHALL NOT BE LIABLE FOR ANY DAMAGES ARISING OUT OF THE USE OF ANY OF ITS GUIDE SPECIFICATIONS.  For Crestron design assistance and design review please contact Sales Support Services Department at   800.237.2041 or techsales@crestron.com.

GENERAL

SUMMARY

Section includes:

Audio-Video Capture Device.

Related Sections:

Division 25 Section “Integrated Automation Software for Control and Monitoring Networks” for server software and control hardware.

Division 25 Section “Integrated Automation Network Servers” for server hardware.

Division 27 Section “Audio-Video Systems” for audio-video hardware.

PRODUCTS

MANUFACTURERS

Basis-of-Design Manufacturer:  Subject to compliance with requirements, provide products of Crestron Electronics, Inc., Rockleigh, NJ 07647,  Phone (800)237-2041, Fax: (201)767-1903, www.crestron.com [or comparable products from a single manufacturer approved by Architect prior to bidding], with the following components and characteristics.

DIGITAL CAPTURE DEVICE

Basis of design products:

Crestron CAPTURE-HD

Crestron CAPTURE-HD-PRO

Description

Capture device shall be a hardware appliance capable of stand alone audio and video capture operation.

Capture device shall not require network connection for audio and video capture to internal memory.

Capture device shall include a built-in audio and video confidence monitor output.

Capture device shall include built-in video scaling and processing circuitry.

Capture device shall include built-in audio mixing and level control.

Capture device parameters shall be accessible by hardware control processor by same manufacturer.  Minimum parameters specified below.

Capture devices requiring subscriptions to enable standard requirements as specified below shall not be accepted.

Performance Criteria

Input and output requirements

Video Inputs

Content:

HDMI, DisplayPort Multimode, DVI, RGBHV

Camera:

Composite, SDI

CAPTURE-HD-PRO includes addition of 3G-SDI camera input.

Audio Inputs

Content:

HDMI

Balanced analog stereo

Unbalanced analog stereo

Speech:

Balanced line level

Unbalanced line level

Video Outputs

HDMI

Audio Outputs

HDMI

Balanced analog stereo

Unbalanced analog stereo

Video processing

Video input scaling

Picture in picture processing

Audio processing

Audio input mixing

Capture Method

File Format:

Video: H.264 high profile @ 720p24, 720p30, 720p60, and 1080p24

Audio: AAC stereo

Bitrate: 2000 to 6000 kbps

File Container:

MPEG-2 transport stream (.ts)

XML (.xml)

File Storage

Built-in memory card slots:

Rear Panel:

MMC flash memory card slot

USB flash memory device slot

Front Panel:

USB flash memory device slot

Network – based on schedule the capture device shall automatically upload captured content to designated network location.

File Delivery

FTP upload to network drive.

Physical removal of internal memory devices.

Live Streaming

Real Time Streaming Protocol (RTSP)

Formats:

H.264 high profile:

720p10

720p15

720p30

720p60

1080p10

1080p15

1080p30

Bitrates:

500 to 6000 kbps

User Control

Front panel control

Remote control - Type 1:

Basis of design product:

Crestron CAPTURE-TPMC-4SM

Touch screen with direct bidirectional communication with CAPTURE device.

Remote control - Type 2:

Basis of design product:

Crestron 2-Series processor

Crestron 3-Series processor

Crestron touch screen

Crestron CRESTRON-MOBILE-PRO

Direct bidirectional communication and control through separate control processor enabling the following:

User interface control:

Wall mount touch screen user interfaces

Wired touch screen user interfaces

Wireless touch screen user interfaces

Mobile device user interfaces

Apple iPad, iphone, itouch, Android 1.6

Control, scheduling and management

Basis of design product:

Crestron SW-FUSION-RV

Communication

Ethernet

10/100/1000Base-T

Auto-switching

Auto-negotiating

Auto-discovery

Full/half duplex

DHCP

Power

External power supply

Environmental

Operating Temperature:

32-104 degrees Fahrenheit (0-40 degrees Celsius)

Humidity:

10-90 percent Relative Humidity (non-condensing)

Heat Dissipation:

69 BTU/hour

Physical architecture

Capture device shall not exceed:

Freestanding or 1U 19-inch rack-mountable

Weight: less than 4.3 lb. (2 kilograms)

EXECUTION

CLOSEOUT ACTIVITIES

Demonstration:  Schedule demonstration with Owner.

Training:  Train Owner's personnel to operate, and maintain capture device.

# END OF SECTION 27 24 23