SECTION 27 41 16

INTEGRATED AUDIO-VIDEO SYSTEMS AND EQUIPMENT

GUIDE SPECIFICATION

Specifier: The Specifier/Design Professional is responsible for the accuracy of all project specifications, including system application and coordination with related sections.  This guide specification is provided as a convenience and requires editing to match actual project requirements.  CRESTRON ELECTRONICS, INC. SHALL NOT BE LIABLE FOR ANY DAMAGES ARISING OUT OF THE USE OF ANY OF ITS GUIDE SPECIFICATIONS.  For Crestron design assistance and design review please contact Sales Support Services Department at   800.237.2041 or techsales@crestron.com.

Specifer:  Please see PART 4 for a listing of products specified in this Guide Specification.

Table of Contents

1	GENERAL	3

1.1	SYSTEM DESCRIPTION	3

1.1.1	Huddle Room	3

1.2	RELATED REQUIREMENTS	3

1.2.1	Section 25 08 00 - Commissioning of Integrated Automation	3

1.2.2	Section 25 10 00 - Integrated Automation Network Equipment	3

1.2.3	Section 25 11 13 - Integrated Automation Network Servers	3

1.2.4	Section 25 13 13 - Integrated Automation Control and Monitoring Network Supervisory Control	3

1.2.5	Section 25 13 16 - Integrated Automation Control and Monitoring Network Integration Panels	3

1.2.6	Section 25 13 19 - Integrated Automation Control and Monitoring Network Interoperability	3

1.2.7	Section 25 15 16 - Integrated Automation Software for Control and Monitoring Networks	3

1.2.8	Section 26 09 43.13 - Digital-Network Lighting Controls	3

1.2.9	Section 27 15 00 - Communications Horizontal Cabling	3

1.2.10	Section 27 41 00 - Audio-Video Systems	3

1.3	REFERENCES	4

1.3.1	Abbreviations and Acronyms	4

2	PRODUCTS	4

2.1	Wireless Presentation Gateway	4

2.1.1	Basis of Design Wireless Presentation Gateway:	4

2.2	Display Device	4

2.2.1	Display device shall be controllable via a built-in RS-232 connection port.	4

2.2.2	Display device shall support 4K video through built-in HDMI input connector(s).	4

2.3	Computer	4

2.3.1	Computer or Laptop shall include an HDMI video output connection port.	4

2.4	HD Scaling Auto-Switcher and Extender	4

2.4.1	Basis of Design HD Scaling Auto-Switcher and Extender	4

2.4.2	The transmitter shall meet the following minimum requirements:	4

2.4.3	The receiver shall meet the following minimum requirements:	5

3	EXECUTION	6

4	APPENDICES	6

4.1	SPECIFIED PRODUCTS	6

4.1.1	Crestron AM-101	6

4.1.2	Crestron HD-MD-400-C-E	6

# GENERAL

## SYSTEM DESCRIPTION

### Huddle Room

#### See AV System Drawings for device quantities and connection details.

Specifier Note: See example AV System Drawing for Huddle Room

#### Display System

##### Video Display Devices - Room shall include one video display.  Display will have the capability of viewing in room video sources.

#### AV Sources

##### Laptop Computer

##### Wireless Presentation Device

#### Audio System

##### Audio shall be provided by display speakers.

#### AV Systems Control

## RELATED REQUIREMENTS

### Section 25 08 00 - Commissioning of Integrated Automation

### Section 25 10 00 - Integrated Automation Network Equipment

### Section 25 11 13 - Integrated Automation Network Servers

### Section 25 13 13 - Integrated Automation Control and Monitoring Network Supervisory Control

### Section 25 13 16 - Integrated Automation Control and Monitoring Network Integration Panels

### Section 25 13 19 - Integrated Automation Control and Monitoring Network Interoperability

### Section 25 15 16 - Integrated Automation Software for Control and Monitoring Networks

### Section 26 09 43.13 - Digital-Network Lighting Controls

### Section 27 15 00 - Communications Horizontal Cabling

### Section 27 41 00 - Audio-Video Systems

## REFERENCES

### Abbreviations and Acronyms

#### CEC:  Consumer Electronics Control

#### EDID:  Extended display identification data

#### HDCP:  High-bandwidth Digital Content Protection

#### HDMI:  High-Definition Multimedia Interface

#### IDF: Intermediate Distribution Frame

#### KSV: Key Selection Vector

#### MDF: Main Distribution Frame

#### NVP: Nominal Velocity of Propagation

# PRODUCTS

## Wireless Presentation Gateway

### Basis of Design Wireless Presentation Gateway:

#### Crestron AM-101

## Display Device

### Display device shall be controllable via a built-in RS-232 connection port.

### Display device shall support 4K video through built-in HDMI input connector(s).

## Computer

### Computer or Laptop shall include an HDMI video output connection port.

## HD Scaling Auto-Switcher and Extender

### Basis of Design HD Scaling Auto-Switcher and Extender

#### Crestron HD-MD-400-C-E

### The transmitter shall meet the following minimum requirements:

#### Ethernet Port

##### One (1) 8-wire RJ-45

###### 10/100 Mbps, auto-switching, auto negotiating, auto-discovery, full/half duplex, DHCP

#### One (1) analog stereo audio input:

##### (1) 3.5mm TRS (L/R unbalanced)

#### One (1) DB15 input:

##### Component (YPbPr)

##### RGB

#### One (1) HDMI video, audio input:

##### Supports DisplayPort Dual-Mode

##### Supports Dolby Digital, Dolby Digital EX, DTS, DTS-ES, DTS 96/24, LPCM up to 8 channels

##### Supports DVI-D with adaptor

##### Supports HDMI

#### Power supply modes:

##### Local or remote DC power source.

##### Remote power supplied by opposite end of TX/RX pair.

#### Single UTP/STP cable transmission connection

##### Signal transmission up to 230 feet.

##### Supports CAT5e.

### The receiver shall meet the following minimum requirements:

#### HDMI digital video, audio, and control output:

##### One (1) 19-pin Type A HDMI female connector

#### HDMI digital video and audio input:

##### One (1) 19-pin Type A HDMI female connector

#### Auto Source Switching

#### Supports HDCP

#### HDMI audio Support: Dolby Digital, Dolby Digital EX, DTS, DTS-ES, DTS 96/24, LPCM up to 8 channels

#### Stereo analog audio output

#### CEC device control.

#### RS-232 control

# EXECUTION

NOT USED in this Guide Specification. Specifier shall Specify PART 3 On-Site work as needed.

# APPENDICES

## SPECIFIED PRODUCTS

Specifier Note: This Article includes Crestron products specified in this Guide Specification document. This Article is for reference only and should not be required in actual project manual unless included in an overall system equipment list.

### Crestron AM-101

### Crestron HD-MD-400-C-E