SECTION 27 41 16

INTEGRATED AUDIO-VIDEO SYSTEMS AND EQUIPMENT

GUIDE SPECIFICATION

Specifier: The Specifier/Design Professional is responsible for the accuracy of all project specifications, including system application and coordination with related sections.  This guide specification is provided as a convenience and requires editing to match actual project requirements.  CRESTRON ELECTRONICS, INC. SHALL NOT BE LIABLE FOR ANY DAMAGES ARISING OUT OF THE USE OF ANY OF ITS GUIDE SPECIFICATIONS.  For Crestron design assistance and design review please contact Sales Support Services Department at   800.237.2041 or techsales@crestron.com.

Specifier:  Please see PART 4 for a listing of products specified in this Guide Specification.

Table of Contents

PART 1	GENERAL	3

PART 2	PRODUCTS	3

2.1	CEILING SPEAKER TYPE 1:	3

A.	Basis of Design:	3

B.	Device Architecture	3

C.	Functions	4

D.	Compliance	4

2.2	CEILING SPEAKER TYPE 2:	4

A.	Basis of Design:	4

B.	Device Architecture	4

C.	Functions	5

D.	Compliance	6

2.3	CEILING SPEAKER TYPE 3:	6

A.	Basis of Design:	6

B.	Device Architecture	6

C.	Functions	7

D.	Compliance	7

2.4	CEILING SPEAKER TYPE 4:	7

A.	Basis of Design:	7

B.	Device Architecture	7

C.	Functions	8

D.	Compliance	9

2.5	CEILING SPEAKER TYPE 5:	9

A.	Basis of Design:	9

B.	Device Architecture	9

C.	Functions	10

D.	Compliance	10

PART 3	EXECUTION	10

PART 4	APPENDICES	10

4.1	SPECIFIED PRODUCTS	10

A.	Crestron ULTIMATE IC6	10

B.	Crestron ULTIMATE IC6-AW	10

C.	Crestron ULTIMATE IC8-AW	10

D.	Crestron ULTIMATE IC8-DT-AW	10

E.	Crestron ULTIMATE ICS8-AW	10

GENERAL

NOT USED in this Guide Specification.  Specifier shall Specify PART 1 administrative and procedural requirements as needed.

PRODUCTS

CEILING SPEAKER TYPE 1:

Specifier Note: The ULTIMATE IC6‑W‑T‑EACH 2-way in‑ceiling speaker features a 6.5 in. (165 mm) carbon fiber woofer for deep, defined low end and articulate midrange, and a 1 in. (25 mm) ceramic coated aluminum tweeter for crisp highs with consistent coverage across a wide listening area. Crestron® Ultimate speakers made by Origin Acoustics deliver the pinnacle of performance for demanding residential installations. Select materials, top shelf components and a high-end appearance underscore the design behind each model.

Basis of Design:

Crestron ULTIMATE IC6

Device Architecture

Baffle: Glass fiber reinforced ABS

Grille:

Electroplated and powder coated steel, paintable.

Magnetically held bezel design.

Mounting:

Tool-less bayonet mounting system.

Cutout diameter: 8.75 inches (222 mm).

Environmental Operating Conditions:

Temperature: 4° to 176°F (-15.5° to 80°C)

Humidity: 10% to 90% RH (non-condensing)

For indoor use only

Optional Accessories:

Flush mount kit

Pre-construction mounting kit

New construction template

Fire rated back box

Acoustic back can

Termination:

Two (2) Gold-plated spring terminals

Maximum wire size: 10 AWG (5.26 mm^2)

Dimensions:

Diameter without grille: 9.61 in. (244 mm)

Depth: 4.13 in. (105 mm)

Round Grille: 10.20 in. (259 mm)

Weight: 4.8 lb. (2.17 kg)

Drivers:

6.5-inch (165 mm) carbon fiber woofer

1-inch (25 mm) ceramic coated aluminum tweeter

Functions

Crossover: 12 dB/oct

Crossover point: 30 dB/oct at 3.5 kHz

Impedance: 8 Ω nominal

Frequency Response: 52 Hz to 30 kHz (± 3 dB)

Frequency Range: 45 Hz to 30 kHz (± 10 dB)

Power Handling

Program: 150 W

Peak: 300 W

Sensitivity: 86 dB at 1 W / 1 m

Compliance

CE

RoHs

FCC Part 15 Class B digital device

Regulatory Model M202046006

CEILING SPEAKER TYPE 2:

Specifier Note: The ULTIMATE IC6‑AW-W‑T‑EACH 2-way all weather in‑ceiling speaker features a 6.5 in. (165 mm) carbon fiber woofer for deep, defined low end and articulate midrange, and a 1 in. (25 mm) ceramic coated aluminum tweeter for crisp highs with consistent coverage across a wide listening area. Crestron® Ultimate speakers made by Origin Acoustics deliver the pinnacle of performance for demanding residential installations. Select materials, top shelf components and a high-end appearance underscore the design behind each model.

Basis of Design:

Crestron ULTIMATE IC6-AW

Device Architecture

Baffle: Glass fiber reinforced ABS

Grille:

Electroplated and powder coated steel, paintable.

Magnetically held bezel design.

Mounting:

Tool-less bayonet mounting system.

Cutout diameter: 8.75 inches (222 mm).

Environmental Operating Conditions:

Temperature: 4° to 220°F (-15.5° to 104°C)

Humidity: 10% to 90% RH (non-condensing)

IP66 rated for water

Optional Accessories:

Flush mount kit

Pre-construction mounting kit

New construction template

Fire rated back box

Acoustic back can

Termination:

Two (2) Gold-plated spring terminals

Maximum wire size: 10 AWG (5.26 mm^2)

Dimensions:

Diameter without grille: 9.61 in. (244 mm)

Depth: 3.70 in. (105 mm)

Round Grille: 10.20 in. (259 mm)

Weight: 4.8 lb. (2.17 kg)

Drivers:

6.5-inch (165 mm) carbon fiber woofer

1-inch (25 mm) ceramic coated aluminum tweeter

Functions

Crossover: 12 dB/oct

Crossover point: 30 dB/oct at 3.5 kHz

Impedance: 8 Ω nominal

Frequency Response: 52 Hz to 30 kHz (± 3 dB)

Frequency Range: 45 Hz to 30 kHz (± 10 dB)

Power Handling

Program: 150 W

Peak: 300 W

Sensitivity: 86 dB at 1 W / 1 m

Compliance

CE

RoHs

FCC Part 15 Class B digital device

Regulatory Model M202046006

CEILING SPEAKER TYPE 3:

Specifier Note: The ULTIMATE IC8-AW-W-T-EACH 2-way all weather in‑ceiling speaker features an 8 in. (203 mm) carbon fiber woofer for deep, defined low end and articulate midrange, and a 1 in. (25 mm) ceramic coated aluminum tweeter for crisp highs with consistent coverage across a wide listening area. Crestron® Ultimate speakers made by Origin Acoustics deliver the pinnacle of performance for demanding residential installations. Select materials, top shelf components and a high-end appearance underscore the design behind each model.

Basis of Design:

Crestron ULTIMATE IC8-AW

Device Architecture

Baffle: Aluminum, brushed and anodized

Grille:

Electroplated and powder coated steel, paintable.

Magnetically held bezel design.

Mounting:

Tool-less bayonet mounting system.

Cutout diameter: 8.75 inches (222 mm).

Environmental Operating Conditions:

Temperature: 4° to 220°F (-15.5° to 104°C)

Humidity: 10% to 90% RH (non-condensing)

For indoor use only

IP66 rated for water

Optional Accessories:

Flush mount kit

Pre-construction mounting kit

New construction template

Fire rated back box

Acoustic back can

Termination:

One (1) 2-pin 5.08 mm pitch 10 A detachable terminal block

Maximum wire size: 12 AWG (3.31 mm^2)

Dimensions:

Diameter without grille: 9.61 in. (244 mm)

Depth: 4.64 in. (105 mm)

Round Grille: 10.20 in. (259 mm)

Weight: 5 lb. (2.26 kg)

Drivers:

8-inch (203 mm) carbon fiber woofer

1-inch (25 mm) ceramic coated aluminum tweeter

Functions

Crossover: 12 dB/oct

Crossover point: 30 dB/oct at 3.5 kHz

Impedance: 8 Ω nominal

Frequency Response: 45 Hz to 30 kHz (± 3 dB)

Frequency Range: 35 Hz to 30 kHz (± 10 dB)

Power Handling

Program: 150 W

Peak: 300 W

Sensitivity: 86 dB at 1 W / 1 m

Compliance

CE

RoHs

FCC Part 15 Class B digital device

CEILING SPEAKER TYPE 4:

Specifier Note: The ULTIMATE IC8‑DT-AW-W‑T‑EACH stereo 2-way all weather in-ceiling dual tweeter speaker features an 8 in. (203 mm) carbon fiber woofer for deep, defined low end and articulate midrange, and dual 1 in. (25 mm) ceramic coated aluminum dome tweeters for crisp highs with consistent coverage across a wide listening area. Crestron® Ultimate speakers made by Origin Acoustics deliver the pinnacle of performance for demanding residential installations. Select materials, top shelf components and a high-end appearance underscore the design behind each model.

Basis of Design:

Crestron ULTIMATE IC8-DT-AW

Device Architecture

Baffle: Glass fiber reinforced ABS

Grille:

Electroplated and powder coated steel, paintable.

Magnetically held bezel design.

Mounting:

Tool-less bayonet mounting system.

Cutout diameter: 8.66 inches (220 mm).

Environmental Operating Conditions:

Temperature: 4° to 176°F (-15.5° to 80°C)

Humidity: 10% to 90% RH (non-condensing)

IP66 rated for water

Optional Accessories:

Flush mount kit

Pre-construction mounting kit

New construction template

Fire rated back box

Termination:

Two (2) 2-pin pluggable screw terminals

Maximum wire size: 12 AWG (3.31 mm^2)

Dimensions:

Diameter without grille: 9.61 in. (244 mm)

Depth: 4.29 in. (109 mm)

Round Grille: 10.20 in. (259 mm)

Weight: 5.29 lb. (2.39 kg)

Drivers:

8-inch (203 mm) carbon fiber woofer

Two (2) 1-inch (25 mm) ceramic coated aluminum dome tweeters

Functions

Crossover: 12 dB/oct

Crossover point: 30 dB/oct at 3.5 kHz

Impedance: 8 Ω nominal

Frequency Response: 38 Hz to 20 kHz (± 3 dB)

Frequency Range: 35 Hz to 30 kHz (± 10 dB)

Power Handling

Program: 150 W

Peak: 300 W

Sensitivity: 86 dB at 1 W / 1 m

Compliance

CE

RoHs

FCC Part 15 Class B digital device

CEILING SPEAKER TYPE 5:

Specifier Note: The ULTIMATE ICS8‑AW-W‑T‑EACH is an in-ceiling subwoofer designed to produce deep, gratifying bass in any room. Crestron® Ultimate speakers made by Origin Acoustics deliver the pinnacle of performance for demanding residential installations.

Basis of Design:

Crestron ULTIMATE ICS8-AW

Device Architecture

Baffle: Fiber reinforced polypropylene

Grille:

Electroplated and powder coated steel, paintable.

Magnetically held bezel design.

Mounting:

Tool-less bayonet mounting system.

Cutout diameter: 8.75 inches (222 mm).

Environmental Operating Conditions:

Temperature: 4° to 176°F (-15.5° to 80°C)

Humidity: 10% to 90% RH (non-condensing)

For indoor use only

IP66 rated for water

Optional Accessories:

Flush mount kit

Pre-construction mounting kit

New construction template

Fire rated back box

Termination:

Two (2) Gold-plated spring terminals

Maximum wire size: 10 AWG (6 mm^2)

Dimensions:

Diameter without grille: 9.61 in. (244 mm)

Depth: 4.72 in. (109 mm)

Round Grille: 10.20 in. (259 mm)

Weight: 5 lb. (2.26 kg)

Drivers:

8-inch (203 mm) carbon fiber woofer

Functions

Low-Pass Filter: 150 Hz and above, Custom 2nd order

Impedance: 8 Ω nominal

Frequency Response: 45 Hz to 150 Hz (± 3 dB)

Frequency Range: 35 Hz to 175 Hz (± 10 dB)

Power Handling

Program: 200 W

Peak: 400 W

Sensitivity: 86 dB at 1 W / 1 m

Compliance

CE

RoHs

FCC Part 15 Class B digital device

Regulatory Model M202046009

EXECUTION

NOT USED in this Guide Specification.  Specifier shall Specify PART 3 On-Site work as needed.

APPENDICES

SPECIFIED PRODUCTS

Crestron ULTIMATE IC6

Crestron ULTIMATE IC6-AW

Crestron ULTIMATE IC8-AW

Crestron ULTIMATE IC8-DT-AW

Crestron ULTIMATE ICS8-AW