SECTION 27 41 16

INTEGRATED AUDIO-VIDEO SYSTEMS AND EQUIPMENT

GUIDE SPECIFICATION

Specifier: The Specifier/Design Professional is responsible for the accuracy of all project specifications, including system application and coordination with related sections.  This guide specification is provided as a convenience and requires editing to match actual project requirements.  CRESTRON ELECTRONICS, INC. SHALL NOT BE LIABLE FOR ANY DAMAGES ARISING OUT OF THE USE OF ANY OF ITS GUIDE SPECIFICATIONS.  For Crestron design assistance and design review please contact Sales Support Services Department at   800.237.2041 or techsales@crestron.com.

Specifier:  Please see PART 4 for a listing of products specified in this Guide Specification.

Table of Contents

1	GENERAL	4

2	PRODUCTS	4

2.1	8.1 in. Wall Mount Touch Screen	4

2.1.1	Basis of Design	4

2.1.2	Device Definition	4

2.1.3	Device Architecture	5

2.1.4	Functions	5

2.1.5	Compliance	9

2.2	10.1 in. Wall Mount Touch Screen	9

2.2.1	Basis of Design	9

2.2.2	Device Definition	9

2.2.3	Device Architecture	10

2.2.4	Functions	11

2.2.5	Compliance	14

2.3	8.1 in. Tabletop Touch Screen	14

2.3.1	Basis of Design	15

2.3.2	Device Definition	15

2.3.3	Device Architecture	15

2.3.4	Functions	16

2.3.5	Compliance	19

2.4	10.1 in. Tabletop Touch Screen	20

2.4.1	Basis of Design	20

2.4.2	Device Definition	20

2.4.3	Device Architecture	21

2.4.4	Functions	21

2.4.5	Compliance	25

2.5	8.1 in. Wall Mount Touch Screen	25

2.5.1	Basis of Design	25

2.5.2	Device Definition	25

2.5.3	Device Architecture	25

2.5.4	Functions	26

2.5.5	Compliance	29

2.6	10.1 in. Wall Mount Touch Screen	29

2.6.1	Basis of Design	29

2.6.2	Device Definition	30

2.6.3	Device Architecture	30

2.6.4	Functions	31

2.6.5	Compliance	34

2.7	8.1 in. Tabletop Touch Screen	34

2.7.1	Basis of Design	34

2.7.2	Device Definition	34

2.7.3	Device Architecture	35

2.7.4	Functions	35

2.7.5	Compliance	38

2.8	10.1 in. Tabletop Touch Screen	38

2.8.1	Basis of Design	38

2.8.2	Device Definition	39

2.8.3	Device Architecture	39

2.8.4	Functions	40

2.8.5	Compliance	43

2.9	8.1 in. Room Scheduling Touch Screen	43

2.9.1	Basis of Design	43

2.9.2	Device Definition	43

2.9.3	Device Architecture	43

2.9.4	Functions	44

2.9.5	Compliance	47

3	EXECUTION	47

4	APPENDICES	47

4.1	SPECIFIED PRODUCTS	47

4.1.1	Crestron TSW-880	48

4.1.2	Crestron TSW-1080	48

4.1.3	Crestron TS-880	48

4.1.4	Crestron TS-1080	48

4.1.5	Crestron TSW-880R	48

4.1.6	Crestron TSW-1080R	48

4.1.7	Crestron TS-880R	48

4.1.8	Crestron TS-1080R	48

4.1.9	Crestron TSS-880	48

# GENERAL

NOT USED in this Guide Specification.  Specifier shall Specify PART 1 administrative and procedural requirements as needed.

# PRODUCTS

## 8.1 in. Wall Mount Touch Screen

Specifier Note:

The Crestron TSW-880 wall mount touch screen is ideal for residential and enterprise applications. The TSW-880 features an 8.1 in. (204 mm) capacitive touch screen display with custom programmable control buttons and support for HTML5-based custom UI projects. PoE and Wi-Fi® network connectivity and a comprehensive assortment of mounting accessories simplify new and retrofit installations.

### Basis of Design

#### Crestron TSW-880

Specifier Note:

TSW-880
https://www.crestron.com/Products/Catalog/Control-and-Management/User-Interfaces/Touchscreen/TSW-880-B

### Device Definition

#### Wall Mount Touch Screen with the following capabilities and features:

##### Ability to serve as user interface for residential, commercial, and government applications

##### Custom programmable virtual buttons

##### Support for custom user interfaces developed using software by same manufacturer

##### Support for streaming video

##### Support for VoIP-based intercom between other touch screens by same manufacturer

##### Room scheduling via third party apps or via app from same manufacturer

##### Support for residential control operating system by same manufacturer

##### Web browsing

##### Ethernet LAN and Wi-Fi connectivity

##### Power over Ethernet (PoE) support

##### Enterprise-grade security (802.1X authentication, MDEP, TLS encryption, HTTPS connectivity, Active Directory® service integration)

##### Support for remote monitoring and managing software by same manufacturer

##### Support for room scheduling accessories by same manufacturer

##### Support for optional mounting kits for installation in a multitude of scenarios

### Device Architecture

#### Environmental

##### Temperature: 32° to 104°F (0° to 40° C)

##### Humidity: 10% to 95% RH (non-condensing)

##### Heat Dissipation: 44 BTU/hr

#### Construction

##### Housing: Die-cast housing, smooth black or white finish, glass with black or white surround

##### Mounting

###### Surface mount over a 2 or 3-gang US electrical box, 2-gang European (DIN 49073) electrical box, or 2-gang UK (BS 4662) electrical box

###### Lectern mount over a 2 in. H x 3-3/4 in. W (51 mm H x 96 mm W) cutout; 1-3/8 in. (35 mm) minimum mounting depth

###### Wall mount, surface mount, and rack mount options available separately

#### Dimensions

##### Height: 4.87 in. (124 mm)

##### Width: 7.93 in. (201 mm)

##### Depth: 0.50 in. (13 mm)

#### Weight: 14.46 oz. (410 g)

### Functions

#### Touch Screen Display

##### Display Type: TFT active matrix color LCD

##### Size: 8.1 in. (204 mm) diagonal

##### Aspect Ratio: 16:10 WXGA

##### Resolution: 1280 x 800 pixels

##### Brightness

###### Minimum: 350 nits (cd/m2)

###### Maximum: 400 nits (cd/m2)

##### Contrast: 1000:1

##### Color Depth: 24 bit, 16.7M colors

##### Illumination: Edge lit LED with auto brightness control

##### Viewing Angle: ±80° horizontal, ±80° vertical

##### Touch Screen: Projected capacitive, 5 point multitouch capable

#### Buttons

##### Virtual Buttons

###### (6) Virtual buttons in collapsible universal tool bar

###### Per button show/hide

###### Preconfigured with icons for Power, Home, Lights, Up, Down, and Microphone

##### Reset: (1) Pin hole on bottom for hardware reset

#### Graphics Engine

##### Proprietary HTML5-based multilanguage web browser

Specifier Note:

Web browsing, weather information, and certain other functions require an Internet connection.

##### Multilanguage onscreen keyboard

##### Screensaver

##### Dual scalable streaming video window

##### Native room scheduling applications

Specifier Note:

Room scheduling mode requires the TSW-880 to be designated exclusively for room scheduling use, which precludes use of certain features and functions described in this spec document. Additionally, CollegeNET® 25Live® scheduling software and Ad Astra™ software do not support scheduling ad hoc meetings from the touch screen.

Refer to the firmware release notes for a list of all scheduling apps that are supported by that firmware release.

Additional subscriptions and/or licenses may be required. Refer to each provider’s website for details about the capabilities and requirements of its scheduling application and services. Third party apps typically run on the TSW-880 as they do on a tablet device. The TSW-880 runs only one app, which is selected at setup. Only the apps approved and delivered by Crestron can run on the TSW-880.

##### Native proprietary residential control operating system app by same manufacturer

##### Touch screen setup and diagnostics via web browser, cloud, or onscreen UI

#### Room Scheduling Support

##### Scheduling app by same manufacturer: Microsoft Exchange Server® and Microsoft 365® software, Google Calendar™ calendaring app and G Suite™ software, CollegeNET® 25Live® software, and Ad Astra™ software

##### Third-Party Apps: Includes support for various third-party scheduling applications

#### Memory

##### RAM: 8 GB LPDDR4

##### Storage: 40 GB eMMC pSLC

##### Maximum Project Size: 1000 MB

#### Wired Communications

##### Ethernet: 1000 Mbps, autoswitching, autonegotiating, autodiscovery, full/half duplex, TCP/IP, UDP/IP, CIP, DHCP, SSL, TLS, SSH, SFTP (SSH File Transfer Protocol), IEEE 802.1X, SNMP, IPv4 or IPv6, Active Directory® service authentication, HTTPS web browser setup, proprietary remote device commissioning and management software client by same manufacturer, IEEE 802.3at compliant

##### USB: USB 2.0 host for room scheduling accessories

#### Wireless Communications

##### Transceiver: IEEE 802.11a/b/g/n/ac/ax Wi-Fi 6E (2.4, 5 GHz and 6 GHz) 2x2 MIMO, static IP or dynamic IP via DHCP

##### Security: 64 and 128-bit WEP, WPA, WPA2, WPA3, WPA Enterprise and WPA2 Enterprise

##### Range: Up to 50 ft (15 m), subject to site-specific conditions

##### Bluetooth® Hardware: BLE, BT5.2, 33 ft (10 m) typical range

#### Streaming Decoder

##### Video Formats: H.265, H.264 (MPEG-4 part 10 AVC, MJPEG)

##### Audio Format: AAC Stereo

##### Bitrates: Up to 25 Mbps (20 Mbps maximum recommended)

##### Input Resolutions: Up to 3840x2160@30fps

##### Protocol: RTSP

#### Audio

##### Features: Built-in microphone and speakers, VoIP-based intercom between other touch screens by same manufacturer

Specifier Note:

The microphone and Bluetooth® transceiver may be defeated programmatically at any time.

#### Connectors

##### LAN PoE

###### (1) 8-pin RJ45 connector, female, with 2 LED indicators

###### 1000BASE-TX Ethernet port

###### PoE+ PD port

###### Green and yellow LEDs indicate Ethernet port status

##### USB

###### (1) USB Type C connector, female

###### USB 2.0 host port

###### USB 3.1 DisplayPort Alt Mode (for future release)

#### Power

##### PoE (Power over Ethernet)

###### IEEE 802.3at Type 2 compliant PoE+ PD (powered device)

###### Requests 30 W (PoE+ Class 4) from an 802.3at Type 2 PSE without LLDP

###### Requests 15.4 W (PoE Class 0) from an 802.3af (or 802.3at Type 1)

Specifier Note:

Enabling the USB port requires use of an 802.3at Type 2 PoE+ PSE.

### Compliance

#### Regulatory Model: M202404001

#### FCC Part 15 Class B, IC Class B, CE, UL® Listed for US and Canada/Intertek®

## 10.1 in. Wall Mount Touch Screen

Specifier Note:

The Crestron TSW-1080 wall mount touch screen is ideal for residential and enterprise applications. The TSW-1080 features a 10.1 in. (257 mm) capacitive touch screen display with custom programmable control buttons and support for HTML5-based custom UI projects. PoE and Wi-Fi® network connectivity and a comprehensive assortment of mounting accessories simplify new and retrofit installations.

### Basis of Design

#### Crestron TSW-1080

Specifier Note:

TSW-1080
https://www.crestron.com/Products/Catalog/Control-and-Management/User-Interfaces/Touchscreen/TSW-1080-B

### Device Definition

#### Wall Mount Touch Screen with the following capabilities and features:

##### Ability to serve as user interface for residential, commercial, and government applications

##### Custom programmable virtual buttons

##### Support for custom user interfaces developed using software by same manufacturer

##### Support for streaming video

##### Support for VoIP-based intercom between other touch screens by same manufacturer

##### Room scheduling via third party apps or via app from same manufacturer

##### Support for residential control operating system by same manufacturer

##### Web browsing

##### Ethernet LAN and Wi-Fi connectivity

##### Power over Ethernet (PoE) support

##### Enterprise-grade security (802.1X authentication, MDEP, TLS encryption, HTTPS connectivity, Active Directory® service integration)

##### Support for remote monitoring and managing software by same manufacturer

##### Support for room scheduling accessories by same manufacturer

##### Support for optional mounting kits for installation in a multitude of scenarios

### Device Architecture

#### Environmental

##### Temperature: 32° to 104°F (0° to 40° C)

##### Humidity: 10% to 95% RH (non-condensing)

##### Heat Dissipation: 44 BTU/hr

#### Construction

##### Housing: Die-cast housing, smooth black or white finish, glass with black or white surround

##### Mounting

###### Surface mount over a 2 or 3-gang US electrical box, 2-gang European (DIN 49073) electrical box, or 2-gang UK (BS 4662) electrical box

###### Lectern mount over a 2 in. H x 3-3/4 in. W (51 mm H x 96 mm W) cutout; 1-3/8 in. (35 mm) minimum mounting depth

###### Wall mount, surface mount, and rack mount options available separately

#### Dimensions

##### Height: 5.96 in. (151 mm)

##### Width: 9.68 in. (246 mm)

##### Depth: 1.18 in. (30 mm)

#### Weight: 19.4 oz. (550 g)

### Functions

#### Touch Screen Display

##### Display Type: TFT active matrix color LCD

##### Size: 10.1 in. (257 mm) diagonal

##### Aspect Ratio: 16:10 WXGA

##### Resolution: 1920 x 1200 pixels

##### Brightness

###### Minimum: 350 nits (cd/m2)

###### Maximum: 400 nits (cd/m2)

##### Contrast: 1000:1

##### Color Depth: 24 bit, 16.7M colors

##### Illumination: Edge lit LED with auto brightness control

##### Viewing Angle: ±80° horizontal, ±80° vertical

##### Touch Screen: Projected capacitive, 5 point multitouch capable

#### Buttons

##### Virtual Buttons

###### (6) Virtual buttons in collapsible universal tool bar

###### Per button show/hide

###### Preconfigured with icons for Power, Home, Lights, Up, Down, and Microphone

##### Reset: (1) Pin hole on bottom for hardware reset

#### Graphics Engine

##### Proprietary HTML5-based multilanguage web browser

Specifier Note:

Web browsing, weather information, and certain other functions require an Internet connection.

##### Multilanguage onscreen keyboard

##### Screensaver

##### Dual scalable streaming video window

##### Native room scheduling applications

Specifier Note:

Room scheduling mode requires the TSW-1080 to be designated exclusively for room scheduling use, which precludes use of certain features and functions described in this spec document. Additionally, CollegeNET® 25Live® scheduling software and Ad Astra™ software do not support scheduling ad hoc meetings from the touch screen.

Refer to the firmware release notes for a list of all scheduling apps that are supported by that firmware release.

Additional subscriptions and/or licenses may be required. Refer to each provider’s website for details about the capabilities and requirements of its scheduling application and services. Third party apps typically run on the TSW-1080 as they do on a tablet device. The TSW-1080 runs only one app, which is selected at setup. Only the apps approved and delivered by Crestron can run on the TSW-1080.

##### Native proprietary residential control operating system app by same manufacturer

##### Touch screen setup and diagnostics via web browser, cloud, or onscreen UI

#### Room Scheduling Support

##### Scheduling app by same manufacturer: Microsoft Exchange Server® and Microsoft 365® software, Google Calendar™ calendaring app and G Suite™ software, CollegeNET® 25Live® software, and Ad Astra™ software

##### Third-Party Apps: Includes support for various third-party scheduling applications

#### Memory

##### RAM: 8 GB LPDDR4

##### Storage: 40 GB eMMC pSLC

##### Maximum Project Size: 1000 MB

#### Wired Communications

##### Ethernet: 1000 Mbps, autoswitching, autonegotiating, autodiscovery, full/half duplex, TCP/IP, UDP/IP, CIP, DHCP, SSL, TLS, SSH, SFTP (SSH File Transfer Protocol), IEEE 802.1X, SNMP, IPv4 or IPv6, Active Directory® service authentication, HTTPS web browser setup, proprietary remote device commissioning and management software client by same manufacturer, IEEE 802.3at compliant

##### USB: USB 2.0 host for room scheduling accessories

#### Wireless Communications

##### Transceiver: IEEE 802.11a/b/g/n/ac/ax Wi-Fi 6E (2.4, 5 GHz and 6 GHz) 2x2 MIMO, static IP or dynamic IP via DHCP

##### Security: 64 and 128-bit WEP, WPA, WPA2, WPA3, WPA Enterprise and WPA2 Enterprise

##### Range: Up to 50 ft (15 m), subject to site-specific conditions

##### Bluetooth® Hardware: BLE, BT5.2, 33 ft (10 m) typical range

#### Streaming Decoder

##### Video Formats: H.265, H.264 (MPEG-4 part 10 AVC, MJPEG)

##### Audio Format: AAC Stereo

##### Bitrates: Up to 25 Mbps (20 Mbps maximum recommended)

##### Input Resolutions: Up to 3840x2160@30fps

##### Protocol: RTSP

#### Audio

##### Features: Built-in microphone and speakers, VoIP-based intercom between other touch screens by same manufacturer

Specifier Note:

The microphone and Bluetooth® transceiver may be defeated programmatically at any time.

#### Connectors

##### LAN PoE

###### (1) 8-pin RJ45 connector, female, with 2 LED indicators

###### 1000BASE-TX Ethernet port

###### PoE+ PD port

###### Green and yellow LEDs indicate Ethernet port status

##### USB

###### (1) USB Type C connector, female

###### USB 2.0 host port

###### USB 3.1 DisplayPort Alt Mode (for future release)

#### Power

##### PoE (Power over Ethernet)

###### IEEE 802.3at Type 2 compliant PoE+ PD (powered device)

###### Requests 30 W (PoE+ Class 4) from an 802.3at Type 2 PSE without LLDP

###### Requests 15.4 W (PoE Class 0) from an 802.3af (or 802.3at Type 1)

Specifier Note:

Enabling the USB port requires use of an 802.3at Type 2 PoE+ PSE.

### Compliance

#### Regulatory Model: M202404002

#### FCC Part 15 Class B, IC Class B, CE, UL® Listed for US and Canada/Intertek®

## 8.1 in. Tabletop Touch Screen

Specifier Note:

The Crestron TS-880 tabletop touch screen is ideal for residential and enterprise applications. The TS-880 features an 8.1 in. (204 mm) capacitive touch screen display with custom programmable control buttons and support for HTML5-based custom UI projects. PoE and Wi-Fi® network connectivity and a comprehensive assortment of mounting accessories simplify new and retrofit installations.

### Basis of Design

#### Crestron TS-880

Specifier Note:

TS-880
https://www.crestron.com/Products/Catalog/Control-and-Management/User-Interfaces/Touchscreen/TS-880-B

### Device Definition

#### Tabletop Touch Screen with the following capabilities and features:

##### Ability to serve as user interface for residential, commercial, and government applications

##### Custom programmable virtual buttons

##### Support for custom user interfaces developed using software by same manufacturer

##### Support for streaming video

##### Support for VoIP-based intercom between other touch screens by same manufacturer

##### Room scheduling via third party apps or via app from same manufacturer

##### Support for residential control operating system by same manufacturer

##### Web browsing

##### Ethernet LAN and Wi-Fi connectivity

##### Power over Ethernet (PoE) support

##### Enterprise-grade security (802.1X authentication, MDEP, TLS encryption, HTTPS connectivity, Active Directory® service integration)

##### Support for remote monitoring and managing software by same manufacturer

##### Support for room scheduling accessories by same manufacturer

### Device Architecture

#### Environmental

##### Temperature: 32° to 104°F (0° to 40° C)

##### Humidity: 10% to 95% RH (non-condensing)

##### Heat Dissipation: 44 BTU/hr

#### Construction

##### Housing: Die-cast housing, smooth black or white finish, glass with black or white surround

#### Dimensions

##### Height: 4.87 in. (124 mm)

##### Width: 7.93 in. (201 mm)

##### Depth: 4.53 in. (115 mm)

#### Weight: 1.76 lb. (799 g)

### Functions

#### Touch Screen Display

##### Display Type: TFT active matrix color LCD

##### Size: 8.1 in. (204 mm) diagonal

##### Aspect Ratio: 16:10 WXGA

##### Resolution: 1280 x 800 pixels

##### Brightness

###### Minimum: 350 nits (cd/m2)

###### Maximum: 400 nits (cd/m2)

##### Contrast: 1000:1

##### Color Depth: 24 bit, 16.7M colors

##### Illumination: Edge lit LED with auto brightness control

##### Viewing Angle: ±80° horizontal, ±80° vertical

##### Touch Screen: Projected capacitive, 5 point multitouch capable

#### Buttons

##### Virtual Buttons

###### (6) Virtual buttons in collapsible universal tool bar

###### Per button show/hide

###### Preconfigured with icons for Power, Home, Lights, Up, Down, and Microphone

##### Reset: (1) Pin hole on bottom for hardware reset

#### Graphics Engine

##### Proprietary HTML5-based multilanguage web browser

Specifier Note:

Web browsing, weather information, and certain other functions require an Internet connection.

##### Multilanguage onscreen keyboard

##### Screensaver

##### Dual scalable streaming video window

##### Native room scheduling applications

Specifier Note:

Room scheduling mode requires the TS-880 to be designated exclusively for room scheduling use, which precludes use of certain features and functions described in this spec document. Additionally, CollegeNET® 25Live® scheduling software and Ad Astra™ software do not support scheduling ad hoc meetings from the touch screen.

Refer to the firmware release notes for a list of all scheduling apps that are supported by that firmware release.

Additional subscriptions and/or licenses may be required. Refer to each provider’s website for details about the capabilities and requirements of its scheduling application and services. Third party apps typically run on the TS-880 as they do on a tablet device. The TS-880 runs only one app, which is selected at setup. Only the apps approved and delivered by Crestron can run on the TS-880.

##### Native proprietary residential control operating system app by same manufacturer

##### Touch screen setup and diagnostics via web browser, cloud, or onscreen UI

#### Room Scheduling Support

##### Scheduling app by same manufacturer: Microsoft Exchange Server® and Microsoft 365® software, Google Calendar™ calendaring app and G Suite™ software, CollegeNET® 25Live® software, and Ad Astra™ software

##### Third-Party Apps: Includes support for various third-party scheduling applications

#### Memory

##### RAM: 8 GB LPDDR4

##### Storage: 40 GB eMMC pSLC

##### Maximum Project Size: 1000 MB

#### Wired Communications

##### Ethernet: 1000 Mbps, autoswitching, autonegotiating, autodiscovery, full/half duplex, TCP/IP, UDP/IP, CIP, DHCP, SSL, TLS, SSH, SFTP (SSH File Transfer Protocol), IEEE 802.1X, SNMP, IPv4 or IPv6, Active Directory® service authentication, HTTPS web browser setup, proprietary remote device commissioning and management software client by same manufacturer, IEEE 802.3at compliant

##### USB: USB 2.0 host for room scheduling accessories

#### Wireless Communications

##### Transceiver: IEEE 802.11a/b/g/n/ac/ax Wi-Fi 6E (2.4, 5 GHz and 6 GHz) 2x2 MIMO, static IP or dynamic IP via DHCP

##### Security: 64 and 128-bit WEP, WPA, WPA2, WPA3, WPA Enterprise and WPA2 Enterprise

##### Range: Up to 50 ft (15 m), subject to site-specific conditions

##### Bluetooth® Hardware: BLE, BT5.2, 33 ft (10 m) typical range

#### Streaming Decoder

##### Video Formats: H.265, H.264 (MPEG-4 part 10 AVC, MJPEG)

##### Audio Format: AAC Stereo

##### Bitrates: Up to 25 Mbps (20 Mbps maximum recommended)

##### Input Resolutions: Up to 3840x2160@30fps

##### Protocol: RTSP

#### Audio

##### Features: Built-in microphone and speakers, VoIP-based intercom between other touch screens by same manufacturer

Specifier Note:

The microphone and Bluetooth® transceiver may be defeated programmatically at any time.

#### Connectors

##### LAN PoE

###### (1) 8-pin RJ45 connector, female, with 2 LED indicators

###### 1000BASE-TX Ethernet port

###### PoE+ PD port

###### Green and yellow LEDs indicate Ethernet port status

##### USB

###### (1) USB Type C connector, female

###### USB 2.0 host port

###### USB 3.1 DisplayPort Alt Mode (for future release)

#### Power

##### PoE (Power over Ethernet)

###### IEEE 802.3at Type 2 compliant PoE+ PD (powered device)

###### Requests 30 W (PoE+ Class 4) from an 802.3at Type 2 PSE without LLDP

###### Requests 15.4 W (PoE Class 0) from an 802.3af (or 802.3at Type 1)

Specifier Note:

Enabling the USB port requires use of an 802.3at Type 2 PoE+ PSE.

### Compliance

#### Regulatory Model: M202404005

#### FCC Part 15 Class B, IC Class B, CE, UL® Listed for US and Canada/Intertek®

## 10.1 in. Tabletop Touch Screen

Specifier Note:

The Crestron TS-1080 tabletop touch screen is ideal for residential and enterprise applications. The TS-1080 features a 10.1 in. (257 mm) capacitive touch screen display with custom programmable control buttons and support for HTML5-based custom UI projects. PoE and Wi-Fi® network connectivity and a comprehensive assortment of mounting accessories simplify new and retrofit installations.

### Basis of Design

#### Crestron TS-1080

Specifier Note:

TS-1080
https://www.crestron.com/Products/Catalog/Control-and-Management/User-Interfaces/Touchscreen/TS-1080-B

### Device Definition

#### Tabletop Touch Screen with the following capabilities and features:

##### Ability to serve as user interface for residential, commercial, and government applications

##### Custom programmable virtual buttons

##### Support for custom user interfaces developed using software by same manufacturer

##### Support for streaming video

##### Support for VoIP-based intercom between other touch screens by same manufacturer

##### Room scheduling via third party apps or via app from same manufacturer

##### Support for residential control operating system by same manufacturer

##### Web browsing

##### Ethernet LAN and Wi-Fi connectivity

##### Power over Ethernet (PoE) support

##### Enterprise-grade security (802.1X authentication, MDEP, TLS encryption, HTTPS connectivity, Active Directory® service integration)

##### Support for remote monitoring and managing software by same manufacturer

##### Support for room scheduling accessories by same manufacturer

### Device Architecture

#### Environmental

##### Temperature: 32° to 104°F (0° to 40° C)

##### Humidity: 10% to 95% RH (non-condensing)

##### Heat Dissipation: 44 BTU/hr

#### Construction

##### Housing: Die-cast housing, smooth black or white finish, glass with black or white surround

#### Dimensions

##### Height: 5.96 in. (151 mm)

##### Width: 9.68 in. (246 mm)

##### Depth: 4.53 in. (115 mm)

#### Weight: 2.23 lb. (464 g)

### Functions

#### Touch Screen Display

##### Display Type: TFT active matrix color LCD

##### Size: 10.1 in. (257 mm) diagonal

##### Aspect Ratio: 16:10 WXGA

##### Resolution: 1920 x 1200 pixels

##### Brightness

###### Minimum: 350 nits (cd/m2)

###### Maximum: 400 nits (cd/m2)

##### Contrast: 1000:1

##### Color Depth: 24 bit, 16.7M colors

##### Illumination: Edge lit LED with auto brightness control

##### Viewing Angle: ±80° horizontal, ±80° vertical

##### Touch Screen: Projected capacitive, 5 point multitouch capable

#### Buttons

##### Virtual Buttons

###### (6) Virtual buttons in collapsible universal tool bar

###### Per button show/hide

###### Preconfigured with icons for Power, Home, Lights, Up, Down, and Microphone

##### Reset: (1) Pin hole on bottom for hardware reset

#### Graphics Engine

##### Proprietary HTML5-based multilanguage web browser

Specifier Note:

Web browsing, weather information, and certain other functions require an Internet connection.

##### Multilanguage onscreen keyboard

##### Screensaver

##### Dual scalable streaming video window

##### Native room scheduling applications

Specifier Note:

Room scheduling mode requires the TS-1080 to be designated exclusively for room scheduling use, which precludes use of certain features and functions described in this spec document. Additionally, CollegeNET® 25Live® scheduling software and Ad Astra™ software do not support scheduling ad hoc meetings from the touch screen.

Refer to the firmware release notes for a list of all scheduling apps that are supported by that firmware release.

Additional subscriptions and/or licenses may be required. Refer to each provider’s website for details about the capabilities and requirements of its scheduling application and services. Third party apps typically run on the TS-1080 as they do on a tablet device. The TS-1080 runs only one app, which is selected at setup. Only the apps approved and delivered by Crestron can run on the TS-1080.

##### Native proprietary residential control operating system app by same manufacturer

##### Touch screen setup and diagnostics via web browser, cloud, or onscreen UI

#### Room Scheduling Support

##### Scheduling app by same manufacturer: Microsoft Exchange Server® and Microsoft 365® software, Google Calendar™ calendaring app and G Suite™ software, CollegeNET® 25Live® software, and Ad Astra™ software

##### Third-Party Apps: Includes support for various third-party scheduling applications

#### Memory

##### RAM: 8 GB LPDDR4

##### Storage: 40 GB eMMC pSLC

##### Maximum Project Size: 1000 MB

#### Wired Communications

##### Ethernet: 1000 Mbps, autoswitching, autonegotiating, autodiscovery, full/half duplex, TCP/IP, UDP/IP, CIP, DHCP, SSL, TLS, SSH, SFTP (SSH File Transfer Protocol), IEEE 802.1X, SNMP, IPv4 or IPv6, Active Directory® service authentication, HTTPS web browser setup, proprietary remote device commissioning and management software client by same manufacturer, IEEE 802.3at compliant

##### USB: USB 2.0 host for room scheduling accessories

#### Wireless Communications

##### Transceiver: IEEE 802.11a/b/g/n/ac/ax Wi-Fi 6E (2.4, 5 GHz and 6 GHz) 2x2 MIMO, static IP or dynamic IP via DHCP

##### Security: 64 and 128-bit WEP, WPA, WPA2, WPA3, WPA Enterprise and WPA2 Enterprise

##### Range: Up to 50 ft (15 m), subject to site-specific conditions

##### Bluetooth® Hardware: BLE, BT5.2, 33 ft (10 m) typical range

#### Streaming Decoder

##### Video Formats: H.265, H.264 (MPEG-4 part 10 AVC, MJPEG)

##### Audio Format: AAC Stereo

##### Bitrates: Up to 25 Mbps (20 Mbps maximum recommended)

##### Input Resolutions: Up to 3840x2160@30fps

##### Protocol: RTSP

#### Audio

##### Features: Built-in microphone and speakers, VoIP-based intercom between other touch screens by same manufacturer

Specifier Note:

The microphone and Bluetooth® transceiver may be defeated programmatically at any time.

#### Connectors

##### LAN PoE

###### (1) 8-pin RJ45 connector, female, with 2 LED indicators

###### 1000BASE-TX Ethernet port

###### PoE+ PD port

###### Green and yellow LEDs indicate Ethernet port status

##### USB

###### (1) USB Type C connector, female

###### USB 2.0 host port

###### USB 3.1 DisplayPort Alt Mode (for future release)

#### Power

##### PoE (Power over Ethernet)

###### IEEE 802.3at Type 2 compliant PoE+ PD (powered device)

###### Requests 30 W (PoE+ Class 4) from an 802.3at Type 2 PSE without LLDP

###### Requests 15.4 W (PoE Class 0) from an 802.3af (or 802.3at Type 1)

Specifier Note:

Enabling the USB port requires use of an 802.3at Type 2 PoE+ PSE.

### Compliance

#### Regulatory Model: M202404006

#### FCC Part 15 Class B, IC Class B, CE, UL® Listed for US and Canada/Intertek®

## 8.1 in. Wall Mount Touch Screen

Specifier Note:

The Crestron TSW-880R touch screen is ideal for residential applications and is designed to run Crestron Home® OS. The TSW-880R features an 8.1 in. (204 mm) capacitive touch screen display. PoE and Wi-Fi® network connectivity simplify new and retrofit installations.

### Basis of Design

#### Crestron TSW-880R

Specifier Note:

TSW-880R
https://www.crestron.com/Products/Catalog/Control-and-Management/User-Interfaces/Touchscreen/TSW-880R-B

### Device Definition

#### Wall Mount Touch Screen with the following capabilities and features:

##### Ability to serve as user interface for residential applications

##### Support for streaming video

##### Support for VoIP-based intercom between other touch screens by same manufacturer

##### Support for residential control operating system by same manufacturer

##### Ethernet LAN and Wi-Fi connectivity

##### Power over Ethernet (PoE) support

##### Enterprise-grade security (802.1X authentication, MDEP, TLS encryption, HTTPS connectivity, Active Directory® service integration)

##### Support for optional mounting kits for installation in a multitude of scenarios

### Device Architecture

#### Environmental

##### Temperature: 32° to 104°F (0° to 40° C)

##### Humidity: 10% to 95% RH (non-condensing)

##### Heat Dissipation: 44 BTU/hr

#### Construction

##### Housing: Die-cast housing, smooth black or white finish, glass with black or white surround

##### Mounting

###### Surface mount over a 2 or 3-gang US electrical box, 2-gang European (DIN 49073) electrical box, or 2-gang UK (BS 4662) electrical box

###### Lectern mount over a 2 in. H x 3-3/4 in. W (51 mm H x 96 mm W) cutout; 1-3/8 in. (35 mm) minimum mounting depth

###### Wall mount option available separately

#### Dimensions

##### Height: 4.87 in. (124 mm)

##### Width: 7.93 in. (201 mm)

##### Depth: 0.50 in. (13 mm)

#### Weight: 14.46 oz. (410 g)

### Functions

#### Touch Screen Display

##### Display Type: TFT active matrix color LCD

##### Size: 8.1 in. (204 mm) diagonal

##### Aspect Ratio: 16:10 WXGA

##### Resolution: 1280 x 800 pixels

##### Brightness

###### Minimum: 350 nits (cd/m2)

###### Maximum: 400 nits (cd/m2)

##### Contrast: 1000:1

##### Color Depth: 24 bit, 16.7M colors

##### Illumination: Edge lit LED with auto brightness control

##### Viewing Angle: ±80° horizontal, ±80° vertical

##### Touch Screen: Projected capacitive, 5 point multitouch capable

#### Buttons

##### Virtual Buttons

###### (6) Virtual buttons in collapsible universal tool bar

###### Per button show/hide

###### Preconfigured with icons for Power, Home, Lights, Up, Down, and Microphone

##### Reset: (1) Pin hole on bottom for hardware reset

#### Graphics Engine

##### Dual scalable streaming video window

##### Native proprietary residential control operating system app by same manufacturer

##### Touch screen setup and diagnostics via web browser, cloud, or onscreen UI

##### Residential control operating system setup and diagnostics via app by same manufacturer

#### Languages

##### Onscreen Keyboard: Arabic, Chinese (Simplified), Croatian, Czech, Danish, Dutch, English (UK), English (US), Finnish, French (Canada), French (Switzerland), German, Hebrew, Hungarian, Italian, Japanese, Norwegian Bokmal, Polish, Portuguese, Russian, Serbian, Spanish, Swedish, Turkish

#### Memory

##### RAM: 8 GB LPDDR4

##### Storage: 40 GB eMMC pSLC

##### Maximum Project Size: 1000 MB

#### Wired Communications

##### Ethernet: 1000 Mbps, autoswitching, autonegotiating, autodiscovery, full/half duplex, TCP/IP, UDP/IP, CIP, DHCP, SSL, TLS, SSH, SFTP (SSH File Transfer Protocol), IEEE 802.1X, SNMP, IPv4 or IPv6, Active Directory® service authentication, HTTPS web browser setup, proprietary remote device commissioning and management software client by same manufacturer, IEEE 802.3at compliant

##### USB: USB 2.0 host

#### Wireless Communications

##### Transceiver: IEEE 802.11a/b/g/n/ac/ax Wi-Fi 6E (2.4, 5 GHz and 6 GHz) 2x2 MIMO, static IP or dynamic IP via DHCP

##### Security: 64 and 128-bit WEP, WPA, WPA2, WPA3, WPA Enterprise and WPA2 Enterprise

##### Range: Up to 50 ft (15 m), subject to site-specific conditions

##### Bluetooth® Hardware: BLE, BT5.2, 33 ft (10 m) typical range

#### Streaming Decoder

##### Video Formats: H.265, H.264 (MPEG-4 part 10 AVC, MJPEG)

##### Audio Format: AAC Stereo

##### Bitrates: Up to 25 Mbps (20 Mbps maximum recommended)

##### Input Resolutions: Up to 3840x2160@30fps

##### Protocol: RTSP

#### Audio

##### Features: Built-in microphone and speakers, VoIP-based intercom between other touch screens by same manufacturer

Specifier Note:

The microphone and Bluetooth® transceiver may be defeated programmatically at any time.

#### Connectors

##### LAN PoE

###### (1) 8-pin RJ45 connector, female, with 2 LED indicators

###### 1000BASE-TX Ethernet port

###### PoE+ PD port

###### Green and yellow LEDs indicate Ethernet port status

##### USB

###### (1) USB Type C connector, female

###### USB 2.0 host port

###### USB 3.1 DisplayPort Alt Mode (for future release)

#### Power

##### PoE (Power over Ethernet)

###### IEEE 802.3at Type 2 compliant PoE+ PD (powered device)

###### Requests 30 W (PoE+ Class 4) from an 802.3at Type 2 PSE without LLDP

###### Requests 15.4 W (PoE Class 0) from an 802.3af (or 802.3at Type 1)

Specifier Note:

Enabling the USB port requires use of an 802.3at Type 2 PoE+ PSE.

### Compliance

#### Regulatory Model: M202404001

#### FCC Part 15 Class B, IC Class B, CE, UL® Listed for US and Canada/Intertek®

## 10.1 in. Wall Mount Touch Screen

Specifier Note:

The Crestron TSW-1080R touch screen is ideal for residential applications and is designed to run Crestron Home® OS. The TSW-1080R features a 10.1 in. (257 mm) capacitive touch screen display. PoE and Wi-Fi® network connectivity simplify new and retrofit installations.

### Basis of Design

#### Crestron TSW-1080R

Specifier Note:

TSW-1080R
https://www.crestron.com/Products/Catalog/Control-and-Management/User-Interfaces/Touchscreen/TSW-1080R-B

### Device Definition

#### Wall Mount Touch Screen with the following capabilities and features:

##### Ability to serve as user interface for residential applications

##### Support for streaming video

##### Support for VoIP-based intercom between other touch screens by same manufacturer

##### Support for residential control operating system by same manufacturer

##### Ethernet LAN and Wi-Fi connectivity

##### Power over Ethernet (PoE) support

##### Enterprise-grade security (802.1X authentication, MDEP, TLS encryption, HTTPS connectivity, Active Directory® service integration)

##### Support for optional mounting kits for installation in a multitude of scenarios

### Device Architecture

#### Environmental

##### Temperature: 32° to 104°F (0° to 40° C)

##### Humidity: 10% to 95% RH (non-condensing)

##### Heat Dissipation: 44 BTU/hr

#### Construction

##### Housing: Die-cast housing, smooth black or white finish, glass with black or white surround

##### Mounting

###### Surface mount over a 2 or 3-gang US electrical box, 2-gang European (DIN 49073) electrical box, or 2-gang UK (BS 4662) electrical box

###### Lectern mount over a 2 in. H x 3-3/4 in. W (51 mm H x 96 mm W) cutout; 1-3/8 in. (35 mm) minimum mounting depth

###### Wall mount option available separately

#### Dimensions

##### Height: 5.96 in. (151 mm)

##### Width: 9.68 in. (246 mm)

##### Depth: 1.18 in. (30 mm)

#### Weight: 19.4 oz. (550 g)

### Functions

#### Touch Screen Display

##### Display Type: TFT active matrix color LCD

##### Size: 10.1 in. (257 mm) diagonal

##### Aspect Ratio: 16:10 WXGA

##### Resolution: 1920 x 1200 pixels

##### Brightness

###### Minimum: 350 nits (cd/m2)

###### Maximum: 400 nits (cd/m2)

##### Contrast: 1000:1

##### Color Depth: 24 bit, 16.7M colors

##### Illumination: Edge lit LED with auto brightness control

##### Viewing Angle: ±80° horizontal, ±80° vertical

##### Touch Screen: Projected capacitive, 5 point multitouch capable

#### Buttons

##### Virtual Buttons

###### (6) Virtual buttons in collapsible universal tool bar

###### Per button show/hide

###### Preconfigured with icons for Power, Home, Lights, Up, Down, and Microphone

##### Reset: (1) Pin hole on bottom for hardware reset

#### Graphics Engine

##### Dual scalable streaming video window

##### Native proprietary residential control operating system app by same manufacturer

##### Touch screen setup and diagnostics via web browser, cloud, or onscreen UI

##### Residential control operating system setup and diagnostics via app by same manufacturer

#### Languages

##### Onscreen Keyboard: Arabic, Chinese (Simplified), Croatian, Czech, Danish, Dutch, English (UK), English (US), Finnish, French (Canada), French (Switzerland), German, Hebrew, Hungarian, Italian, Japanese, Norwegian Bokmal, Polish, Portuguese, Russian, Serbian, Spanish, Swedish, Turkish

#### Memory

##### RAM: 8 GB LPDDR4

##### Storage: 40 GB eMMC pSLC

##### Maximum Project Size: 1000 MB

#### Wired Communications

##### Ethernet: 1000 Mbps, autoswitching, autonegotiating, autodiscovery, full/half duplex, TCP/IP, UDP/IP, CIP, DHCP, SSL, TLS, SSH, SFTP (SSH File Transfer Protocol), IEEE 802.1X, SNMP, IPv4 or IPv6, Active Directory® service authentication, HTTPS web browser setup, proprietary remote device commissioning and management software client by same manufacturer, IEEE 802.3at compliant

##### USB: USB 2.0 host

#### Wireless Communications

##### Transceiver: IEEE 802.11a/b/g/n/ac/ax Wi-Fi 6E (2.4, 5 GHz and 6 GHz) 2x2 MIMO, static IP or dynamic IP via DHCP

##### Security: 64 and 128-bit WEP, WPA, WPA2, WPA3, WPA Enterprise and WPA2 Enterprise

##### Range: Up to 50 ft (15 m), subject to site-specific conditions

##### Bluetooth® Hardware: BLE, BT5.2, 33 ft (10 m) typical range

#### Streaming Decoder

##### Video Formats: H.265, H.264 (MPEG-4 part 10 AVC, MJPEG)

##### Audio Format: AAC Stereo

##### Bitrates: Up to 25 Mbps (20 Mbps maximum recommended)

##### Input Resolutions: Up to 3840x2160@30fps

##### Protocol: RTSP

#### Audio

##### Features: Built-in microphone and speakers, VoIP-based intercom between other touch screens by same manufacturer

Specifier Note:

The microphone may be defeated programmatically at any time.

#### Connectors

##### LAN PoE

###### (1) 8-pin RJ45 connector, female, with 2 LED indicators

###### 1000BASE-TX Ethernet port

###### PoE+ PD port

###### Green and yellow LEDs indicate Ethernet port status

##### USB

###### (1) USB Type C connector, female

###### USB 2.0 host port

###### USB 3.1 DisplayPort Alt Mode (for future release)

#### Power

##### PoE (Power over Ethernet)

###### IEEE 802.3at Type 2 compliant PoE+ PD (powered device)

###### Requests 30 W (PoE+ Class 4) from an 802.3at Type 2 PSE without LLDP

###### Requests 15.4 W (PoE Class 0) from an 802.3af (or 802.3at Type 1)

Specifier Note:

Enabling the USB port requires use of an 802.3at Type 2 PoE+ PSE.

### Compliance

#### Regulatory Model: M202404002

#### FCC Part 15 Class B, IC Class B, CE, UL® Listed for US and Canada/Intertek®

## 8.1 in. Tabletop Touch Screen

Specifier Note:

The Crestron TS-880R touch screen is ideal for residential applications and is designed to run Crestron Home® OS. The TS-880R features an 8.1 in. (204 mm) capacitive touch screen display. PoE and Wi-Fi® network connectivity and a comprehensive assortment of mounting accessories simplify new and retrofit installations.

### Basis of Design

#### Crestron TS-880R

Specifier Note:

TS-880R
https://www.crestron.com/Products/Catalog/Control-and-Management/User-Interfaces/Touchscreen/TS-880R-B

### Device Definition

#### Tabletop Touch Screen with the following capabilities and features:

##### Ability to serve as user interface for residential applications

##### Support for streaming video

##### Support for VoIP-based intercom between other touch screens by same manufacturer

##### Support for residential control operating system by same manufacturer

##### Ethernet LAN and Wi-Fi connectivity

##### Power over Ethernet (PoE) support

##### Enterprise-grade security (802.1X authentication, MDEP, TLS encryption, HTTPS connectivity, Active Directory® service integration)

### Device Architecture

#### Environmental

##### Temperature: 32° to 104°F (0° to 40° C)

##### Humidity: 10% to 95% RH (non-condensing)

##### Heat Dissipation: 44 BTU/hr

#### Construction

##### Housing: Die-cast housing, smooth black or white finish, glass with black or white surround

#### Dimensions

##### Height: 4.87 in. (124 mm)

##### Width: 7.93 in. (201 mm)

##### Depth: 4.53 in. (115 mm)

#### Weight: 1.76 lb. (799 g)

### Functions

#### Touch Screen Display

##### Display Type: TFT active matrix color LCD

##### Size: 8.1 in. (204 mm) diagonal

##### Aspect Ratio: 16:10 WXGA

##### Resolution: 1280 x 800 pixels

##### Brightness

###### Minimum: 350 nits (cd/m2)

###### Maximum: 400 nits (cd/m2)

##### Contrast: 1000:1

##### Color Depth: 24 bit, 16.7M colors

##### Illumination: Edge lit LED with auto brightness control

##### Viewing Angle: ±80° horizontal, ±80° vertical

##### Touch Screen: Projected capacitive, 5 point multitouch capable

#### Buttons

##### Virtual Buttons

###### (6) Virtual buttons in collapsible universal tool bar

###### Per button show/hide

###### Preconfigured with icons for Power, Home, Lights, Up, Down, and Microphone

##### Reset: (1) Pin hole on bottom for hardware reset

#### Graphics Engine

##### Dual scalable streaming video window

##### Native proprietary residential control operating system app by same manufacturer

##### Touch screen setup and diagnostics via web browser, cloud, or onscreen UI

##### Residential control operating system setup and diagnostics via app by same manufacturer

#### Languages

##### Onscreen Keyboard: Arabic, Chinese (Simplified), Croatian, Czech, Danish, Dutch, English (UK), English (US), Finnish, French (Canada), French (Switzerland), German, Hebrew, Hungarian, Italian, Japanese, Norwegian Bokmal, Polish, Portuguese, Russian, Serbian, Spanish, Swedish, Turkish

#### Memory

##### RAM: 8 GB LPDDR4

##### Storage: 40 GB eMMC pSLC

##### Maximum Project Size: 1000 MB

#### Wired Communications

##### Ethernet: 1000 Mbps, autoswitching, autonegotiating, autodiscovery, full/half duplex, TCP/IP, UDP/IP, CIP, DHCP, SSL, TLS, SSH, SFTP (SSH File Transfer Protocol), IEEE 802.1X, SNMP, IPv4 or IPv6, Active Directory® service authentication, HTTPS web browser setup, proprietary remote device commissioning and management software client by same manufacturer, IEEE 802.3at compliant

##### USB: USB 2.0 host

#### Wireless Communications

##### Transceiver: IEEE 802.11a/b/g/n/ac/ax Wi-Fi 6E (2.4, 5 GHz and 6 GHz) 2x2 MIMO, static IP or dynamic IP via DHCP

##### Security: 64 and 128-bit WEP, WPA, WPA2, WPA3, WPA Enterprise and WPA2 Enterprise

##### Range: Up to 50 ft (15 m), subject to site-specific conditions

##### Bluetooth® Hardware: BLE, BT5.2, 33 ft (10 m) typical range

#### Streaming Decoder

##### Video Formats: H.265, H.264 (MPEG-4 part 10 AVC, MJPEG)

##### Audio Format: AAC Stereo

##### Bitrates: Up to 25 Mbps (20 Mbps maximum recommended)

##### Input Resolutions: Up to 3840x2160@30fps

##### Protocol: RTSP

#### Audio

##### Features: Built-in microphone and speakers, VoIP-based intercom between other touch screens by same manufacturer

Specifier Note:

The microphone may be defeated programmatically at any time.

#### Connectors

##### LAN PoE

###### (1) 8-pin RJ45 connector, female, with 2 LED indicators

###### 1000BASE-TX Ethernet port

###### PoE+ PD port

###### Green and yellow LEDs indicate Ethernet port status

##### USB

###### (1) USB Type C connector, female

###### USB 2.0 host port

###### USB 3.1 DisplayPort Alt Mode (for future release)

#### Power

##### PoE (Power over Ethernet)

###### IEEE 802.3at Type 2 compliant PoE+ PD (powered device)

###### Requests 30 W (PoE+ Class 4) from an 802.3at Type 2 PSE without LLDP

###### Requests 15.4 W (PoE Class 0) from an 802.3af (or 802.3at Type 1)

Specifier Note:

Enabling the USB port requires use of an 802.3at Type 2 PoE+ PSE.

### Compliance

#### Regulatory Model: M202404005

#### FCC Part 15 Class B, IC Class B, CE, UL® Listed for US and Canada/Intertek®

## 10.1 in. Tabletop Touch Screen

Specifier Note:

The Crestron TS-1080R touch screen is ideal for residential applications and is designed to run Crestron Home® OS. The TS-1080R features a 10.1 in. (257 mm) capacitive touch screen display. PoE and Wi-Fi® network connectivity and a comprehensive assortment of mounting accessories simplify new and retrofit installations.

### Basis of Design

#### Crestron TS-1080R

Specifier Note:

TS-1080R
https://www.crestron.com/Products/Catalog/Control-and-Management/User-Interfaces/Touchscreen/TS-1080R-B

### Device Definition

#### Tabletop Touch Screen with the following capabilities and features:

##### Ability to serve as user interface for residential applications

##### Support for streaming video

##### Support for VoIP-based intercom between other touch screens by same manufacturer

##### Support for residential control operating system by same manufacturer

##### Ethernet LAN and Wi-Fi connectivity

##### Power over Ethernet (PoE) support

##### Enterprise-grade security (802.1X authentication, MDEP, TLS encryption, HTTPS connectivity, Active Directory® service integration)

### Device Architecture

#### Environmental

##### Temperature: 32° to 104°F (0° to 40° C)

##### Humidity: 10% to 95% RH (non-condensing)

##### Heat Dissipation: 44 BTU/hr

#### Construction

##### Housing: Die-cast housing, smooth black or white finish, glass with black or white surround

#### Dimensions

##### Height: 5.96 in. (151 mm)

##### Width: 9.68 in. (246 mm)

##### Depth: 4.53 in. (115 mm)

#### Weight: 2.23 lb. (464 g)

### Functions

#### Touch Screen Display

##### Display Type: TFT active matrix color LCD

##### Size: 10.1 in. (257 mm) diagonal

##### Aspect Ratio: 16:10 WXGA

##### Resolution: 1920 x 1200 pixels

##### Brightness

###### Minimum: 350 nits (cd/m2)

###### Maximum: 400 nits (cd/m2)

##### Contrast: 1000:1

##### Color Depth: 24 bit, 16.7M colors

##### Illumination: Edge lit LED with auto brightness control

##### Viewing Angle: ±80° horizontal, ±80° vertical

##### Touch Screen: Projected capacitive, 5 point multitouch capable

#### Buttons

##### Virtual Buttons

###### (6) Virtual buttons in collapsible universal tool bar

###### Per button show/hide

###### Preconfigured with icons for Power, Home, Lights, Up, Down, and Microphone

##### Reset: (1) Pin hole on bottom for hardware reset

#### Graphics Engine

##### Dual scalable streaming video window

##### Native proprietary residential control operating system app by same manufacturer

##### Touch screen setup and diagnostics via web browser, cloud, or onscreen UI

##### Residential control operating system setup and diagnostics via app by same manufacturer

#### Languages

##### Onscreen Keyboard: Arabic, Chinese (Simplified), Croatian, Czech, Danish, Dutch, English (UK), English (US), Finnish, French (Canada), French (Switzerland), German, Hebrew, Hungarian, Italian, Japanese, Norwegian Bokmal, Polish, Portuguese, Russian, Serbian, Spanish, Swedish, Turkish

#### Memory

##### RAM: 8 GB LPDDR4

##### Storage: 40 GB eMMC pSLC

##### Maximum Project Size: 1000 MB

#### Wired Communications

##### Ethernet: 1000 Mbps, autoswitching, autonegotiating, autodiscovery, full/half duplex, TCP/IP, UDP/IP, CIP, DHCP, SSL, TLS, SSH, SFTP (SSH File Transfer Protocol), IEEE 802.1X, SNMP, IPv4 or IPv6, Active Directory® service authentication, HTTPS web browser setup, proprietary remote device commissioning and management software client by same manufacturer, IEEE 802.3at compliant

##### USB: USB 2.0 host

#### Wireless Communications

##### Transceiver: IEEE 802.11a/b/g/n/ac/ax Wi-Fi 6E (2.4, 5 GHz and 6 GHz) 2x2 MIMO, static IP or dynamic IP via DHCP

##### Security: 64 and 128-bit WEP, WPA, WPA2, WPA3, WPA Enterprise and WPA2 Enterprise

##### Range: Up to 50 ft (15 m), subject to site-specific conditions

##### Bluetooth® Hardware: BLE, BT5.2, 33 ft (10 m) typical range

#### Streaming Decoder

##### Video Formats: H.265, H.264 (MPEG-4 part 10 AVC, MJPEG)

##### Audio Format: AAC Stereo

##### Bitrates: Up to 25 Mbps (20 Mbps maximum recommended)

##### Input Resolutions: Up to 3840x2160@30fps

##### Protocol: RTSP

#### Audio

##### Features: Built-in microphone and speakers, VoIP-based intercom between other touch screens by same manufacturer

Specifier Note:

The microphone may be defeated programmatically at any time.

#### Connectors

##### LAN PoE

###### (1) 8-pin RJ45 connector, female, with 2 LED indicators

###### 1000BASE-TX Ethernet port

###### PoE+ PD port

###### Green and yellow LEDs indicate Ethernet port status

##### USB

###### (1) USB Type C connector, female

###### USB 2.0 host port

###### USB 3.1 DisplayPort Alt Mode (for future release)

#### Power

##### PoE (Power over Ethernet)

###### IEEE 802.3at Type 2 compliant PoE+ PD (powered device)

###### Requests 30 W (PoE+ Class 4) from an 802.3at Type 2 PSE without LLDP

###### Requests 15.4 W (PoE Class 0) from an 802.3af (or 802.3at Type 1)

Specifier Note:

Enabling the USB port requires use of an 802.3at Type 2 PoE+ PSE.

### Compliance

#### Regulatory Model: M202404006

#### FCC Part 15 Class B, IC Class B, CE, UL® Listed for US and Canada/Intertek®

## 8.1 in. Room Scheduling Touch Screen

Specifier Note:

Crestron TSS-880 series touch screens are designed to enhance and simplify room scheduling across an enterprise. The TSS-880 installs near a meeting room or huddle space to provide a clear indication of its availability and scheduling and integrates seamlessly with a variety of popular scheduling services.

### Basis of Design

#### Crestron TSS-880

Specifier Note:

TSS-880
https://www.crestron.com/Products/Catalog/Scheduling/Room-Scheduling/Panel/TSS-880-B

### Device Definition

#### Wall Mount Touch Screen with the following capabilities and features:

##### Room scheduling via third party apps or via app from same manufacturer

##### Integrated light bar for at-a-glance indication of room availability

##### Ethernet LAN and Wi-Fi connectivity

##### Power over Ethernet (PoE) support

##### Enterprise-grade security (802.1X authentication, MDEP, TLS encryption, HTTPS connectivity, Active Directory® service integration)

##### Support for remote monitoring and managing software by same manufacturer

##### Support for room scheduling accessories by same manufacturer

##### Support for optional mounting kits for installation in a multitude of scenarios

### Device Architecture

#### Environmental

##### Temperature: 32° to 104°F (0° to 40° C)

##### Humidity: 10% to 95% RH (non-condensing)

##### Heat Dissipation: 44 BTU/hr

#### Construction

##### Housing: Die-cast housing, smooth black or white finish, glass with black or white surround

##### Mounting

###### Surface mount over a 2 or 3-gang US electrical box, 2-gang European (DIN 49073) electrical box, or 2-gang UK (BS 4662) electrical box

###### Lectern mount over a 2 in. H x 3-3/4 in. W (51 mm H x 96 mm W) cutout; 1-3/8 in. (35 mm) minimum mounting depth

###### Wall mount, surface mount, and rack mount options available separately

#### Dimensions

##### Height: 4.87 in. (124 mm)

##### Width: 7.93 in. (201 mm)

##### Depth: 0.50 in. (13 mm)

#### Weight: 14.6 oz. (410 g)

### Functions

#### Touch Screen Display

##### Display Type: TFT active matrix color LCD

##### Size: 8.1 in. (204 mm) diagonal

##### Aspect Ratio: 16:10 WXGA

##### Resolution: 1280 x 800 pixels

##### Brightness

###### Minimum: 350 nits (cd/m2)

###### Maximum: 400 nits (cd/m2)

##### Contrast: 1000:1

##### Color Depth: 24 bit, 16.7M colors

##### Illumination: Edge lit LED with auto brightness control

##### Viewing Angle: ±80° horizontal, ±80° vertical

##### Touch Screen: Projected capacitive, 5 point multitouch capable

#### Buttons

##### Virtual Buttons

###### (6) Virtual buttons in collapsible universal tool bar

###### Per button show/hide

###### Preconfigured with icons for Power, Home, Lights, Up, Down, and Microphone

##### Reset: (1) Pin hole on bottom for hardware reset

#### Room Scheduling Support

##### Scheduling app by same manufacturer: Microsoft Exchange Server® and Microsoft 365® software, Google Calendar™ calendaring app and G Suite™ software, CollegeNET® 25Live® software, and Ad Astra™ software

##### Third-Party Apps: Includes support for various third-party scheduling applications

Specifier Note:

Room scheduling mode requires the TSS-880 to be designated exclusively for room scheduling use, which precludes use of certain features and functions described in this spec document. Additionally, CollegeNET® 25Live® scheduling software and Ad Astra™ software do not support scheduling ad hoc meetings from the touch screen.

Refer to the firmware release notes for a list of all scheduling apps that are supported by that firmware release.

Additional subscriptions and/or licenses may be required. Refer to each provider’s website for details about the capabilities and requirements of its scheduling application and services. Third party apps typically run on the TSS-880 as they do on a tablet device. The TSS-880 runs only one app, which is selected at setup. Only the apps approved and delivered by Crestron can run on the TSS-880.

#### Memory

##### RAM: 8 GB LPDDR4

##### Storage: 40 GB eMMC pSLC

##### Maximum Project Size: 1000 MB

#### Wired Communications

##### Ethernet: 1000 Mbps, autoswitching, autonegotiating, autodiscovery, full/half duplex, TCP/IP, UDP/IP, CIP, DHCP, SSL, TLS, SSH, SFTP (SSH File Transfer Protocol), IEEE 802.1X, SNMP, IPv4 or IPv6, Active Directory® service authentication, HTTPS web browser setup, proprietary remote device commissioning and management software client by same manufacturer, IEEE 802.3at compliant

##### USB: USB 2.0 host for room scheduling accessories

#### Wireless Communications

##### Transceiver: IEEE 802.11a/b/g/n/ac/ax Wi-Fi 6E (2.4, 5 GHz and 6 GHz) 2x2 MIMO, static IP or dynamic IP via DHCP

##### Security: 64 and 128-bit WEP, WPA, WPA2, WPA3, WPA Enterprise and WPA2 Enterprise

##### Range: Up to 50 ft (15 m), subject to site-specific conditions

##### Bluetooth® Hardware: Bluetooth beacon by same manufacturer

#### Streaming Decoder

##### Video Formats: H.265, H.264 (MPEG-4 part 10 AVC, MJPEG)

##### Audio Format: AAC Stereo

##### Bitrates: Up to 25 Mbps (20 Mbps maximum recommended)

##### Input Resolutions: Up to 3840x2160@30fps

##### Protocol: RTSP

#### Audio

##### Features: Built-in speakers

#### Connectors

##### LAN PoE

###### (1) 8-pin RJ45 connector, female, with 2 LED indicators

###### 1000BASE-TX Ethernet port

###### PoE+ PD port

###### Green and yellow LEDs indicate Ethernet port status

##### USB

###### (1) USB Type C connector, female

###### USB 2.0 host port

#### Power

##### PoE (Power over Ethernet)

###### IEEE 802.3at Type 2 compliant PoE+ PD (powered device)

###### Requests 30 W (PoE+ Class 4) from an 802.3at Type 2 PSE without LLDP

###### Requests 15.4 W (PoE Class 0) from an 802.3af (or 802.3at Type 1)

Specifier Note:

Enabling the USB port requires use of an 802.3at Type 2 PoE+ PSE.

### Compliance

#### Regulatory Model: M202404001

#### FCC Part 15 Class B, IC Class B, CE, UL® Listed for US and Canada/Intertek®

# EXECUTION

NOT USED in this Guide Specification.  Specifier shall Specify PART 3 On-Site work as needed.

# APPENDICES

## SPECIFIED PRODUCTS

Specifier Note: This Article includes Crestron products specified in this Guide Specification document.  This Article is for reference only and should not be required in actual project manual unless included in an overall system equipment list.

### Crestron TSW-880

### Crestron TSW-1080

### Crestron TS-880

### Crestron TS-1080

### Crestron TSW-880R

### Crestron TSW-1080R

### Crestron TS-880R

### Crestron TS-1080R

### Crestron TSS-880