SECTION 27 41 16

INTEGRATED AUDIO-VIDEO SYSTEMS AND EQUIPMENT

GUIDE SPECIFICATION

Specifier: The Specifier/Design Professional is responsible for the accuracy of all project specifications, including system application and coordination with related sections.  This guide specification is provided as a convenience and requires editing to match actual project requirements.  CRESTRON ELECTRONICS, INC. SHALL NOT BE LIABLE FOR ANY DAMAGES ARISING OUT OF THE USE OF ANY OF ITS GUIDE SPECIFICATIONS.  For Crestron design assistance and design review please contact Sales Support Services Department at   800.237.2041 or techsales@crestron.com.

Specifier:  Please see PART 4 for a listing of products specified in this Guide Specification.

Table of Contents

PART 1	GENERAL	4

PART 2	PRODUCTS	4

2.1	CEILING SPEAKER TYPE 1:	4

A.	Basis of Design:	4

B.	Device Architecture	4

C.	Functions	5

D.	Compliance	6

2.2	CEILING SPEAKER TYPE 2:	6

A.	Basis of Design:	6

B.	Device Architecture	6

C.	Functions	7

D.	Compliance	8

2.3	CEILING SPEAKER TYPE 3:	8

A.	Basis of Design:	8

B.	Device Architecture	8

C.	Functions	9

D.	Compliance	10

2.4	CEILING SPEAKER TYPE 4:	10

A.	Basis of Design:	10

B.	Device Architecture	10

C.	Functions	11

D.	Compliance	12

2.5	CEILING SPEAKER TYPE 5:	12

A.	Basis of Design Product:	12

B.	Device Architecture	12

C.	Functions	13

D.	Compliance	14

PART 3	EXECUTION	14

PART 4	APPENDICES	14

4.1	SPECIFIED PRODUCTS	14

A.	Crestron SAROS ICE4T	14

B.	Crestron SAROS ICE6T	14

C.	Crestron SAROS ICE6LPT	14

D.	Crestron SAROS ICE8T	15

E.	Crestron SAROS ICE8LPT	15

GENERAL

NOT USED in this Guide Specification.  Specifier shall Specify PART 1 administrative and procedural requirements as needed.

PRODUCTS

CEILING SPEAKER TYPE 1:

Specifier Note: The Saros Express ICE4T model is a 2-way in-ceiling speaker featuring a 4 inch woofer and wide-dispersion, horn-loaded 3/4 inch dome tweeter. Saros® Express speakers by Crestron® deliver easy installation and excellent performance in a range of popular sizes for cost-sensitive commercial applications.

Basis of Design:

Crestron SAROS ICE4T

Device Architecture

Enclosure material: Zinc-plated steel, plenum rated.

Baffle: ABS UL94V-0 plastic.

Grille:

Steel

Magnetically held frameless design

White or black textured finish, paintable

Mounting:

Flush ceiling mount.

Four integral two-step toggle mounting clamps.

Maximum ceiling thickness: 2.4 inches (61 mm).

Cutout diameter: 7.75 inches (197 mm).

Minimum ceiling cavity depth: 6.9 inches (176 mm).

Environmental Operating Conditions:

Temperature: -2° to 120°F (-19° to 49°C)

Humidity: 5% to 95% RH (non-condensing)

Included Accessories:

Support bridge for tile ceiling installation.

Adjustable for off center installation.

Optional Accessories:

Safety Tether

Have a minimum of two (2) safety wire installation points.

Black Grille

Termination:

4-pin 5 mm screw down conductor detachable terminal block with speaker input and parallel pass-through connections.

Maximum wire size: 12 AWG.

Conduit knockouts:

One 0.5-inch knockout.

One 0.75-inch knockout.

Dimensions:

Diameter: 8.53 inches (217 mm), not including toggles.

Depth: 7.13 inches (181 mm).

Weight: 7.0 lb. (3.2 kg)

Drivers:

4-inch (102 mm) treated paper woofer with ring mode decoupled cloth surround and steel basket

.79-inch (20 mm) treated cloth dome horn loaded tweeter

Functions

Speaker unit shall be equipped for Low Impedance 8 Ω or High Impedance 70 V or 100 V operation.

Accessible front mounted low impedance, high impedance selector switch.

High Impedance Line Transformer:

Four level taps at 70 V:

3.75 Watts

7.5 Watts

15 Watts

30 Watts

Three level taps at 100 V:

7.5 Watts

15 Watts

30 Watts

Low Impedance transformer by-passed:

8 Ω

Sensitivity: 89 dB at 1 W, 1 m away.

Frequency Response: Within plus or minus 3 dB from 85 to 20,000 Hz.

Frequency Range: Within minus 10 dB from 75 to 20,000 Hz.

Crossover Frequency: 2 kHz

Nominal Coverage: 95 degrees conical.

Power Handling: 40 Watts Program at 8 Ω.

Compliance

UL Listed for US & Canada

UL 1480

UL 2043

CE

FCC Part 15 Class B

CEILING SPEAKER TYPE 2:

Specifier Note: The Saros Express ICE6T model is a 2-way in-ceiling speaker featuring a 6-1/2 inch woofer and wide-dispersion, horn-loaded 1 inch dome tweeter. Saros® Express speakers by Crestron® deliver easy installation and excellent performance in a range of popular sizes for cost-sensitive commercial applications.

Basis of Design:

Crestron SAROS ICE6T

Device Architecture

Enclosure material: Zinc-plated steel, plenum rated.

Baffle: ABS UL94V-0 plastic.

Grille:

Steel

Magnetically held frameless design

White or black textured finish, paintable

Mounting:

Flush ceiling mount.

Four integral two-step toggle mounting clamps.

Maximum ceiling thickness: 2.4 inches (61 mm).

Cutout diameter: 9.875 inches (250 mm).

Minimum ceiling cavity depth: 8.1 inches (206 mm).

Environmental Operating Conditions:

Temperature: -2° to 120°F (-19° to 49°C)

Humidity: 5% to 95% RH (non-condensing)

Included Accessories:

Support bridge for tile ceiling installation.

Adjustable for off center installation.

Optional Accessories:

Safety Tether

Have a minimum of two (2) safety wire installation points.

Black Grille

Termination:

4-pin 5 mm screw down conductor detachable terminal block with speaker input and parallel pass-through connections.

Maximum wire size: 12 AWG.

Conduit knockouts:

One 0.5-inch knockout.

One 0.75-inch knockout.

Dimensions:

Diameter: 10.57 inches (269 mm), not including mounting dogs.

Depth: 8.31 inches (211 mm).

Weight: 10.0 lb. (4.5 kg)

Drivers:

6-inch (165 mm) treated paper woofer with ring mode decoupled cloth surround and steel basket

- .98-inch (25 mm) treated cloth dome horn loaded tweeter

Functions

Speaker unit shall be equipped for Low Impedance 8 Ω or High Impedance 70 V or 100 V operation.

Accessible front mounted low impedance, high impedance selector switch.

High Impedance Line Transformer:

Four level taps at 70 V:

3.75 Watts

7.5 Watts

15 Watts

30 Watts

Three level taps at 100 V:

7.5 Watts

15 Watts

30 Watts

Low Impedance transformer by-passed:

8 Ω

Sensitivity: 89 dB at 1 W, 1 m away.

Frequency Response: Within plus or minus 3 dB from 75 to 20,000 Hz.

Frequency Range: Within minus 10 dB from 60 to 20,000 Hz.

Crossover Frequency: 2.5 kHz.

Nominal Coverage: 95 degrees conical.

Power Handling: 60 Watts Program at 8 Ω.

Compliance

UL Listed for US & Canada

UL 1480

UL 1480A

UL 2043

CE

FCC Part 15 Class B

CEILING SPEAKER TYPE 3:

Specifier Note: The Saros Express ICE6LPT model is a 2-way in-ceiling speaker featuring a 6-1/2 inch woofer and wide-dispersion, horn-loaded 1 inch dome tweeter. Its low-profile enclosure allows for installation in constricted ceiling spaces as shallow as 5-1/8 inches (130 mm). Saros® Express speakers by Crestron® deliver easy installation and excellent performance in a range of popular sizes for cost-sensitive commercial applications.

Basis of Design:

Crestron SAROS ICE6LPT

Device Architecture

Enclosure material: Zinc-plated steel, plenum rated with side entry cable clamp

Baffle:

ABS UL94V-0 plastic.

Grille:

Steel

Magnetically held frameless design

White textured finish, paintable

Mounting:

Flush ceiling mount.

Four integral two-step toggle clamps

Maximum ceiling thickness: 2.4 inches.

Recommended cutout diameter: 9.875 inches.

Minimum ceiling cavity depth: 5.2 inches.

Environmental Operating Conditions:

Temperature: -2° to 120°F (-19° to 49°C)

Humidity: 5% to 95% RH (non-condensing)

Included Accessories:

Support bridge for tile ceiling installation.

Adjustable for off center installation.

Optional Accessories:

Safety Tether

Have a minimum of two (2) safety wire installation points.

Black Grille

Termination:

4-pin 5 mm screw down conductor detachable terminal block with speaker input and parallel pass-through connections.

Maximum wire size: 12 AWG.

Dimensions:

Diameter: 10.64 inches, not including mounting dogs.

Depth: 5.30 inches.

Weight: 9.2 lb.

Drivers:

6.5-inch (165 mm) treated paper woofer with ring mode decoupled cloth surround and steel basket

.98-inch (25 mm) treated cloth dome horn loaded tweeter

Functions

Speaker unit shall be equipped for Low Impedance 8 Ω or High Impedance 70 V or 100 V operation.

Accessible front mounted low impedance, high impedance selector switch.

High Impedance Line Transformer:

Four level taps at 70 V:

3.75 Watts

7.5 Watts

15 Watts

30 Watts

Three level taps at 100 V:

7.5 Watts

15 Watts

30 Watts

Low Impedance transformer:

8 Ω

Sensitivity: 89 dB at 1 W, 1 m away.

Frequency Response: Within plus or minus 3 dB from 85 to 20,000 Hz.

Frequency Range: Within minus 10 dB from 70 to 20,000 Hz.

Crossover frequency: 2.5 kHz.

Nominal Coverage: 95 degrees conical.

Power Handling: 60 W program at 8 Ω

Compliance

UL Listed for US & Canada

UL 1480

UL 1480A

UL 2043

CE

FCC Part 15 Class B

CEILING SPEAKER TYPE 4:

Specifier Note: The Saros Express ICE8T model is a 2-way in-ceiling speaker featuring an 8 inch woofer and wide-dispersion, horn-loaded 1 inch dome tweeter. Saros® Express speakers by Crestron® deliver easy installation and excellent performance in a range of popular sizes for cost-sensitive commercial applications.

Basis of Design:

Crestron SAROS ICE8T

Device Architecture

Enclosure material: Zinc-plated steel, plenum rated.

Baffle: ABS UL94V-0 plastic.

Grille:

Steel

Magnetically held frameless design

White textured finish, paintable

Mounting:

Flush ceiling mount.

Four integral two-step toggle mounting clamps.

Maximum ceiling thickness: 2.4 inches (61 mm).

Cutout diameter: 11.3125 inches (287 mm).

Minimum ceiling cavity depth: 8.9 inches (226 mm).

Environmental Operating Conditions:

Temperature: -2° to 120°F (-19° to 49°C)

Humidity: 5% to 95% RH (non-condensing)

Included Accessories:

Support bridge for tile ceiling installation.

Adjustable for off center installation.

Optional Accessories:

Safety Tether

Have a minimum of two (2) safety wire installation points.

Black Grille

Termination:

4-pin 5 mm screw down conductor detachable terminal block with speaker input and parallel pass-through connections.

Maximum wire size: 12 AWG.

Conduit knockouts:

One 0.5-inch knockout.

One 0.75-inch knockout.

Dimensions:

Diameter: 12.07 inches (307 mm), not including toggles.

Depth: 9.10 inches (231 mm).

Weight: 12.2 lb (5.7 kg).

Drivers:

8-inch (203 mm) treated paper woofer with ring mode decoupled cloth surround and steel basket

.98-inch (25 mm) treated cloth dome horn loaded tweeter

Functions

Speaker unit shall be equipped for Low Impedance 8 Ω or High Impedance 70 V or 100 V operation.

Accessible front mounted low impedance, high impedance selector switch.

High Impedance Line Transformer:

Five level taps at 70 V:

3.75 Watts

7.5 Watts

15 Watts

30 Watts

60 Watts

Four level taps at 100 V:

7.5 Watts

15 Watts

30 Watts

60 Watts

Low Impedance transformer by-passed:

8 Ω

Sensitivity: 89 dB at 1 W, 1 m away.

Frequency Response: Within plus or minus 3 dB from 70 to 20,000 Hz.

Frequency Range: Within minus 10 dB from 55 to 20,000 Hz.

Crossover Frequency: 2.5 kHz

Nominal Coverage: 95 degrees conical.

Power Handling: 75 Watts Program at 8 Ω.

Compliance

UL Listed for US & Canada

UL 1480

UL 1480A

UL 2043

CE

FCC Part 15 Class B

CEILING SPEAKER TYPE 5:

Specifier Note: The Saros Express ICE8LPT model is a 2-way in-ceiling speaker featuring an 8 inch woofer and wide-dispersion, horn-loaded 1 inch dome tweeter. Its low-profile enclosure allows for installation in constricted ceiling spaces as shallow as 5-1/8 inches (130 mm). Saros® Express speakers by Crestron® deliver easy installation and excellent performance in a range of popular sizes for cost-sensitive commercial applications.

Basis of Design Product:

Crestron SAROS ICE8LPT

Device Architecture

Enclosure material: Zinc-plated steel, plenum rated with side entry cable clamp

Baffle: ABS UL94V-0 plastic.

Grille:

Steel

Magnetically held frameless design

White textured finish, paintable

Mounting:

Flush ceiling mount.

Four integral two-step toggle clamps

Maximum ceiling thickness: 2.4 inches (61 mm).

Recommended cutout diameter: 11.3125 inches (287 mm).

Minimum ceiling cavity depth: 5.2 inches (130 mm).

Environmental Operating Conditions:

Temperature: -2° to 120°F (-19° to 49°C)

Humidity: 5% to 95% RH (non-condensing)

Included Accessories:

Support bridge for tile ceiling installation.

Adjustable for off center installation.

Optional Accessories:

Safety Tether

Have a minimum of two (2) safety wire installation points.

Black Grille

Termination:

4-pin 5 mm screw down conductor detachable terminal block with speaker input and parallel pass-through connections.

Maximum wire size: 12 AWG.

Dimensions:

Diameter: 12.13 inches (308 mm), not including mounting dogs

Depth: 5.30 inches (135 mm).

Weight: 10.8 lb. (4.9 kg)

Drivers:

8-inch (203 mm) treated paper woofer with ring mode decoupled cloth surround and steel basket

.98-inch (25 mm) treated cloth dome horn loaded tweeter

Functions

Speaker unit shall be equipped for Low Impedance 8 Ω or High Impedance 70 V or 100 V operation.

Accessible front mounted low impedance, high impedance selector switch.

High Impedance Line Transformer:

Four level taps at 70 V:

3.75 Watts

7.5 Watts

15 Watts

30 Watts

Four level taps at 100 V:

7.5 Watts

15 Watts

30 Watts

Low Impedance transformer:

8 Ω

Sensitivity: 89 dB at 1 W, 1 m away.

Frequency Response: Within plus or minus 3 dB from 80 to 20,000 Hz.

Frequency Range: Within minus 10 dB from 65 to 20,000 Hz.

Crossover frequency: 2.5 kHz.

Nominal Coverage: 95 degrees conical.

Power Handling: 60 W program at 8 Ω

Compliance

UL Listed for US & Canada

UL 1480

UL 1480A

UL 2043

CE

FCC Part 15 Class B

EXECUTION

NOT USED in this Guide Specification.  Specifier shall Specify PART 3 On-Site work as needed.

APPENDICES

SPECIFIED PRODUCTS

Crestron SAROS ICE4T

Crestron SAROS ICE6T

Crestron SAROS ICE6LPT

Crestron SAROS ICE8T

Crestron SAROS ICE8LPT