SECTION 27 41 16

INTEGRATED AUDIO-VIDEO SYSTEMS AND EQUIPMENT

GUIDE SPECIFICATION

Specifier: The Specifier/Design Professional is responsible for the accuracy of all project specifications, including system application and coordination with related sections.  This guide specification is provided as a convenience and requires editing to match actual project requirements.  CRESTRON ELECTRONICS, INC. SHALL NOT BE LIABLE FOR ANY DAMAGES ARISING OUT OF THE USE OF ANY OF ITS GUIDE SPECIFICATIONS.  For Crestron design assistance and design review please contact Sales Support Services Department at   800.237.2041 or techsales@crestron.com.

Specifier: Please see PART 4 for a listing of products specified in this Guide Specification.

Table of Contents

1	GENERAL	3

2	PRODUCTS	3

2.1	Network AV Switching Appliance	3

2.1.1	Basis of Design	3

2.1.2	Device Definition	3

2.1.3	Device Architecture	4

2.1.4	Functions	4

3	EXECUTION	7

4	APPENDICES	7

4.1	SPECIFIED PRODUCTS	7

4.1.1	Crestron DM-NVX-DIR2	7

# GENERAL

NOT USED in this Guide Specification.  Specifier shall Specify PART 1 administrative and procedural requirements as needed.

# PRODUCTS

## Network AV Switching Appliance

Specifier Note:

The Crestron DM-NVX-DIR2 is an enterprise-grade network appliance that facilitates configuration, control, and management of a large-scale AV network using DM NVX® encoder and decoder endpoints. The DM-NVX-DIR2 virtually emulates the functionality of a traditional hardware-based DM® matrix switcher, routing high-quality 4K streaming AV signals throughout a room, building, or campus.

### Basis of Design

#### Crestron DM-NVX-DIR2

Specifier Note:

DM-NVX-DIR2
https://www.crestron.com/Products/Catalog/AV-Over-IP/DM-NVX-AV-Over-IP/Management/DM-NVX-DIR2

### Device Definition

#### Network AV Switching Appliance with the following capabilities and features:

##### Network AV system configuration, management, and signal routing

##### Compatible with Network AV system endpoints by same manufacturer

##### Support for up to 100 endpoints and 20 domains

##### Interdomain routing

##### Web-based graphical user interface

##### Full programmable control of virtual matrices and physical endpoints

##### Automatic endpoint device discovery

##### Multicast address control

##### Credential management of endpoints

##### Custom naming and search tools

##### XML device map file import and export

##### Built-in logging

##### Support for multiple control systems by same manufacturer

##### External power adapter support

### Device Architecture

#### Environmental

##### Operating Temperature: 50 to 95°F (10 to 35° C)

##### Storage Temperature: -40 to 149°F (-40 to 65° C)

##### Heat Dissipation: 42.9 BTU/hr (short idle)

#### Construction

##### Enclosure: Metal, plastic

##### Mounting: Freestanding, optional VESA® mount and Kensington® lock capabilities

#### Dimensions

##### Height: 1.40 in. (36 mm)

##### Width: 7.20 in. (183 mm)

##### Depth: 7.00 in. (178 mm)

#### Weight: 2.87 lb (1.30 kg)

### Functions

#### Computer

##### Model: Dell® OptiPlex® 7080 Micro Desktop computer

##### Processor: Intel Core® i5-10500T CPU @ 2.30 GHz

##### RAM: 8 GB DDR4 2666 MT/s

##### Storage: 256 GB SSD

##### Graphics: Intel® UHD Graphics 630

##### Network: Intel I219-LM 100/1000 Mbps Ethernet

##### Operating System: AlmaLinux OS® 9.5 software

#### Device Support

##### Endpoints: Supports 100 AV-over-IP endpoints by same manufacturer, each configured as an encoder or decoder

##### Domains: Supports 20 domains (allows grouping of endpoints in up to 20 individual subsystems)

#### Communications

##### Ethernet: 100/1000 Mbps, auto-switching, auto-negotiating, auto-discovery, full/half duplex, TCP/IP, UDP/IP, CIP, DHCP, SSL, TLS, SSH, IPv4, HTTPS web browser setup and control, integration with control system by same manufacturer

#### Buttons and Indicators

##### Power: (1) Push button with LED backlight; For power on/off and reset

##### LAN: (2) LEDs on LAN port; Indicate Ethernet link status and activity

#### Connectors

##### Audio (Front): (1) 3.5 mm bidirectional line level audio connector (not used)

##### Line Out (Front): (1) 3.5 mm stereo line level output audio connector (not used)

##### USB-C (Front): (1) USB Type-C® 3.2 (Gen 2) connector, female (not used)

##### SSUSB (Front)

###### (1) USB Type-A 3.2 (Gen 2) connector, with PowerShare, female

###### Connects to a keyboard for optional command line configuration

##### Antenna: (2) Wireless antenna slots (not used)

##### HDMI

###### (1) HDMI® connector, female

###### Connects to a monitor for optional command line configuration

##### LAN

###### (1) 8-pin RJ45 connector, female

###### 100BASE-TX/1000BASE-T Ethernet port

##### SSUSB 3.2 Gen 1

###### (2) USB Type-A 3.2 (Gen 1) connectors, female

###### Provides one port with Smart Power

###### Connects to a keyboard or mouse for optional command line configuration

###### Ethernet adapter by same manufacturer can be connected for additional Ethernet ports

##### SSUSB 3.2 Gen 2

###### (2) USB Type-A 3.2 (Gen 2) connectors, female

###### Connects to a keyboard or mouse for optional command line configuration

###### Ethernet adapter by same manufacturer can be connected for additional Ethernet ports

##### Kensington Lock: (1) Slot for optional Kensington® lock (not included)

##### DisplayPort

###### (2) DisplayPort™ 1.4 connectors, female

###### Connects to a monitor for optional command line configuration

##### 19.5 VDC

###### (1) DC power connector

###### 19.5 VDC power input

###### For included power adapter

#### Power

##### Power Adapter (included)

###### Input: 100-240 VAC, 50/60 Hz

###### Output: 130 W @ 19.5 VDC

# EXECUTION

NOT USED in this Guide Specification.  Specifier shall Specify PART 3 On-Site work as needed.

# APPENDICES

## SPECIFIED PRODUCTS

Specifier Note: This Article includes Crestron products specified in this Guide Specification document.  This Article is for reference only and should not be required in actual project manual unless included in an overall system equipment list.

### Crestron DM-NVX-DIR2