# SECTION 23 09 23

DIRECT-DIGITAL CONTROL SYSTEM FOR HVAC

Major Equipment Specified in this section:

Heating and Cooling Thermostat: Crestron CHV-TSTATEX-FCU

Table of Contents

PART 1	GENERAL	3

1.1	SUMMARY	3

A.	Section Includes	3

B.	Related Requirements:	3

PART 2	PRODUCTS	3

2.1	HEATING AND COOLING THERMOSTAT	3

A.	Manufacturers	3

B.	Basis of Design Product:	4

C.	Minimum Characteristics:	4

D.	Communication:	4

E.	Mounting	4

2.2	SYSTEM FUNCTIONS	4

A.	Control Functions:	4

B.	Climate control functions:	4

C.	Temperature sensors.	5

D.	Operation modes:	5

2.3	USER INTERFACE	5

A.	Controls	5

B.	Built-in LCD status display	5

2.4	INTEGRATION AND COMMUNICATION	5

A.	Direct Communication:	5

B.	BACnet IP Integration:	5

END OF SECTION 23 09 23	5

# SECTION 23 09 23

DIRECT-DIGITAL CONTROL SYSTEM FOR HVAC

Specifier: The Specifier/Design Professional is responsible for the accuracy of all project specifications, including system application and coordination with related sections.  This guide specification is provided as a convenience and requires editing to match actual project requirements.  CRESTRON ELECTRONICS, INC. SHALL NOT BE LIABLE FOR ANY DAMAGES ARISING OUT OF THE USE OF ANY OF ITS GUIDE SPECIFICATIONS.  For Crestron design assistance and design review please contact Sales Support Services Department at   800.237.2041 or techsales@crestron.com.

GENERAL

SUMMARY

Section Includes

Heating and Cooling Thermostat with wireless control processor communication capability.  Thermostat integrates with control processor and networked BACnet IP servers and devices for a complete automated system.

Related Requirements:

Section 12 24 13 Roller Window Shades

Section 25 08 00 Commissioning of Integrated Automation

Section 25 10 00 Integrated Automation Network Equipment

Section 25 11 13 Integrated Automation Network Servers

Section 25 13 13 Integrated Automation Control and Monitoring Network Supervisory Control

Section 25 13 19 Integrated Automation Control and Monitoring Network Interoperability

Section 25 15 16 Integrated Automation Software for Control and Monitoring Networks

Section 25 50 00 Integrated Automation Facility Controls

Section 26 05 00 Common Work Results For Electrical

Section 26 27 26 Wiring Devices

Section 26 51 00 Interior Lighting

Section 27 15 00 Communications Horizontal Cabling

Section 27 41 00 Audio-Video Systems

PRODUCTS

HEATING AND COOLING THERMOSTAT

Manufacturers

Basis-of-Design Manufacturer:  Subject to compliance with requirements, provide products of Crestron Electronics, Inc., Rockleigh, NJ 07647,  Phone (800)237-2041, Fax: (201)767-1903, www.crestron.com [or comparable products from a single manufacturer approved by Architect prior to bidding], with the following components and characteristics.

Basis of Design Product:

Crestron CHV-TSTATEX-FCU

Minimum Characteristics:

Four pipe fan coil system control:

Relay control of heat pipe and cool pipe valves.

Relay control of High/Medium/Low fan speeds.

Automatic fan mode.

Automatic heating and cooling mode.

Communication:

Bidirectional wireless communication capability with any or all of the following:

Control processor

Control processor and integrated BACnet devices.

Control processor and networked management servers.

Mounting

1-gang horizontally mounted standard electrical box.

SYSTEM FUNCTIONS

Control Functions:

Heating and cooling operation modes:

Off

Heat

Cool

Auto

Fan speed:

Auto

High

Medium

Low

Climate control functions:

Separate heating and cooling set points.

Adjustable anticipators.

Increase circulation continuous fan operation mode.

Temperature sensors.

Built-in temperature sensor.

Support for two remote temperature sensors.

Operation modes:

Stand-alone

Separate master control processor integration.

Fail-safe mode – master controller integrated with stand-alone as secondary mode.

USER INTERFACE

Controls

Operation mode select.

Fan speed select.

Temperature set point.

Built-in LCD status display

Current Temperature.

Set Point Temperature.

System Mode.

Fan Indicator.

INTEGRATION AND COMMUNICATION

Direct Communication:

The thermostat shall be capable of communicating wirelessly with a master control processor using a single native communication protocol.  Protocol translators, and protocol adaptors, shall not be permitted.

BACnet IP Integration:

The thermostat and control processor shall be capable of bidirectional BACnet IP communications with networked BACnet IP devices and equipment.

END OF SECTION 23 09 23