# SECTION 27 41 16

INTEGRATED AUDIO-VIDEO SYSTEMS AND EQUIPMENT

Equipment Specified in this section:

DM-MD8X8, DM-MD16X16, DM-MD32X32

DM-MD8X8-RPS, DM-MD16X16-RPS, DM-MD32X32-RPS

DM-MD64X64, DM-MD128X128

DMB-4K-I-HD

DMB-4K-I-HD-DNT

DMB-4K-I-C

DMB-4K-O-C

DMB-4K-O-HD

DMB-4K-O-HD-DNT

DMB-I-S

DMB-I-S2

DMB-O-S

DMB-O-S2

DMC-4K-C-DSP-HDCP2

DMC-4K-C-HDCP2

DMC-4K-CO-HD-HDCP2

DMC-4K-HD-DSP-HDCP2

DMC-4K-HD-HDCP2

DMC-4K-HDO

DMC-DVI

DMC-HDO

DMCI

DMC-VID-RCA-A

DMC-VID-RCA-D

DMC-S

DMC-S2

DMC-S2-DSP

DMC-S2O-HD

DMC-SDI

DMC-S-DSP

DMC-SO-HD

DMC-STRO

DMC-VGA

DMC-VID4

DMC-VID-BNC

DM-RMC-100-S

DM-RMC-150-S

DM-RMC-200-C

DM-RMC-200-S

DM-RMC-200-S2

DM-RMC-4K-100-C

DM-RMC-4K-100-C-1G

DM-RMC-4K-SCALER-C

DM-RMC-4K-SCALER-C-DSP

DM-RMC-SCALER-C

DM-RMC-SCALER-S

DM-RMC-SCALER-S2

DM-TX-200-C-2G

DM-TX-201C

DM-TX-201-S

DM-TX-201-S2

DM-TX-401-C

DM-TX-401-S

DM-TX-401-S2

DM-TX-4K-100-C-1G

DM-TX-4K-202-C

DM-TX-4K-302-C

Table of Contents

PART 1	GENERAL	10

1.1	SUMMARY	10

A.	Section Includes	10

PART 2	PRODUCTS	10

2.1	DIGITAL SIGNAL MATRIX SWITCHER AND TRANSMISSION SYSTEM	10

A.	Manufacturers:	10

B.	System Description	10

C.	System Capacity	10

D.	System Operation	11

E.	Integrated Operating System Software Architecture	14

2.2	SYSTEM MAIN CHASSIS TYPE 1	15

A.	Manufacturers:	15

B.	Description	15

C.	Basis-of-Design Products:	15

D.	System Capacity	16

E.	Input and Output Modules	16

F.	Front Panel Control Interface	16

G.	Video Preview	16

H.	Web Browser Interface	16

I.	Matrix Power	16

J.	Matrix Cooling	16

2.3	4K HDMI INPUT BLADE	17

A.	Description	17

B.	Basis of design product: Crestron DMB-4K-I-HD DigitalMedia Input Blade.	17

C.	Modular Design:  Input Blade shall be a modular unit with the following characteristics:	17

D.	HDMI inputs:	17

E.	Analog Audio Insert:	18

F.	Video outputs:	18

G.	Audio/surround formats:	18

2.4	4K HDMI AND DANTE INPUT BLADE	18

A.	Description	18

B.	Basis of design product: Crestron DMB-4K-I-HD-DNT DigitalMedia Input Blade.	18

C.	Modular Design:  Input Blade shall be a modular unit with the following characteristics:	18

D.	HDMI inputs:	19

E.	Analog Audio Insert:	19

F.	Dante Audio Input and Send	19

G.	Dante Communication	19

H.	Video outputs:	19

I.	Audio/surround formats:	20

2.5	SINGLE UTP/STP CABLE 4K TRANSMISSION INPUT BLADE	20

A.	Description	20

B.	Basis of design product: Crestron DMB-4K-I-C DigitalMedia input blade.	20

C.	Transmitter receive input:	20

D.	Video output:	21

E.	Audio/surround formats:	21

2.6	MULTIMODE FIBER INPUT BLADE	21

A.	Description	21

B.	Basis of design product: Crestron DMB-I-S DigitalMedia input blade.	21

C.	Transmitter receive input:	21

D.	Video output:	22

E.	Audio/surround formats:	22

2.7	SINGLE-MODE FIBER INPUT BLADE	22

A.	Description	22

B.	Basis of design product: Crestron DMB-I-S2 DigitalMedia input blade.	22

C.	Transmitter receive input:	22

D.	Video output:	23

E.	Audio/surround formats:	23

2.8	4K HDMI OUTPUT BLADE	23

A.	Description	23

B.	Basis of design product: Crestron DMB-4K-O-HD	23

C.	Modular Design:	23

D.	Output Card Source:	24

E.	Analog Audio De-Embedding:	24

F.	Outputs:	24

2.9	4K HDMI AND DANTE OUTPUT BLADE	24

A.	Description	24

B.	Basis of design product: Crestron DMB-4K-O-HD-DNT	24

C.	Modular Design:	24

D.	Output Card Source:	25

E.	Analog Audio De-Embedding:	25

F.	Dante Audio Output and Send/Return	25

G.	Dante Communication	25

H.	Outputs:	25

2.10	SINGLE UTP/STP CABLE 4K TRANSMISSION OUTPUT BLADE	25

A.	Description	25

B.	Basis of design product: Crestron DMB-4K-O-C	26

C.	Modular Design:	26

D.	Output Card Source:	26

E.	Outputs:	26

2.11	SINGLE MULTIMODE FIBER CABLE TRANSMISSION OUTPUT CARD	26

A.	Description	26

B.	Basis of design product: Crestron DMB-O-S	27

C.	Modular Design:	27

D.	Output Card Source:	27

E.	Outputs:	27

2.12	SINGLE-MODE FIBER CABLE TRANSMISSION OUTPUT CARD	27

A.	Description	27

B.	Basis of design product: Crestron DMB-O-S2	28

C.	Modular Design:	28

D.	Output Card Source:	28

E.	Outputs:	28

2.13	SYSTEM MAIN CHASSIS TYPE 2	28

A.	Manufacturers:	28

B.	Description	28

C.	Basis-of-Design Products: Crestron DM-MD8X8, DM-MD16X16, and DM-MD32X32	29

D.	System Capacity	29

E.	Input and Output Modules	29

F.	Multi-Format Audio Backplain	29

G.	Audio Breakaway	29

2.14	HDMI 4K INPUT CARD	29

A.	Description	29

B.	Basis of design product: Crestron DMC-4K-HD-HDCP2 DigitalMedia input card.	30

C.	Modular Design:  Input card shall be a modular unit with the following characteristics:	30

D.	HDMI input:	30

E.	Video output:	31

F.	USB:	31

G.	Audio/surround formats:	31

H.	Analog audio output.	31

2.15	DVI INPUT CARD	31

A.	Description	31

B.	Basis of design product: Crestron DMC-DVI DigitalMedia input card.	31

C.	Modular Design:  Input card shall be a modular unit with the following characteristics:	32

D.	Video input:	32

E.	Audio input:	33

F.	Video output:	33

G.	USB:	33

2.16	VGA INPUT CARD	33

A.	Description	33

B.	Basis of design product: Crestron DMC-VGA DigitalMedia input card.	34

C.	Modular Design:  Input card shall be a modular unit with the following characteristics:	34

D.	Video input:	34

E.	Audio input:	34

F.	Video output:	35

G.	USB:	35

2.17	HD-SDI INPUT CARD	35

A.	Description	35

B.	Basis of design product: Crestron HD-SDI DigitalMedia input card.	35

C.	Modular Design:  Input card shall be a modular unit with the following characteristics:	35

D.	HD-SDI input:	35

E.	HD-SDI Pass-through Output:	36

F.	Video output:	36

G.	Audio formats:	36

H.	Analog audio output.	36

2.18	BNC VIDEO INPUT CARD	36

A.	Description	36

B.	Basis of design product: Crestron DMC-VID-BNC DigitalMedia input card.	37

C.	Modular Design:  Input card shall be a modular unit with the following characteristics:	37

D.	Video input:	37

E.	Audio input:	37

F.	Video output:	37

2.19	VIDEO INPUT CARD	38

A.	Description	38

B.	Basis of design product: Crestron DMC-VID4 DigitalMedia input card.	38

C.	Modular Design:  Input card shall be a modular unit with the following characteristics:	38

D.	Functions	38

E.	Video input:	38

F.	Video output:	39

2.20	VIDEO WITH S/PDIF VIDEO INPUT CARD	39

A.	Description	39

B.	Basis of design product: Crestron DMC-VID-RCA-D DigitalMedia input card.	39

C.	Modular Design:  Input card shall be a modular unit with the following characteristics:	39

D.	Video input:	39

E.	Audio input:	40

F.	Video output:	40

2.21	VIDEO WITH STEREO INPUT CARD	40

A.	Description	40

B.	Basis of design product: Crestron DMC-VID-RCA-A DigitalMedia input card.	40

C.	Modular Design:  Input card shall be a modular unit with the following characteristics:	40

D.	Video input:	40

E.	Audio input:	41

F.	Video output:	41

2.22	SINGLE CABLE UTP/STP INPUT CARD	41

A.	Description	41

B.	Basis of design product: Crestron DMC-4K-C-HDCP2 DigitalMedia input card.	41

C.	Modular Design:  Input card shall be a modular unit with the following characteristics:	42

D.	Transmitter receive input:	42

E.	Transmitter remote power supply capability	42

F.	Video output:	42

G.	Audio/surround formats:	43

H.	Analog audio output.	43

2.23	MULTIMODE FIBER INPUT CARD	43

A.	Description	43

B.	Basis of design product: Crestron DMC-S DigitalMedia input card.	43

C.	Transmitter receive input:	43

D.	Video output:	44

E.	Audio/surround formats:	44

F.	Analog audio output.	44

2.24	SINGLE-MODE FIBER INPUT CARD	45

A.	Description	45

B.	Basis of design product: Crestron DMC-S2 DigitalMedia input card.	45

C.	Transmitter receive input:	45

D.	Video output:	45

E.	Audio/surround formats:	46

F.	Analog audio output.	46

2.25	TRANSMISSION SYSTEM CARD INTERFACE	46

A.	Description	46

B.	Basis of design product: Crestron DMCI DigitalMedia input card interface.	46

2.26	SINGLE UTP/STP CABLE 4K TRANSMISSION OUTPUT CARD	46

A.	Description	46

B.	Basis of design product: Crestron DMC-4K-CO-HD-HDCP2	47

C.	Modular Design:	47

D.	Output Card Source:	47

E.	Outputs:	47

2.27	SINGLE MULTIMODE FIBER CABLE TRANSMISSION OUTPUT CARD	48

A.	Description	48

B.	Basis of design product: Crestron DMC-SO-HD	48

C.	Modular Design:	48

D.	Output Card Source:	48

E.	Outputs:	48

2.28	SINGLEMODE FIBER CABLE TRANSMISSION OUTPUT CARD	48

A.	Description	48

B.	Basis of design product: Crestron DMC-S2O	49

C.	Modular Design:	49

D.	Output Card Source:	49

E.	Outputs:	49

2.29	HDMI WITH ANALOG AUDIO OUTPUT CARD	49

A.	Description	49

B.	Basis of design product: Crestron DMC-HDO	50

C.	Modular Design:	50

D.	Output Card Source:	50

E.	Outputs:	50

2.30	4K HDMI WITH ANALOG AUDIO OUTPUT CARD	50

A.	Description	50

B.	Basis of design product: Crestron DMC-4K-HDO	51

C.	Modular Design:	51

D.	Output Card Source:	51

E.	Outputs:	51

2.31	H.264 STREAMING OUTPUT CARD	52

A.	Description	52

B.	Basis of design product: Crestron DMC-STRO	52

C.	Modular Design:	52

D.	Output Card Source:	52

E.	Output:	52

2.32	4K TRANSMITTER TYPE 1	53

A.	Description	53

B.	Basis of design product: Crestron DM-TX-4K-302-C DigitalMedia transmitter.	53

C.	Performance	53

2.33	4K TRANSMITTER TYPE 2	55

A.	Description	55

B.	Basis of design product: Crestron DM-TX-4K-202-C DigitalMedia transmitter.	55

C.	Performance	55

2.34	4K TRANSMITTER TYPE 3	56

A.	Description	56

B.	Basis of design product: Crestron DM-TX-4K-100-C-1G DigitalMedia transmitter	57

C.	Performance	57

2.35	TRANSMITTER TYPE C1	58

A.	Description	58

B.	Basis of design product: Crestron DM-TX-201C DigitalMedia transmitter.	58

C.	Performance	58

2.36	TRANSMITTER TYPE C2	59

A.	Description	59

B.	Basis of design product: Crestron DM-TX-200-C-2G DigitalMedia transmitter.	60

C.	Performance	60

2.37	TRANSMITTER TYPE C3	61

A.	Description	61

B.	Basis of design product: Crestron DM-TX-401-C DigitalMedia transmitter.	61

C.	Performance	61

2.38	“S” TRANSMITTERS	63

A.	The signal transmitters shall extend HDMI video and audio, DVI-I, RGBHV, YPbPr, Y/C, Composite, Analog 2-channel audio, and USB HID (Human Interface Device) data to compatible transmission receiver modules or ports.	63

B.	Switching:	63

2.39	TRANSMITTER TYPE S1	63

A.	Description	63

B.	Basis of design product: Crestron DM-TX-201S DigitalMedia transmitter.	63

C.	Performance	63

2.40	TRANSMITTER TYPE S3	64

A.	Description	64

B.	Basis of design product: Crestron DM-TX-401-S DigitalMedia transmitter.	65

C.	Performance	65

2.41	“S2” TRANSMITTERS	66

A.	The signal transmitters shall extend HDMI video and audio, DVI-I, RGBHV, YPbPr, Y/C, Composite, Analog 2-channel audio, and USB HID (Human Interface Device) data to compatible transmission receiver modules or ports.	66

B.	Switching:	66

2.42	TRANSMITTER TYPE S2-1	67

A.	Description	67

B.	Basis of design product: Crestron DM-TX-201-S2 DigitalMedia transmitter.	67

C.	Performance	67

2.43	TRANSMITTER TYPE S2-3	68

A.	Description	68

B.	Basis of design product: Crestron DM-TX-401-S2 DigitalMedia transmitter.	69

C.	Performance	69

2.44	“C” RECEIVERS	70

A.	Receivers shall accept a multi-signal transmission over a single UTP or STP cable.	70

B.	Multi-Signal transmission shall be converted to standard HDMI video and control signals.	70

C.	Receiver shall support device control and communication when integrated with compatible control processors.	70

2.45	RECEIVER TYPE C1	70

A.	Description	70

B.	Basis of design product: Crestron DM-RMC-200-C	71

C.	Performance	71

2.46	RECEIVER TYPE C2	72

A.	Description	72

B.	Basis of design product: Crestron DM-RMC-SCALER-C	72

C.	Performance	73

2.47	RECEIVER TYPE 4K	74

A.	Description	74

B.	Basis of design product: Crestron DM-RMC-4K-100-C	74

C.	Performance	74

2.48	RECEIVER TYPE 4K WALL	75

A.	Description	75

B.	Basis of design product: Crestron DM-RMC-4K-100-C-1G	75

C.	Performance	75

2.49	RECEIVER TYPE 4K SCALER	77

A.	Description	77

B.	Basis of design product: Crestron DM-RMC-4K-SCALER-C	77

C.	Performance	77

2.50	RECEIVER TYPE 4K SCALER WITH DSP	78

A.	Description	78

B.	Basis of design product: Crestron DM-RMC-4K-SCALER-C-DSP	78

C.	Performance	78

2.51	“S” RECEIVERS	79

A.	Receivers shall accept a multi-signal transmission over a single multimode fiber cable.	79

B.	Multi-Signal transmission shall be converted to standard HDMI video and control signals.	79

C.	Receiver shall support device control and communication when integrated with compatible  control processors.	80

2.52	RECEIVER TYPE S1	80

A.	Description	80

B.	Basis of design product: Crestron DM-RMC-200-S	80

C.	Performance	80

2.53	RECEIVER TYPE S2	82

A.	Description	82

B.	Basis of design product: Crestron DM-RMC-SCALER-S	82

C.	Performance	82

2.54	RECEIVER TYPE S3	83

A.	Description	83

B.	Basis of design product: Crestron DM-RMC-100-S	83

C.	Performance	83

2.55	RECEIVER TYPE S4	84

A.	Description	84

B.	Basis of design product: Crestron DM-RMC-150-S	84

C.	Performance	84

2.56	“S2” RECEIVERS	86

A.	Receivers shall accept a multi-signal transmission over a singlemode fiber cable.	86

B.	Multi-Signal transmission shall be converted to standard HDMI video and control signals.	86

C.	Receiver shall support device control and communication when integrated with compatible  control processors	86

2.57	RECEIVER TYPE S2-1	86

A.	Description	86

B.	Basis of design product: Crestron DM-RMC-200-S2	86

C.	Performance	86

2.58	RECEIVER TYPE S2-2	88

A.	Description	88

B.	Basis of design product: Crestron DM-RMC-SCALER-S2	88

C.	Performance	88

PART 3	NOT USED	89

END OF SECTION 27 41 16	89

# SECTION 27 41 16

INTEGRATED AUDIO-VIDEO SYSTEMS AND EQUIPMENT

Specifier: The Specifier/Design Professional is responsible for the accuracy of all project specifications, including system application and coordination with related sections.  This guide specification is provided as a convenience and requires editing to match actual project requirements.  CRESTRON ELECTRONICS, INC. SHALL NOT BE LIABLE FOR ANY DAMAGES ARISING OUT OF THE USE OF ANY OF ITS GUIDE SPECIFICATIONS.  For Crestron design assistance and design review please contact Sales Support Services Department at   800.237.2041 or techsales@crestron.com.

GENERAL

SUMMARY

Section Includes

High Definition configurable Audio-Video distribution systems, components and accessories.

PRODUCTS

DIGITAL SIGNAL MATRIX SWITCHER AND TRANSMISSION SYSTEM

Manufacturers:

Basis-of-Design Manufacturer: Subject to compliance with requirements, provide products of Crestron Electronics, Inc., Rockleigh, NJ 07647, Phone (800)237-2041, Fax: (201)767-1903, www.crestron.com, with the following components and characteristics.

Specifier: The following paragraph describes a DM system, i.e. DM-MD series configurable switchers), not specific to matrix switcher frame size.  Edit to suit specific project “system” requirements.

System Description

The Digital AV Matrix system utilizes card-cage architecture.  Main chassis, input and output cards shall be selected based on overall system requirements. [see system drawings].  Matrix System shall support cascading of matrix units, and long distance transmitter and receiver devices by same manufacturer.

Chassis shall support switching for 4K Ultra HD built in.

Basis-of-Design Product Manufacturer: Crestron DigitalMedia Matrix Switchers

System Capacity

Configurable Chassis Options—Main chassis shall be available in the following input to output capacities:

8X8

Maximum input capacity: 8 discrete inputs

Minimum input capacity: 1 discrete inputs

Maximum output capacity: 8 discrete outputs

Minimum output capacity: 2 discrete outputs

16X16

Maximum input capacity: 16 discrete inputs

Minimum input capacity: 1 discrete inputs

Maximum output capacity: 16 discrete outputs

Minimum output capacity: 2 discrete outputs

32X32

Maximum input capacity: 32 discrete inputs

Minimum input capacity: 1 discrete inputs

Maximum output capacity: 32 discrete outputs

Minimum output capacity: 2 discrete outputs

64X64

Maximum input capacity: 64 discrete inputs

Minimum input capacity: 8 discrete inputs

Maximum output capacity: 64 discrete outputs

Minimum output capacity: 8 discrete outputs

128X128

Maximum input capacity: 128 discrete inputs

Minimum input capacity: 8 discrete inputs

Maximum output capacity: 128 discrete outputs

Minimum output capacity: 8 discrete outputs

System Operation

Switching and Routing

Switch Type:

Audio-Video Matrix switching—Any source input switchable to one or more outputs

USB HID Routing—Any USB type-A port routable to one or more type-B ports

Media Signal Types—The Matrix Switcher shall provide matrix switching for the following video formats and standards through custom selected interchangeable input modules.

Analog Video:

Standard encoding systems:

NTSC.

PAL.

Analog Signal Formats:

Composite video.

S-Video.

RGBHV video.

Component video.

Digital Video:

Digital Standards and formats:

HDCP v. 1.2.

Deep Color.

3D format.

Digital Signal Formats:

HDMI.

Dual-Mode DisplayPort.

DVI.

SDI.

HDBaseT.

3-Cable Transmission Protocol.

Single Cable Transmission Protocol.

Dual Fiber Transmission Protocol.

Single Fiber Transmission Protocol.

Single-Mode Long Distance Fiber Transmission Protocol.

Resolutions—The Matrix Switcher shall have native support for the following resolutions:

640x480@60Hz.

20x480@60Hz (480p).

720x576@50Hz (576p).

800x600@60Hz.

848x480@60Hz.

852x480@60Hz.

854x480@60Hz.

1024x768@60Hz.

1024x852@60Hz.

1024x1024@60Hz.

1280x720@50Hz (720p50).

1280x720@60Hz (720p60).

1280x768@60Hz.

1280x800@60Hz.

1280x960@60Hz.

1280x1024@60Hz.

1360x768@60Hz.

1365x1024@60Hz.

1366x768@60Hz.

1400x1050@60Hz.

1440x900@60Hz.

1600x900@60Hz.

1600x1200@60Hz.

1680x1050@60Hz.

1920x1080@24Hz (1080p24).

1920x1080@25Hz (1080p25).

1920x1080@50Hz (1080p50).

1920x1080@60Hz (1080p60).

1920x1200@60Hz.

2048x1080@24Hz.

2048x1152@60Hz.

720x480@30Hz (480i).

720x576@25Hz (576i).

1920x1080@25Hz (1080i25).

1920x1080@30Hz (1080i30).

Other resolutions allowed by HDMI up to 165MHz pixel clock

SMPTE 425M (3G-SDI) 4:2:2 Colorspace: 1920x1080@50Hz (1080p50), 1920x1080@60Hz (1080p60).

SMPTE 425M (3G-SDI) 4:4:4 Colorspace: 1280x720@50Hz (720p50), 1280x720@60Hz (720p60), 1920x1080@24Hz (1080p24), 1920x1080@25Hz (1080p25), 1920x1080@30Hz (1080p30), 1920x1080@50Hz (1080i50 or 1080sF25), 1920x1080@60Hz (1080i60 or 1080sF30).

SMPTE 260M (HD-SDI): 1920x1035@60Hz (1035i60).

SMPTE 295M (HD-SDI): 1920x1080@50Hz (1080i50).

SMPTE 274M (HD-SDI): 1920x1080@24Hz (1080p24), 1920x1080@24Hz (1080sF24), 1920x1080@25Hz (1080p25), 1920x1080@30Hz (1080p30), 1920x1080@50Hz (1080i50 or 1080sF25), 1920x1080@60Hz (1080i60 or 1080sF30).

SMPTE 296M (HD-SDI): 1280x720@50Hz (720p50), 1280x720@60Hz (720p60).

SMPTE 259M-C (SD-SDI): 720x480@59.94 (NTSC), 720x576@50i (PAL).

Audio Matrix Switching.

The Matrix Switcher shall provide matrix switching for the following audio formats and standards through custom selected interchangeable input modules.

Analog Stereo.

Analog 2-channel.

Dolby Digital Plus.

Dolby TrueHD.

DTS-HD High Res.

DTS HD Master Audio.

8 channel PCM.

Dolby Digital.

Dolby Digital EX.

DTS.

DTS-EX.

DTS 96/24.

2 channel PCM.

USB Matrix Switching.

The Matrix Switcher shall provide USB matrix switching for the following devices through custom selected interchangeable input modules and transmitter and receiver units.

USB HID compatible keyboard.

USB HID compatible mouse.

The Matrix Switcher shall route connected USB type A host devices to USB type-B controller devices.

Long Distance Transmission

System shall achieve long distance transmission of all switched and managed signals and data through the selected input/output cards over the following cable types:

UTP and STP cable

Multi-mode fiber

Single-mode fiber

Transmission shall require one cable per channel.

The single cable transmission format shall be capable simultaneously passing the following:

Video

Audio

USB HID

Ethernet

Special purpose device control signals:

Infrared (IR) control signal data

Bidirectional serial control data

Digital relay control data

Network integration: 10/100 Ethernet integrated into long distance transmission protocol.

Integrated Operating System Software Architecture

Stand-Alone System Architecture—System shall be capable of performing all matrix switching and content management functions without external processing or control equipment.

System shall natively control and manage the following using a single software platform:

System Configuration

Setup of overall input and output types and quantities.

System Setup

Individual sources and sink setup.

System Operation

Routing

Selection and execution of source to sink or source to multiple sinks routes.

Management

EDID

CEC

System Status Monitor

Resolution and format information for inputs and outputs

Input resolution and format information

Output resolution and format information

System diagnostics

SYSTEM MAIN CHASSIS TYPE 1

Manufacturers:

Basis-of-Design Manufacturer: Subject to compliance with requirements, provide products of Crestron Electronics, Inc., Rockleigh, NJ 07647, Phone (800)237-2041, Fax: (201)767-1903, www.crestron.com, with the following components and characteristics.

Description

Card-cage matrix switchers with configurable input and output card modules.  Through the use of interchangeable input and output modules the matrix switcher shall support input and output formats up to 4K  HDMI.  Matrix Card-cage shall support local device input and output card types and special purpose cards for use with long distance transmitter and receiver devices by same manufacturer and HDBaseT compatible devices.  Switcher shall include a built-in Ethernet switch.  Switcher shall be available in 64x64 and 128x128 chassis sizes.  Switcher shall be capable of stand-alone operation or integrated operation using a control processor from the same manufacturer.  Input and output cards shall be field-interchangeable.  Any input slot shall be routable to any output slot.  Setup and diagnostics tools shall be built-in and accessible through the front panel interface.  Additional configuration and management tools shall be available through software applications provided by same manufacturer.

Basis-of-Design Products:

Crestron modular matrix switchers: DM-MD64X64, and DM-MD128X128

System Capacity

Main chassis shall be available in the following input and output capacities:

64 Input x 64 Output

128 Input x 128 Output

Input and Output Modules

The Switcher shall support the following input and output signal card types:

Input blade:

8 input connections.

Output blade:

8 output connections.

Front Panel Control Interface

Chassis shall include a built-in control touch screen.

Video Preview

Built-in touch screen interface shall be capable of displaying a live video preview image of any connected source.

Web Browser Interface

Chassis shall include a web server to facilitate browser control of the matrix system.

Browser control interface shall provide the same control and management functionality as the built-in chassis front panel touch screen user interface, with exception of video preview.

Matrix Power

Standard factory configuration shall include hot swappable power supplies.

One redundant power supply shall be included and installed in the switcher chassis.

All power Supply Modules shall be hot swappable.

Removal of chassis from rack shall not be required to replace a power supply module.

Replacing a power supply module shall not require powering down the entire chassis.

Tools shall not be required to replace power supply modules.

The switcher chassis shall be capable of housing one redundant power supply above and beyond power requirements of a fully loaded chassis.

Matrix Cooling

Standard factory configuration shall include a hot swappable fan tray.

Removal of chassis from rack shall not be required to replace a fan tray module.

Replacing the fan tray module shall not require powering down the entire chassis.

Tools shall not be required to replace fan tray modules.

The switcher chassis shall be capable of housing one redundant power supply above and beyond power requirements of a fully loaded chassis.

SPECIFIER:

The following blades will be available soon, please see www.crestron.com for updated information.

Input Blades:

DM 8G (UTP/STP) input blade with 8 inputs and support for 4K.

Model: DMB-4K-I-C

Output Blades:

HDMI output blade with 8 outputs, built-in scalers and support for 4K.

Model: DMB-4K-O-HD-SCALER

4K HDMI INPUT BLADE

Description

The HDMI input blade shall accept up to eight 4K HDMI signals.

4K Operation:

When installed in the matrix chassis, the input signals shall be switchable to installed 4K output blades for 4K signal transmission.

Basis of design product: Crestron DMB-4K-I-HD DigitalMedia Input Blade.

Modular Design:  Input Blade shall be a modular unit with the following characteristics:

Input Blade module shall be one of many available input modules available from same manufacturer.

Modular component of compatible matrix switcher.

Occupy 1 input blade slot in compatible matrix switcher.

Include 1 stereo analog audio input point per channel.

Field installable plug-in card.

HDMI inputs:

Connector:  eight (8) 19-pin type A female HDMI

Format:

HDMI with Deep Color and 3D

HDCP content protection

Input video resolutions:

Progressive video resolutions:

640x480@60Hz, 720x480@60Hz (480p), 720x576@50Hz (576p), 800x600@60Hz, 848x480@60Hz, 852x480@60Hz, 854x480@60Hz, 1024x768@60Hz, 1024x852@60Hz, 1024x1024@60Hz, 1280x720@50Hz (720p50), 1280x720@60Hz (720p60), 1280x768@60Hz, 1280x800@60Hz, 1280x960@60Hz, 1280x1024@60Hz, 1360x768@60Hz, 1365x1024@60Hz, 1366x768@60Hz, 1400x1050@60Hz, 1440x900@60Hz, 1600x900@60Hz, 1600x1200@60Hz, 1680x1050@60Hz, 1920x1080@24Hz (1080p24), 1920x1080@25Hz (1080p25), 1920x1080@50Hz (1080p50), 1920x1080@60Hz (1080p60), 1920x1200@60Hz, 2048x1080@24Hz, 2048x1152@60Hz, 2560x1440@60Hz, 2560x1600@60Hz, 3840x2160@24Hz, 3840x2160@30Hz, 4096x2160@24Hz, plus any other resolution allowed by HDMI up to 300MHz pixel clock.

Interlaced video resolutions:

720x480@30Hz (480i), 720x576@25Hz (576i), 1920x1080@25Hz (1080i25), 1920x1080@30Hz (1080i30), plus any other resolution allowed by HDMI up to 300MHz pixel clock.

Analog Audio Insert:

Connector:  68-pin VHDCI female

Specifier: Audio Breakout Box model: AUD-BOB-1602 is required for analog audio insertion, see http://www.crestron.com/products/model/AUD-BOB-1602  for additional information.

Video outputs:

Matrix backplane:

HDMI signal includes:

Source video at native resolution.

Embedded source audio signal.

Audio/surround formats:

Dolby Digital®, Dolby Digital EX, Dolby Digital Plus, Dolby® TrueHD, DTS®, DTS-ES, DTS 96/24, DTS-HD High Res, DTS-HD Master Audio™, Up to 8ch PCM.

4K HDMI AND DANTE INPUT BLADE

Description

The HDMI input blade shall accept up to eight 4K HDMI signals.

4K Operation:

When installed in the matrix chassis, the input signals shall be switchable to installed 4K output blades for 4K signal transmission.

The HDMI input blade shall accept up to eight stereo audio Dante signals.

Basis of design product: Crestron DMB-4K-I-HD-DNT DigitalMedia Input Blade.

Modular Design:  Input Blade shall be a modular unit with the following characteristics:

Input Blade module shall be one of many available input modules available from same manufacturer.

Modular component of compatible matrix switcher.

Occupy 1 input blade slot in compatible matrix switcher.

Include 1 stereo analog audio input point per channel.

Field installable plug-in card.

HDMI inputs:

Connector:  eight (8) 19-pin type A female HDMI

Format:

HDMI with Deep Color and 3D

HDCP content protection

Input video resolutions:

Progressive video resolutions:

640x480@60Hz, 720x480@60Hz (480p), 720x576@50Hz (576p), 800x600@60Hz, 848x480@60Hz, 852x480@60Hz, 854x480@60Hz, 1024x768@60Hz, 1024x852@60Hz, 1024x1024@60Hz, 1280x720@50Hz (720p50), 1280x720@60Hz (720p60), 1280x768@60Hz, 1280x800@60Hz, 1280x960@60Hz, 1280x1024@60Hz, 1360x768@60Hz, 1365x1024@60Hz, 1366x768@60Hz, 1400x1050@60Hz, 1440x900@60Hz, 1600x900@60Hz, 1600x1200@60Hz, 1680x1050@60Hz, 1920x1080@24Hz (1080p24), 1920x1080@25Hz (1080p25), 1920x1080@50Hz (1080p50), 1920x1080@60Hz (1080p60), 1920x1200@60Hz, 2048x1080@24Hz, 2048x1152@60Hz, 2560x1440@60Hz, 2560x1600@60Hz, 3840x2160@24Hz, 3840x2160@30Hz, 4096x2160@24Hz, plus any other resolution allowed by HDMI up to 300MHz pixel clock.

Interlaced video resolutions:

720x480@30Hz (480i), 720x576@25Hz (576i), 1920x1080@25Hz (1080i25), 1920x1080@30Hz (1080i30), plus any other resolution allowed by HDMI up to 300MHz pixel clock.

Analog Audio Insert:

Connector:  68-pin VHDCI female

Specifier: Audio Breakout Box model: AUD-BOB-1602 is required for analog audio insertion, see http://www.crestron.com/products/model/AUD-BOB-1602 for additional information.

Dante Audio Input and Send

Stereo 2-channel send/output and return/input per channel at up to 24-bit 48 kHz

Dante Communication

The Dante network interface shall utilize the host switcher’s LAN port.

Switcher shall support up to 120 stereo audio channels.

Video outputs:

Matrix backplane:

HDMI signal includes:

Source video at native resolution.

Embedded source audio signal.

Audio/surround formats:

Dolby Digital®, Dolby Digital EX, Dolby Digital Plus, Dolby® TrueHD, DTS®, DTS-ES, DTS 96/24, DTS-HD High Res, DTS-HD Master Audio™, Up to 8ch PCM.

SINGLE UTP/STP CABLE 4K TRANSMISSION INPUT BLADE

Description

The Input Blade shall receive transmission of any compatible source signal connected.

The Input Blade shall receive transmission of signals over manufacturer suggested STP cable, CAT5e or CAT6 cable, or better.

Contact manufacturer prior to cable installation and use manufacturers suggested transmission cable, connectors, and installation methods for all segments of signal path.

The Input Blade shall provide eight (8) discrete inputs.

The Input Blade shall interface with HDBaseT supported single cable transmitters and multi-function transmission transmitters by same manufacturer.

4K Operation:

The Input Blade shall receive transmission of 4K HDMI signals from compatible sources.

The input blade shall receive transmissions from compatible transmitter, supporting distance of up to 330 feet (100 m).

The input blade shall accept transmitted signal and convert to HDMI signals. This signal shall be available as an output on the matrix.

Basis of design product: Crestron DMB-4K-I-C DigitalMedia input blade.

Transmitter receive input:

Connector: 8 female RJ-45 connectors

Format:

HDMI with Deep Color and 3D

HDCP content protection

Input video resolutions:

Progressive video resolutions:

640x480@60Hz, 720x480@60Hz (480p), 720x576@50Hz (576p), 800x600@60Hz, 848x480@60Hz, 852x480@60Hz, 854x480@60Hz, 1024x768@60Hz, 1024x852@60Hz, 1024x1024@60Hz, 1280x720@50Hz (720p50), 1280x720@60Hz (720p60), 1280x768@60Hz, 1280x800@60Hz, 1280x960@60Hz, 1280x1024@60Hz, 1360x768@60Hz, 1365x1024@60Hz, 1366x768@60Hz, 1400x1050@60Hz, 1440x900@60Hz, 1600x900@60Hz, 1600x1200@60Hz, 1680x1050@60Hz, 1920x1080@24Hz (1080p24), 1920x1080@25Hz (1080p25), 1920x1080@50Hz (1080p50), 1920x1080@60Hz (1080p60), 1920x1200@60Hz, 2048x1080@24Hz, 2048x1152@60Hz, plus any other resolution allowed by HDMI up to 165MHz pixel clock.

Interlaced video resolutions:

720x480@30Hz (480i), 720x576@25Hz (576i), 1920x1080@25Hz (1080i25), 1920x1080@30Hz (1080i30), plus any other resolution allowed by HDMI up to 165MHz pixel clock.

Video output:

Matrix backplane:

HDMI signal includes:

Source video at native resolution.

Embedded source audio signal.

Audio/surround formats:

Dolby Digital®, Dolby Digital EX, Dolby Digital Plus, Dolby® TrueHD, DTS®, DTS-ES, DTS 96/24, DTS-HD High Res, DTS-HD Master Audio™, Up to 8ch PCM.

MULTIMODE FIBER INPUT BLADE

Description

The multimode fiber input card shall receive transmissions from compatible transmitter, supporting distance of up to 1,000 feet (300 km).

The single-mode fiber input card shall accept transmitted signal and convert to HDMI signals. This signal shall be available as an output on the matrix.

Basis of design product: Crestron DMB-I-S DigitalMedia input blade.

Transmitter receive input:

Connector: 8 female SC connector

Format:

HDMI with Deep Color and 3D

HDCP content protection

Input video resolutions:

Progressive video resolutions:

640x480@60Hz, 720x480@60Hz (480p), 720x576@50Hz (576p), 800x600@60Hz, 848x480@60Hz, 852x480@60Hz, 854x480@60Hz, 1024x768@60Hz, 1024x852@60Hz, 1024x1024@60Hz, 1280x720@50Hz (720p50), 1280x720@60Hz (720p60), 1280x768@60Hz, 1280x800@60Hz, 1280x960@60Hz, 1280x1024@60Hz, 1360x768@60Hz, 1365x1024@60Hz, 1366x768@60Hz, 1400x1050@60Hz, 1440x900@60Hz, 1600x900@60Hz, 1600x1200@60Hz, 1680x1050@60Hz, 1920x1080@24Hz (1080p24), 1920x1080@25Hz (1080p25), 1920x1080@50Hz (1080p50), 1920x1080@60Hz (1080p60), 1920x1200@60Hz, 2048x1080@24Hz, 2048x1152@60Hz, plus any other resolution allowed by HDMI up to 165MHz pixel clock.

Interlaced video resolutions:

720x480@30Hz (480i), 720x576@25Hz (576i), 1920x1080@25Hz (1080i25), 1920x1080@30Hz (1080i30), plus any other resolution allowed by HDMI up to 165MHz pixel clock.

Video output:

Matrix backplane:

HDMI signal includes:

Source video at native resolution.

Embedded source audio signal.

Audio/surround formats:

Dolby Digital®, Dolby Digital EX, Dolby Digital Plus, Dolby® TrueHD, DTS®, DTS-ES, DTS 96/24, DTS-HD High Res, DTS-HD Master Audio™, Up to 8ch PCM.

SINGLE-MODE FIBER INPUT BLADE

Description

The single-mode fiber input card shall receive transmissions from compatible transmitter, supporting distance of up to 7.5 miles (12 km).

The single-mode fiber input card shall accept transmitted signal and convert to HDMI signals. This signal shall be available as an output on the matrix.

Basis of design product: Crestron DMB-I-S2 DigitalMedia input blade.

Transmitter receive input:

Connector: 8 female LC connector

Format:

HDMI with Deep Color and 3D

HDCP content protection

Input video resolutions:

Progressive video resolutions:

640x480@60Hz, 720x480@60Hz (480p), 720x576@50Hz (576p), 800x600@60Hz, 848x480@60Hz, 852x480@60Hz, 854x480@60Hz, 1024x768@60Hz, 1024x852@60Hz, 1024x1024@60Hz, 1280x720@50Hz (720p50), 1280x720@60Hz (720p60), 1280x768@60Hz, 1280x800@60Hz, 1280x960@60Hz, 1280x1024@60Hz, 1360x768@60Hz, 1365x1024@60Hz, 1366x768@60Hz, 1400x1050@60Hz, 1440x900@60Hz, 1600x900@60Hz, 1600x1200@60Hz, 1680x1050@60Hz, 1920x1080@24Hz (1080p24), 1920x1080@25Hz (1080p25), 1920x1080@50Hz (1080p50), 1920x1080@60Hz (1080p60), 1920x1200@60Hz, 2048x1080@24Hz, 2048x1152@60Hz, plus any other resolution allowed by HDMI up to 165MHz pixel clock.

Interlaced video resolutions:

720x480@30Hz (480i), 720x576@25Hz (576i), 1920x1080@25Hz (1080i25), 1920x1080@30Hz (1080i30), plus any other resolution allowed by HDMI up to 165MHz pixel clock.

Video output:

Matrix backplane:

HDMI signal includes:

Source video at native resolution.

Embedded source audio signal.

Audio/surround formats:

Dolby Digital®, Dolby Digital EX, Dolby Digital Plus, Dolby® TrueHD, DTS®, DTS-ES, DTS 96/24, DTS-HD High Res, DTS-HD Master Audio™, Up to 8ch PCM.

4K HDMI OUTPUT BLADE

Description

The Output Blade shall provide 4K HDMI output switched from any video source signal within the matrix chassis.

The Output Blade shall provide eight (8) discrete outputs.

4K Operation:

The Output Blade shall support 4K HDMI signals from compatible source to compatible sink using 4K switching, transmission and interfacing components from Output Card manufacturer.

The Output Blade shall provide eight (8) discrete outputs.

Basis of design product: Crestron DMB-4K-O-HD

Modular Design:

Output Blade shall be a modular unit with the following characteristics:

Output Blade module shall be one of multiple available output modules available from same manufacturer.

Modular component of compatible blade frame matrix switcher.

Occupy output blade slot space in compatible matrix switcher.

Field installable plug-in blade.

Output Card Source:

All outputs shall be sourced by the matrix backplane.  Matrix backplane shall include all sources connected to matrix input blades.

Analog Audio De-Embedding:

Connector:  68-pin VHDCI female

Specifier: Audio Breakout Box model: AUD-BOB-1602 is required for analog audio output see http://www.crestron.com/products/model/AUD-BOB-1602 for additional information.

Outputs:

Connectors:

Eight (8) 19-pin type A female HDMI

Format:

HDMI with Deep Color and 3D

HDCP content protection

Signal includes:

Video source signal at native resolution.

Embedded audio signal

4K HDMI AND DANTE OUTPUT BLADE

Description

The Output Blade shall provide 4K HDMI output switched from any video source signal within the matrix chassis.

The Output Blade shall provide eight (8) discrete outputs.

4K Operation:

The Output Blade shall support 4K HDMI signals from compatible source to compatible sink using 4K switching, transmission and interfacing components from Output Card manufacturer.

The HDMI output blade shall output up to eight stereo audio Dante signals from any or all of the blade’s outputs.

Basis of design product: Crestron DMB-4K-O-HD-DNT

Modular Design:

Output Blade shall be a modular unit with the following characteristics:

Output Blade module shall be one of multiple available output modules available from same manufacturer.

Modular component of compatible blade frame matrix switcher.

Occupy output blade slot space in compatible matrix switcher.

Field installable plug-in blade.

Output Card Source:

All outputs shall be sourced by the matrix backplane.  Matrix backplane shall include all sources connected to matrix input blades.

Analog Audio De-Embedding:

Connector:  68-pin VHDCI female

Specifier: Audio Breakout Box model: AUD-BOB-1602 is required for analog audio output see http://www.crestron.com/products/model/AUD-BOB-1602 for additional information.

Dante Audio Output and Send/Return

Stereo 2-channel send/output and return/input per channel at up to 24-bit 48 kHz

Dante Communication

The Dante network interface shall utilize the host switcher’s LAN port.

Switcher shall support up to 120 stereo audio channels.

Outputs:

Connectors:

Eight (8) 19-pin type A female HDMI

Format:

HDMI with Deep Color and 3D

HDCP content protection

Signal includes:

Video source signal at native resolution.

Embedded audio signal

SINGLE UTP/STP CABLE 4K TRANSMISSION OUTPUT BLADE

Description

The Output Blade shall provide transmission of any routed source signal within the matrix chassis.

The Output Blade shall transmit over manufacturer suggested STP cable, CAT5e or CAT6 cable, or better.

Contact manufacturer prior to cable installation and use manufacturers suggested transmission cable, connectors, and installation methods for all segments of signal path.

The Output Blade shall provide eight (8) discrete outputs.

The Output Blade shall interface with HDBaseT supported single cable transmission receivers and multi-function transmission receivers by same manufacturer.

4K Operation:

The Output Blade shall support transmission of 4K HDMI signals from compatible source to compatible sink using 4K switching, transmission and interfacing components from Output Card manufacturer.

The Output Blade shall provide eight (8) discrete outputs.

Basis of design product: Crestron DMB-4K-O-C

Modular Design:

Output Blade shall be a modular unit with the following characteristics:

Output Blade module shall be one of multiple available output modules available from same manufacturer.

Modular component of compatible blade frame matrix switcher.

Occupy output blade slot space in compatible matrix switcher.

Field installable plug-in blade.

Output Card Source:

All outputs shall be sourced by the matrix backplane.  Matrix backplane shall include all sources connected to matrix input blades.

Outputs:

Connectors:

Female shielded RJ-45 connector for each transmission output.

Format:

HDMI with Deep Color and 3D

HDCP content protection

Signal includes:

Video source signal at native resolution.

Embedded audio signal

Embedded Ethernet

Remote Power Sourcing:

Output Blade through RJ-45 output connection shall be capable of remotely powering transmission receivers through optional power supply.

Specifier: PoDM power supply model: PW-4830-DU is an optional unit, see http://www.crestron.com/products/model/PW-4830DUS  for additional information.

SINGLE MULTIMODE FIBER CABLE TRANSMISSION OUTPUT CARD

Description

The Output Card shall provide transmission of any routed source signal within the matrix chassis.

The output card shall transmit over one multimode fiber optic strand, contact manufacturer for current list of suggested cable types.

The multimode fiber output blade shall transmit to compatible modules or receivers, supporting distances of up to 1,000 feet (300 km).

The Output Card shall provide eight (8) discrete outputs.

Basis of design product: Crestron DMB-O-S

Modular Design:

Output card shall be a modular unit with the following characteristics:

Output card module shall be one of multiple available output modules available from same manufacturer.

Modular component of compatible card frame matrix switcher.

Occupy output blade slot space in compatible matrix switcher.

Field installable plug-in card.

Output Card Source:

All outputs shall be sourced by the matrix backplane.  Matrix backplane shall include all sources connected to matrix input cards.

Outputs:

Connectors:

Eight (8) SC female optical fiber connector for each transmission output.

Format:

HDMI with Deep Color and 3D

HDCP content protection

Signal includes:

Video source signal at native resolution.

Embedded audio signal

Embedded Ethernet

SINGLE-MODE FIBER CABLE TRANSMISSION OUTPUT CARD

Description

The Output Card shall provide transmission of any routed source signal within the matrix chassis.

The output card shall transmit over one singlemode fiber optic strand, contact manufacturer for current list of suggested cable types.

The singlemode fiber output blade shall transmit to compatible modules or receivers, supporting distances of up to 7.5 miles (12km).

The Output Card shall provide eight (8) discrete outputs.

Basis of design product: Crestron DMB-O-S2

Modular Design:

Output card shall be a modular unit with the following characteristics:

Output card module shall be one of multiple available output modules available from same manufacturer.

Modular component of compatible card frame matrix switcher.

Occupy output blade slot space in compatible matrix switcher.

Field installable plug-in card.

Output Card Source:

All outputs shall be sourced by the matrix backplane.  Matrix backplane shall include all sources connected to matrix input cards.

Outputs:

Connectors:

Eight (8) LC female optical fiber connector for each transmission output.

Format:

HDMI with Deep Color and 3D

HDCP content protection

Signal includes:

Video source signal at native resolution.

Embedded audio signal

Embedded Ethernet

SYSTEM MAIN CHASSIS TYPE 2

Manufacturers:

Basis-of-Design Manufacturer: Subject to compliance with requirements, provide products of Crestron Electronics, Inc., Rockleigh, NJ 07647, Phone (800)237-2041, Fax: (201)767-1903, www.crestron.com, with the following components and characteristics.

Description

Card-cage matrix switchers with configurable input and output card modules.  Through the use of interchangeable input and output modules the matrix switcher shall support various input formats up to 4K  HDMI and H.264, HDMI and HDMI 4K output formats.  Matrix Card-cage shall support local device input and output card types and special purpose cards for use with long distance transmitter and receiver devices by same manufacturer and HDBaseT compatible devices.  Switcher shall include a built-in Ethernet switch.  Switcher shall be available in 8x8, 16x16, and 32x32 chassis sizes.  Switcher shall be capable of stand-alone operation or integrated operation using a control processor from the same manufacturer.  Input and output cards shall be field-interchangeable.  Any input slot shall be routable to any output slot.  Setup and diagnostics tools shall be built-in and accessible through the front panel interface.  Additional configuration and management tools shall be available through software applications provided by same manufacturer.

Basis-of-Design Products: Crestron DM-MD8X8, DM-MD16X16, and DM-MD32X32

Crestron modular matrix switchers: DM-MD8X8, DM-MD16X16, and DM-MD32X32 ;

Specifier:  Redundant power supply versions are also available: DM-MD8X8-RPS, DM-MD16X16-RPS, and DM-MD32X32-RPS

System Capacity

Main chassis shall be available in the following input and output capacities:

8 Input x 8 Output

16 Input x 16 Output

32 Input x 32 Output

Input and Output Modules

The Switcher shall support the following input and output signal card types:

Input card:

Single input connection.

Output card:

One, two, or four output connections, configurable in groups of two.

Multi-Format Audio Backplain

The matrix Switcher shall support the option to maintain two audio formats for a single surround sound source device using a single input module.

Audio format 1: Native multi-channel audio generated by source.

Audio format 2: Stereo down-mix of native multi-channel source audio.

Audio Breakaway

Within a single switcher chassis, source audio inputs shall be routable to any output separately or combined with source video.

Switcher shall be capable of routing stereo audio and surround audio separately when an input slot is equipped with an input card with audio processing functionality.

HDMI 4K INPUT CARD

SPECIFIER:

Surround Sound DSP Mix down is available on an optional version of this card,

Use model: [DMC-4K-HD-HDCP2-DSP]

Description

The HDMI input card shall accept one 4K HDCP2 HDMI signal.

4K Operation:

When installed in the matrix chassis, the input signal shall be switchable to installed 4K output cards for 4K signal transmission.

Specifier: The DMC-4K-HD-HDCP2(DSP) card is compatible with original/existing and current DM-MD8X8, DM-MD16X16, and DM-MD32X32 chassis units.  Existing chassis units may be upgraded to 4K capability by replacing existing i/o cards and DM receivers with 4K capable models.

Basis of design product: Crestron DMC-4K-HD-HDCP2 DigitalMedia input card.

Modular Design:  Input card shall be a modular unit with the following characteristics:

Input card module shall be one of many available input modules available from same manufacturer.

Modular component of compatible card frame matrix switcher.

Occupy 1 input card slot in compatible matrix switcher.

Include 1 HDMI pass through output.

HDMI output includes audio and video from source.

Include 1 stereo analog audio output:

Stereo audio extracted from HDMI source signal.

Specifier: To retain surround source audio for downstream HDMI signal as well as creating stereo mix-down, use model DMC-4K-HD-HDCP2-DSP.

Field installable plug-in card.

HDMI input:

Connector: 19-pin type A female HDMI

Format:

HDMI with Deep Color and 3D

4K HDMI

HDCP2.2 compliant

Input video resolutions:

Progressive video resolutions:

640x480@60Hz, 720x480@60Hz (480p), 720x576@50Hz (576p), 800x600@60Hz, 848x480@60Hz, 852x480@60Hz, 854x480@60Hz, 1024x768@60Hz, 1024x852@60Hz, 1024x1024@60Hz, 1280x720@50Hz (720p50), 1280x720@60Hz (720p60), 1280x768@60Hz, 1280x800@60Hz, 1280x960@60Hz, 1280x1024@60Hz, 1360x768@60Hz, 1365x1024@60Hz, 1366x768@60Hz, 1400x1050@60Hz, 1440x900@60Hz, 1600x900@60Hz, 1600x1200@60Hz, 1680x1050@60Hz, 1920x1080@24Hz (1080p24), 1920x1080@25Hz (1080p25), 1920x1080@50Hz (1080p50), 1920x1080@60Hz (1080p60), 1920x1200@60Hz, 2048x1080@24Hz, 2048x1152@60Hz, 2560x1440@60Hz, 2560x1600@60Hz, 3840x2160@24Hz, 3840x2160@30Hz, 4096x2160@24Hz, plus any other resolution allowed by HDMI up to 300MHz pixel clock.

Interlaced video resolutions:

720x480@30Hz (480i), 720x576@25Hz (576i), 1920x1080@25Hz (1080i25), 1920x1080@30Hz (1080i30), plus any other resolution allowed by HDMI up to 300MHz pixel clock.

Video output:

Local HDMI output:

Connector: 19-pin Type A female HDMI.

HDMI signal includes:

Video source signal at native resolution.

Embedded source audio signal.

Matrix backplane:

HDMI signal includes:

Source video at native resolution.

Embedded source audio signal.

USB:

Local USB port

Connector: USB Type A female connector

Support for:

USB HID devices

Audio/surround formats:

Dolby Digital®, Dolby Digital EX, Dolby Digital Plus, Dolby® TrueHD, DTS®, DTS-ES, DTS 96/24, DTS-HD High Res, DTS-HD Master Audio™, Up to 8ch PCM.

Analog audio output.

2 female RCA connectors.

Stereo source pass through.

Digital to analog conversion:

24 bit 48 kHz

Surround formats shall be mixed to stereo in optional version of input card.

Specifier: To retain surround source audio for downstream HDMI signal as well as creating stereo mix-down, use model DMC-4K-HD-HDCP2-DSP.

DVI INPUT CARD

Description

The Video Input Card shall accept a DVI video signal, injected audio and convert to HDMI signal. This signal shall be available as an output on the matrix.

Basis of design product: Crestron DMC-DVI DigitalMedia input card.

Modular Design:  Input card shall be a modular unit with the following characteristics:

Input card module shall be one of many available input modules available from same manufacturer.

Modular component of compatible card frame matrix switcher.

Occupy 1 input card slot in compatible matrix switcher.

Include 1 HDMI pass through output.

HDMI output includes audio and video from source.

Field installable plug-in card.

Video input:

Connector:  One DVI-I female connector

Auto-sensing multi-format analog video input.

Support for the following video types:

DVI-D

DVI-I

YPbPr (component).

Y/C (S-Video).

RGB

HDCP content protection

Input video resolutions:

Progressive video resolutions:

640x480@60Hz, 720x480@60Hz (480p), 720x576@50Hz (576p), 800x600@60Hz, 848x480@60Hz, 852x480@60Hz, 854x480@60Hz, 1024x768@60Hz, 1024x852@60Hz, 1024x1024@60Hz, 1280x720@50Hz (720p50), 1280x720@60Hz (720p60), 1280x768@60Hz, 1280x800@60Hz, 1280x960@60Hz, 1280x1024@60Hz, 1360x768@60Hz, 1365x1024@60Hz, 1366x768@60Hz, 1400x1050@60Hz, 1440x900@60Hz, 1600x900@60Hz, 1600x1200@60Hz, 1680x1050@60Hz, 1920x1080@24Hz (1080p24), 1920x1080@25Hz (1080p25), 1920x1080@50Hz (1080p50), 1920x1080@60Hz (1080p60), 1920x1200@60Hz, 2048x1080@24Hz, 2048x1152@60Hz, plus any other resolution allowed by HDMI up to 165MHz pixel clock.

Interlaced video resolutions:

720x480@30Hz (480i), 720x576@25Hz (576i), 1920x1080@25Hz (1080i25), 1920x1080@30Hz (1080i30), plus any other resolution allowed by HDMI up to 165MHz pixel clock.

RGB video resolutions:

640x480@60Hz, 720x480@60Hz (480p), 720x576@50Hz (576p), 800x600@60Hz, 848x480@60Hz, 1024x768@60Hz, 1280x720@50Hz (720p50), 1280x720@60Hz (720p60), 1280x768@60Hz, 1280x800@60Hz, 1280x960@60Hz, 1280x1024@60Hz, 1360x768@60Hz, 1366x768@60Hz, 1400x1050@60Hz, 1440x900@60Hz, 1600x1200@60Hz, 1680x1050@60Hz, 1920x1080@24Hz (1080p24), 1920x1080@50Hz (1080p50), 1920x1080@60Hz (1080p60), 1920x1200@60Hz, 2048x1080@24Hz, 2048x1152@60Hz

Component video resolutions:

480i, 576i, 480p, 576p, 720p50, 720p60, 1080i25 (1125 lines), 1080i30, 1080p30, 1080p50 (1125 lines), 1080p60

Composite and Y/C (S-Video) video resolutions:

480i, 576i

Audio input:

Connector:  One 5 position terminal block

Format:

Analog balanced stereo; line level

Analog unbalanced stereo; line level

Video output:

Local HDMI output:

Connector: 19-pin Type A female HDMI.

HDMI signal includes:

Video source signal at native resolution.

Embedded stereo audio signal.

Matrix backplane:

HDMI signal includes:

Source video at native resolution.

Embedded source audio signal.

USB:

Local USB 1.1 port

Connector: USB Type A female connector

Support for:

USB HID devices

VGA INPUT CARD

Description

The Video Input Card shall accept a VGA video signal, injected audio and convert to HDMI signal. This signal shall be available as an output on the matrix.

Basis of design product: Crestron DMC-VGA DigitalMedia input card.

Modular Design:  Input card shall be a modular unit with the following characteristics:

Input card module shall be one of many available input modules available from same manufacturer.

Modular component of compatible card frame matrix switcher.

Occupy 1 input card slot in compatible matrix switcher.

Include 1 HDMI pass through output.

HDMI output includes audio and video from source.

Field installable plug-in card.

Video input:

Connector:  One DB15HD female connector

Auto-sensing multi-format analog video input.

Support for the following video types:

VGA

YPbPr (component).

Y/C (S-Video).

RGB

HDCP content protection

Input video resolutions:

RGB video resolutions:

640x480@60Hz, 720x480@60Hz (480p), 720x576@50Hz (576p), 800x600@60Hz, 848x480@60Hz, 1024x768@60Hz, 1280x720@50Hz (720p50), 1280x720@60Hz (720p60), 1280x768@60Hz, 1280x800@60Hz, 1280x960@60Hz, 1280x1024@60Hz, 1360x768@60Hz, 1366x768@60Hz, 1400x1050@60Hz, 1440x900@60Hz, 1600x1200@60Hz, 1680x1050@60Hz, 1920x1080@24Hz (1080p24), 1920x1080@50Hz (1080p50), 1920x1080@60Hz (1080p60), 1920x1200@60Hz, 2048x1080@24Hz, 2048x1152@60Hz

Component video resolutions:

480i, 576i, 480p, 576p, 720p50, 720p60, 1080i25 (1125 lines), 1080i30, 1080p30, 1080p50 (1125 lines), 1080p60

Composite and Y/C (S-Video) video resolutions:

480i, 576i

Audio input:

Connector:  One 5 position terminal block

Format:

Analog balanced stereo; line level

Analog unbalanced stereo; line level

Video output:

Local HDMI output:

Connector: 19-pin Type A female HDMI.

HDMI signal includes:

Video source signal at native resolution.

Embedded stereo audio signal.

Matrix backplane:

HDMI signal includes:

Source video at native resolution.

Embedded source audio signal.

USB:

Local USB 1.1 port

Connector: USB Type A female connector

Support for:

USB HID devices

HD-SDI INPUT CARD

Description

The HD-SDI input card shall accept an HD-SD signal.  This signal shall be available as an output on the matrix.

Basis of design product: Crestron HD-SDI DigitalMedia input card.

Modular Design:  Input card shall be a modular unit with the following characteristics:

Input card module shall be one of many available input modules available from same manufacturer.

Modular component of compatible card frame matrix switcher.

Occupy 1 input card slot in compatible matrix switcher.

Include 1 HDMI pass through output.

HDMI output includes audio and video from source.

Include 1 stereo analog audio output:

Two channels of audio extracted from SDI source signal.

Field installable plug-in card.

HD-SDI input:

Connector: female BNC

Format:

SD-SDI (SMPTE 259M)

HD-SDI (SMPTE 292M)

3G-SDI (SMPTE 424M)

Input video resolutions:

SMPTE 425M (3G-SDI) 4:2:2 Colorspace: 1920x1080@50Hz (1080p50), 1920x1080@60Hz (1080p60);
SMPTE 425M (3G-SDI) 4:4:4 Colorspace: 1280x720@50Hz (720p50), 1280x720@60Hz (720p60), 1920x1080@24Hz (1080p24), 1920x1080@25Hz (1080p25), 1920x1080@30Hz (1080p30), 1920x1080@50Hz (1080i50 or 1080sF25), 1920x1080@60Hz (1080i60 or 1080sF30);
SMPTE 260M (HD-SDI): 1920x1035@60Hz (1035i60);
SMPTE 295M (HD-SDI): 1920x1080@50Hz (1080i50);
SMPTE 274M (HD-SDI): 1920x1080@24Hz (1080p24), 1920x1080@24Hz (1080sF24), 1920x1080@25Hz (1080p25), 1920x1080@30Hz (1080p30), 1920x1080@50Hz (1080i50 or 1080sF25), 1920x1080@60Hz (1080i60 or 1080sF30);
SMPTE 296M (HD-SDI): 1280x720@50Hz (720p50), 1280x720@60Hz (720p60);
SMPTE 259M-C (SD-SDI): 720x480@59.94 (NTSC), 720x576@50i (PAL)

HD-SDI Pass-through Output:

Connector: female BNC

Video output:

Local HDMI output:

Connector: 19-pin Type A female HDMI.

HDMI signal includes:

Video source signal at native resolution.

Embedded 2-channel audio signal.

Matrix backplane:

HDMI signal includes:

Source video at native resolution.

Embedded source audio signal.

Audio formats:

2 channel PCM via SDI, HDMI, & DM; 2-channel analog

Analog audio output.

2 female RCA connectors.

Digital to analog conversion:

24 bit 48 kHz

BNC VIDEO INPUT CARD

Description

The Video Input Card shall accept an analog audio/video signal and convert it to an HDMI signal. This signal shall be available as an output on the matrix.

Basis of design product: Crestron DMC-VID-BNC DigitalMedia input card.

Modular Design:  Input card shall be a modular unit with the following characteristics:

Input card module shall be one of many available input modules available from same manufacturer.

Modular component of compatible card frame matrix switcher.

Occupy 1 input card slot in compatible matrix switcher.

Include 1 HDMI pass through output.

HDMI output includes audio and video from source.

Field installable plug-in card.

Video input:

Connector:  Three BNC female connectors

Auto-sensing multi-format analog video input.

Support for the following video types:

YPbPr (component).

Y/C (S-Video).

Composite

Input video resolutions:

Component video resolutions:

480i, 576i, 480p, 576p, 720p50, 720p60, 1080i25 (1125 lines), 1080i30, 1080p30, 1080p50 (1125 lines), 1080p60

Composite video resolutions:

480i, 576i

Audio input:

Connector:  One 5 position terminal block

Format:

Analog balanced stereo; line level

Analog unbalanced stereo; line level

Video output:

Local HDMI output:

Connector: 19-pin Type A female HDMI.

HDMI signal includes:

Video source signal at native resolution.

Embedded stereo audio signal.

Matrix backplane:

HDMI signal includes:

Source video at native resolution.

Embedded source audio signal.

VIDEO INPUT CARD

Description

The Security Camera Input Card shall accept analog video signal, provide image processing, and output the result to an HDMI signal. This signal shall be available as an output on the matrix.

Basis of design product: Crestron DMC-VID4 DigitalMedia input card.

Modular Design:  Input card shall be a modular unit with the following characteristics:

Input card module shall be one of many available input modules available from same manufacturer.

Modular component of compatible card frame matrix switcher.

Occupy 1 input card slot in compatible matrix switcher.

Include 1 HDMI pass through output.

Field installable plug-in card.

Functions

The input card shall provide an on-board video processor.

Shall support the following display modes:

Quad Screen

Dual Screen

Full Screen

Sequential input switching

Shall support text overlay

Dynamic colored window labeling

Time/date.

Video input:

Connectors:  Four RCA female connectors

Support for Composite video

Input video resolutions:

Composite video resolutions:

480i, 576i

Video output:

Local HDMI output:

Connector: 19-pin Type A female HDMI.

HDMI signal includes:

Video source signal at native resolution.

Matrix backplane:

HDMI signal includes:

Source video at native resolution.

VIDEO WITH S/PDIF VIDEO INPUT CARD

Description

The video w/S/PDIF Input Card shall accept an analog audio/video signal and convert it to an HDMI signal. This signal shall be available as an output on the matrix.

Basis of design product: Crestron DMC-VID-RCA-D DigitalMedia input card.

Modular Design:  Input card shall be a modular unit with the following characteristics:

Input card module shall be one of many available input modules available from same manufacturer.

Modular component of compatible card frame matrix switcher.

Occupy 1 input card slot in compatible matrix switcher.

Include 1 HDMI pass through output.

HDMI output includes audio and video from source.

Field installable plug-in card.

Video input:

Connector:  Three RCA female connectors

Auto-sensing multi-format analog video input.

Support for the following video types:

YPbPr (component).

Y/C (S-Video).

Composite

Input video resolutions:

Component video resolutions:

480i, 576i, 480p, 576p, 720p50, 720p60, 1080i25 (1125 lines), 1080i30, 1080p30, 1080p50 (1125 lines), 1080p60

Composite video resolutions:

480i, 576i

Audio input:

Connector:  One RCA female

Format:

S/PDIF coaxial

Dolby Digital®, Dolby Digital EX, DTS®, DTS-ES, DTS 96/24, 2ch PCM

Video output:

Local HDMI output:

Connector: 19-pin Type A female HDMI.

HDMI signal includes:

Video source signal at native resolution.

Embedded audio signal.

Matrix backplane:

HDMI signal includes:

Source video at native resolution.

Embedded source audio signal.

VIDEO WITH STEREO INPUT CARD

Description

The Video Input Card shall accept an analog audio/video signal and convert it to an HDMI signal. This signal shall be available as an output on the matrix.

Basis of design product: Crestron DMC-VID-RCA-A DigitalMedia input card.

Modular Design:  Input card shall be a modular unit with the following characteristics:

Input card module shall be one of many available input modules available from same manufacturer.

Modular component of compatible card frame matrix switcher.

Occupy 1 input card slot in compatible matrix switcher.

Include 1 HDMI pass through output.

HDMI output includes audio and video from source.

Field installable plug-in card.

Video input:

Connector:  Three RCA female connectors

Auto-sensing multi-format analog video input.

Support for the following video types:

YPbPr (component).

Y/C (S-Video).

Composite

Input video resolutions:

Component video resolutions:

480i, 576i, 480p, 576p, 720p50, 720p60, 1080i25 (1125 lines), 1080i30, 1080p30, 1080p50 (1125 lines), 1080p60

Composite video resolutions:

480i, 576i

Audio input:

Connector:  Two RCA female

Format:

Analog stereo

Video output:

Local HDMI output:

Connector: 19-pin Type A female HDMI.

HDMI signal includes:

Video source signal at native resolution.

Embedded audio signal.

Matrix backplane:

HDMI signal includes:

Source video at native resolution.

Embedded source audio signal.

SINGLE CABLE UTP/STP INPUT CARD

SPECIFIER:

Surround Sound DSP Mix down is available on an optional version of this card,

use model: [DMC-4K-C-DSP-HDCP2]

Description

The single cable UTP or STP input card shall receive transmission from compatible transmitters, supporting distance of up to 330 feet (100 km).

The HDMI input card shall accept one 4K HDCP2 signal from compatible 4K HDCP2 transmitter.

The single cable UTP or STP input card shall accept transmitted signal and convert to HDMI signal and separate audio output. This signal shall be available as an output on the matrix.

Basis of design product: Crestron DMC-4K-C-HDCP2 DigitalMedia input card.

Modular Design:  Input card shall be a modular unit with the following characteristics:

Input card module shall be one of many available input modules available from same manufacturer.

Modular component of compatible card frame matrix switcher.

Occupy 1 input card slot in compatible matrix switcher.

Include 1 HDMI pass through output.

HDMI output includes audio and video from transmission source.

Include 1 stereo analog audio output:

Stereo audio extracted from HDMI source signal.

Specifier: To retain surround source audio for downstream HDMI signal as well as creating stereo mix-down, use model DMC-4K-C-HDCP2-DSP.

Field installable plug-in card.

Transmitter receive input:

Connector: 1 female shielded RJ-45 connector

Convert compatible transmitter signal to HDMI and stereo audio for local output.

Convert compatible transmitter signal to HDMI, surround/stereo audio, and Ethernet link for connection to matrix switcher backplane.

Format:

HDMI with Deep Color and 3D

HDCP content protection

Input video resolutions:

Progressive video resolutions:

640x480@60Hz, 720x480@60Hz (480p), 720x576@50Hz (576p), 800x600@60Hz, 848x480@60Hz, 852x480@60Hz, 854x480@60Hz, 1024x768@60Hz, 1024x852@60Hz, 1024x1024@60Hz, 1280x720@50Hz (720p50), 1280x720@60Hz (720p60), 1280x768@60Hz, 1280x800@60Hz, 1280x960@60Hz, 1280x1024@60Hz, 1360x768@60Hz, 1365x1024@60Hz, 1366x768@60Hz, 1400x1050@60Hz, 1440x900@60Hz, 1600x900@60Hz, 1600x1200@60Hz, 1680x1050@60Hz, 1920x1080@24Hz (1080p24), 1920x1080@25Hz (1080p25), 1920x1080@50Hz (1080p50), 1920x1080@60Hz (1080p60), 1920x1200@60Hz, 2048x1080@24Hz, 2048x1152@60Hz, plus any other resolution allowed by HDMI up to 165MHz pixel clock.

Interlaced video resolutions:

720x480@30Hz (480i), 720x576@25Hz (576i), 1920x1080@25Hz (1080i25), 1920x1080@30Hz (1080i30), plus any other resolution allowed by HDMI up to 165MHz pixel clock.

Transmitter remote power supply capability

Power compatible transmitter through single CAT5e cable.

Video output:

Local HDMI output:

Connector: 19-pin Type A female HDMI.

HDMI signal includes:

Video source signal at native resolution.

Embedded stereo audio signal.

Matrix backplane:

HDMI signal includes:

Source video at native resolution.

Embedded source audio signal.

Audio/surround formats:

Dolby Digital®, Dolby Digital EX, Dolby Digital Plus, Dolby® TrueHD, DTS®, DTS-ES, DTS 96/24, DTS-HD High Res, DTS-HD Master Audio™, Up to 8ch PCM.

Analog audio output.

2 female RCA connectors.

Stereo source pass through.

Digital to analog conversion:

24 bit 48 kHz

Surround formats shall be mixed to stereo in optional version of input card.

MULTIMODE FIBER INPUT CARD

SPECIFIER:

Surround Sound DSP Mix down is available on an optional version of this card,

Use model: [DMC-S-DSP]

Description

The multimode fiber input card shall receive transmissions from compatible transmitter, supporting distance of up to 1,000 feet (300 km).

The single-mode fiber input card shall accept transmitted signal and convert to HDMI signal and separate audio output. This signal shall be available as an output on the matrix.

Basis of design product: Crestron DMC-S DigitalMedia input card.

Transmitter receive input:

Connector: 1 female SC connector

Convert compatible transmitter signal to HDMI and stereo audio for local output.

Convert compatible transmitter signal to HDMI, surround/stereo audio, and Ethernet link for connection to matrix switcher backplane.

Format:

HDMI with Deep Color and 3D

HDCP content protection

Input video resolutions:

Progressive video resolutions:

640x480@60Hz, 720x480@60Hz (480p), 720x576@50Hz (576p), 800x600@60Hz, 848x480@60Hz, 852x480@60Hz, 854x480@60Hz, 1024x768@60Hz, 1024x852@60Hz, 1024x1024@60Hz, 1280x720@50Hz (720p50), 1280x720@60Hz (720p60), 1280x768@60Hz, 1280x800@60Hz, 1280x960@60Hz, 1280x1024@60Hz, 1360x768@60Hz, 1365x1024@60Hz, 1366x768@60Hz, 1400x1050@60Hz, 1440x900@60Hz, 1600x900@60Hz, 1600x1200@60Hz, 1680x1050@60Hz, 1920x1080@24Hz (1080p24), 1920x1080@25Hz (1080p25), 1920x1080@50Hz (1080p50), 1920x1080@60Hz (1080p60), 1920x1200@60Hz, 2048x1080@24Hz, 2048x1152@60Hz, plus any other resolution allowed by HDMI up to 165MHz pixel clock.

Interlaced video resolutions:

720x480@30Hz (480i), 720x576@25Hz (576i), 1920x1080@25Hz (1080i25), 1920x1080@30Hz (1080i30), plus any other resolution allowed by HDMI up to 165MHz pixel clock.

Video output:

Local HDMI output:

Connector: 19-pin Type A female HDMI.

HDMI signal includes:

Video source signal at native resolution.

Embedded stereo audio signal.

Matrix backplane:

HDMI signal includes:

Source video at native resolution.

Embedded source audio signal.

Audio/surround formats:

Dolby Digital®, Dolby Digital EX, Dolby Digital Plus, Dolby® TrueHD, DTS®, DTS-ES, DTS 96/24, DTS-HD High Res, DTS-HD Master Audio™, Up to 8ch PCM.

Analog audio output.

2 female RCA connectors.

Stereo source pass through.

Digital to analog conversion:

24 bit 48 kHz

Surround formats shall be mixed to stereo in optional version of input card.

SINGLE-MODE FIBER INPUT CARD

SPECIFIER:

Surround Sound DSP Mix down is available on an optional version of this card,

Use model: [DMC-S2-DSP]

Description

The single-mode fiber input card shall receive transmissions from compatible transmitter, supporting distance of up to 7.5 miles (12 km).

The single-mode fiber input card shall accept transmitted signal and convert to HDMI signal and separate audio output. This signal shall be available as an output on the matrix.

Basis of design product: Crestron DMC-S2 DigitalMedia input card.

Transmitter receive input:

Connector: 1 female LC connector

Convert compatible transmitter signal to HDMI and stereo audio for local output.

Convert compatible transmitter signal to HDMI, surround/stereo audio, and Ethernet link for connection to matrix switcher backplane.

Format:

HDMI with Deep Color and 3D

HDCP content protection

Input video resolutions:

Progressive video resolutions:

640x480@60Hz, 720x480@60Hz (480p), 720x576@50Hz (576p), 800x600@60Hz, 848x480@60Hz, 852x480@60Hz, 854x480@60Hz, 1024x768@60Hz, 1024x852@60Hz, 1024x1024@60Hz, 1280x720@50Hz (720p50), 1280x720@60Hz (720p60), 1280x768@60Hz, 1280x800@60Hz, 1280x960@60Hz, 1280x1024@60Hz, 1360x768@60Hz, 1365x1024@60Hz, 1366x768@60Hz, 1400x1050@60Hz, 1440x900@60Hz, 1600x900@60Hz, 1600x1200@60Hz, 1680x1050@60Hz, 1920x1080@24Hz (1080p24), 1920x1080@25Hz (1080p25), 1920x1080@50Hz (1080p50), 1920x1080@60Hz (1080p60), 1920x1200@60Hz, 2048x1080@24Hz, 2048x1152@60Hz, plus any other resolution allowed by HDMI up to 165MHz pixel clock.

Interlaced video resolutions:

720x480@30Hz (480i), 720x576@25Hz (576i), 1920x1080@25Hz (1080i25), 1920x1080@30Hz (1080i30), plus any other resolution allowed by HDMI up to 165MHz pixel clock.

Video output:

Local HDMI output:

Connector: 19-pin Type A female HDMI.

HDMI signal includes:

Video source signal at native resolution.

Embedded stereo audio signal.

Matrix backplane:

HDMI signal includes:

Source video at native resolution.

Embedded source audio signal.

Audio/surround formats:

Dolby Digital®, Dolby Digital EX, Dolby Digital Plus, Dolby® TrueHD, DTS®, DTS-ES, DTS 96/24, DTS-HD High Res, DTS-HD Master Audio™, Up to 8ch PCM.

Analog audio output.

2 female RCA connectors.

Stereo source pass through.

Digital to analog conversion:

24 bit 48 kHz

Surround formats shall be mixed to stereo in optional version of input card.

TRANSMISSION SYSTEM CARD INTERFACE

Description

The interface is a card-cage unit capable of interfacing with single input slot input cards.

The interface enables front panel connections of compatible inputs cards.

The interface shall power and provide limited communication	to input card features.

Basis of design product: Crestron DMCI DigitalMedia input card interface.

SINGLE UTP/STP CABLE 4K TRANSMISSION OUTPUT CARD

Description

The Output Card shall provide transmission of any routed source signal, within the matrix chassis.

The output card shall transmit over manufacturer suggested STP cable, CAT5e or CAT6 cable, or better.

Contact manufacturer prior to cable installation and use manufacturers suggested transmission cable, connectors, and installation methods for all segments of signal path.

The Output Card shall provide 2 discrete outputs.

The Output Card shall interface with HDBaseT supported single cable transmission receivers.

The card shall provide 1 HDMI output for every 2 single cable transmission outputs:

HDMI output shall be a parallel signal to the single cable transmission output.

4K Operation:

The Output Card shall support transmission of 4K HDMI signals from compatible source to compatible sink using 4K switching, transmission and interfacing components from Output Card manufacturer.

Specifier: The DMC-4K-CO-HD-HDCP2 card is compatible with original/existing and current DM-MD8X8, DM-MD16X16, and DM-MD32X32 chassis units.  Existing chassis units may be upgraded to 4K capability by replacing existing i/o cards and DM receivers with 4K capable models.

Basis of design product: Crestron DMC-4K-CO-HD-HDCP2

Crestron DMC-4K-CO-HD-HDCP2 DigitalMedia output card.

Modular Design:

Output card shall be a modular unit with the following characteristics:

Output card module shall be one of many available output modules available from same manufacturer.

Modular component of compatible card frame matrix switcher.

Occupy output card slot space in compatible matrix switcher.

Field installable plug-in card.

Output Card Source:

All outputs shall be sourced by the matrix backplane.  Matrix backplane shall include all sources connected to matrix input cards.

Outputs:

Connectors:

Female shielded RJ-45 connector for each transmission output.

Female 19-pin Type A HDMI for parallel output channels.

Format:

HDMI with Deep Color and 3D

4K HDMI

HDCP2.2 content protection

Signal includes:

Video source signal at native resolution.

Embedded audio signal

Embedded Ethernet

Remote Power Sourcing:

Output Card, through RJ-45 output connection shall be capable of remotely powering transmission receivers through optional power supply.

Specifier: Use PoDM power supply model(s): DM-PSU-ULTRA-MIDSPAN, CEN-SWPOE-16, and DM-PSU-8/16 for remote powering compatible transmitters and receivers.

SINGLE MULTIMODE FIBER CABLE TRANSMISSION OUTPUT CARD

Description

The Output Card shall provide transmission of any routed source signal within the matrix chassis.

The output card shall transmit over one multimode fiber optic strand, contact manufacturer for current list of suggested cable types.

The Output Card shall provide 2 discrete outputs.

The card shall provide 1 HDMI output for every 2 single cable transmission outputs:

HDMI output shall be a parallel signal to the single cable transmission output.

Basis of design product: Crestron DMC-SO-HD

Crestron DMC-SO-HD DigitalMedia output card.

Modular Design:

Output card shall be a modular unit with the following characteristics:

Output card module shall be one of many available output modules available from same manufacturer.

Modular component of compatible card frame matrix switcher.

Occupy output card slot space in compatible matrix switcher.

Field installable plug-in card.

Output Card Source:

All outputs shall be sourced by the matrix backplane.  Matrix backplane shall include all sources connected to matrix input cards.

Outputs:

Connectors:

SC female optical fiber connector for each transmission output.

Female 19-pin Type A HDMI for parallel output channels.

Format:

HDMI with Deep Color and 3D

HDCP content protection

Signal includes:

Video source signal at native resolution.

Embedded audio signal

Embedded Ethernet

SINGLEMODE FIBER CABLE TRANSMISSION OUTPUT CARD

Description

The Output Card shall provide transmission of any routed source signal within the matrix chassis.

The output card shall transmit over one singlemode fiber optic strand, contact manufacturer for current list of suggested cable types.

The Output Card shall provide 2 discrete outputs.

The card shall provide 1 HDMI output for every 2 single cable transmission outputs:

HDMI output shall be a parallel signal to the single cable transmission output.

Basis of design product: Crestron DMC-S2O

Crestron DMC-S2O DigitalMedia output card.

Modular Design:

Output card shall be a modular unit with the following characteristics:

Output card module shall be one of many available output modules available from same manufacturer.

Modular component of compatible card frame matrix switcher.

Occupy output card slot space in compatible matrix switcher.

Field installable plug-in card.

Output Card Source:

All outputs shall be sourced by the matrix backplane.  Matrix backplane shall include all sources connected to matrix input cards.

Outputs:

Connectors:

LC female optical fiber connector for each transmission output.

Female 19-pin Type A HDMI for parallel output channels.

Format:

HDMI with Deep Color and 3D

HDCP content protection

Signal includes:

Video source signal at native resolution.

Embedded audio signal

Embedded Ethernet

HDMI WITH ANALOG AUDIO OUTPUT CARD

Description

The Output Card shall provide an HDMI with balanced analog audio output of any routed source signal within the matrix chassis.

Analog audio level shall be controllable via programming and control processor by same manufacturer.

The Output Card shall provide 2 discrete outputs.

Basis of design product: Crestron DMC-HDO

Crestron DMC-HDO DigitalMedia output card.

Modular Design:

Output card shall be a modular unit with the following characteristics:

Output card module shall be one of many available output modules available from same manufacturer.

Modular component of compatible card frame matrix switcher.

Occupy one output card slot space in compatible matrix switcher.

Field installable plug-in card.

Output Card Source:

All outputs shall be sourced by the matrix backplane.  Matrix backplane shall include all sources connected to matrix input cards.

Outputs:

Connectors:

LC female optical fiber connector for each transmission output.

Female 19-pin Type A HDMI

5 position terminal block

Audio Format:

Analog balanced stereo

Analog unbalanced stereo

Video Format:

HDMI with Deep Color and 3D

HDCP content protection

Signal includes:

Video source signal at native resolution.

Embedded audio signal

Embedded Ethernet

4K HDMI WITH ANALOG AUDIO OUTPUT CARD

Description

The Output Card shall provide an HDMI with balanced analog audio output of any routed source signal within the matrix chassis.

Analog audio level shall be controllable via programming and control processor by same manufacturer.

The Output Card shall provide 2 discrete outputs.

4K Operation:

The Output Card shall support output of 4K HDMI signals from compatible source to compatible sink.

Specifier: The DMC-4K-HDO card is compatible with original/existing and current DM-MD8X8, DM-MD16X16, and DM-MD32X32 chassis units.  Existing chassis units may be upgraded to 4K capability by replacing existing i/o cards and DM transmitters and receivers with 4K capable models.

Basis of design product: Crestron DMC-4K-HDO

Crestron DMC-4K-HDO DigitalMedia output card.

Modular Design:

Output card shall be a modular unit with the following characteristics:

Output card module shall be one of many available output modules available from same manufacturer.

Modular component of compatible card frame matrix switcher.

Occupy one output card slot space in compatible matrix switcher.

Field installable plug-in card.

Output Card Source:

All outputs shall be sourced by the matrix backplane.  Matrix backplane shall include all sources connected to matrix input cards.

Outputs:

Connectors:

LC female optical fiber connector for each transmission output.

Female 19-pin Type A HDMI

5 position terminal block

Audio Format:

Analog balanced stereo

Analog unbalanced stereo

Video Format:

4K HDMI with Deep Color and 3D

HDCP2.2 content protection

Signal includes:

Video source signal at native resolution.

Embedded audio signal

Embedded Ethernet

H.264 STREAMING OUTPUT CARD

Description

The Output Card shall provide an H.264 stream of any one or two routed source signals within the matrix chassis.

Analog audio level shall be controllable via programming and control processor by same manufacturer.

The Output Card shall provide one output that may be composed of any one or two input sources.

The single stream output shall be capable of combining two separate sources in the following methods:

Picture-in-picture

Side-by-side.

Basis of design product: Crestron DMC-STRO

Crestron DMC-STRO DigitalMedia output card.

Modular Design:

Output card shall be a modular unit with the following characteristics:

Output card module shall be one of many available output modules available from same manufacturer.

Modular component of compatible card frame matrix switcher.

Occupy output card slot space in compatible matrix switcher.

Field installable plug-in card.

Output Card Source:

All outputs shall be sourced by the matrix backplane.  Matrix backplane shall include all sources connected to matrix input cards.

Output:

Connectors:

CONTENT LAN: (1, 2, 3, or 4) 8-wire RJ45 female; 10Base-T/100Base-TX/1000Base-T Ethernet ports; Each port provides a dedicated LAN connection per streaming output

LAN Link Indicators: (1 set per each CONTENT LAN port)

Audio:

AAC stereo

Audio Controls:

Volume: Adjustable 0% to 100%, plus mute

Delay: 0.0 to 40.0 ms

Source 1 – 2 Mix Level: -80.0 to 0.0 dB

Video:

Video Streaming Formats: H.264 @ 360p60, 480p30, 480p60, 720p10, 720p15, 720p30, 720p60, 1080p10, 1080p15, and 1080p30

Video Streaming Bitrate: Up to 25 Mbps

Streaming Protocols: RTP, RTSP, SDP

Container: MPEG-2 transport stream (.ts) or none

Session Initiation Modes:

By receiver (unicast)

By transmitter (unicast)

Multicast via RTSP

Multicast via UDP

4K TRANSMITTER TYPE 1

Description

The signal transmitters shall extend 4K HDMI video, audio, and data over a single UTP/STP cable to compatible transmission receiver modules or ports.  The following source formats shall be supported:

4K HDMI, HDCP2.2 compliant

DVI-I

Dual-Mode Display Port

RGBHV

YPbPr

VGA

Y/C

Composite

Analog 2-channel audio

USB HID (Human Interface Device)

Switching:

- Transmitter shall include integrated switching with signal sensing.

Switching modes:

Automatic: switcher shall switch to the last detected input.

Controlled: control processor controls source switching and audio break-away switching.

Basis of design product: Crestron DM-TX-4K-302-C DigitalMedia transmitter.

Performance

The transmitter shall meet the following minimum requirements:

Two (2) HDMI video, audio, and control input:

Supports 4K HDMI

Supports HDCP 2.2

Supports Dolby Digital, Dolby Digital EX, DTS, DTS-ES, DTS 96/24, up to 8 channel PCM.

Supports DVI-D with adaptor.

Supports Dual-Mode DisplayPort with adaptor.

CEC device control

One (1) DB15 input:

Component (YPbPr)

RGB

S-Video (Y/C)

Composite Video

One (1) analog stereo audio input:

(1) 3.5mm TRS (L/R unbalanced)

One (1) USB HID port.

Supports USB HID class devices

HDMI digital video/audio output

One (1) 19-pin Type A HDMI female connector

Supports 4K HDMI

Supports HDCP 2.2

Ethernet Port

One (1) 8-wire RJ-45

10/100 Mbps, auto-switching, auto negotiating, auto-discovery, full/half duplex, DHCP

Single UTP/STP cable transmission connection

Supports HDBaseT signal specifications.

Supports remote power injection through matrix switcher.

Supports CAT5e.

Signal transmission up to 330 feet.

Power supply modes:

Remote power supplied by matrix switcher through UTP/STP transmission cable.

Local or remote DC power source.

Mounting:

Freestanding.

Surface mount.

Rack rail mount.

One (1) bidirectional RS-232 port:

One (1) 5-pin 3.5mm detachable terminal block.

GND, TX, RX, CTS, RTS support.

Up to 115.2k baud, hardware and software handshaking support.

One (1) IR/Serial port:

One (1) 2-pin 3.5mm detachable terminal block.

IR output up to 1.1 MHz.

1-way serial TTL/RS-232 (0-5 Volts) up to 19200 baud.

4K TRANSMITTER TYPE 2

Description

The signal transmitters shall extend 4K HDMI video, audio, and data over a single UTP/STP cable to compatible transmission receiver modules or ports.  The following source formats shall be supported:

4K HDMI, HDCP2.2 compliant

DVI-I

Dual-Mode Display Port

USB HID (Human Interface Device)

Switching:

- Transmitter shall include integrated switching with signal sensing.

Switching modes:

Automatic: switcher shall switch to the last detected input.

Controlled: control processor controls source switching.

Basis of design product: Crestron DM-TX-4K-202-C DigitalMedia transmitter.

Performance

The transmitter shall meet the following minimum requirements:

Two (2) HDMI video, audio, and control input:

Supports 4K HDMI

Supports HDCP 2.2

Supports Dolby Digital, Dolby Digital EX, DTS, DTS-ES, DTS 96/24, up to 8 channel PCM.

Supports DVI-D with adaptor.

Supports Dual-Mode DisplayPort with adaptor.

CEC device control

One (1) USB HID port.

Supports USB HID class devices

HDMI digital video/audio output

One (1) 19-pin Type A HDMI female connector

Supports 4K HDMI

Supports HDCP 2.2

Ethernet Port

One (1) 8-wire RJ-45

10/100 Mbps, auto-switching, auto negotiating, auto-discovery, full/half duplex, DHCP.

Single UTP/STP cable transmission connection

Supports HDBaseT signal specifications.

Supports remote power injection through matrix switcher.

Supports CAT5e.

Signal transmission up to 330 feet.

Power supply modes:

Remote power supplied by matrix switcher through UTP/STP transmission cable.

Local or remote DC power source.

Mounting:

Freestanding.

Surface mount.

Rack rail mount.

One (1) bidirectional RS-232 port:

One (1) 5-pin 3.5mm detachable terminal block.

GND, TX, RX, CTS, RTS support.

Up to 115.2k baud, hardware and software handshaking support.

One (1) IR/Serial port:

One (1) 2-pin 3.5mm detachable terminal block.

IR output up to 1.1 MHz.

1-way serial TTL/RS-232 (0-5 Volts) up to 19200 baud.

4K TRANSMITTER TYPE 3

Description

The wall mount signal transmitters shall extend 4K HDMI video and audio over a single UTP/STP cable to compatible transmission receiver modules or ports.  The following source formats shall be supported:

4K HDMI, HDCP2.2 compliant.

DVI-I

Dual-Mode Display Port.

Basis of design product: Crestron DM-TX-4K-100-C-1G DigitalMedia transmitter

Performance

The transmitter shall meet the following minimum requirements:

One (1) HDMI video, audio, and control input:

Supports 4K HDMI.

Supports HDCP 2.2

Supports Dolby Digital, Dolby Digital EX, DTS, DTS-ES, DTS 96/24, up to 8 channel PCM.

Supports DVI-D with adaptor.

Supports Dual-Mode DisplayPort with adaptor.

CEC device control.

Single UTP/STP cable transmission connection

Supports HDBaseT signal specifications.

Supports remote power injection through matrix switcher.

Supports CAT5e.

Signal transmission up to 330 feet.

Power supply modes:

Remote power supplied by matrix switcher through UTP/STP transmission cable.

Local or remote DC power source.

Mounting:

1-gang wall box mount

Surface mount

Rack rail mount

One (1) bidirectional RS-232 port:

One (1) 5-pin 3.5mm detachable terminal block

GND, TX, RX, CTS, RTS support

Up to 115.2k baud, hardware and software handshaking support.

One (1) IR/Serial port:

One (1) 2-pin 3.5mm detachable terminal block.

IR output up to 1.1 MHz.

1-way serial TTL/RS-232 (0-5 Volts) up to 19200 baud.

TRANSMITTER TYPE C1

Description

The signal transmitters shall extend HDMI video, audio, and data over a single UTP/STP cable to compatible transmission receiver modules or ports.  The following source formats shall be supported:

HDMI

DVI-I

RGBHV

YPbPr

VGA

Y/C

Composite

Analog 2-channel audio

USB HID (Human Interface Device)

Switching:

- Transmitter shall include integrated switching with signal sensing.

Switching modes:

Automatic: switcher shall switch to the last detected input.

Controlled: control processor controls source switching and audio break-away switching.

Basis of design product: Crestron DM-TX-201C DigitalMedia transmitter.

Performance

The transmitter shall meet the following minimum requirements:

One (1) HDMI video, audio, and control input:

Supports HDMI.

Supports HDCP.

Supports Dolby Digital, Dolby Digital EX, DTS, DTS-ES, DTS 96/24, up to 8 channel PCM.

Supports DVI-D with adaptor.

Supports Dual-Mode DisplayPort.

CEC device control.

One (1) DB15 input:

Component (YPbPr)

RGB

S-Video (Y/C)

Composite Video

One (1) analog stereo audio input:

(1) 3.5mm TRS (L/R unbalanced)

One (1) USB HID port.

Supports USB HID class devices

HDMI digital video/audio output

One (1) 19-pin Type A HDMI female connector

Ethernet Port

One (1) 8-wire RJ-45

10/100 Mbps, auto-switching, auto negotiating, auto-discovery, full/half duplex, DHCP

Single UTP/STP cable transmission connection

Supports HDBaseT signal specifications.

Supports remote power injection through matrix switcher.

Supports CAT5e.

Signal transmission up to 330 feet.

Power supply modes:

Remote power supplied by matrix switcher through UTP/STP transmission cable.

Local or remote DC power source.

Mounting:

Freestanding.

Surface mount.

Rack rail mount.

TRANSMITTER TYPE C2

Description

The signal transmitters shall extend HDMI video, audio, and data over a single UTP/STP cable to compatible transmission receiver modules or ports.  The following source formats shall be supported:

HDMI

DVI-I

RGBHV

VGA

YPbPr

Y/C

Composite

Analog 2-channel audio

USB HID (Human Interface Device)

Switching:

- Transmitter shall include integrated switching with signal sensing.

Switching modes:

Automatic: switcher shall switch to the last detected input.

Controlled: control processor controls source switching and audio break-away switching.

Basis of design product: Crestron DM-TX-200-C-2G DigitalMedia transmitter.

Performance

The transmitter shall meet the following minimum requirements:

One (1) HDMI video, audio, and control input:

Supports HDMI.

Supports HDCP.

Supports Dolby Digital, Dolby Digital EX, DTS, DTS-ES, DTS 96/24, up to 8 channel PCM.

Supports DVI-D with adaptor.

Supports Dual-Mode DisplayPort.

CEC device control.

One (1) DB15 input:

Component (YPbPr)

RGB

S-Video (Y/C)

Composite Video

One (1) analog stereo audio input:

(1) 3.5mm TRS (L/R unbalanced)

One (1) USB HID port.

Supports USB HID class devices

Single UTP/STP cable transmission connection

Supports HDBaseT signal specifications.

Supports remote power injection through matrix switcher.

Supports CAT5e.

Signal transmission up to 330 feet.

Power supply modes:

Remote power supplied by matrix switcher through UTP/STP transmission cable.

Local or remote DC power source.

Mounting:

2-gang wall box mount.

2-gang floor box mount.

TRANSMITTER TYPE C3

Description

The signal transmitters shall extend HDMI video, audio, and data over a single UTP/STP cable to compatible transmission receiver modules or ports.  Transmitter shall support connection of four separate sources simultaneously.  The following source formats shall be supported:

Display Port

HDMI

DVI-I

RGBHV

VGA

YPbPr

Y/C

Composite

Analog 2-channel audio

USB HID (Human Interface Device)

Switching: Transmitter shall include integrated switching with signal sensing.

Switching modes:

Automatic: switcher shall switch to the last detected input.

Controlled: control processor controls source switching and audio break-away switching.

Transmitter shall include the following control ports for remote device control.

One (1) 5-position terminal block for Serial RS-232 communication.

One (1) 2-position terminal block for Infrared (IR) control.

Basis of design product: Crestron DM-TX-401-C DigitalMedia transmitter.

Performance

The transmitter shall meet the following minimum requirements:

One (1) HDMI video, audio, and control input:

Supports HDMI.

Supports HDCP.

Supports Dolby Digital, Dolby Digital EX, DTS, DTS-ES, DTS 96/24, up to 8 channel PCM.

Supports DVI-D with adaptor.

Supports Dual-Mode DisplayPort.

CEC device control.

One (1) Display Port video, audio, and control input:

Supports DisplayPort.

Supports HDCP.

Supports Dolby Digital®, Dolby Digital EX, DTS®, DTS-ES, DTS 96/24, Up to 8ch PCM

One (1) DB15 input:

Component (YPbPr)

RGB

S-Video (Y/C)

Composite Video

One (1) analog stereo audio input:

(1) 3.5mm TRS (L/R unbalanced)

One (1) analog stereo audio input:

(2) Female RCA (L/R unbalanced)

One (1) USB HID port.

Supports USB HID class devices

HDMI digital video/audio output

One (1) 19-pin Type A HDMI female connector

Ethernet Port

One (1) 8-wire RJ-45

10/100 Mbps, auto-switching, auto negotiating, auto-discovery, full/half duplex, DHCP

Single UTP/STP cable transmission connection

Supports HDBaseT signal specifications.

Supports remote power injection through matrix switcher.

Supports CAT5e.

Signal transmission up to 330 feet.

Power supply modes:

Remote power supplied by matrix switcher through UTP/STP transmission cable.

Local or remote DC power source.

Mounting:

Freestanding

1U, 1/2-width 19-inch rack-mountable

“S” TRANSMITTERS

The signal transmitters shall extend HDMI video and audio, DVI-I, RGBHV, YPbPr, Y/C, Composite, Analog 2-channel audio, and USB HID (Human Interface Device) data to compatible transmission receiver modules or ports.

Switching:

Multiple input transmitters shall include integrated switching with signal sensing.

Switching modes:

Automatic: switcher shall switch to the last detected input.

Controlled: control processor controls source switching and audio break-away switching.

TRANSMITTER TYPE S1

Description

The signal transmitters shall extend HDMI video, audio, and data over a single strand of multimode fiber cable to compatible transmission receiver modules or ports.  The following source formats shall be supported:

HDMI

DVI-I

RGBHV

YPbPr

VGA

Y/C

Composite

Analog 2-channel audio

USB HID (Human Interface Device)

Switching:

- Transmitter shall include integrated switching with signal sensing.

Switching modes:

Automatic: switcher shall switch to the last detected input.

Controlled: control processor controls source switching and audio break-away switching.

Basis of design product: Crestron DM-TX-201S DigitalMedia transmitter.

Performance

The transmitter shall meet the following minimum requirements:

One (1) HDMI video, audio, and control input:

Supports HDMI.

Supports HDCP.

Supports Dolby Digital, Dolby Digital EX, DTS, DTS-ES, DTS 96/24, up to 8 channel PCM.

Supports DVI-D with adaptor.

Supports Dual-Mode DisplayPort.

CEC device control.

One (1) DB15 input:

Component (YPbPr)

RGB

S-Video (Y/C)

Composite Video

One (1) analog stereo audio input:

(1) 3.5mm TRS (L/R unbalanced)

One (1) USB HID port.

Supports USB HID class devices

HDMI digital video/audio output

One (1) 19-pin Type A HDMI female connector

Ethernet Port

One (1) 8-wire RJ-45

10/100 Mbps, auto-switching, auto negotiating, auto-discovery, full/half duplex, DHCP

Single multimode fiber cable transmission connection

One (1) SC female connector.

Signal transmission up to 1000 feet (300m).

Power supply modes:

Remote power supplied by matrix switcher through UTP/STP transmission cable.

Local or remote DC power source.

Mounting:

Freestanding.

Surface mount.

Rack rail mount.

TRANSMITTER TYPE S3

Description

The signal transmitters shall extend HDMI video, audio, and data over a single strand of multimode fiber cable to compatible transmission receiver modules or ports.  Transmitter shall support connection of four separate sources simultaneously.  The following source formats shall be supported:

Display Port

HDMI

DVI-I

RGBHV

VGA

YPbPr

Y/C

Composite

Analog 2-channel audio

USB HID (Human Interface Device)

Switching: Transmitter shall include integrated switching with signal sensing.

Switching modes:

Automatic: switcher shall switch to the last detected input.

Controlled: control processor controls source switching and audio break-away switching.

Transmitter shall include the following control ports for remote device control.

One (1) 5-position terminal block for Serial RS-232 communication.

One (1) 2-position terminal block for Infrared (IR) control.

Basis of design product: Crestron DM-TX-401-S DigitalMedia transmitter.

Performance

The transmitter shall meet the following minimum requirements:

One (1) HDMI video, audio, and control input:

Supports HDMI.

Supports HDCP.

Supports Dolby Digital, Dolby Digital EX, DTS, DTS-ES, DTS 96/24, up to 8 channel PCM.

Supports DVI-D with adaptor.

Supports Dual-Mode DisplayPort.

CEC device control.

One (1) Display Port video, audio, and control input:

Supports DisplayPort Multimode.

Supports HDCP.

Supports Dolby Digital®, Dolby Digital EX, DTS®, DTS-ES, DTS 96/24, Up to 8ch PCM

One (1) DB15 input:

Component (YPbPr)

RGB

S-Video (Y/C)

Composite Video

One (1) analog stereo audio input:

(1) 3.5mm TRS (L/R unbalanced)

One (1) analog stereo audio input:

(2) Female RCA (L/R unbalanced)

One (1) USB HID port.

Supports USB HID class devices

HDMI digital video/audio output

One (1) 19-pin Type A HDMI female connector

Ethernet Port

One (1) 8-wire RJ-45

10/100 Mbps, auto-switching, auto negotiating, auto-discovery, full/half duplex, DHCP

Single multimode fiber cable transmission connection

One (1) SC female connector.

Signal transmission up to 1000 feet (300m).

Power supply modes:

Remote power supplied by matrix switcher through UTP/STP transmission cable.

Local or remote DC power source.

Mounting:

Freestanding

1U, 1/2-width 19-inch rack-mountable

“S2” TRANSMITTERS

The signal transmitters shall extend HDMI video and audio, DVI-I, RGBHV, YPbPr, Y/C, Composite, Analog 2-channel audio, and USB HID (Human Interface Device) data to compatible transmission receiver modules or ports.

Switching:

Multiple input transmitters shall include integrated switching with signal sensing.

Switching modes:

Automatic: switcher shall switch to the last detected input.

Controlled: control processor controls source switching and audio break-away switching.

TRANSMITTER TYPE S2-1

Description

The signal transmitters shall extend HDMI video, audio, and data over a single strand of multimode fiber cable to compatible transmission receiver modules or ports.  The following source formats shall be supported:

HDMI

DVI-I

RGBHV

YPbPr

VGA

Y/C

Composite

Analog 2-channel audio

USB HID (Human Interface Device)

Switching:

- Transmitter shall include integrated switching with signal sensing.

Switching modes:

Automatic: switcher shall switch to the last detected input.

Controlled: control processor controls source switching and audio break-away switching.

Basis of design product: Crestron DM-TX-201-S2 DigitalMedia transmitter.

Performance

The transmitter shall meet the following minimum requirements:

One (1) HDMI video, audio, and control input:

Supports HDMI.

Supports HDCP.

Supports Dolby Digital, Dolby Digital EX, DTS, DTS-ES, DTS 96/24, up to 8 channel PCM.

Supports DVI-D with adaptor.

Supports Dual-Mode DisplayPort.

CEC device control.

One (1) DB15 input:

Component (YPbPr)

RGB

S-Video (Y/C)

Composite Video

One (1) analog stereo audio input:

(1) 3.5mm TRS (L/R unbalanced)

One (1) USB HID port.

Supports USB HID class devices

HDMI digital video/audio output

One (1) 19-pin Type A HDMI female connector

Ethernet Port

One (1) 8-wire RJ-45

10/100 Mbps, auto-switching, auto negotiating, auto-discovery, full/half duplex, DHCP

Single mode fiber cable transmission connection

One (1) LC female connector

Supports G.652.D single-mode fiber optic cable

Signal transmission up to 7.5 miles (12 km).

Power supply modes:

Remote power supplied by matrix switcher through UTP/STP transmission cable.

Local or remote DC power source.

Mounting:

Freestanding.

Surface mount.

Rack rail mount.

TRANSMITTER TYPE S2-3

Description

The signal transmitters shall extend HDMI video, audio, and data over a single strand of multimode fiber cable to compatible transmission receiver modules or ports.  Transmitter shall support connection of four separate sources simultaneously.  The following source formats shall be supported:

Display Port

HDMI

DVI-I

RGBHV

VGA

YPbPr

Y/C

Composite

Analog 2-channel audio

USB HID (Human Interface Device)

Switching: Transmitter shall include integrated switching with signal sensing.

Switching modes:

Automatic: switcher shall switch to the last detected input.

Controlled: control processor controls source switching and audio break-away switching.

Transmitter shall include the following control ports for remote device control.

One (1) 5-position terminal block for Serial RS-232 communication.

One (1) 2-position terminal block for Infrared (IR) control.

Basis of design product: Crestron DM-TX-401-S2 DigitalMedia transmitter.

Performance

The transmitter shall meet the following minimum requirements:

One (1) HDMI video, audio, and control input:

Supports HDMI.

Supports HDCP.

Supports Dolby Digital, Dolby Digital EX, DTS, DTS-ES, DTS 96/24, up to 8 channel PCM.

Supports DVI-D with adaptor.

Supports Dual-Mode DisplayPort.

CEC device control.

One (1) Display Port video, audio, and control input:

Supports Dual-Mode DisplayPort.

Supports HDCP.

Supports Dolby Digital®, Dolby Digital EX, DTS®, DTS-ES, DTS 96/24, Up to 8ch PCM

One (1) DB15 input:

Component (YPbPr)

RGB

S-Video (Y/C)

Composite Video

One (1) analog stereo audio input:

(1) 3.5mm TRS (L/R unbalanced)

One (1) analog stereo audio input:

(2) Female RCA (L/R unbalanced)

One (1) USB HID port.

Supports USB HID class devices

HDMI digital video/audio output

One (1) 19-pin Type A HDMI female connector

Ethernet Port

One (1) 8-wire RJ-45

10/100 Mbps, auto-switching, auto negotiating, auto-discovery, full/half duplex, DHCP

Single mode fiber cable transmission connection

One (1) LC female connector

Supports G.652.D single-mode fiber optic cable

Signal transmission up to 7.5 miles (12 km).

Power supply modes:

Remote power supplied by matrix switcher through UTP/STP transmission cable.

Local or remote DC power source.

Mounting:

Freestanding

1U, 1/2-width 19-inch rack-mountable

“C” RECEIVERS

Receivers shall accept a multi-signal transmission over a single UTP or STP cable.

Multi-Signal transmission shall be converted to standard HDMI video and control signals.

Receiver shall support device control and communication when integrated with compatible control processors.

RECEIVER TYPE C1

Description

The signal receiver shall receive long distance transmission from compatible transmitter modules or ports.  Receiver shall include the following outputs types and connections:

HDMI

Analog stereo audio

Amplified stereo audio

USB HID (Human Interface Device)

Receiver shall include the following control port types for remote device control.

Serial RS-232 communication.

Infrared (IR) control.

Relays for contact closure control.

Basis of design product: Crestron DM-RMC-200-C

Performance

The receiver shall meet the following minimum requirements:

HDMI digital video, audio, and control output:

One (1) 19-pin Type A HDMI female connector

Supports HDMI with Deep Color and 3D.

Supports DVI-D with adaptor.

Supports HDCP.

HDMI audio Support:

Dolby Digital, Dolby Digital EX, Dolby TrueHD, DTS, DTS-ES, DTS 96/24, DTS-HD Master Audio, and up to 8 channel PCM.

CEC device control.

Integrated HD video scaling:

Deinterlacing and interlacing.

Frame rate conversion.

Deep Color support.

3D to 2D conversion.

Content adaptive noise reduction.

Wide screen format selection:

Zoom.

Stretch.

Maintain source aspect ratio.

1:1.

Video wall processing:

2x2.

3x2.

3x3.

4x3.

4x4.

Analog audio output:

Digital to analog conversion: 24-bit 48 kHz.

Volume gain range: -80db to 0db.

Stereo line level.

Stereo amplified, 15 Watts per channel at 8 ohms.

One (1) bidirectional RS-232 port:

One (1) 5-pin 3.5mm detachable terminal block.

GND, TX, RX, CTS, RTS support.

Up to 115.2k baud, hardware and software handshaking support.

Two (2) IR/Serial ports:

One (1) 4-pin 3.5mm detachable terminal block.

IR output up to 1.1 MHz.

1-way serial TTL/RS-232 (0-5 Volts) up to 19200 baud.

Two (2) normally open isolated relay ports:

One (1) 4-pin 3.5mm detachable terminal block.

Maximum ratings:  1 Amp, 30 Volts AC/DC.

One (1) digital contact closure sensing port:

One (1) 2-pin 3.5mm detachable terminal block.

Rating:  0-24 Volts DC, referenced to ground.

One (1) USB HID port.

USB type A female.

One (1) 10/100 LAN port.

Single UTP/STP cable transmission connection

Supports HDBaseT signal specifications.

Supports CAT5e

Signal transmission up to 330 feet

Power supply:

Local or remote DC power source.

Mounts on a US 2-gang electrical box

RECEIVER TYPE C2

Description

The signal receiver shall receive long distance transmission from compatible transmitter modules or ports.  Receiver shall include the following outputs types and connections:

HDMI

USB HID (Human Interface Device)

Receiver shall include the following control port types for remote device control.

Serial RS-232 communication.

Infrared (IR) control.

Basis of design product: Crestron DM-RMC-SCALER-C

Performance

The receiver shall meet the following minimum requirements:

HDMI digital video, audio, and control output:

One (1) 19-pin Type A HDMI female connector

Supports HDMI with Deep Color and 3D.

Supports DVI-D with adaptor.

Supports HDCP.

HDMI audio Support:

Dolby Digital, Dolby Digital EX, Dolby TrueHD, DTS, DTS-ES, DTS 96/24, DTS-HD Master Audio, and up to 8 channel PCM.

CEC device control.

Integrated HD video scaling:

Deinterlacing and interlacing.

Frame rate conversion.

Deep Color support.

3D to 2D conversion.

Content adaptive noise reduction.

Wide screen format selection:

Zoom.

Stretch.

Maintain source aspect ratio.

1:1.

Video wall processing:

2x2.

3x2.

3x3.

4x3.

4x4.

One (1) bidirectional RS-232 port:

One (1) 5-pin 3.5mm detachable terminal block.

GND, TX, RX, CTS, RTS support.

Up to 115.2k baud, hardware and software handshaking support.

Two (2) IR/Serial ports:

One (1) 4-pin 3.5mm detachable terminal block.

IR output up to 1.1 MHz.

1-way serial TTL/RS-232 (0-5 Volts) up to 19200 baud.

One (1) USB HID port.

USB type A female.

One (1) 10/100 LAN port.

Single UTP/STP cable transmission connection

Supports HDBaseT signal specifications.

Supports CAT5e

Signal transmission up to 330 feet

Power supply:

Local or remote DC power source.

Mounts on a US 2-gang electrical box

RECEIVER TYPE 4K

Description

The signal receiver shall receive long distance 4K HDMI transmission from compatible transmitter modules or ports.  Receiver shall include the following outputs types and connections:

4K HDMI

Receiver shall include the following control port types for remote device control.

Serial RS-232 communication.

Infrared (IR) control.

Basis of design product: Crestron DM-RMC-4K-100-C

Performance

The receiver shall meet the following minimum requirements:

HDMI digital video, audio, and control output:

One (1) 19-pin Type A HDMI female connector

Supports HDMI with Deep Color and 3D.

Supports 4K.

Supports DVI-D with adaptor.

Supports HDCP.

HDMI audio Support:

Dolby Digital, Dolby Digital EX, Dolby TrueHD, DTS, DTS-ES, DTS 96/24, DTS-HD Master Audio, and up to 8 channel PCM.

CEC device control.

One (1) bidirectional RS-232 port:

One (1) 5-pin 3.5mm detachable terminal block.

GND, TX, RX, CTS, RTS support.

Up to 115.2k baud, hardware and software handshaking support.

Two (2) IR/Serial ports:

One (1) 4-pin 3.5mm detachable terminal block.

IR output up to 1.1 MHz.

1-way serial TTL/RS-232 (0-5 Volts) up to 19200 baud.

One (1) digital contact closure sensing port:

One (1) 2-pin 3.5mm detachable terminal block.

Rating:  0-24 Volts DC, referenced to ground.

One (1) 10/100 LAN port.

Single UTP/STP cable transmission connection

Supports HDBaseT signal specifications.

Supports remote power injection through matrix switcher.

The receiver shall support transmissions over manufacturer suggested STP cable, CAT5e or CAT6 cable, or better.

Contact manufacturer prior to cable installation and use manufacturers suggested transmission cable, connectors, and installation methods for all segments of signal path.

2K Signal transmission up to 330 feet.

4K Signal transmission up to 230 feet.

Specifier: For maximum 4K signal transmission distance, specify Crestron DM-8G cable.  Contact manufacturer prior to cable installation and use manufacturers current suggested transmission cable, connectors, and installation methods for all segments of signal path.

Power supply:

Remote power supplied by matrix switcher through UTP/STP transmission cable.

Local or remote DC power source.

Mounts on a US 2-gang electrical box

RECEIVER TYPE 4K WALL

Description

The signal receiver shall receive long distance 4K HDMI transmission from compatible transmitter modules or ports.  Receiver shall include the following outputs types and connections:

4K HDMI

Receiver shall include the following control port types for remote device control.

Serial RS-232 communication.

Infrared (IR) control.

Basis of design product: Crestron DM-RMC-4K-100-C-1G

Performance

The receiver shall meet the following minimum requirements:

HDMI digital video, audio, and control output:

One (1) 19-pin Type A HDMI female connector

Supports HDMI with Deep Color and 3D.

Supports 4K.

Supports DVI-D with adaptor.

Supports HDCP2.2

HDMI audio Support:

Dolby Digital, Dolby Digital EX, Dolby TrueHD, DTS, DTS-ES, DTS 96/24, DTS-HD Master Audio, and up to 8 channel PCM.

CEC device control.

One (1) bidirectional RS-232 port:

One (1) 3-pin 3.5mm detachable terminal block.

GND, TX, RX support.

Up to 115.2k baud, hardware and software handshaking support.

One (1) IR/Serial port:

One (1) 2-pin 3.5mm detachable terminal block.

IR output up to 1.1 MHz.

1-way serial TTL/RS-232 (0-5 Volts) up to 19200 baud.

Single UTP/STP cable transmission connection

Supports HDBaseT signal specifications.

Supports remote power injection through matrix switcher.

The receiver shall support transmissions over manufacturer suggested STP cable, CAT5e or CAT6 cable, or better.

Contact manufacturer prior to cable installation and use manufacturers suggested transmission cable, connectors, and installation methods for all segments of signal path.

2K Signal transmission up to 330 feet.

4K Signal transmission up to 230 feet.

Specifier: For maximum 4K signal transmission distance, specify Crestron DM-CBL-ULTRA cable.  Contact manufacturer prior to cable installation and use manufacturers current suggested transmission cable, connectors, and installation methods for all segments of signal path.

Power supply:

Remote power supplied by matrix switcher through UTP/STP transmission cable.

Local or remote DC power source.

Mounts in US 1-gang electrical box

RECEIVER TYPE 4K SCALER

Description

The signal receiver shall receive long distance transmission from compatible transmitter modules or ports.  Receiver shall include the following outputs types and connections:

4K HDMI

HDCP2.2 Compliant

Receiver shall include the following control port types for remote device control.

Serial RS-232 communication.

Infrared (IR) control.

Basis of design product: Crestron DM-RMC-4K-SCALER-C

Performance

The receiver shall meet the following minimum requirements:

HDMI digital video, audio, and control output:

One (1) 19-pin Type A HDMI female connector

Supports HDMI with Deep Color and 3D.

Supports DVI-D with adaptor.

Supports HDCP2.2

HDMI audio Support:

Dolby Digital, Dolby Digital EX, Dolby TrueHD, DTS, DTS-ES, DTS 96/24, DTS-HD Master Audio, and up to 8 channel PCM.

CEC device control.

Integrated HD video scaling:

4K/60 Scaling

Deinterlacing and interlacing.

Frame rate conversion.

Deep Color support.

Content adaptive noise reduction.

Wide screen format selection:

Zoom.

Stretch.

Maintain source aspect ratio.

1:1.

Video wall processing:

Up to 64 individual displays

One (1) bidirectional RS-232 port:

One (1) 5-pin 3.5mm detachable terminal block.

GND, TX, RX, CTS, RTS support.

Up to 115.2k baud, hardware and software handshaking support.

Two (2) IR/Serial ports:

One (1) 4-pin 3.5mm detachable terminal block.

IR output up to 1.1 MHz.

1-way serial TTL/RS-232 (0-5 Volts) up to 19200 baud.

One (1) 10/100 LAN port.

Single UTP/STP cable transmission connection

Supports HDBaseT signal specifications.

Supports CAT5e

Signal transmission up to 330 feet

Power supply:

Local or remote DC power source.

Power over HDBaseT

Power over DM

RECEIVER TYPE 4K SCALER WITH DSP

Description

The signal receiver shall receive long distance transmission from compatible transmitter modules or ports.  Receiver shall include the following outputs types and connections:

4K HDMI

HDCP2.2 Compliant

Audio De-embedding

Audio Downmixing

Receiver shall include the following control port types for remote device control.

Serial RS-232 communication.

Infrared (IR) control.

Basis of design product: Crestron DM-RMC-4K-SCALER-C-DSP

Performance

The receiver shall meet the following minimum requirements:

HDMI digital video, audio, and control output:

One (1) 19-pin Type A HDMI female connector

Supports HDMI with Deep Color and 3D.

Supports DVI-D with adaptor.

Supports HDCP2.2

HDMI audio Support:

Dolby Digital, Dolby Digital EX, Dolby TrueHD, DTS, DTS-ES, DTS 96/24, DTS-HD Master Audio, and up to 8 channel PCM.

CEC device control.

Integrated HD video scaling:

4K/60 Scaling

Deinterlacing and interlacing.

Frame rate conversion.

Deep Color support.

Content adaptive noise reduction.

Wide screen format selection:

Zoom.

Stretch.

Maintain source aspect ratio.

1:1.

Video wall processing:

Up to 64 individual displays

One (1) bidirectional RS-232 port:

One (1) 5-pin 3.5mm detachable terminal block.

GND, TX, RX, CTS, RTS support.

Up to 115.2k baud, hardware and software handshaking support.

Two (2) IR/Serial ports:

One (1) 4-pin 3.5mm detachable terminal block.

IR output up to 1.1 MHz.

1-way serial TTL/RS-232 (0-5 Volts) up to 19200 baud.

One (1) 10/100 LAN port.

Single UTP/STP cable transmission connection

Supports HDBaseT signal specifications.

Supports CAT5e

Signal transmission up to 330 feet

Power supply:

Local or remote DC power source.

Power over HDBaseT

Power over DM

“S” RECEIVERS

Receivers shall accept a multi-signal transmission over a single multimode fiber cable.

Multi-Signal transmission shall be converted to standard HDMI video and control signals.

Receiver shall support device control and communication when integrated with compatible  control processors.

RECEIVER TYPE S1

Description

The signal receiver shall receive long distance transmission from compatible transmitter modules or ports.  Receiver shall include the following outputs types and connections:

HDMI

Analog stereo audio

Amplified stereo audio

USB HID (Human Interface Device)

Receiver shall include the following control port types for remote device control.

Serial RS-232 communication.

Infrared (IR) control.

Relays for contact closure control.

Basis of design product: Crestron DM-RMC-200-S

Performance

The receiver shall meet the following minimum requirements:

HDMI digital video, audio, and control output:

One (1) 19-pin Type A HDMI female connector

Supports HDMI with Deep Color and 3D.

Supports DVI-D with adaptor.

Supports HDCP.

HDMI audio Support:

Dolby Digital, Dolby Digital EX, Dolby TrueHD, DTS, DTS-ES, DTS 96/24, DTS-HD Master Audio, and up to 8 channel PCM.

CEC device control.

Integrated HD video scaling:

Deinterlacing and interlacing.

Frame rate conversion.

Deep Color support.

3D to 2D conversion.

Content adaptive noise reduction.

Wide screen format selection:

Zoom.

Stretch.

Maintain source aspect ratio.

1:1.

Video wall processing:

2x2.

3x2.

3x3.

4x3.

4x4.

Analog audio output:

Digital to analog conversion: 24-bit 48 kHz.

Volume gain range: -80db to 0db.

Stereo line level.

Stereo amplified, 15 Watts per channel at 8 ohms.

One (1) bidirectional RS-232 port:

One (1) 5-pin 3.5mm detachable terminal block.

GND, TX, RX, CTS, RTS support.

Up to 115.2k baud, hardware and software handshaking support.

Two (2) IR/Serial ports:

One (1) 4-pin 3.5mm detachable terminal block.

IR output up to 1.1 MHz.

1-way serial TTL/RS-232 (0-5 Volts) up to 19200 baud.

Two (2) normally open isolated relay ports:

One (1) 4-pin 3.5mm detachable terminal block.

Maximum ratings:  1 Amp, 30 Volts AC/DC.

One (1) digital contact closure sensing port:

One (1) 2-pin 3.5mm detachable terminal block.

Rating:  0-24 Volts DC, referenced to ground.

One (1) USB HID port.

USB type A female.

One (1) 10/100 LAN port.

Single multimode fiber cable transmission connection

One (1) SC female connector

Signal transmission up to 1000 feet (300 m)

Power supply:

Local DC power source.

Mounts on a US 2-gang electrical box

RECEIVER TYPE S2

Description

The signal receiver shall receive long distance transmission from compatible transmitter modules or ports.  Receiver shall include the following outputs types and connections:

HDMI

USB HID (Human Interface Device)

Receiver shall include the following control port types for remote device control.

Serial RS-232 communication.

Infrared (IR) control.

Basis of design product: Crestron DM-RMC-SCALER-S

Performance

The receiver shall meet the following minimum requirements:

HDMI digital video, audio, and control output:

One (1) 19-pin Type A HDMI female connector

Supports HDMI with Deep Color and 3D.

Supports DVI-D with adaptor.

Supports HDCP.

HDMI audio Support:

Dolby Digital, Dolby Digital EX, Dolby TrueHD, DTS, DTS-ES, DTS 96/24, DTS-HD Master Audio, and up to 8 channel PCM.

CEC device control.

Integrated HD video scaling:

Deinterlacing and interlacing.

Frame rate conversion.

Deep Color support.

3D to 2D conversion.

Content adaptive noise reduction.

Wide screen format selection:

Zoom.

Stretch.

Maintain source aspect ratio.

1:1.

Video wall processing:

2x2.

3x2.

3x3.

4x3.

4x4.

One (1) bidirectional RS-232 port:

One (1) 5-pin 3.5mm detachable terminal block.

GND, TX, RX, CTS, RTS support.

Up to 115.2k baud, hardware and software handshaking support.

Two (2) IR/Serial ports:

One (1) 4-pin 3.5mm detachable terminal block.

IR output up to 1.1 MHz.

1-way serial TTL/RS-232 (0-5 Volts) up to 19200 baud.

One (1) USB HID port.

USB type A female.

One (1) 10/100 LAN port.

Single multimode fiber cable transmission connection

One (1) SC female connector

Signal transmission up to 1000 feet (300 m)

Power supply:

Local DC power source.

Mounts on a US 2-gang electrical box

RECEIVER TYPE S3

Description

The signal receiver shall receive long distance transmission from compatible transmitter modules or ports.  Receiver shall include the following outputs types and connections:

HDMI

Receiver shall include the following control port types for remote device control.

Serial RS-232 communication.

Infrared (IR) control.

Basis of design product: Crestron DM-RMC-100-S

Performance

The receiver shall meet the following minimum requirements:

HDMI digital video, audio, and control output:

One (1) 19-pin Type A HDMI female connector

Supports HDMI with Deep Color and 3D.

Supports DVI-D with adaptor.

Supports HDCP.

HDMI audio Support:

Dolby Digital, Dolby Digital EX, Dolby TrueHD, DTS, DTS-ES, DTS 96/24, DTS-HD Master Audio, and up to 8 channel PCM.

CEC device control.

One (1) bidirectional RS-232 port:

One (1) 5-pin 3.5mm detachable terminal block.

GND, TX, RX, CTS, RTS support.

Up to 115.2k baud, hardware and software handshaking support.

Two (2) IR/Serial ports:

One (1) 4-pin 3.5mm detachable terminal block.

IR output up to 1.1 MHz.

1-way serial TTL/RS-232 (0-5 Volts) up to 19200 baud.

One (1) digital contact closure sensing port:

One (1) 2-pin 3.5mm detachable terminal block.

Rating:  0-24 Volts DC, referenced to ground.

One (1) 10/100 LAN port.

Single multimode fiber cable transmission connection

One (1) SC female connector

Signal transmission up to 1000 feet (300 m)

Power supply:

Remote power supplied by matrix switcher through UTP/STP transmission cable.

Local DC power source.

Mounts on a US 2-gang electrical box

RECEIVER TYPE S4

Description

The signal receiver shall receive long distance transmission from compatible transmitter modules or ports.  Receiver shall include the following outputs types and connections:

HDMI

Analog stereo audio

USB HID (Human Interface Device)

Receiver shall include the following control port types for remote device control.

Serial RS-232 communication.

Infrared (IR) control.

Relays for contact closure control.

Basis of design product: Crestron DM-RMC-150-S

Performance

The receiver shall meet the following minimum requirements:

HDMI digital video, audio, and control output:

One (1) 19-pin Type A HDMI female connector

Supports HDMI with Deep Color and 3D.

Supports DVI-D with adaptor.

Supports HDCP.

HDMI audio Support:

Dolby Digital, Dolby Digital EX, Dolby TrueHD, DTS, DTS-ES, DTS 96/24, DTS-HD Master Audio, and up to 8 channel PCM.

CEC device control.

Analog audio output:

Digital to analog conversion: 24-bit 48 kHz.

Volume gain range: -80db to 0db.

Stereo line level.

Stereo amplified, 15 Watts per channel at 8 ohms.

One (1) bidirectional RS-232 port:

One (1) 5-pin 3.5mm detachable terminal block.

GND, TX, RX, CTS, RTS support.

Up to 115.2k baud, hardware and software handshaking support.

Two (2) IR/Serial ports:

One (1) 4-pin 3.5mm detachable terminal block.

IR output up to 1.1 MHz.

1-way serial TTL/RS-232 (0-5 Volts) up to 19200 baud.

Two (2) normally open isolated relay ports:

One (1) 4-pin 3.5mm detachable terminal block.

Maximum ratings:  1 Amp, 30 Volts AC/DC.

One (1) digital contact closure sensing port:

One (1) 2-pin 3.5mm detachable terminal block.

Rating:  0-24 Volts DC, referenced to ground.

One (1) USB HID port.

USB type A female.

One (1) 10/100 LAN port.

Single multimode fiber cable transmission connection

One (1) SC female connector

Signal transmission up to 1000 feet (300 m)

Power supply:

Local DC power source.

Mounts on a US 2-gang electrical box

“S2” RECEIVERS

Receivers shall accept a multi-signal transmission over a singlemode fiber cable.

Multi-Signal transmission shall be converted to standard HDMI video and control signals.

Receiver shall support device control and communication when integrated with compatible  control processors

RECEIVER TYPE S2-1

Description

The signal receiver shall receive long distance transmission from compatible transmitter modules or ports.  Receiver shall include the following outputs types and connections:

HDMI

Analog stereo audio

Amplified stereo audio

USB HID (Human Interface Device)

Receiver shall include the following control port types for remote device control.

Serial RS-232 communication.

Infrared (IR) control.

Relays for contact closure control.

Basis of design product: Crestron DM-RMC-200-S2

Performance

The receiver shall meet the following minimum requirements:

HDMI digital video, audio, and control output:

One (1) 19-pin Type A HDMI female connector

Supports HDMI with Deep Color and 3D.

Supports DVI-D with adaptor.

Supports HDCP.

HDMI audio Support:

Dolby Digital, Dolby Digital EX, Dolby TrueHD, DTS, DTS-ES, DTS 96/24, DTS-HD Master Audio, and up to 8 channel PCM.

CEC device control.

Integrated HD video scaling:

Deinterlacing and interlacing.

Frame rate conversion.

Deep Color support.

3D to 2D conversion.

Content adaptive noise reduction.

Wide screen format selection:

Zoom.

Stretch.

Maintain source aspect ratio.

1:1.

Video wall processing:

2x2.

3x2.

3x3.

4x3.

4x4.

Analog audio output:

Digital to analog conversion: 24-bit 48 kHz.

Volume gain range: -80db to 0db.

Stereo line level.

Stereo amplified, 15 Watts per channel at 8 ohms.

One (1) bidirectional RS-232 port:

One (1) 5-pin 3.5mm detachable terminal block.

GND, TX, RX, CTS, RTS support.

Up to 115.2k baud, hardware and software handshaking support.

Two (2) IR/Serial ports:

One (1) 4-pin 3.5mm detachable terminal block.

IR output up to 1.1 MHz.

1-way serial TTL/RS-232 (0-5 Volts) up to 19200 baud.

Two (2) normally open isolated relay ports:

One (1) 4-pin 3.5mm detachable terminal block.

Maximum ratings:  1 Amp, 30 Volts AC/DC.

One (1) digital contact closure sensing port:

One (1) 2-pin 3.5mm detachable terminal block.

Rating:  0-24 Volts DC, referenced to ground.

One (1) USB HID port.

USB type A female.

One (1) 10/100 LAN port.

Singlemode fiber cable transmission connection

One (1) LC female connector.

Supports G.652.D single-mode fiber optic cable.

Signal transmission up to 7.5 miles (12 km).

Power supply:

Local DC power source.

Mounts on a US 2-gang electrical box

RECEIVER TYPE S2-2

Description

The signal receiver shall receive long distance transmission from compatible transmitter modules or ports.  Receiver shall include the following outputs types and connections:

HDMI

USB HID (Human Interface Device)

Receiver shall include the following control port types for remote device control.

Serial RS-232 communication.

Infrared (IR) control.

Basis of design product: Crestron DM-RMC-SCALER-S2

Performance

The receiver shall meet the following minimum requirements:

HDMI digital video, audio, and control output:

One (1) 19-pin Type A HDMI female connector

Supports HDMI with Deep Color and 3D.

Supports DVI-D with adaptor.

Supports HDCP.

HDMI audio Support:

Dolby Digital, Dolby Digital EX, Dolby TrueHD, DTS, DTS-ES, DTS 96/24, DTS-HD Master Audio, and up to 8 channel PCM.

CEC device control.

Integrated HD video scaling:

Deinterlacing and interlacing.

Frame rate conversion.

Deep Color support.

3D to 2D conversion.

Content adaptive noise reduction.

Wide screen format selection:

Zoom.

Stretch.

Maintain source aspect ratio.

1:1.

Video wall processing:

2x2.

3x2.

3x3.

4x3.

4x4.

One (1) bidirectional RS-232 port:

One (1) 5-pin 3.5mm detachable terminal block.

GND, TX, RX, CTS, RTS support.

Up to 115.2k baud, hardware and software handshaking support.

Two (2) IR/Serial ports:

One (1) 4-pin 3.5mm detachable terminal block.

IR output up to 1.1 MHz.

1-way serial TTL/RS-232 (0-5 Volts) up to 19200 baud.

One (1) USB HID port.

USB type A female.

One (1) 10/100 LAN port.

Singlemode fiber cable transmission connection

One (1) LC female connector.

Supports G.652.D single-mode fiber optic cable.

Signal transmission up to 7.5 miles (12 km).

Power supply:

Local DC power source.

Mounts on a US 2-gang electrical box

NOT USED

END OF SECTION 27 41 16