SECTION 27 41 16

INTEGRATED AUDIO-VIDEO SYSTEMS AND EQUIPMENT

GUIDE SPECIFICATION

Specifier: The Specifier/Design Professional is responsible for the accuracy of all project specifications, including system application and coordination with related sections.  This guide specification is provided as a convenience and requires editing to match actual project requirements.  CRESTRON ELECTRONICS, INC. SHALL NOT BE LIABLE FOR ANY DAMAGES ARISING OUT OF THE USE OF ANY OF ITS GUIDE SPECIFICATIONS.  For Crestron design assistance and design review please contact Sales Support Services Department at   800.237.2041 or techsales@crestron.com.

Specifer:  Please see PART 4 for a listing of products specified in this Guide Specification.

Table of Contents

1	GENERAL	5

1.1	SYSTEM DESCRIPTION	5

1.1.1	Enterprise Conference 10-20 People	5

1.2	RELATED REQUIREMENTS	5

1.2.1	Section 25 08 00 - Commissioning of Integrated Automation	6

1.2.2	Section 25 10 00 - Integrated Automation Network Equipment	6

1.2.3	Section 25 11 13 - Integrated Automation Network Servers	6

1.2.4	Section 25 13 13 - Integrated Automation Control and Monitoring Network Supervisory Control	6

1.2.5	Section 25 13 16 - Integrated Automation Control and Monitoring Network Integration Panels	6

1.2.6	Section 25 13 19 - Integrated Automation Control and Monitoring Network Interoperability	6

1.2.7	Section 25 15 16 - Integrated Automation Software for Control and Monitoring Networks	6

1.2.8	Section 26 09 43.13 - Digital-Network Lighting Controls	6

1.2.9	Section 27 15 00 - Communications Horizontal Cabling	6

1.2.10	Section 27 41 00 - Audio-Video Systems	6

1.3	REFERENCES	6

1.3.1	Abbreviations and Acronyms	6

2	PRODUCTS	6

2.1	AV Multi-Function Presentation Processor	6

2.1.1	Basis-of-Design Manufacturer:	6

2.1.2	Basis of Design Product:	7

2.1.3	System Architecture - The Presentation Processor unit shall be a single unit composed of internally integrated components providing multiple functions.  Included sub-components:	7

2.1.4	AV Multi-Function Presentation System - A single unit integrated digital media presentation system.  The single central switching and control unit integrates audio-video switching, audio mixing and amplification, and system control.	7

2.1.5	Video System	7

2.1.6	Audio System	7

2.1.7	Single Cable Transmission System	9

2.1.8	Control Processor	9

2.1.9	Multimedia Presentation Gateway	10

2.2	Amplifier	10

2.2.1	Basis of Design Amplifier:	10

2.2.2	Amplifier Input/Output Requirements	10

2.2.3	Audio Configuration	10

2.2.4	Audio Input and Output	11

2.2.5	Communication	11

2.2.6	User Interface Export	11

2.3	Audio DSP	11

2.3.1	Basis of Design Audio DSP:	11

2.3.2	User Interface Export	11

2.3.3	DSP Functions	12

2.3.4	Audio I/O	12

2.3.5	Audio Configuration	12

2.3.6	Filters shall include	13

2.3.7	Auxiliary channel strips	13

2.4	User Interfaces	13

2.4.1	Touch Screen	13

2.5	Wall mounted Hallway Room Availability Sign	14

2.5.1	Basis of Design Room Availability SIgn:	14

2.5.2	Functional Requirements	14

2.5.3	Physical	14

2.6	Network Occupancy Sensor:	14

2.6.1	Basis of Design Occupancy Sensor:	15

2.6.2	Functional	15

2.6.3	Physical	15

2.7	Speakers	15

2.7.1	Basis of Design Speaker:	15

2.7.2	Ceiling mounted 2-way speaker with the following specifications:	15

2.8	Surface Mounted Connection Interface	17

2.8.1	Basis of Design Connection Interface	17

2.8.2	The Connection interface shall be available in black anodized and brushed aluminum finishes.	17

2.8.3	The Connection interface shall provide configurable connectivity in a flush mount tabletop unit.	17

2.8.4	The Connection interface shall support custom combinations of pullout cables, cable retractors, connector panels, and AC power outlets.	17

2.8.5	The Connection interface shall utilize a universal cutout size that will fit other interfaces from same manufacturer.	17

2.8.6	17

2.9	Display Device	17

2.9.1	Display device shall be controllable via a built-in RS-232 connection port.	17

2.9.2	Display device shall support 4K video through built-in HDMI input connector(s).	17

2.10	Audio Source Devices	17

2.10.1	Microphones	17

2.11	Cable Tuner	17

2.11.1	HDMI AV source device shall be controllable via IR control protocol.	17

2.12	Computer	17

2.12.1	Computer or Laptop shall include an HDMI video output connection port.	17

2.13	Graphics Engine HDBaseT Receiver	17

2.13.1	Basis of Design Graphics Engine HDBaseT receiver	17

2.13.2	Functions	17

2.13.3	Performance	18

2.14	HDBaseT Transmitter	18

2.14.1	Basis of Design HDBaseT Transmitter:	18

2.14.2	The transmitter shall meet the following minimum requirements:	18

2.15	PTZ Camera	20

2.15.1	PTZ video camera shall be compatible with input connection on VTC codec unit.	20

2.16	VTC Codec	20

2.16.1	VTC Codec shall support connections as shown on system drawings.	20

2.17	Power Conditioner	20

2.17.1	Basis of Design Power Conditioner:	20

2.17.2	The Power Conditioner shall meet the following minimum requirements:	20

3	EXECUTION	20

4	APPENDICES	20

4.1	SPECIFIED PRODUCTS	20

4.1.1	Crestron DMPS3-4K-350-C	20

4.1.2	Crestron TSW-1060	20

4.1.3	Crestron SSW	20

4.1.4	Crestron DM-TX-4K-302-C	20

4.1.5	Crestron DM-DGE-200-C	20

4.1.6	Crestron DSP-1283	20

4.1.7	Crestron AMP-8075	21

4.1.8	Crestron SAROS IC6T	21

4.1.9	Crestron FT-600	21

4.1.10	Crestron PC-300	21

4.1.11	Crestron GLS-ODT-C-CN	21

# GENERAL

## SYSTEM DESCRIPTION

### Enterprise Conference 10-20 People

#### See AV System Drawings for device quantities and connection details.

Specifier Note: See example AV System Drawing for Conference 10-20 People

#### Display System

##### Video Display Devices - Room shall include two video displays.  Each with the capability of viewing any in room video source or VTC codec video source.

#### AV Sources

##### Computer (OFE)

##### IPTV Player/Decoder

##### Laptop Computers

#### Video Conferencing

##### Video Tele-Conference - Room shall include a VTC codec with

###### VTC Camera - Room shall include a video camera integrated with the VTC codec.

#### Audio System

##### Audio DSP - Room shall include an integrated audio processor, microphones, and amplification system supporting all in-room and far-end audio source and microphones.

#### AV Systems Control

##### Control User Interface - Room shall include a table top touch screen user interface.

##### Device Controller - Device and equipment controller for scheduling and management servers.

SPECIFIER: Retain this article if the AV controller specified in this section is connected to or integrated with a system scheduling and management server such as Crestron FUSION, where the controller in this section provides interfacing and control of equipment being scheduled and managed.

##### Room System Control - Room system and scheduling through touch screen user interface(s) with on-screen information display viewable on room display(s).

Specifier: Retain this article if room system includes Fusion integration.

##### Wall Mounted Control User Interface - Room shall include a wall mounted scheduling and control user interface with an integrated room occupancy indicator sign.

## RELATED REQUIREMENTS

### Section 25 08 00 - Commissioning of Integrated Automation

### Section 25 10 00 - Integrated Automation Network Equipment

### Section 25 11 13 - Integrated Automation Network Servers

### Section 25 13 13 - Integrated Automation Control and Monitoring Network Supervisory Control

### Section 25 13 16 - Integrated Automation Control and Monitoring Network Integration Panels

### Section 25 13 19 - Integrated Automation Control and Monitoring Network Interoperability

### Section 25 15 16 - Integrated Automation Software for Control and Monitoring Networks

### Section 26 09 43.13 - Digital-Network Lighting Controls

### Section 27 15 00 - Communications Horizontal Cabling

### Section 27 41 00 - Audio-Video Systems

## REFERENCES

### Abbreviations and Acronyms

#### CEC:  Consumer Electronics Control

#### EDID:  Extended display identification data

#### HDCP:  High-bandwidth Digital Content Protection

#### HDMI:  High-Definition Multimedia Interface

#### IDF: Intermediate Distribution Frame

#### KSV: Key Selection Vector

#### MDF: Main Distribution Frame

#### NVP: Nominal Velocity of Propagation

# PRODUCTS

## AV Multi-Function Presentation Processor

### Basis-of-Design Manufacturer:

Subject to compliance with requirements, provide products of Crestron Electronics, Inc., Rockleigh, NJ 07647, Phone 800-237-2041, Fax: 201-767 1903, www.crestron.com

### Basis of Design Product:

#### Crestron DMPS3-4K-[200/300/350]-C

SPECIFIER: See the following links for product selection: https://www.crestron.com/products/model/DMPS3-4K-200-C https://www.crestron.com/products/model/DMPS3-4K-300-C

### System Architecture - The Presentation Processor unit shall be a single unit composed of internally integrated components providing multiple functions.  Included sub-components:

#### Front Panel Control Interface shall provide basic control of the Presentation Processor.

#### Audio Matrix/Mixer

#### Video Matrix

#### Single Cable Transmission Matrix

Single cable 4K HDMI transmission transmitter and receiver ports with integrated switching matrix.

#### Control Processor

### AV Multi-Function Presentation System - A single unit integrated digital media presentation system.  The single central switching and control unit integrates audio-video switching, audio mixing and amplification, and system control.

### Video System

#### Video Inputs

##### Video inputs include HDMI and HDBaseT type input.

SPECIFIER: the DMPS3-4K-200 includes INPUTS: (6) HDMI, (1) DM; OUTPUTS: (1) HDMI, (1) DM

The DMPS3-4K-300/350 include INPUTS: (6) HDMI, (2) DM; OUTPUTS: (2) HDMI, (2) DM

###### HDBaseT type inputs shall be compatible with HDBaseT and manufacturers proprietary format supporting additional control functionality.

###### HDMI inputs are compatible with DVI and Dual-Mode DisplayPort sources

#### 4K Video Switcher

##### Built-in video matrix switching allows video sources to be routed simultaneously to output connectors.

##### Internal switcher shall support routing of HDMI and other AV sources to HDMI and HDBaseT outputs.

SPECIFIER: the DMPS3-4K-200 includes (1) HDMI and (1) DM output, the DMPS3-4K-300/350 include (2) HDMI and (2) DM outputs,

##### The HDMI outputs are compatible with DVI and the HDBaseT outputs are compatible with HDBaseT and manufacturer proprietary format supporting additional control functionality.

##### 4K/60 Video Scaling - Unit shall include an independent, 4K scaler on each HDMI output.

### Audio System

#### Audio Inputs - Each HDMI and analog audio input includes adjustable input compensation to accommodate a range of signals and maintain consistent volume levels when switching between sources.

#### Audio Amplifier - Unit shall include a built-in power amplifier.

##### Amplifier shall support three mutually exclusive amplifier modes.

###### 100V mode: mono, 40 Watts RMS per channel.

###### 4 ohm, 8 ohm mode: Stereo, 20 Watts RMS per channel at 4 ohms or 8 ohms.

###### 70V mode: mono, 40 Watts RMS per channel.

#### Audio Matrix Functions:

##### Any analog input, digital audio input, or HDBaseT audio input signal shall be routable to:

###### Amplified Speaker Output

###### Analog line level outputs

###### HDBaseT type output

###### HDMI output

#### Microphone Preamplifier

##### Microphone Inputs - Each internal microphone preamplifier input shall be connected to two mutually exclusive input connections:

###### Detachable terminal block - Balanced microphone level analog audio with switchable 48 volt DC phantom power.

###### Detachable terminal block - Balanced or unbalanced line level analog audio.

#### Audio Mixer

##### All stereo audio outputs shall be capable of outputting independent microphone and program audio mixes.

###### All stereo sources and microphone sources shall be available simultaneously.

###### All stereo sources and microphone sources shall have controllable levels in mixed output signal.

#### Audio DSP

##### Each analog audio output shall include DSP processing, allowing each output to be configured separately.  DSP parameters include:

###### real-time adjustable volume, bass, treble, and mute controls

###### 10-band graphic equalization

###### 4-band parametric equalization

###### Adjustable limiting

###### Up to 85 ms of speaker delay adjustment

### Single Cable Transmission System

#### HDBaseT type inputs and outputs allow for connecting to remote sources and display devices, and integrating with larger systems via a single CAT5 type cable.

#### The one-wire interface supports transmission of ultra high-definition video, audio, control, power, and networking signals over CAT type cable at distances up to 330 feet (100 meters).

#### Unit shall be HDBaseT Certified

##### The processor shall be designed using HDBaseT Alliance specifications, and shall support interoperability with other HDBaseT certified products.

### Control Processor

Unit shall include a built-in control processor with onboard control ports for control of external devices.

#### The Central Switching And Control Unit shall include an integrated microprocessor based control processor.

#### The built-in control processor shall support customizable control of integrated AV devices, room lighting hardware, window shades, and projection screens.

#### Controller shall include the following onboard control ports:

#### Ethernet

#### Four IR ports

#### two RS-232 COM ports

#### four relay ports

#### four digital input ports

#### RS-422 type network control bus

#### Control Subnet Networking

Unit shall include a built-in control subnet network port.

#### The controller shall support a network management system by the same manufacturer, supporting overall remote system controlling, monitoring, and managing through network computers and mobile devices.

#### The controller shall support touch screens, keypads, and wireless remotes and mobile device Apps from the same manufacturer.

### Multimedia Presentation Gateway

#### Multimedia Gateway allows the wireless presentation of content from a laptop, smartphone, or tablet device via a Wi-Fi wireless network or wired LAN connections.

#### Multimedia gateway functionality shall be a built-in feature.

## Amplifier

### Basis of Design Amplifier:

#### Crestron AMP-8075

### Amplifier Input/Output Requirements

#### Amplifier shall deliver a maximum 75 watts per channel, low or high impedance.

#### Amplifier shall have an input sensitivity of 1.29V, +4dBu balanced for the rated power.

#### Output Impedance:  Amplifier shall include audio controls to service low impedance and high impedance speaker configurations.

#### Amplifier shall be a single rack space unit.

### Audio Configuration

#### The amplifier object in the Amplifier software tool shall be configurable for instant audio delivery.

#### The amplifier object in the software tool shall provide:

##### Stereo grouping of adjacent channel pairs

##### Output level control

##### Bridging of adjacent channel pairs

##### Mute

##### Status indicators for Clip, DC Protect, Over Current & Thermal events

#### The Amplifier software tool presets shall recall any system configuration.

#### The Amplifier software tool views shall recall any system control screen configuration.

### Audio Input and Output

#### Amplifier shall provide eight balanced line level input connections.

#### and eight speaker level output connections

#### The amplifier shall provide low impedance output or 70V high impedance outputs for distributed speaker systems.

##### High impedance mode selection shall be available in the Amplifier software tool.

##### High impedance outputs shall be direct coupled.

##### A front panel indicator LED shall illuminate when in high impedance mode.

### Communication

#### Remote communications with the amplifier software tool and Touch Panels by same manufacturer will be via standard Ethernet.

#### The amplifier shall be configured, monitored and controlled by the Amplifier software tool.

#### The amplifier shall provide a local Universal Serial Bus (USB) console port connection on a standard USB-B type connector.

#### The amplifier shall support: 10/100/1000 Mbps, auto-switching, auto-negotiating, auto-discovery, full/half duplex, TCP/IP, UDP/IP, CIP, DHCP, SSL, SSH, SFTP (SSH File Transfer Protocol)

#### The amplifier software tool shall allow multiple amplifier instances to be active on a common network.

### User Interface Export

#### Graphic control elements including digital attenuators and VU meters shall be user selectable and exportable directly from the amplifier software tool to a touch screen user interface by the same manufacturer.

#### The User Interface Export file shall enable the building of touch screens by the same manufacturer with a drag and drop process.

## Audio DSP

### Basis of Design Audio DSP:

#### Crestron DSP-1283

### User Interface Export

#### The User Interface Export file shall enable the building of touch panels with a drag and drop process.

#### Touch Panels by same manufacturer shall integrate natively with the DSP for a rich graphic user interface.

#### Graphic control elements will be selectable and exportable directly from the DSP software tool.

### DSP Functions

#### Acoustic Echo Cancellation on all 12 input channels

#### Dante audio connectivity

#### Front panel 5 segment LED indicators referencing:

##### Line Out

##### Microphone and line in

#### Separate POTS connector RJ11

#### Separate Primary and Secondary Dante network ports, RJ45

#### Separate VoIP connector RJ45

#### USB 2.0 audio 8x8 with type B connector

### Audio I/O

#### Each individual channel shall have its own dedicated connection.  Analog to Digital and Digital to Analog Conversion

#### Input connections shall be selectable for balanced line level analog audio signals on Phoenix style removable connectors.

#### Shall provide 12 balanced input connections with selectable phantom power for condenser microphones.

#### Shall provide 8 balanced output connections.

### Audio Configuration

#### Channel Strips shall allow for the creation of unique configurations that are repeatable and portable.

#### The Digital Signal Processor shall provide:

##### Any-to-Any Matrix Routing

##### Gain controls (Input/Output)

##### Matrix In/Out level trim

##### Matrix crosspoint level

##### Spectrum analyzer

#### Presets shall recall any system configuration

#### The channel strip software shall be configurable for instant audio delivery without requiring configuration code compile.

#### Views shall recall any system control screen configuration.

### Filters shall include

#### Compressors

#### Crossovers

#### Delays

#### Equalization

#### Gates

#### Gating style Auto Mixer

#### Hi Pass filters

#### Limiters

#### Lo and Hi Shelf

#### Low Pass filters

### Auxiliary channel strips

#### An additional 8 Auxiliary channel strips with comprehensive signal processing shall be provided.

##### All Auxiliary Strips shall be routable to any Matrix crosspoint or output Bus.

## User Interfaces

### Touch Screen

#### Basis of Design Touch Screen User Interface:

##### Crestron TSW-1060

#### Functions

##### Provide custom user interface with controls for AV systems, integrated hardware, and integrated scheduling and management system.

#### Communication:

##### Bidirectional 10/100 Mbps Ethernet communication.

#### Physical

##### 10.1 inch TFT active-matrix color LCD touch screen 1280 by 800 WXGA resolution display.

##### Power:  IEEE 802.3af Class 3 PoE Powered Device

## Wall mounted Hallway Room Availability Sign

### Basis of Design Room Availability SIgn:

#### Crestron SSW

### Functional Requirements

#### The sign shall illuminate green when the room is available and red when the room is occupied.

#### Custom RGB Color Mixing - The sign shall support custom colors through control system programming.

#### The sign shall support custom color alternating functions, color fade in and out functions, or blink on and off functions through custom system programming.

#### For room scheduling applications, the sign will connect to a compatible system touch screen via USB. No programming or configuration is required. The sign color shall be controlled by the Room Scheduling system by the same manufacturer.

### Physical

#### Sign shall mount to the wall outside a meeting room to indicate room status.

#### The sign shall include tri-color RGB LEDs to allow for a full spectrum of colors by blending the red, green, and blue elements in 1% increments per color.

#### Custom Engraving Option

##### The sign shall be available with or without custom engraving. The engraving shall appear on both sides of the acrylic sign face to identify the room by name or number. When illuminated, the engraving shall be visible at a distance of 50 feet or 15 meters.

## Network Occupancy Sensor:

### Basis of Design Occupancy Sensor:

#### Crestron GLS-ODT-C-CN

### Functional

#### Combination of ultrasonic motion detection and passive infrared detection with internal microprocessor.

#### Coverage:  360 deg., 2000 sq. ft.

#### Delayed time off adjustment.

#### Detects movement within space while reducing false triggering or shutoffs while space is occupied.

#### Sensor shall be independently adjustable for installed conditions.

#### Walk-through mode.

### Physical

#### Additional Interfacing: includes connection port for additional photocell.

#### Equipped with 4-wire interface for direct connection to control bus

#### Mounting:  3” octagon box

#### Photocell: Built-in ambient light photocell

#### Set-up and commissioning: parameters shall be configurable via a handheld wireless remote.

## Speakers

### Basis of Design Speaker:

#### Crestron SAROS IC6T

### Ceiling mounted 2-way speaker with the following specifications:

#### Performance:

##### Sensitivity: 88.5 dB at one meter, with 1-W input

##### Frequency Response: Within plus or minus 3 dB from 50 to 20,000 Hz

##### Frequency Range: Within minus 10 dB from 40 to 20,000Hz

##### Nominal Dispersion Angle: 100 degrees conical

##### Power Handling: 125 Watts Program at 8-Ohms.

##### Speaker unit shall be equipped for Low Impedance 8-Ohm or High Impedance 70V or 100V operation.

###### Accessible front mounted low impedance, high impedance selector switch.

##### High Impedance Line Transformer level taps at 70V and 100V:

###### 3.75 Watts; 70 Watt only

###### 7.5 Watts

###### 15 Watts

###### 30 Watts

###### 60 Watts

#### Enclosures: Aluminum, plenum rated.

#### Termination:

##### 4-pin screw down conductor terminal removable terminal block with speaker input and parallel pass-through connections.

##### Conduit knockouts:

###### One 0.5-inch knockout

###### One 0.75-inch knockout

#### Grille:

##### Steel

##### Frameless magnetically affixed with safety tether.

##### Finish: Re-paintable, powder coated.

#### Drivers:

##### 6.5-inch polypropylene woofer with steel basket

##### 0	.98-inch titanium dome tweeter

Specifier: Crestron Saros speakers utilize a tweeter protection method that does not re-quire replacement of internal components. In the event of the tweeter being “over-powered” the protection device will limit power to the tweeter as long as the condition exists. Many competitor speakers utilize a fuse or fuse-bulb type component, which is capable of absorbing some over-limit power, but will eventually rupture, requiring the entire unit to be removed and disassemble for replacement of the fuse/fuse-bulb.

##### Crossover Frequency: 2,500 Hz.

## Surface Mounted Connection Interface

### Basis of Design Connection Interface

#### Crestron FT-600

### The Connection interface shall be available in black anodized and brushed aluminum finishes.

### The Connection interface shall provide configurable connectivity in a flush mount tabletop unit.

### The Connection interface shall support custom combinations of pullout cables, cable retractors, connector panels, and AC power outlets.

### The Connection interface shall utilize a universal cutout size that will fit other interfaces from same manufacturer.

## Display Device

### Display device shall be controllable via a built-in RS-232 connection port.

### Display device shall support 4K video through built-in HDMI input connector(s).

## Audio Source Devices

### Microphones

#### Dante Microphone

## Cable Tuner

### HDMI AV source device shall be controllable via IR control protocol.

## Computer

### Computer or Laptop shall include an HDMI video output connection port.

## Graphics Engine HDBaseT Receiver

### Basis of Design Graphics Engine HDBaseT receiver

#### Crestron DM-DGE-200-C

### Functions

#### The Digital Graphics Engine adds touch screen controller capability to HD or Ultra HD touch screen displays.

#### The Digital Graphics Engine with related software by same manufacturer supports the creation of a custom graphical user environment capable of combining controls of integrated equipment and systems with dual-window video display.

#### The Digital Graphics Engine shall include an HDBaseT input for support of 4K UHD and DCI 4K sources connected at an HDBaseT transmitter or switcher.

#### The Digital Graphics Engine shall support H.264 streaming video signals received over a high-speed Ethernet connection.

#### The Digital Graphics Engine shall include an Ethernet port to interface with a control system by the same manufacturer.

#### The Digital Graphics Engine shall include built-in RS-232 and IR control ports to allow for programmable control of the connected display and other devices via the control system.

### Performance

#### The receiver shall meet the following minimum requirements:

##### HDMI digital video, audio, and control output:

###### One (1) 19-pin Type A HDMI female connector

##### HDMI output supports HDCP 2.2

##### Streaming Decoder Video: 	H.264 (MPEG-4 part 10 AVC), MJPEG

##### Supports HDCP.

##### HDMI audio Support:  Up to 8 channel LPCM.

##### CEC device control.

## HDBaseT Transmitter

### Basis of Design HDBaseT Transmitter:

#### Crestron DM-TX-4K-302-C

### The transmitter shall meet the following minimum requirements:

#### Ethernet Port

##### One (1) 8-wire RJ-45

###### 10/100 Mbps, auto-switching, auto negotiating, auto-discovery, full/half duplex, DHCP

#### HDMI digital video/audio output

##### One (1) 19-pin Type A HDMI female connector

#### One (1) analog stereo audio input:

##### (1) 3.5mm TRS (L/R unbalanced)

#### One (1) DB15 input:

##### Component (YPbPr)

##### Composite Video

##### RGB

##### S-Video (Y/C)

#### One (1) USB HID port.

##### Supports USB HID class devices

#### Power supply modes:

##### Local or remote DC power source.

##### Remote power supplied by matrix switcher through UTP/STP transmission cable.

#### Single UTP/STP cable transmission connection

##### Signal transmission up to 330 feet.

##### Supports CAT5e.

##### Supports HDBaseT signal specifications.

##### Supports remote power injection through matrix switcher.

#### Two (2) HDMI video, audio, and control input:

##### CEC device control

##### Supports 4K HDMI

##### Supports DisplayPort Dual-Mode

##### Supports Dolby Digital, Dolby Digital EX, Dolby Digital Plus, Dolby TrueHD, Dolby Atmos, DTS, DTS-ES, DTS 96/24, DTS-HD High Res, DTS-HD Master Audio, LPCM up to 8 channels

##### Supports DVI-D with adaptor

##### Supports HDCP 2.2

## PTZ Camera

### PTZ video camera shall be compatible with input connection on VTC codec unit.

## VTC Codec

### VTC Codec shall support connections as shown on system drawings.

## Power Conditioner

### Basis of Design Power Conditioner:

#### Crestron PC-300

### The Power Conditioner shall meet the following minimum requirements:

#### Features an LCD front control panel, network connectivity, web browser setup, event logging, ping monitoring, temperature sensing, and UL 1449 compliance.

#### Provides comprehensive energy monitoring and switching of individual outlets.

#### Rack-mountable 15A, 120V AC power controller, conditioner, and surge protector

# EXECUTION

NOT USED in this Guide Specification. Specifier shall Specify PART 3 On-Site work as needed.

# APPENDICES

## SPECIFIED PRODUCTS

Specifier Note: This Article includes Crestron products specified in this Guide Specification document. This Article is for reference only and should not be required in actual project manual unless included in an overall system equipment list.

### Crestron DMPS3-4K-350-C

### Crestron TSW-1060

### Crestron SSW

### Crestron DM-TX-4K-302-C

### Crestron DM-DGE-200-C

### Crestron DSP-1283

### Crestron AMP-8075

### Crestron SAROS IC6T

### Crestron FT-600

### Crestron PC-300

### Crestron GLS-ODT-C-CN