SECTION 27 41 16

INTEGRATED AUDIO-VIDEO SYSTEMS AND EQUIPMENT

Equipment Specified in this section:

DMPS3-4K-100-C

Table of Contents

SECTION 27 41 16	4

PART 1	GENERAL	4

1.1	SECTION INCLUDES	4

A.	Presentation System –	4

B.	Integrated Controller –	4

C.	Host Processor –	4

D.	Device Controller –	4

1.2	RELATED REQUIREMENTS	4

A.	Section 25 08 00	4

B.	Section 25 10 00	4

C.	Section 25 11 13	5

D.	Section 25 13 13	5

E.	Section 25 13 16	5

F.	Section 25 13 19	5

G.	Section 25 15 16	5

H.	Section 26 09 43.13	5

I.	Section 27 15 00	5

J.	Section 27 41 00	5

1.3	REFERENCES	5

A.	Abbreviations and Acronyms	5

B.	Definitions	5

C.	Reference Standards	6

PART 2	PRODUCTS	7

2.1	MANUFACTURERS	7

A.	Basis-of-Design Manufacturer:	7

2.2	DIGITAL MEDIA PRESENTATION SYSTEM	7

A.	Basis of Design Product:	7

B.	System Architecture	7

C.	Front Panel Control Interface	7

D.	Out of box functionality	8

2.3	AUDIO MATRIX SWITCHER	8

A.	Switcher Signal Routing	8

2.4	MICROPHONE PREAMPLIFIER	8

A.	Microphone Input	8

2.5	MICROPHONE AND PROGRAM AUDIO MIXER	8

A.	Independent Mixes	8

2.6	VIDEO SWITCHER	9

A.	Auto detecting	9

B.	Switcher Signal Routing	9

2.7	ANALOG TO DIGITAL VIDEO TRANSCODER	9

A.	Resolution	9

2.8	SINGLE CABLE SIGNAL TRANSMISSION	9

A.	Transmitter	9

B.	Supported signal and data types:	9

C.	Cable and Connectors	10

D.	Maximum Cable Lengths	10

E.	Device Power	10

2.9	SYSTEM CONTROL PROCESSOR	10

A.	Control Processor	10

B.	Minimum Characteristics:	10

C.	External Ports	11

2.10	SYSTEM FUNCTIONS AND SEQUENCES	11

A.	Audio-Video Control Functions	11

B.	Lighting and Shade Control Functions	12

2.11	USER INTERFACE CONTROL FUNCTIONS	12

A.	As Specified in Division 25.	12

2.12	USER INTERFACE CONTROLLERS	12

A.	As Specified in Division 25.	12

PART 3	EXECUTION	13

3.1	INSTALLERS / TECHNICIANS / ENGINEERS	13

A.	Digital Media System Contractors/Sub Contractors	13

3.2	SYSTEM TESTING AND COMMISSIONING	13

A.	Testing	13

B.	Commissioning	13

END OF SECTION 27 41 16	14

# SECTION 27 41 16

INTEGRATED AUDIO-VIDEO SYSTEMS AND EQUIPMENT

Specifier: The Specifier/Design Professional is responsible for the accuracy of all project specifications, including system application and coordination with related sections.  This guide specification is provided as a convenience and requires editing to match actual project requirements.  CRESTRON ELECTRONICS, INC. SHALL NOT BE LIABLE FOR ANY DAMAGES ARISING OUT OF THE USE OF ANY OF ITS GUIDE SPECIFICATIONS.  For Crestron design assistance and design review please contact Sales Support Services Department at   800.237.2041 or techsales@crestron.com.

GENERAL

SECTION INCLUDES

Presentation System –

A single unit integrated digital media presentation system.  The single central switching and control unit integrates audio switching, 4K video switching, audio mixing and amplification, and complete system control.

SPECIFIER: Retain the following article if the AV controller specified in this section is controlling:

- A building lighting system (lighting system may be a stand-alone system connected via network interface).

- Lighting equipment connected to other systems (any lighting equipment whose primary controller is not the controller specified in this section but is connected to it via some type of control interface).

- Lighting equipment specified in other sections.

- Existing Lighting equipment.

Integrated Controller –

Advanced control functions and sequences for integrated lighting [systems] [equipment].

SPECIFIER:  Retain the following article if the AV controller specified in this section is connected to or integrated with user interfaces (keypads, touchpanels, Browser GUIs, etc.) specified in other sections.

Host Processor –

Host control processor for user control interfaces.

SPECIFIER:  Retain the following article if the AV controller specified in this section is connected to or integrated with a system scheduling and management server such as Crestron FUSION, where the controller in this section provides interfacing and control of equipment being scheduled and managed.

Device Controller –

Device and equipment controller for scheduling and management servers.

RELATED REQUIREMENTS

Section 25 08 00

Commissioning of Integrated Automation

Section 25 10 00

Integrated Automation Network Equipment

Section 25 11 13

Integrated Automation Network Servers

Section 25 13 13

Integrated Automation Control and Monitoring Network Supervisory Control

Section 25 13 16

Integrated Automation Control and Monitoring Network Integration Panels

Section 25 13 19

Integrated Automation Control and Monitoring Network Interoperability

Section 25 15 16

Integrated Automation Software for Control and Monitoring Networks

Section 26 09 43.13

Digital-Network Lighting Controls

Section 27 15 00

Communications Horizontal Cabling

Section 27 41 00

Audio-Video Systems

REFERENCES

Abbreviations and Acronyms

CEC:  Consumer Electronics Control

EDID:  Extended display identification data

HDCP:  High-bandwidth Digital Content Protection

HDMI:  High-Definition Multimedia Interface

IDF: Intermediate Distribution Frame

KSV: Key Selection Vector

MDF: Main Distribution Frame

NVP: Nominal Velocity of Propagation

Definitions

Channel: The end-to-end transmission path between two points including all patch or extension cords.  The path at which application-specific equipment is connected.

Permanent Link: The installed twisted pair cable link excluding test and patch cords.

Permalink: see Permanent Link.

Nominal Velocity of Propagation: (NVP) expresses the speed of the electrical signals along the cabling link in relation to the speed of light in vacuum (3x108 m/second). Insulation characteristics and twist rate of the wire pair influence NVP in minor ways.

Reference Standards

ANSI/TIA-568-C  “Requirements for Field Test Instruments and Measurements for Balanced Twisted-Pair Cabling”

ANSI/TIA-568-C.2 “Commercial Balanced Twisted-Pair Telecommunications Cabling and Components Standard”.

ANSI/TIA-1152  “Generic Telecommunications Cabling for Customer Premises”

PRODUCTS

MANUFACTURERS

Basis-of-Design Manufacturer:

Subject to compliance with requirements, provide products of Crestron Electronics, Inc., Rockleigh, NJ 07647, Phone 800-237-2041, Fax: 201-767-1903, www.crestron.com [or comparable products from a single manufacturer approved by A/E prior to bidding], with the following components and characteristics.

DIGITAL MEDIA PRESENTATION SYSTEM

Basis of Design Product:

Crestron, DMPS3-4K-100-C 3-Series, 4K Digital Media Presentation System.

System Architecture

The Digital Media Presentation system shall be a single central switching and control unit integrating the following:

Audio matrix switching

Microphone pre-amplification

Microphone and program audio mixing

Front panel user interface

4K Scaling (both up to 4K and down from 4K)

Analog to digital video transcoding

4K multi-format video switching

Single Cable HDBaseT compatible signal transmission

System control processing

Built-in power supply

The Digital Media Presentation system shall integrate with the following:

External USB one-touch table top user interfaces

External touch screen user interfaces

External keypad user interfaces

Browser based user interfaces

Mobile iOS and Android apps

External Single Cable HDBaseT compatible signal transmission transmitter and receiver units

AV source devices

HDMI Video display devices

System devices under control

Front Panel Control Interface

The front panel user interface shall control the following system functions:

Volume control

Source select

Out of box functionality

External USB one touch table top user interfaces by same manufacturer, providing source selection

External Ethernet touch screen user interface by same manufacturer

Reporting to enterprise management platform by same manufacturer

AUDIO MATRIX SWITCHER

Switcher Signal Routing

Any stereo input signal shall be routable to:

Stereo analog output

Digital outputs

HDMI output

External Single Cable HDBaseT compatible signal transmission receiver units

The dual internal stereo switchers shall have multiple stereo inputs.

Each internal stereo switcher input shall be internally connected to one of three external input connection types.

Type 1

19-pin Type A HDMI female connector with stereo audio

Type 2

3.5mm TRS mini phone jack with stereo line level analog audio

MICROPHONE PREAMPLIFIER

Microphone Input

The internal microphone preamplifier input shall be connected to a detachable terminal block supporting balanced microphone level analog audio with switchable 48 volt DC phantom power.

MICROPHONE AND PROGRAM AUDIO MIXER

Independent Mixes

The analog audio output and the digital audio outputs shall be capable of outputting independent audio mixes.

All stereo sources and the microphone source shall be available simultaneously.

All stereo sources and microphone source shall have controllable levels in each of the two output mixes.

VIDEO SWITCHER

Auto detecting

Video switcher shall support auto signal detecting.

Switcher Signal Routing

The internal video switcher shall have multiple inputs:

Four analog VGA inputs

Four 4K HDMI inputs

All video input signals shall be routable to video outputs.

Each internal switcher input shall be internally connected to one of four external input connection types.

Type 1

DB15HD female connector.

Type 2

19-pin Type A HDMI female connector.

ANALOG TO DIGITAL VIDEO TRANSCODER

Resolution

Convert all analog video to digital video at source resolution

Analog to digital conversion: 10-bit 165 MHz

SINGLE CABLE SIGNAL TRANSMISSION

Transmitter

Integrated transmitter connections for single cable signal transmission.

Single cable signal transmission shall be HDBaseT compatible

Single cable signal transmission shall support the following communication standards between presentation system and transmitters and receivers by same manufacturer:

Ethernet

Serial RS-232

Infrared control

Supported signal and data types:

Uncompressed video and audio

HDMI with HDCP

100Mbps Ethernet

USB HID

Bidirectional device control signals

Cable and Connectors

Cat5e or Cat6 LAN cable.

RJ-45 modular connectors.

Maximum Cable Lengths

Cable Type 1 – Manufacturer Ultra type

330 feet (100 meters)

Cable Type 2 – Manufacturer 8G type

330 feet (100 meters) for 2K resolution

230 feet (70 meters) for 4K resolution

Cable Type 3 – CAT5e

330 feet (100 meters) for 2K resolution

165 feet (50 meters) for 4K resolution

Device Power

The single cable signal transmission technology shall be capable of providing power for compatible remote transmitter and receiver devices.

SYSTEM CONTROL PROCESSOR

Control Processor

The Central Switching And Control Unit shall include an integrated microprocessor based control processor.

Minimum Characteristics:

Utilize a real time, preemptive, multi-threaded/multi-tasking kernel; Transaction-Safe Extended FAT file system; supports up to 10 simultaneously running programs. Processor shall communicate directly with Ethernet, control ports and proprietary control network utilizing high-speed, parallel bus infrastructure.

Control processor shall utilize Transaction-Safe Extended FAT file system.

Control System shall support the following Ethernet features:

10/100/1000 Mbps

Auto-switching

Auto-negotiating

Auto-discovery

Full/half duplex

Industry-standard TCP/IP stack

UDP/IP

CIP

DHCP

Private Network Mode

SSL

IEEE 802.1X

SNMP

BACnet/IP

IPv4 or IPv6

Active Directory authentication

RSTP

SMTP e-mail client

IIS v.6.0 Web Server

Installer setup via manufacturer software or MSIE

External Ports

The control system shall be equipped with the following external connection ports:

Infrared/Serial

One-way device control

IR output up to 1.2 MHz

One-way serial TTL/RS-232 (0-5 Volts) up to 115.2k baud

Infrared Input

Built-in front panel IR receiver

Supports RC-5 IR commands via external IR transmitters

Serial Communication Port

One 5-pin 3.5mm detachable terminal block

bidirectional RS-232 ports up to 115.2k baud, hardware and software handshaking support for communication with serial devices.

Ethernet

One 8-wire RJ45

10/100/1000BaseT Ethernet port

Communication Network

One 4-pin 3.5mm detachable terminal blocks

Master net communications ports

24W available 24VDC device power

SYSTEM FUNCTIONS AND SEQUENCES

Audio-Video Control Functions

Specifier:

Retain the following article if Division 26 Lighting System integration is required in the project.

All Room/area sensor feedback (occupancy, daylight, etc.) provided by the lighting system control processors via the Crestron Remote System Definition (.rsd) file.

Room occupancy status shall be based on sensor data provided by Lighting system as specified in Section 26 09 43.13

Specifier: AV control system programming functionality is project specific, add required functionality here.

Room modes: TBD

System control functions: TBD

Lighting and Shade Control Functions

Specifier:

Retain the following articles if Division 26 Lighting System integration is required in the project.

All lighting and shade systems are to be controlled by the lighting system(s) specified in Section 26 09 43.13.  This system (AV Control system) will have complete control of the lighting and shades system via the Remote System Definition (.rsd) file provided by the lighting system contractor/installer.  Coordination with lighting system contractor is required in order to integrate required functionality into the .rsd intersystem communication file.

Access to full control capability of integrated lighting and shade systems shall be provided by: Custom Software Control Interface as specified in Section 26 09 43.13.

Basic System Control Functions - Lighting system functions as defined by Section 26 09 43.13 shall be accessible by the AV Automation Control Processor.  All stored information shall be maintained by the lighting system control processors and only accessed and edited by the AV Automation Control Processors.

Advanced System Control Functions – Lighting system functions as defined by Section 26 09 43.13 shall be accessible by the AV Automation Control Processor.  All stored information shall be maintained by the lighting system control processors and only accessed and edited by the AV Automation Control Processors.

USER INTERFACE CONTROL FUNCTIONS

As Specified in Division 25.

USER INTERFACE CONTROLLERS

As Specified in Division 25.

EXECUTION

INSTALLERS / TECHNICIANS / ENGINEERS

Digital Media System Contractors/Sub Contractors

Trained technicians who have successfully attended an appropriate training program and have obtained a certificate as proof thereof shall execute the tests specified in this section.

SPECIFIER: Insert additional certifications/credentials as required based on project needs.

Training Certifications:

SPECIFIER: Crestron offers three DigitalMedia training courses/certificaitons.

DigitalMedia Certified Designer (DMC-D-4K) - A DM Certified Designer understands the fundamental differences between analog and digital systems and the unique design considerations needed to ensure reliable operation.

DigitalMedia Certified Technician (DMC-T-4K) - A DM Certified Technician can perform all of the copper and fiber termination options available for DM systems, perform cable plant certification and install and test the performance capabilities of a Crestron DigitalMedia system.

DigitalMedia Certified Engineer (DMC-E-4K) – The DMC-E-4K Certified Engineer program includes the DMC-D-4K certification and DMC-T-4K Certification. A DM Certified Engineer can perform DMC-D-4K and DMC-T-4K roles as well as commissioning, system setup, diagnostics, testing and reporting. Only a DMC-E-4K is equipped to fully execute and support a DM project.

Technicians performing Cable Installation and termination shall hold a Crestron DigitalMedia Certified Technician (DMC-T-4K) certification.

Engineers Commissioning the digital media system shall hold a Crestron DigitalMedia Certified Engineer (DMC-E-4K) certification.

SYSTEM TESTING AND COMMISSIONING

Testing

A DigitalMedia Certified Engineer (DMC-E-4K) shall perform the contractor verification tests.

Contractor shall verify that all components of the system are installed according to manufacturers specifications and are compliant with Division 27 specifications.

Commissioning

A DigitalMedia Certified Engineer (DMC-E-4K) shall perform acceptance testing and commissioning.

SPECIFIER: Retain the following for DigitalMedia modular i/o matrix switchers.

The contractor shall provide a copy of the system commissioning Test Report in electronic format.

All reported information shall be generated by the digital media matrix unit and the configuration software.

Commissioning engineer shall run all available tests and include all installed system components.

Commissioning Test Report shall include the following:

Tests Failures and Notices

Sink Device EDID Test – Open items or failures shall not be accepted.

Cable Length Test - Open items or failures shall not be accepted.

HDCP KSV Limitations – Limitations shall not be accepted.

Cable Limitations – Limitations shall not be accepted.

EDID Limitations – Limitations shall not be accepted.

Cable Length Limits exceeded – Failing cables shall not be accepted.

Device Model Number, Serial Number, and Firmware Version for main chassis and each input and output card.

Device Model Number, Serial Number, and Firmware Version for connected transmitter and receiver devices.

EDID – Input Resolution and 3D support status for each input.

EDID – Supported Output Resolution and 3D support status for devices connected to each output.

EDID – Supported Audio formats for each input.

EDID – Supported Audio formats for devices connected to each output.

END OF SECTION 27 41 16