SECTION 27 41 16

INTEGRATED AUDIO-VIDEO SYSTEMS AND EQUIPMENT

GUIDE SPECIFICATION

Specifier: The Specifier/Design Professional is responsible for the accuracy of all project specifications, including system application and coordination with related sections.  This guide specification is provided as a convenience and requires editing to match actual project requirements.  CRESTRON ELECTRONICS, INC. SHALL NOT BE LIABLE FOR ANY DAMAGES ARISING OUT OF THE USE OF ANY OF ITS GUIDE SPECIFICATIONS.  For Crestron design assistance and design review please contact Sales Support Services Department at   800.237.2041 or techsales@crestron.com.

Specifer:  Please see PART 4 for a listing of products specified in this Guide Specification.

Table of Contents

1	GENERAL	4

1.1	SYSTEM DESCRIPTION	4

1.1.1	Present 4-12 People	4

1.2	RELATED REQUIREMENTS	4

1.2.1	Section 25 08 00 - Commissioning of Integrated Automation	4

1.2.2	Section 25 10 00 - Integrated Automation Network Equipment	5

1.2.3	Section 25 11 13 - Integrated Automation Network Servers	5

1.2.4	Section 25 13 13 - Integrated Automation Control and Monitoring Network Supervisory Control	5

1.2.5	Section 25 13 16 - Integrated Automation Control and Monitoring Network Integration Panels	5

1.2.6	Section 25 13 19 - Integrated Automation Control and Monitoring Network Interoperability	5

1.2.7	Section 25 15 16 - Integrated Automation Software for Control and Monitoring Networks	5

1.2.8	Section 26 09 43.13 - Digital-Network Lighting Controls	5

1.2.9	Section 27 15 00 - Communications Horizontal Cabling	5

1.2.10	Section 27 41 00 - Audio-Video Systems	5

1.3	REFERENCES	5

1.3.1	Abbreviations and Acronyms	5

2	PRODUCTS	5

2.1	AV Multi-Function Presentation Processor	5

2.1.1	Basis-of-Design Manufacturer:	5

2.1.2	Basis of Design Product:	5

2.1.3	System Architecture - The Presentation Processor unit shall be a single unit composed of internally integrated components providing multiple functions.  Included sub-components:	6

2.1.4	AV Multi-Function Presentation System - A single unit integrated digital media presentation system.  The single central switching and control unit integrates audio-video switching, audio mixing and amplification, and system control.	6

2.1.5	Video System	6

2.1.6	Audio System	6

2.1.7	Single Cable Transmission System	7

2.1.8	Control Processor	7

2.2	User Interfaces	8

2.2.1	Touch Screen	8

2.3	Wall mounted Hallway Room Availability Sign	8

2.3.1	Basis of Design Room Availability SIgn:	8

2.3.2	Functional Requirements	8

2.3.3	Physical	9

2.4	Network Occupancy Sensor:	9

2.4.1	Basis of Design Occupancy Sensor:	9

2.4.2	Functional	9

2.4.3	Physical	9

2.5	Speaker System	9

2.5.1	Basis of Design Speaker:	9

2.5.2	Amplified Sound Bar Speaker with the following specifications:	10

2.6	Surface Mounted Connection Interface	10

2.6.1	Basis of Design Connection Interface	10

2.6.2	The Connection interface shall be available in black a finish.	10

2.6.3	The Connection interface shall provide configurable connectivity in a flush mount tabletop unit.	10

2.6.4	The Connection interface shall support custom combinations of cables, and an AC power outlet.	10

2.7	Display Device	10

2.7.1	Display device shall be controllable via a built-in RS-232 connection port.	10

2.7.2	Display device shall support 4K video through built-in HDMI input connector(s).	10

2.8	Cable Tuner	10

2.8.1	HDMI AV source device shall be controllable via IR control protocol.	10

2.9	Computer	10

2.9.1	Computer or Laptop shall include an HDMI video output connection port.	10

2.10	Graphics Engine HDBaseT Receiver	10

2.10.1	Basis of Design Graphics Engine HDBaseT receiver	11

2.10.2	Functions	11

2.10.3	Performance	11

2.11	Power Conditioner	12

2.11.1	Basis of Design Power Conditioner:	12

2.11.2	The Power Conditioner shall meet the following minimum requirements:	12

3	EXECUTION	12

4	APPENDICES	12

4.1	SPECIFIED PRODUCTS	12

4.1.1	Crestron DMPS3-100-C	12

4.1.2	Crestron TSW-1060	12

4.1.3	Crestron SSW	12

4.1.4	Crestron CCS-FF-2	12

4.1.5	Crestron TSW-760	12

4.1.6	Crestron SAROS SB-200-P	12

4.1.7	Crestron TT-101	12

4.1.8	Crestron PC-100	12

4.1.9	Crestron GLS-ODT-C-CN	12

# GENERAL

## SYSTEM DESCRIPTION

### Present 4-12 People

#### See AV System Drawings for device quantities and connection details.

Specifier Note: See example AV System Drawing for Present 4-12 People

#### Display System

##### Video Display Devices - Room shall include two video displays.  Each with the capability of viewing any in room video source or VTC codec video source.

#### AV Sources

##### Computer (OFE)

##### IPTV Player/Decoder

##### Laptop Computer

##### Wireless Presentation Device

#### Audio System

##### Audio processing shall be provided by AV switcher and speakers and amplification shall be through an amplified speaker bar.

#### AV Systems Control

##### Control User Interface - Room shall include a table top touch screen user interface.

##### Device Controller - Device and equipment controller for scheduling and management servers.

SPECIFIER: Retain this article if the AV controller specified in this section is connected to or integrated with a system scheduling and management server such as Crestron FUSION, where the controller in this section provides interfacing and control of equipment being scheduled and managed.

##### Room System Control - Room system and scheduling through touch screen user interface(s) with on-screen information display viewable on room display(s).

Specifier: Retain this article if room system includes Fusion integration.

##### Wall Mounted Control User Interface - Room shall include a wall mounted scheduling and control user interface with an integrated room occupancy indicator sign.

##### Table Top Control User Interface - Room shall include a table top control user interface.

## RELATED REQUIREMENTS

### Section 25 08 00 - Commissioning of Integrated Automation

### Section 25 10 00 - Integrated Automation Network Equipment

### Section 25 11 13 - Integrated Automation Network Servers

### Section 25 13 13 - Integrated Automation Control and Monitoring Network Supervisory Control

### Section 25 13 16 - Integrated Automation Control and Monitoring Network Integration Panels

### Section 25 13 19 - Integrated Automation Control and Monitoring Network Interoperability

### Section 25 15 16 - Integrated Automation Software for Control and Monitoring Networks

### Section 26 09 43.13 - Digital-Network Lighting Controls

### Section 27 15 00 - Communications Horizontal Cabling

### Section 27 41 00 - Audio-Video Systems

## REFERENCES

### Abbreviations and Acronyms

#### CEC:  Consumer Electronics Control

#### EDID:  Extended display identification data

#### HDCP:  High-bandwidth Digital Content Protection

#### HDMI:  High-Definition Multimedia Interface

#### IDF: Intermediate Distribution Frame

#### KSV: Key Selection Vector

#### MDF: Main Distribution Frame

#### NVP: Nominal Velocity of Propagation

# PRODUCTS

## AV Multi-Function Presentation Processor

### Basis-of-Design Manufacturer:

Subject to compliance with requirements, provide products of Crestron Electronics, Inc., Rockleigh, NJ 07647, Phone 800-237-2041, Fax: 201-767 1903, www.crestron.com

### Basis of Design Product:

#### Crestron DMPS3-4K-100-C

SPECIFIER: See the following links for product selection: https://www.crestron.com/products/model/DMPS3-4K-200-C https://www.crestron.com/products/model/DMPS3-4K-300-C

### System Architecture - The Presentation Processor unit shall be a single unit composed of internally integrated components providing multiple functions.  Included sub-components:

#### Front Panel Control Interface shall provide basic control of the Presentation Processor.

#### Audio Matrix/Mixer

#### Video Matrix

#### Single Cable Transmission Matrix

Single cable 4K HDMI transmission transmitter port with integrated switching matrix.

#### Control Processor

### AV Multi-Function Presentation System - A single unit integrated digital media presentation system.  The single central switching and control unit integrates audio-video switching, audio mixing and amplification, and system control.

### Video System

#### Video Inputs

##### Video inputs include HDMI and VGA type inputs.

SPECIFIER: the DMPS3-4K-100 includes INPUTS: (4) HDMI, (0) DM; OUTPUTS: (1) HDMI, (1) DM

#### 4K Video Switcher

##### Built-in video matrix switching allows video sources to be routed simultaneously to output connectors.

##### Internal switcher shall support routing of HDMI and other AV sources to HDMI and HDBaseT outputs.

SPECIFIER: the DMPS3-4K-100-C includes (1) HDMI and (1) DM output

##### The HDMI output are compatible with DVI and the HDBaseT output are compatible with HDBaseT and manufacturer proprietary format supporting additional control functionality.

##### 4K/60 Video Scaling - Unit shall include an independent, 4K scaler for the outputs.

### Audio System

#### Audio Inputs - Each HDMI and analog audio input includes adjustable input compensation to accommodate a range of signals and maintain consistent volume levels when switching between sources.

#### Audio Matrix Functions:

##### Any analog input or digital audio input signal shall be routable to:

###### Analog line level output

###### HDMI output and HDBaseT type output

#### Microphone Preamplifier

##### Detachable terminal block - Balanced microphone level analog audio with switchable 48 volt DC phantom power.

#### Audio Mixer

##### Stereo audio output shall be capable of outputting microphone and program audio mixes.

###### All stereo sources and microphone source shall be available simultaneously.

###### All stereo sources and microphone source shall have controllable levels in mixed output signal.

### Single Cable Transmission System

#### HDBaseT type output allows for connecting to remote sources and display devices, and integrating with larger systems via a single CAT5 type cable.

#### The one-wire interface supports transmission of ultra high-definition video, audio, control, power, and networking signals over CAT type cable at distances up to 330 feet (100 meters).

#### Unit shall be HDBaseT Certified

##### The processor shall be designed using HDBaseT Alliance specifications, and shall support interoperability with other HDBaseT certified products.

### Control Processor

Unit shall include a built-in control processor with onboard control ports for control of external devices.

#### The Central Switching And Control Unit shall include an integrated microprocessor based control processor.

#### The built-in control processor shall support customizable control of integrated AV devices, room lighting hardware, window shades, and projection screens.

#### Controller shall include the following onboard control ports:

#### Ethernet

#### One IR port

#### One RS-232 COM port

#### RS-422 type network control bus

#### The controller shall support a network management system by the same manufacturer, supporting overall remote system controlling, monitoring, and managing through network computers and mobile devices.

#### The controller shall support touch screens, keypads, and wireless remotes and mobile device Apps from the same manufacturer.

## User Interfaces

### Touch Screen

#### Basis of Design Touch Screen User Interface:

##### Crestron TSW-760

##### Crestron TSW-1060

#### Functions

##### Provide custom user interface with controls for AV systems, integrated hardware, and integrated scheduling and management system.

#### Communication:

##### Bidirectional 10/100 Mbps Ethernet communication.

#### Physical

##### 7 inch TFT active-matrix color LCD touch screen 1280 by 800 WXGA resolution display.

Specifier Note: 7 inch version: Crestron TSW-760

##### 10.1 inch TFT active-matrix color LCD touch screen 1280 by 800 WXGA resolution display.

Specifier Note: 10.1 inch version: Crestron TSW-1060

##### Power:  IEEE 802.3af Class 3 PoE Powered Device

## Wall mounted Hallway Room Availability Sign

### Basis of Design Room Availability SIgn:

#### Crestron SSW

### Functional Requirements

#### The sign shall illuminate green when the room is available and red when the room is occupied.

#### Custom RGB Color Mixing - The sign shall support custom colors through control system programming.

#### The sign shall support custom color alternating functions, color fade in and out functions, or blink on and off functions through custom system programming.

#### For room scheduling applications, the sign will connect to a compatible system touch screen via USB. No programming or configuration is required. The sign color shall be controlled by the Room Scheduling system by the same manufacturer.

### Physical

#### Sign shall mount to the wall outside a meeting room to indicate room status.

#### The sign shall include tri-color RGB LEDs to allow for a full spectrum of colors by blending the red, green, and blue elements in 1% increments per color.

#### Custom Engraving Option

##### The sign shall be available with or without custom engraving. The engraving shall appear on both sides of the acrylic sign face to identify the room by name or number. When illuminated, the engraving shall be visible at a distance of 50 feet or 15 meters.

## Network Occupancy Sensor:

### Basis of Design Occupancy Sensor:

#### Crestron GLS-ODT-C-CN

### Functional

#### Combination of ultrasonic motion detection and passive infrared detection with internal microprocessor.

#### Coverage:  360 deg., 2000 sq. ft.

#### Delayed time off adjustment.

#### Detects movement within space while reducing false triggering or shutoffs while space is occupied.

#### Sensor shall be independently adjustable for installed conditions.

#### Walk-through mode.

### Physical

#### Additional Interfacing: includes connection port for additional photocell.

#### Equipped with 4-wire interface for direct connection to control bus

#### Mounting:  3” octagon box

#### Photocell: Built-in ambient light photocell

#### Set-up and commissioning: parameters shall be configurable via a handheld wireless remote.

## Speaker System

### Basis of Design Speaker:

#### Crestron SAROS SB-200-P

### Amplified Sound Bar Speaker with the following specifications:

#### Performance:

##### Input Sensitivity: 0.75 Vrms per channel for full rated output

##### Frequency Response: Within plus or minus 3B from 100 to 20,000 Hz

##### Output Sensitivity: 83 dB SPL @ 1 meter, both channels driven @ 1 Watt each

#### Enclosures: Plastic housing with perforate metal grille, black

#### Termination:

##### Inputs: Balanced  audio, terminal block

##### Power Requirements:  50 Watts (500 mA maximum) @ 100-240 Volts AC, 50/60 Hz

## Surface Mounted Connection Interface

### Basis of Design Connection Interface

#### Crestron TT-101

### The Connection interface shall be available in black a finish.

### The Connection interface shall provide configurable connectivity in a flush mount tabletop unit.

### The Connection interface shall support custom combinations of cables, and an AC power outlet.

## Display Device

### Display device shall be controllable via a built-in RS-232 connection port.

### Display device shall support 4K video through built-in HDMI input connector(s).

## Cable Tuner

### HDMI AV source device shall be controllable via IR control protocol.

## Computer

### Computer or Laptop shall include an HDMI video output connection port.

## Graphics Engine HDBaseT Receiver

### Basis of Design Graphics Engine HDBaseT receiver

#### Crestron CCS-FF-2

### Functions

#### The Digital Graphics Engine adds touch screen controller capability to HD or Ultra HD touch screen displays.

#### The Digital Graphics Engine with related software by same manufacturer supports the creation of a custom graphical user environment capable of combining controls of integrated equipment and systems with dual-window video display.

#### The Digital Graphics Engine shall include an HDBaseT input for support of 4K UHD and DCI 4K sources connected at an HDBaseT transmitter or switcher.

#### The Digital Graphics Engine shall support H.264 streaming video signals received over a high-speed Ethernet connection.

#### The Digital Graphics Engine shall include an Ethernet port to interface with a control system by the same manufacturer.

#### The Digital Graphics Engine shall include built-in RS-232 and IR control ports to allow for programmable control of the connected display and other devices via the control system.

#### Multimedia Presentation Gateway

##### Multimedia Gateway allows the wireless presentation of content from a laptop, smartphone, or tablet device via a Wi-Fi wireless network or wired LAN connections.

##### Multimedia gateway functionality shall be a built-in feature.

### Performance

#### The receiver shall meet the following minimum requirements:

##### HDMI digital video, audio, and control output:

###### One (1) 19-pin Type A HDMI female connector

##### HDMI output supports HDCP 2.2

##### Streaming Decoder Video: 	H.264 (MPEG-4 part 10 AVC), MJPEG

##### Supports HDCP.

##### HDMI audio Support:  Up to 8 channel LPCM.

##### CEC device control.

## Power Conditioner

### Basis of Design Power Conditioner:

#### Crestron PC-100

### The Power Conditioner shall meet the following minimum requirements:

#### Provides three-Stage Sequential Turn-On Delay.

#### Rack-mountable 15A, 120V AC power conditioner, and surge protector

# EXECUTION

NOT USED in this Guide Specification. Specifier shall Specify PART 3 On-Site work as needed.

# APPENDICES

## SPECIFIED PRODUCTS

Specifier Note: This Article includes Crestron products specified in this Guide Specification document. This Article is for reference only and should not be required in actual project manual unless included in an overall system equipment list.

### Crestron DMPS3-100-C

### Crestron TSW-1060

### Crestron SSW

### Crestron CCS-FF-2

### Crestron TSW-760

### Crestron SAROS SB-200-P

### Crestron TT-101

### Crestron PC-100

### Crestron GLS-ODT-C-CN