SECTION 27 41 16

INTEGRATED AUDIO-VIDEO SYSTEMS AND EQUIPMENT

GUIDE SPECIFICATION

Specifier: The Specifier/Design Professional is responsible for the accuracy of all project specifications, including system application and coordination with related sections.  This guide specification is provided as a convenience and requires editing to match actual project requirements.  CRESTRON ELECTRONICS, INC. SHALL NOT BE LIABLE FOR ANY DAMAGES ARISING OUT OF THE USE OF ANY OF ITS GUIDE SPECIFICATIONS.  For Crestron design assistance and design review please contact Sales Support Services Department at   800.237.2041 or techsales@crestron.com.

Specifier: Please see PART 4 for a listing of products specified in this Guide Specification.

Table of Contents

1	GENERAL	3

2	PRODUCTS	3

2.1	Dual Camera Speaker Tracking Solution	3

2.1.1	Basis of Design	3

2.1.2	Device Definition	3

2.1.3	Device Architecture	4

2.1.4	Functions	4

2.1.5	Compliance	6

3	EXECUTION	6

4	APPENDICES	6

4.1	SPECIFIED PRODUCTS	6

4.1.1	Crestron IV-CAM-I12D-B	7

# GENERAL

NOT USED in this Guide Specification.  Specifier shall Specify PART 1 administrative and procedural requirements as needed.

# PRODUCTS

## Dual Camera Speaker Tracking Solution

Specifier Note:

The IV-CAM-I12D-B is an intelligent speaker tracking solution that utilizes a pair of 4K PTZ cameras to automatically follow the active speaking participant. Featuring a 4K reference camera and integrated microphone array for spatial audio detection, the IV-CAM-I12D-B is suited for medium to large spaces requiring close-up views of speaking participants. Capable of speaker tracking, group framing, and dynamic composition camera shots, the IV-CAM-I12D-B fits perfectly into a meeting space. All Visual AI is built into the camera – no external system is needed.

### Basis of Design

#### Crestron IV-CAM-I12D-B

Specifier Note:

IV-CAM-I12D-B
https://www.crestron.com/Products/Catalog/Unified-Communications/Intelligent-Video/Camera/IV-CAM-I12D-B

### Device Definition

#### Dual Camera Speaker Tracking Solution with the following capabilities and features:

##### Visual AI for speaker tracking and group framing

##### Dual Camera automatic pan, tilt, and zoom without connection to external system

##### Camera support for resolutions up to 4K30

##### Support for 12x optical zoom for speaker tracking distances up to 25 ft from participants

##### Connects directly to a codec running Microsoft Teams® or Zoom Rooms™ software

##### Configuration with camera management software by same manufacturer

##### Viewing and control via single Ethernet connection

##### Ability to serve as primary camera in an intelligent switching system

##### Compatibility with a variety of audio solutions

##### Dynamic lightbar that displays current camera status

### Device Architecture

#### Environmental

##### Temperature: 32° to 104°F (0° to 40° C)

##### Humidity: 10% to 90% RH (noncondensing)

#### Construction

##### Mounting Options: Wall mount bracket (included)

##### Mounting Thread: ¼ in. threaded mount holes

#### Dimensions

##### Height: 7.61 in. (193 mm)

##### Width: 19.69 in. (500 mm)

##### Depth: 6.10 in. (155 mm)

#### Weight: 9.00 lb (4.08 kg)

### Functions

#### Optics

##### Image Sensor: 1/2.8 4K Sony® CMOS Sensors

##### Optical Zoom: 12x

##### Focal Length: F=34-40.3 mm

##### Field of View (PTZ): Horizontal: 80.5°

##### Field of View (Reference): Horizontal: 110°

##### Shutter Speed: 1/60 – 1/10000 sec

##### Iris: F1.8 – F3.6

##### Minimum Illumination: 2 lux

##### Pan/Tilt Angle: Pan: -130° - 130°, Tilt: -30° - 90°

##### Pan/Tilt Speed: Pan: 0.2° - 90°/s, Tilt: 0.2° - 70°/s

##### White Balance: Auto, One Push, Manual, Static

##### Exposure: Auto, Manual, Shutter Priority, Iris Priority, Brightness Priority

##### Signal to Noise Ratio: ≥50dB

#### Camera Processing

##### Intelligent Video Functions: Speaker Tracking, Group Framing, Conversation Mode, Intelligent Switching

##### Recommended Range: 5-25 ft

##### Presets: Up to 512 configurable presets

#### Video

##### Video Outputs: HDMI®, USB, RTSP

##### Signal Formats

###### HDMI: 4K30/25, 1080p59.94/50/30/29.94/25, 720p59.94/50

###### USB (NV12 and MJPEG): 4K30/25, 1440p30, 1080p30/25, 720p30/25, 360p30

#### Communications

##### Ethernet: RJ45, 1000Mbps

##### Serial: RS-232, RS-485

##### Control Protocol: VISCA Passthrough, VISCA over IP, PELCO-D, PELCO-P, UVC, AES67 (input)

##### AUDIO REF: (1) 3-pin 3.5 mm detachable terminal block; Balanced audio input for analog connection

##### COM: (1) 5-pin 3.5 mm detachable terminal block; Bidirectional RS-232/485 port

##### USB2 TO DEVICE: (1) USB Type A connector, female (not used)

##### HDMI OUT: (1) HDMI Type A connector; HDMI digital video output

##### USB3 TO HOST: (1) USB Type C connector, female; USB 3.0/2.0 device port for video and control

##### 24VDC 2.5A: (1) 5.5 mm DC power connector; 24 VDC power input

#### Management Client

##### Software: Camera management software by same manufacturer (Windows® OS computer required)

#### Controls and Indicators

##### Status Bar

###### (1) Multi segment tricolor LED bar graph

###### Half green LED bar graph indicates live camera feed

###### Full green LED bar graph indicates camera tracking

###### Full amber LED bar graph indicates firmware upgrade

###### Half red LED bar graph indicates Privacy Mode

##### RESET: (1) Recessed push button for hardware reset

#### Power

##### Power Source Options: Power Pack

##### Power Pack: 24 VDC, 2.5 A, 50/60 Hz

### Compliance

#### Regulatory Model: M202306005

#### FCC Part 15 Class B, IC Class B, CE, Intertek® Listed for US and Canada

# EXECUTION

NOT USED in this Guide Specification.  Specifier shall Specify PART 3 On-Site work as needed.

# APPENDICES

## SPECIFIED PRODUCTS

Specifier Note: This Article includes Crestron products specified in this Guide Specification document.  This Article is for reference only and should not be required in actual project manual unless included in an overall system equipment list.

### Crestron IV-CAM-I12D-B